3GPP TSG RAN WG1 Meeting #101-e	          						R1-2004601

E-meeting, May 25 – June 5, 2020

Title:	[DRAFT] Reply LS on LS reply to RAN2 on UE capability

Response to:	R2-2004081

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	Huawei [RAN1]

To:	RAN2

Cc:

Contact Person:

#### Name:	Chao Li

Tel. Number:

E-mail Address:	lichao.cn@hisilicon.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

- Overall Description:

RAN1 would like to thank RAN2 for the LS on UE capability.

The maximum data rate for sidelink is determined by using baseband capabilities, similar as in NR Uu:

data ratein Mbps=10-6∙vLayers∙Qm∙f∙Rmax∙NPRBBW,μ∙12Tsμ∙(1-OH)

Where

vLayers is the maximum supported layers for transmission and reception.

Qm is the maximum supported modulation order for transmission and reception.

Rmax is the maximum coding rate.

NPRBBW,μ is maximum RB allocation in bandwidth BW with numerology μ, as defined is 5.3 TS 38.101-1, where BW is the UE supported maximum bandwidth in the given band or band combination.

Tsμ is the average OFDM symbol duration in a slot for numerology μ.

- Tsμ=10-314*2μ for NCP

- Tsμ=10-312*2μ for ECP

f is the scaling factor. Considering the association of RF capabilities and UE baseband capabilities, the down scaling factor can be 1 and 0.75 for 256-QAM and 64-QAM, respectively.

OH is the overhead. Considering the different PSFCH, DMRS patterns, CSI-RS and PT-RS configurations, the value of OH is determined as follows:

- OH=[0.28] when sl-PSFCH-Period is 0 for NCP

- OH=[0.33] when sl-PSFCH-Period- is 4 for NCP

- OH=[0.38] when sl-PSFCH-Period is 2 for NCP

- OH=[0.49] when sl-PSFCH-Period is 1 for NCP

- OH=[0.32] when sl-PSFCH-Period- is 0 for ECP

- OH=[0.39] when sl-PSFCH-Period is 4 for ECP

- OH=[0.45] when sl-PSFCH-Period is 2 for ECP

- OH=[0.57] when sl-PSFCH-Period- is 1 for ECP

The values of overhead also additionally consider the average RS overhead, the AGC symbol, and the GP.

2. Actions:

To RAN2 group

RAN1 respectfully asks RAN2 to take the above information into consideration for determination of L2 buffer sizes.

3. Date of Next TSG-RAN1 Meetings:

RAN1#102-e	                         August 2020	            			Electronic

RAN1#102bis	                         12-16 October 2020	               	China