3GPP TSG RAN WG1 Meeting #101-e			R1-2004625

25 May – 5 June, 2020

Title:	Draft reply LS on RAN2 DCP Open Issues

Response to:	R2-2004175

Release:	Rel-16

Work Item:	NR_UE_pow_sav-Core

Source:	Huawei [RAN WG1]

To:	RAN WG2

Cc:

Contact Person:

#### Name:	Xiaolei TIE

#### E-mail Address:	tiexiaolei@huawei.com

1. Overall Description:

In the LS, RAN2 asks questions as follows:

- Q1: To confirm RAN2 understanding that if DCP and RAR search spaces are not quasi-collocated, a collision between DCP and RAR addressed to C-RNTI will impact legacy RAR handling.

  - From RAN2 point of view, the understanding is that RAR addressed all RNTIs should be prioritized over DCP by the UE.

- Q2: RAN2 would like to ask if RAN1 has any concerns with the understanding above? If RAN1 doesn’t have any concerns, what is RAN1 preference on where to capture this behavior e.g. TS 38.213 or in TS 38.321 via a DCP monitoring exception rule similar to overlap with DRX Active time?

The corresponding answers to the RAN2 questions are as follows:

Answers:

  - A1: On whether the legacy RAR handling is impacted, it depends on what search space sets are configured to be monitored for DCI format 2_6 and RAR addressed to C-RNTI respectively.

  - A2: There is no need to specify any additional behaviors to prioritize the DCI for RAR over the DCP monitoring when there is collision. It is up to gNB implementation to handle the collision as in legacy.

2. Actions:

RAN1 respectfully asks RAN2 to take the above information into account in their work.

3. Date of Next RAN1 Meetings:

RAN1#102         	24 August – 4 September 2020       E-Meeting