| 3GPP TR 38.808 V0.0.1 (2020-06) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on supporting NR from 52.6 GHz to 71 GHz (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Study of Required Changes to NR	8

4.1	RAN1 Aspects	8

4.1.1	Candidate numerology and bandwidth	8

4.2	RAN4 aspects	9

5	Study of channel access mechanism for 60GHz	9

5.1	Identification of regulatory aspects for consideration	9

Annex <A> (informative): Evaluation Methodology	10

A.1	Link level evaluation assumptions	10

A.2	System level evaluation assumptions	10

A.2.1	[Evaluation A]	10

A.2.1	[Evaluation B]	10

Annex <B> (informative): Change history	11

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it shall be the second unnumbered clause.

# 1	Scope

In order to support wide range of services, 5G NR system aims to be flexible enough to meet the connectivity requirements of a range of existing and future (yet unknown) services to be deployable in an efficient manner. NR considers supporting potential use of frequency range up to 100 GHz (ref. TR 38.913).

NR specifications that have been developed in Rel-15 and Rel-16 define operation for frequencies up to 52.6GHz, where all physical layer channels, signals, procedures, and protocols are designed to be optimized for uses under 52.6GHz.

However, frequencies above 52.6GHz are faced with more difficult challenges, such as higher phase noise, larger propagation loss due to high atmospheric absorption, lower power amplifier efficiency, and strong power spectral density regulatory requirements in unlicensed bands, compared to lower frequency bands. Additionally, the frequency ranges above 52.6 GHz potentially contain larger spectrum allocations and larger bandwidths that are not available for bands lower than 52.6 GHz.

As an initial effort to enable and optimize 3GPP NR system for operation in above 52.6GHz, 3GPP RAN has studied requirements for NR beyond 52.6GHz up to 114.25GHz including global spectrum availability and regulatory requirements (including channelization and licensing regimes), potential use cases and deployment scenarios, and NR system design requirements and considerations on top of regulatory requirements (ref. TR 38.807).  The potential use cases identified in the study include high data rate eMBB, mobile data offloading, short range high-data rate D2D communications, broadband distribution networks, integrated access backhaul (IAB), factory automation, industrial IoT (IIoT), wireless display transfer, augmented reality (AR)/virtual reality (VR) wearables, intelligent transport systems (ITS) and V2X, data center inter-rack connectivity, smart grid automation, private networks, and support of high positioning accuracy. The use cases span over several deployment scenarios identified in the study. The deployment scenarios include, but not limited to, indoor hotspot, dense urban, urban micro, urban macro, rural, factor hall, and indoor D2D scenarios. The study also identified several system design requirements around waveform, MIMO operation, device power consumption, channelization, bandwidth, range, availability, connectivity, spectrum regime considerations, and others.

Among the frequencies of interest, frequencies between 52.6 GHz and 71 GHz are especially interesting relatively in the short term because of their proximity to sub-52.6GHz for which the current NR system is optimized and the imminent commercial opportunities for high data rate communications, e.g., unlicensed spectrum but also licensed spectrum between 57GHz and 71GHz. Therefore, it would be beneficial to make a study focused on feasibility of using existing waveforms and required changes for frequencies between 52.6 GHz and 71 GHz, so as to take advantage of imminent commercial opportunities for the specific frequency regime by minimizing the specification burden and maximizing the leverage of FR2 based implementations.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 38.807: " Study on requirements for NR beyond 52.6 GHz".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

B	transmission bandwidth

G	antenna gain

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BS	Base Station

EIRP	Equivalent Isotropic Radiated Power

FDD	Frequency Duplex Division

IAB	Integrated Access Backhaul

ISM	Industrial, Scientific and Medical

ITU	International Telecommunication Union

LBT	Listen Before Talk

MCOT	Maximum Channel Occupancy Time

NR	New Radio

OCB	Occupied Bandwidth

OOBE	Out-Of-Band Emission

PSD	Power Spectral Density

PTP	Point to point

SI	Study Item

SID	Study Item Description

TDD	Time Duplex Division

UE	User Equipment

V2X	Vehicle to Everything

WAN	Wide Area Network

# 4	Study of Required Changes to NR

## 4.1	RAN1 Aspects

Editor’s Note: This section will be further categorized into sub-sections depending on discussions

### 4.1.1	Candidate numerology and bandwidth

Editor’s Note: This section can include discussion on CP length, subcarrier spacing, and channel bandwidth issues

## 4.2	RAN4 aspects

Editor’s Note: This section will be further categorized into sub-sections depending on discussions

# 5	Study of channel access mechanism for 60GHz

Editor’s Note: This section will be further categorized into sub-sections depending on discussions

## 5.1	Identification of regulatory aspects for consideration

Editor’s Note: This section can include list of identified regulatory aspects that is needed for consideration of channel access mechanism for 60 GHz unlicensed operation.

# Annex <A> (informative):
Evaluation Methodology

## A.1	Link level evaluation assumptions

Editor’s Note: add parameters and assumptions for link level simulations

## A.2	System level evaluation assumptions

Editor’s Note: add parameters and assumptions for link level simulations. Parameters and assumptions can be categorized into scenarios or evaluation purpose (that may cover multiple scenarios).

### A.2.1	[Evaluation A]

Editor’s Note: Section title to be replaced with appropriate scenario name or evaluation purpose

### A.2.1	[Evaluation B]

Editor’s Note: Section title to be replaced with appropriate scenario name or evaluation purpose

# Annex <B> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-06 | RAN# 88 | RP-20xxxx |  |  |  | Draft skeleton TR | V0.0.1 |