3GPP TSG RAN WG1 #101			R1-2004762

e-Meeting, May 25th – June 5th, 2020

Agenda item:	7.2.6.5

Source: 	Moderator (Ericsson)

Title: 	TP for eMIMO lowPAPR thread #2

Document for:	Discussion and Decision

- Analysis

| # Reasons for change | # A sentence that the low-PAPR sequence of type 2 changes for different values of α and δ in Section 5.2.3 is incorrect and should be removed/replaced. |
| --- | --- |
| # Summary of changes | # Remove the incorrect sentence and introduce a clarification on fixed alpha delta values. |
| # Specs/Sections impacted | # TS 38.211 V16.1.0, section 5.2.3, 6.4.1.1.1.2 and 6.4.1.3.3.1 |
| # Consequences if not approved | # The spec is erroneous as multiple sequences are not defined for type 2 |

- Text proposal

TP for TS 38.211 V16.1.0

< Start TP for TS 38.211 V16.1.0> =========================================================================== ### 5.2.3 Low-PAPR sequence generation type 2 The low-PAPR sequence ru,v(α,δ)n is defined by a base sequence ru,vn according to ru,v(α,δ)n=ru,vn, 0≤n<M where M=mNscRB2δ is the length of the sequence. Multiple sequences are defined from a single base sequence through different values of α and δ. < Unchanged Part Deleted > 6.4.1.1.1.2 Sequence generation when transform precoding is enabled If transform precoding for PUSCH is enabled, the reference-signal sequence shall be generated according to where ru,vα,δ(n) with δ=1 and α=0 depends on the configuration: - if the higher-layer parameter dmrsUplinkTransformPrecoding-r16 is configured, π/2-BPSK modulation is used for PUSCH, and the PUSCH transmission is not a msg3 transmission, and the transmission is not scheduled using DCI format 0_0 in a common search space,ru,vα,δ(n) is given by clause 5.2.3 withcinit given by < Unchanged Part Deleted > - otherwise,ru,vα,δ(n) is given by clause 5.2.2 with α=0. < Unchanged Part Deleted > ###### 6.4.1.3.3.1 Sequence generation The reference-signal sequence rlm shall be generated according to where MscPUCCH,s is given by clause 6.3.2.6.3 and ru,vα,δ(m) depends on the configuration: - if the higher-layer parameter dmrsUplinkTransformPrecodingPUCCH-r16 is configured, ru,vα,δ(m) is given by clause 5.2.3, with δ=0 and with cinit given by clause 6.4.1.3.2.1. The sequence group u and the sequence number v depend on the sequence hopping in clause 6.3.2.2.1. =========================================================================== < End TP for TS 38.211 V16.1.0>