3GPP TSG-RAN WG1 Meeting #101-e                 	     R1-2004789

e-Meeting, May 25th – June 5th, 2020

Source: 	Moderator (CMCC)

Title:	Outcome of email discussion of [101-e-NR-LTE_NR_DC_CA-Unaligned_CA]

Agenda item:	7.2.10.5

Document for:	Discussion & Decision

- Summary

TP1 for TS 38.213 section 9.1.2.1

Reason for change: It is not clear how to determine value of the parameters Nslot,offset,cDL and μoffset,DL,c, or Nslot,offsetUL  and μoffset, UL. Comma is missing from the parameter μoffsetUL, which should be μoffset, UL, as what is used in the subsequent formulae. Parameters misaligned between specifications.

Summary of change:

Clarify the determination of the parameters Nslot,offset,cDL and μoffset,DL,c, or Nslot,offsetUL  and μoffset, UL

Replace μoffsetUL with μoffset, UL.

Replace CA-slot-offset with ca-SlotOffset.

Replace “higher-layer” with “higher layer”.

Consequences if not approved:

It is not clear how to determine the values of the parameters Nslot,offset,cDL and μoffset,DL,c, or Nslot,offsetUL  and μoffset, UL.

Ambiguity of the parameter μoffset, UL.

Ambiguity of the parameter CA-slot-offset.

Inconsistency among context.

Clauses affected: 38.213, section 9.1.2.1

- Proposals

Proposed TP 1

<unchanged text omitted>

9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

For a serving cell , an active DL BWP, and an active UL BWP, as described in Clause 12, the UE determines a set of  occasions for candidate PDSCH receptions for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot . If serving cell  is deactivated, the UE uses as the active DL BWP for determining the set of  occasions for candidate PDSCH receptions a DL BWP provided by firstActiveDownlinkBWP-Id. The determination is based:

a)	on a set of slot timing values  associated with the active UL BWP

a)	If the UE is configured to monitor PDCCH for DCI format 1_0 and is not configured to monitor PDCCH for DCI format 1_1 on serving cell ,  is provided by the slot timing values {1, 2, 3, 4, 5, 6, 7, 8} for DCI format 1_0

b)	If the UE is configured to monitor PDCCH for DCI format 1_1 for serving cell ,  is provided by dl-DataToUL-ACK for DCI format 1_1

b)	on a set of row indexes  of a table that is provided either by a first set of row indexes of a table that is provided by pdsch-TimeDomainAllocationList in pdsch-ConfigCommon or by Default PDSCH time domain resource allocation A [6, TS 38.214], or by the union of the first set of row indexes and a second set of row indexes, if provided by pdsch-TimeDomainAllocationList in pdsch-Config, associated with the active DL BWP and defining respective sets of slot offsets , start and length indicators SLIV, and PDSCH mapping types for PDSCH reception as described in [6, TS 38.214]

c)	on the ratio  between the downlink SCS configuration  and the uplink SCS configuration  provided by subcarrierSpacing in BWP-Downlink and BWP-Uplink for the active DL BWP and the active UL BWP, respectively

d)	if provided, on tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated as described in Clause 11.1

e)	if CA-slot-offset ca-SlotOffset is provided, on Nslot,offset,cDL and μoffset,DL,c for serving cell c, where Nslot,offset,cDL and μoffset,DL,c, are the Nslot, offsetCA and the, respectively, which are determined by higher layer configured ca-SlotOffset for the serving cell c, or on Nslot,offsetUL  and μoffset, UL for the cell of PUCCH transmission, where Nslot,offsetUL  and μoffset,UL are the Nslot, offsetCA and the, respectively, which are determined by higher layer configured ca-SlotOffset for the cell of PUCCH transmission, as described in [4, TS 38.211].

<unchanged text omitted>