3GPP TSG RAN WG1 Meeting #101-e	R1-2004850

E-Meeting, May 25 – June 5, 2020

Agenda Item:	6.2.3.1.1

Source:	Moderator (Huawei)

Title:	Text proposal on indexing of additional SRS symbols

Document for:	Discussion and Decision

Introduction

This document provides the text proposal as outcomes of the following email discussion [1]:

[101-e-LTE-LTE_DL_MIMO_EE-02] Indexing of additional SRS symbols by 5/29 – Yubo (Huawei)

- Issues 1 in R1-2004706

Discussion

TP on resource reservation to special subframes

Reason for changes:

The parameter l in nSRS=lR may be misleading as the range of the parameter l is not defined.

Summary of changes:

The parameter l in nSRS=lR is the additional SRS transmission number within the subframe.

Specs/sections impacted:

36.211 sections 5.5.3.2.2

Consequences if not approved:

The parameter l in nSRS=lR may be interpreted as the OFDM symbol index within the subframe, which leads to an incorrect transmission pattern.

-----------------------------------------------------Start of Text Proposal-----------------------------------

<unchanged parts are omitted>

##### 5.5.3.2.2	Mapping to physical resources for additional SRS

An additional SRS spans one or more OFDM symbols in the time domain, where

-	the starting OFDM symbol l0 within the subframe is given by the higher-layer parameter additionalSRS-startPos;

-	the duration N in number of OFDM symbols, including potential guard symbols, is given by the higher-layer parameter additionalSRS-duration;

Mapping to physical resources shall be done according to clause 5.5.3.2.1 with the following exceptions:

-	frequency hopping between OFDM symbols is supported and if a UE is configured by higher layer parameter additionalSRS-GuardSymbolFH, a guard symbol is added between every frequency hop;

-	antenna switching within a subframe is supported and if a UE is configured by higher layer parameter additionalSRS-GuardSymbolAS, a guard symbol is added between every antenna switching;

-	nSRS=l'R where l' is the additional SRS transmission number not counting guard symbol(s) within the subframe with l'=0 corresponding to the starting OFDM symbol l0, and R∈1,2,3,4,6,7,8,9,12,13 is the repetition factor given by the higher-layer parameter additionalSRS-RepNum;

------------------------------------------------------End of Text Proposal------------------------------------

- References

- R1-2004846	Feature lead summary on 101-e-LTE-LTE_DL_MIMO_EE-02	Moderator (Huawei)