3GPP TSG RAN WG1 #101		                         R1-2004983

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.2.2.5

Source: 	Moderator (LG Electronics)

Title: 	Text proposal as outcome of email discussions [101-e-NR-unlic-NRU-WB-01] and [101-e-NR-unlic-NRU-WB-02]

Document for:	Discussion and decision

- Introduction

This document is to capture the following agreements in specification. The corresponding text proposal is provided in Section 2.

Agreement: (RAN1#100bis-e)

To support UL bandwidth part wider than 20 MHz with no intra-cell guard band, UE can be configured with zero GBs by setting GB width to 0 when configuring intraCellGuardBandUL-r16 (e.g., such gNB creates 4 RB-sets in 80MHz UL carrier).

- Inform RAN2 of this agreement

Agreement: (RAN1#100bis-e)

For an UL carrier without intra-cell guard bands when the parameter useInterlacePUCCH-PUCCH is configured in any of BWP-UplinkCommon and BWP-UplinkDedicated:

- The UL carrier can be configured with NRB-set,UL≥1non-overlapping RB set(s)

- For each RB set except for RB set 0, the starting CRB index is given by startCRB-r16

  - For RB set 0, the starting CRB index is given by Ngrid,ULstart,μ

- The UE expects nrofCRBs-r16 set to 0 for all GBs between two adjacent RB sets within the UL carrier.

- The UE expects N RBs contained in each interlace of each RB set, wherein 10 <= N <= 11.

  - For 30 kHz SCS, the number of RBs within any RB set is between 50 and 55, and for 15 kHz SCS, the number of RBs within any RB set is between 100 and 110

- Note: This configuration may be used for the case where transmission only occurs in a BWP if LBT is successful in all RB sets within the BWP (from RAN1#99 agreement)

- Note: It’s up to gNB’s configuration to fulfill RAN4 requirement with  e.g., on maximum transmission bandwidth configuration, spectral emission mask, and so on.

- Note: In order to reuse existing PUCCH/PUSCH resource allocation mechanisms, this proposal applies to all supported carrier bandwidths except 10 MHz

- FFS: Whether BWP can be configured to be partially overlapping with a RB set

Agreement: (RAN1#101-e [101-e-NR-unlic-NRU-WB-01])

For a DL carrier where no intra-cell guard bands are configured with intraCellGuardBandDL-r16,

- The DL carrier can be configured with NRB-set,DL>1  non-overlapping RB set(s).

- For each RB set except for RB set 0, the starting CRB index is given by startCRB-r16 and Ngrid,DLstart,μ

  - For RB set 0, the starting CRB index is given by Ngrid,DLstart,μ

- The UE expects nrofCRBs-r16 set to 0 for all GBs between two adjacent RB sets within the DL carrier.

- For 30 kHz SCS, the number of RBs within any RB set is between 50 and 55, and for 15 kHz SCS, the number of RBs within any RB set is between 100 and 110

  - For 30 kHz SCS, the UE may be configured with intraCellGuardBandDL-r16 such that one of the RB sets contain 56 PRBs

Agreement: (RAN1#101-e [101-e-NR-unlic-NRU-WB-01])

For an UL carrier without intra-cell guard bands when the parameter useInterlacePUCCH-PUCCH is configured in any of BWP-UplinkCommon and BWP-UplinkDedicated,

- The UE does not expect that UL BWP within the UL carrier is configured to include parts of a RB set.

- For 30 kHz SCS, the UE may be configured with intraCellGuardBandUL-r16 such that one of the RB sets contain 56 PRBs

  - Note: the number of RBs for the other RB sets is between 50 and 55 as previously agreed

- The UL carrier can be configured with NRB-set,UL>1  non-overlapping RB set(s) if intraCellGuardBandUL-r16 is provided.

- This agreement and the corresponding agreement from RAN1#100bis-e also apply to the case when useInterlacePUCCH-PUCCH is not configured in either of BWP-UplinkCommon and BWP-UplinkDedicated

Agreement: (RAN1#101-e [101-e-NR-unlic-NRU-WB-02])

For GuardBand-r16, the value range of startCRB-r16 is from 0 to 274.

- Note: This requires the change from RB s,xend,μ=GB s,xstart,μ-1 and RB s+1,xstart,μ=GB s,xstart,μ+GB s,xsize,μ to RB s,xend,μ=Ngrid,xstart,μ+GB s,xstart,μ-1 and RB s+1,xstart,μ=Ngrid,xstart,μ+GB s,xstart,μ+GB s,xsize,μ, respectively, in TS 38.214 Section 7.

Agreement: (RAN1#101-e [101-e-NR-unlic-NRU-WB-02])

For GuardBand-r16, the value range of nrofCRBs-r16 is from 0 to 15.

- UE does not expect that nrofCRBs-r16 is configured with non-zero value smaller than the default guard band size defined in RAN4 specifications.

Agreement: (RAN1#101-e [101-e-NR-unlic-NRU-WB-02])

When intraCellGuardBandUL-r16/intraCellGuardBandDL-r16 is absent for an UL/DL carrier and the default configuration in 38.101-1 indicates that there are no intra-cell guard bands for the carrier (i.e., 20 MHz carrier), then the number of RB sets for the carrier is 1 with index 0. When interlacing is configured for the UL carrier, the BWP spans the whole carrier, and the RB set index is 0 within the UL BWP.

- Text proposal

Reason for changes

To capture the agreements for intra-cell guard band configuration and DL or UL carrier without intra-cell guard bands.

Summary of changes

- Modified from RB s,xend,μ=GB s,xstart,μ-1 and RB s+1,xstart,μ=GB s,xstart,μ+GB s,xsize,μ to RB s,xend,μ=Ngrid,xstart,μ+GB s,xstart,μ-1 and RB s+1,xstart,μ=Ngrid,xstart,μ+GB s,xstart,μ+GB s,xsize,μ, respectively

- Clarified that UE does not expect that nrofCRBs-r16 is configured with non-zero value smaller than the default guard band size defined in RAN4 specifications

- Removed the square brackets based on RAN1 agreements

- Clarified that the number of RB set for a carrier where intra-cell guard band configuration is absent and default pattern contains no guard band, is equal to 1

- Clarified that for a carrier regardless of with intra-cell guard band or without intra-cell guard band, UE does not expect that BWP within the carrier is configured to include parts of a RB set

- Clarified that when a UE is provided with nrofCRBs-r16=0 for all intra-cell guard band(s) on a carrier, the UE is indicated that no intra-cell guard-bands are configured for the carrier

- Clarified that when no intra-cell guard-bands are configured for a carrier, the UE expects more than 1 RB sets, 100-110 RBs for a RB set with 15 kHz SCS, and 50-55 RBs for a RB set with 30 kHz SCS (except for at most one RB set containing 56 RBs)

Specs/Sections impacted

TS 38.214 Clause 7

Consequences if not approved

The corresponding agreements are not reflected in specification.

# 7 UE procedures for transmitting and receiving on a carrier with intra-cell guard bands For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandUL-r16 for UL carrier and intraCellGuardBandDL-r16 for DL carrier, the UE is provided with NRB-set,x-1 intra-cell guard bands on a carrier, each defined by start CRB and size in number of CRBs, GB s,xstart,μ and GB s,xsize,μ , provided by higher layer parameters startCRB-r16 and nrofCRBs-r16, respectively. The subscript x is set to DL and UL for the downlink and uplink, respectively. Where there is no risk of confusion, the subscript x can be dropped. The intra-cell guard bands separate NRB-set,x RB sets, each defined by start and end CRB, RB s,xstart,μ and RB s,xend,μ, respectively. UE does not expect that nrofCRBs-r16 is configured with non-zero value smaller than the applicable intra-cell guard bands as specified in [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ. UE determines RB 0,xstart,μ=Ngrid,xstart,μ, RBNRB-set-1,xend,μ=Ngrid,xstart,μ+Ngrid,xsize,μ-1, and the remaining start and end CRBs as RB s,xend,μ=Ngrid,xstart,μ+GB s,xstart,μ-1 and RB s+1,xstart,μ=Ngrid,xstart,μ+GB s,xstart,μ+GB s,xsize,μ. The RB set s consists of RBs,xsize,μ resource blocks where RBs,xsize,μ=RB s,xend,μ-RB s,xstart,μ+1. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines the CRB indices for the intra-cell guard band(s), if any, and corresponding RB set(s) according to the nominal intra-cell guard band and RB set pattern as specified in [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines the CRB indices for the intra-cell guard band(s), if any, and corresponding RB set(s) according to the nominal intra-cell guard band and RB set pattern as specified in [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ. For either or both DL and UL, if the nominal intra-cell guard band and RB set pattern as specified in [8, TS 38.101-1] contains no intra-cell guard bands, the number of RB sets for the carrier is NRB-set,x=1. For a carrier, the UE expects N BWP,istart,μ=RB s0,xstart,μ, and N BWP,isize,μ=RB s1,xend,μ-RB s0,xstart,μ+1 where 0≤s0≤s1≤NRB-set,x-1for a BWP i configured by BWP-DownlinkCommon or BWP-DownlinkDedicated for the DL BWP, or BWP-UplinkCommon or BWP-UplinkDedicated for the UL BWP. Within the BWP i, RB sets are numbered in increasing order from 0 to NRB-set,xBWP-1 where NRB-set,xBWP is the number of RB sets contained in the BWP i and RB set 0 within the BWP i corresponds to RB set s0 in the carrier and RB set NRB-set,xBWP-1 within the BWP i corresponds to RB set s1 in the carrier. When a UE is provided with nrofCRBs-r16=0 for all intra-cell guard band(s) on a carrier, the UE is indicated that no intra-cell guard-bands are configured for the carrier, and expects NRB-set,x>1. For μ=0, the UE expects the number of RBs within a RB set is between 100 and 110. For μ=1, the UE expects the number of RBs within a RB set is between 50 and 55 except for at most one RB set which may contain 56 RBs.