3GPP TSG RAN WG1 #101-e Meeting	            R1-2005020

e-Meeting, May 25th – June 5th, 2020

Agenda item:	7.2.4.1

Source: 	Moderator (Samsung)

Title: 	Text Proposals for [101-e-NR-5G_V2X_NRSL-PHYstructure-03]

Document for:	Endorsement

Introduction

During WI and RAN1#101-e meeting, the following were agreed.

Agreement: - The CRC length for 2nd SCI is 24 bits. Agreements: - Duplication and discontinuous scrambling sequence for 2-layer mapping of the 2nd SCI is fixed in TS38.211 and 212. - How to fix it is up to editors Update on 6/1: to check on 6/2 6/3 Update from GTW 6/3: Agreements: W.r.t. Qm and beta offset for 2nd SCI mapping - Alt 1-1: Qm = 2 - Alt 1-2: Qm as indicated by MCS - Alt 2-1: beta offset value range as NR Uu for HARQ-ACK - Alt 2-2: define beta offset value set per Qm Take {Alt 1-1 + Alt 2-1} Agreements: - W.r.t. number of layers for 2nd SCI mapping: - Not using the number of layers in the equation, where Q'_SCI is defined as "the number of coded modulation symbols generated for 2nd-stage SCI transmission (prior to duplication for the 2nd layer, if present)" Agreements: Indication of beta offset in 1st SCI among 4 (pre-)configured values (2 bits) - The possible values for beta offset refer to the same table as for Uu in Table 9.3-2 of TS38.213 - A UE is configured with a single set of beta values, applicable to both singel-layer and two-layer PSSCH transmissions Agreements: - Remove M_sc^CSI-RS(l) in calculation of M_sc^SCI2(l) Agreements: In Rel-16, a UE is not expected to have the number of bits after rate matching for 2nd SCI more than K = [2048 or 4096] - To down-select one of the above two values in the next meeting Agreements: - The mapping for 2nd-stage SCI takes PSFCH overhead indication into account

Summary of TPs

| TP1 | Target specification and version, Clauses affected | TS38.212 V16.1.0 Clause 8.4.4 |
| --- | --- | --- |
|  | Reason for change | It was agreed to modify the number of coded modulation symbol for 2nd SCI. |
|  | Summary of change | The determination of the number of coded modulation symbols for 2nd SCI is modified. |
|  | Consequences if not approved | Incomplete specification |

| TP2 | Target specification and version, Clauses affected | TS38.212 V16.1.0 Clause 8.3.1.1 |
| --- | --- | --- |
|  | Reason for change | How to indicate beta offset is defined. |
|  | Summary of change | The square bracket in beta offset indication is removed and the table related to beta offset is now added. |
|  | Consequences if not approved | Incomplete specification |

Text Proposal 1 for TS38.212

This is a TP for the following agreement.

Agreement:

- The CRC length for 2nd SCI is 24 bits.

Agreements:

W.r.t. Qm and beta offset for 2nd SCI mapping

  - Alt 1-1: Qm = 2

  - Alt 1-2: Qm as indicated by MCS

  - Alt 2-1: beta offset value range as NR Uu for HARQ-ACK

  - Alt 2-2: define beta offset value set per Qm

Take {Alt 1-1 + Alt 2-1}

Agreements:

- W.r.t. number of layers for 2nd SCI mapping:

  - Not using the number of layers in the equation, where Q'_SCI is defined as "the number of coded modulation symbols generated for 2nd-stage SCI transmission (prior to duplication for the 2nd layer, if present)"

Agreements:

- Remove M_sc^CSI-RS(l) in calculation of M_sc^SCI2(l)

Agreements:

The mapping for 2nd-stage SCI takes PSFCH overhead indication into account

Agreements:

In Rel-16, a UE is not expected to have the number of bits after rate matching for 2nd SCI more than K = [2048 or 4096]

- To down-select one of the above two values in the next meeting

-----------------------Begin of text proposal for 38.212 Section 8.4.4--------------------

- 8.4.4 Rate Matching

For 2nd-stage SCI transmission on PSSCH with SL-SCH, the number of coded modulation symbols generated for 2nd-stage SCI transmission prior to duplication for the 2nd layer, if present, denoted as QSCI2', is determined as follows:

QSCI2'=minOSCI2+LSCI2∙βoffsetSCI2∙l=0NsymbolPSSCH-1MscSCI2(l)r=0CSL-SCH-1Kr2∙R, αl=0NsymbolPSSCH-1MscSCI2(l)+γ

where

-	OSCI2 is the number of the SCI format 0-2 bits

-	LSCI2 is the number of CRC bits for SCI format 0-2, which is 24 bits.

-	βoffsetSCI2 is indicated in the corresponding SCI format 0-1.

-	CSL-SCH

-	MscPSSCH(l) is the scheduled bandwidth of PSSCH transmission, expressed as a number of subcarriers;

-	MscDMRS(l) is the number of subcarriers in OFDM symbol l that carries DMRS, in the PSSCH transmission.

-	MscPT-RS(l) is the number of subcarriers in OFDM symbol l that carries PT-RS, in the PSSCH transmission.

MscCSI-RS(l)l

-	MscSCI2(l) is the number of resource elements that can be used for transmission of the SCI format 0-2 in OFDM symbol l, for l=0,1,2⋯NsymbolPSSCH-1,NsymbolPSSCH and for NsymbolPSSCH=Nsymbsh-NsymbPSFCH, in PSSCH transmission, where NsymbshNsymbslot = sl-lengthSLsymbols -2, where sl-lengthSLsymbols is the number of sidelink symbols within the slot provided by higher layers as defined in [6, TS 38.214], and where  NsymbPSFCH = 3 if “PSFCH overhead indication” field of SCI format 1-A indicates “1”, and NsymbPSFCH = 0 otherwise, if higher layer parameter sl-PSFCH-Period is 2 or 4. If higher layer parameter sl-PSFCH-Period is 0, NsymbPSFCH=0. If higher layer parameter sl-PSFCH-Period is 1, NsymbPSFCH=3.

-		MscSCI2(l) = MscPSSCH(l) - MscDMRS(l) - MscPT-RS(l) MscCSI-RS(l)

-	γ is the number of otherwise vacant resource elements in the resource block to which the last coded symbol of the SCI format 0-2 belongs.

-	Krr R is the coding rate as indicated by MCS index in SCI format 1-A.

-	α is configured by higher layer parameter sl-Scaling.

The input bit sequence to rate matching is d0, d1,d2,d3, ⋯,dN-1, where N is the number of coded bits.

Rate matching is performed according to Clause 5.4.1 by setting IBIL=1.

The output bit sequence after rate matching is denoted as g0SCI2, g1SCI2, g2SCI2, g3SCI2,⋯, gGSCI2GSCI2-1SCI2, where GSCI2=QSCI2'∙ QmSCI2 and QmSCI2 is modulation order of SCI format 0-2. A UE is not expected to have GSCI2>K.

-----------------------End of text proposal for 38.212 Section 8.4.4--------------------

Text Proposal 2 for TS38.212

This is a TP for the following agreement.

Agreements:

Indication of beta offset in 1st SCI among 4 (pre-)configured values (2 bits)

- The possible values for beta offset refer to the same table as for Uu in Table 9.3-2 of TS38.213

- A UE is configured with a single set of beta values, applicable to both singel-layer and two-layer PSSCH transmissions

----------------------------------Begin of text proposal for 38.212 Section 8.3.1.1---------------------------------

- 8.3.1.1	SCI format 0-1

SCI format 0-1 is used for the scheduling of PSSCH and 2nd-stage-SCI on PSSCH

The following information is transmitted by means of the SCI format 0-1:

-	Priority – 3 bits as defined in clause x.x.x of [6, TS 38.214].

-	Frequency resource assignment – log2(N subChannel SLN subChannel SL + 12) bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2; otherwise log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16) bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, as defined in clause x.x.x of [6, TS 38.214].

-	Time resource assignment – 5 bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2; otherwise 9 bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, as defined in clause x.x.x of [6, TS 38.214].

-	Resource reservation period – log2(NreservPeriod) bits as defined in clause x.x.x of [6, TS 38.214], if higher layer parameter sl-MultiReserveResource is configured; 0 bit otherwise.

-	DMRS pattern – [x] bits as defined in clause 8.4.1.1.2 of [4, TS 38.211], if more than one DMRS patterns are configured by higher layer parameter sl-PSSCH-DMRS-TimePattern; 0 bit otherwise.

-	2nd-stage SCI format – [x] bits as defined in clause x.x.x of [6, TS 38.214].

-	Beta_offset indicator –2 bits as provided by higher layer parameter sl-BetaOffsets2ndSCI and Table 8.3.1.1-x.

-	Number of DMRS port – 1 bit as defined in Table 8.3.1.1-1.

-	Modulation and coding scheme – 5 bits as defined in clause 8.1.3 of [6, TS 38.214].

-	Reserved – [2 - 4] bits as determined by higher layer parameter sl-NumReservedBits, with value set to zero.

Table 8.3.1.1-1: Number of DMRS port

| Value of the Number of DMRS port field | Antenna ports |
| --- | --- |
| 0 | 1000 |
| 1 | 1000 and 1001 |

Table 8.3.1.1-x Mapping of four beta_offset indicator values to offset indexes

| beta_offset indicator | Beta offset index in Table 9.3-2 of TS38.213 |
| --- | --- |
| 00 | 1st offset index provided higher layers |
| 01 | 2nd offset index provided higher layers |
| 10 | 3rd offset index provided higher layers |
| 11 | 4th offset index provided higher layers |

----------------------------------End of text proposal for 38.214 Section 8.3.1.1---------------------------------