3GPP TSG RAN WG1 #102-e					  	R1-2005756

e-Meeting, August 17th – 28th, 2020

Agenda item:	7.2.2.2.2

Source:	NEC

Title:              	TPs on DCI reserved bits length in NR-U

Document for:	Discussion and Decision

- Introduction

In this contribution, we have some text proposals on bit length of DCI format 1_0 with CRC scrambled by RA-RNTI or MsgB-RNTI.

- Discussion

In current TS 38.212-g20 [1], the number of DCI format 1_0 bits except for “Frequency domain resource assignment” field is

- For operation in a cell with shared spectrum channel access

  - 28 bits scrambled by RA-RNTI or MsgB-RNTI.

    - Note: 4 + 1 + 5 + 2 + 2 + 14.

  - 30 bits scrambled by other RNTI (C-RNTI, SI-RNTI, P-RNTI, TC-RNTI).

- For operation in a cell without shared spectrum channel access

  - 28 bits scrambled by all RNTI.

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by RA-RNTI or MsgB-RNTI: - Frequency domain resource assignment – bits - is the size of CORESET 0 if CORESET 0 is configured for the cell and is the size of initial DL bandwidth part if CORESET 0 is not configured for the cell - Time domain resource assignment – 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214] - VRB-to-PRB mapping – 1 bit according to Table 7.3.1.2.2-5 - Modulation and coding scheme – 5 bits as defined in Clause 5.1.3 of [6, TS38.214], using Table 5.1.3.1-1 - TB scaling – 2 bits as defined in Clause 5.1.3.2 of [6, TS38.214] - LSBs of SFN – 2 bits for the DCI format 1_0 with CRC scrambled by MsgB-RNTI as defined in Clause 8.2A of [5, TS 38.213]; or 2 bits for the DCI format 1_0 with CRC scrambled by RA-RNTI as defined in Clause 8.2 of [5, TS 38.213] for operation in a cell with shared spectrum channel access; 0 bit otherwise - Reserved bits – 14 bits for the DCI format 1_0 with CRC scrambled by MsgB-RNTI; or 14 bits for the DCI format 1_0 with CRC scrambled by RA-RNTI for operation in a cell with shared spectrum channel access; otherwise 16 bits

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by TC-RNTI: - Identifier for DCI formats – 1 bit - The value of this bit field is always set to 1, indicating a DL DCI format - Frequency domain resource assignment – bits - is the size of CORESET 0 - Time domain resource assignment – 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214] - VRB-to-PRB mapping – 1 bit according to Table 7.3.1.2.2-5 - Modulation and coding scheme – 5 bits as defined in Clause 5.1.3 of [6, TS38.214], using Table 5.1.3.1-1 - New data indicator – 1 bit - Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2 - HARQ process number – 4 bits - Downlink assignment index – 2 bits, reserved - TPC command for scheduled PUCCH – 2 bits as defined in Clause 7.2.1 of [5, TS38.213] - PUCCH resource indicator – 3 bits as defined in Clause 9.2.3 of [5, TS38.213] - PDSCH-to-HARQ_feedback timing indicator – 3 bits as defined in Clause 9.2.3 of [5, TS38.213] - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 for operation in a cell with shared spectrum channel access; otherwise 0 bit

The bits length for DCI format 1-0 by different RNTI should be the same. So the reserved bits for the DCI format 1_0 with CRC scrambled by RA-RNTI for operation in a cell with shared spectrum channel access should be 16 and the reserved bits for the DCI format 1_0 with CRC scrambled by MsgB-RNTI for operation in a cell with shared spectrum channel access should also be 16.

So we propose that:

- Text proposal 1

>>> Text proposal for 38.212 Section 7.3.1.2.1 >>>

< Unchanged parts are omitted >

-	Reserved bits – 14 bits for the DCI format 1_0 with CRC scrambled by MsgB-RNTI for operation in a cell without shared spectrum channel access; otherwise 16 bits

< Unchanged parts are omitted >

>>> End Text proposal >>>

- Conclusion

In this contribution, we provide some text proposals on bit length of DCI format 1_0 with CRC scrambled by RA-RNTI or MsgB-RNTI.

- References

[1] TS38.212-g20.