3GPP TSG-RAN WG1 Meeting #102-e	Tdoc R1-2005918

e-Meeting, 17th – 28th August, 2020

Agenda Item:	7.2.2.2.5

Source:	Ericsson

Title:	Wideband Operation

Document for:	Discussion, Decision

# 1	Introduction

This document is for the purposes of NR-U maintenance under the wideband operation agenda item.

# 2	Corrections

## 2.1	Corrections to Notation for Intra-Cell Guard Bands and RB Sets

In 38.214 Section 7 [1], there can be some ambiguity since the same index s is used to index intra-cell guard bands and RB sets, even though the number of guard bands is one less than the number of RB sets. This can be easily corrected by using different indices.

In addition, the formatting of the variables in this section is not consistent with other specs, e.g., 38.211. Subscripts and superscripts should not be in italics, e.g., N BWP,istart,μ should be NBWP,istart,μ and RBNRB-set-1,xend,μ should be RBNRB-set-1,xend,μ. These can be highlighted and left to the editor to correct.

In 38.214 Section 7, correct notation for indexing guard bands within a carrier. Correct notation of variables to be consistent with 38.211.

A text proposal implementing Proposal 1 is as follows.

---------------------------------------------- Text Proposal (TP#1) for 38.214, Section 7 ----------------------------------------

*** Unchanged text omitted ***

7	UE procedures for transmitting and receiving on a carrier with intra-cell guard bands

Note to editor: to be consistent with other specs, e.g., 38.211, the subscripts/superscripts in the highlighted variables should be formatted so they are not italicized, e.g., N BWP,istart,μ should be NBWP,istart,μ and RBNRB-set-1,xend,μ should be RBNRB-set-1,xend,μ.

For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandUL-r16 for UL carrier and intraCellGuardBandDL-r16 for DL carrier, the UE is provided with  NRB-set,x-1  intra-cell guard bands on a carrier, each defined by start CRB and size in number of CRBs, GBr,xstart,μ GB s,xstart,μ  and GBr,xsize,μ GB s,xsize,μ , provided by higher layer parameters startCRB-r16 and nrofCRBs-r16, respectively, where r∈0,1,…,NRB-set,x-2. The subscript x is set to DL and UL for the downlink and uplink, respectively. Where there is no risk of confusion, the subscript x can be dropped. The intra-cell guard bands separate NRB-set,x RB sets, each defined by start and end CRB, RB s,xstart,μ and RB s,xend,μ, respectively. UE does not expect that nrofCRBs-r16 is configured with non-zero value smaller than the applicable intra-cell guard bands as specified in [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ. UE determines RB 0,xstart,μ=Ngrid,xstart,μ, RBNRB-set-1,xend,μ=Ngrid,xstart,μ+Ngrid,xsize,μ-1, and the remaining start and end CRBs for s∈1,…,NRB-set,x-2 as RB s,xend,μ=Ngrid,xstart,μ+GB s,xstart,μ-1 and RB s+1,xstart,μ=Ngrid,xstart,μ+GB s,xstart,μ+GB s,xsize,μ. The RB set with index s consists of  RBs,xsize,μ resource blocks where  RBs,xsize,μ=RB s,xend,μ-RB s,xstart,μ+1. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines the CRB indices for the intra-cell guard band(s), if any, and corresponding RB set(s) according to the nominal intra-cell guard band and RB set pattern as specified in [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines the CRB indices for the intra-cell guard band(s), if any, and corresponding RB set(s) according to the nominal intra-cell guard band and RB set pattern as specified in [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ. For either or both DL and UL, if the nominal intra-cell guard band and RB set pattern as specified in [8, TS 38.101-1] contains no intra-cell guard bands, the number of RB sets for the carrier is NRB-set,x=1.

For a carrier, the UE expects  N BWP,istart,μ=RB s0,xstart,μ, and N BWP,isize,μ=RB s1,xend,μ-RB s0,xstart,μ+1 where 0≤s0≤s1≤NRB-set,x-1for a BWP i configured by BWP-DownlinkCommon or BWP-DownlinkDedicated for the DL BWP, or BWP-UplinkCommon or BWP-UplinkDedicated for the UL BWP.  Within the BWP i, RB sets are numbered in increasing order from 0 to NRB-set,xBWP-1 where NRB-set,xBWP is the number of RB sets contained in the BWP i and RB set 0 within the BWP i corresponds to RB set s0 in the carrier and RB set NRB-set,xBWP-1 within the BWP i corresponds to RB set s1 in the carrier.

When a UE is provided with nrofCRBs-r16=0 for all intra-cell guard band(s) on a carrier, the UE is indicated that no intra-cell guard-bands are configured for the carrier, and expects NRB-set,x>1. For μ=0, the UE expects the number of RBs within a RB set is between 100 and 110. For μ=1, the UE expects the number of RBs within a RB set is between 50 and 55 except for at most one RB set which may contain 56 RBs.

*** Unchanged text omitted ***

----------------------------------------------------------- End Text Proposal -----------------------------------------------------------

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	In 38.214 Section 7, correct notation for indexing guard bands within a carrier. Correct notation of variables to be consistent with 38.211.

# References

3GPP TS 38.214, "Physical layer procedures for data," v16.2.0, June 2020.