3GPP TSG RAN WG1 #102-e	R1-2005960

e-Meeting, August 17th – August 28th, 2020

Agenda item:	8.6.5

Source:	NEC

Title: 	CBW for RedCap

Document for:	Discussion and Decision

Introduction

In RAN1#101-e emeeting, the following agreements were made [1]:

Agreements:

- For FR1, study at least 20MHz maximum UE bandwidth at least for initial access

  - Other bandwidths FFS

- For FR2, study 50MHz and 100 MHz maximum UE bandwidth at least for initial access

  - Other bandwidths FFS

In this contribution, we discuss about this agreement in FR2 and make proposal on it.

Discussion

Maximum transmission BW [RB] and minimum guardband [kHz] are specified by RAN4 spec [2] for channel bandwidth 50 MHz and 100 MHz in FR2 as follows:

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N.A |
| 120 | 32 | 66 | 132 | 264 |

Table 5.3.3-1: Minimum guardband for each UE channel bandwidth and SCS (kHz)

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 1210 | 2450 | 4930 | N. A |
| 120 | 1900 | 2420 | 4900 | 9860 |

Table: 5.3.3-2: Minimum guardband (kHz) of SCS 240 kHz SS/PBCH block

| SCS (kHz) | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- |
| 240 | 3800 | 7720 | 15560 |

50 MHz and 100 MHz CBW were agreed for study instead of other CBW such as 80 MHz in RAN1#101-e. One reason would be these CBW have already been specified by RAN4. On the other hand, as seen from above tables, nothing has been specified for 240 kHz SCS for 50 MHz CBW.

Considering process of the agreements, 240 kHz SCS for 50 MHz CBW should be out of scope of the study.

Proposal:

- For 50 MHz CBW in FR2, 240 kHz SCS is out of scope of the study due to lack of RAN4 spec

Summary

Proposal:

- For 50 MHz CBW in FR2, 240 kHz SCS is out of scope of the study due to lack of RAN4 spec

Reference

[1] Chairman’s notes RAN1#101-e

[2] TS38.101-2