3GPP TSG RAN WG1 #102-e	        		             R1-2006140

e-Meeting, August 17th – 28th, 2020

Agenda item:	8.3.1.2

Source: 	Samsung

Title: 				    CSI feedback enhancements for URLLC

Document for:	Discussion and Decision

# Introduction

This contribution considers potential CSI feedback enhancements for URLLC in Rel-17.

# CSI feedback enhancements

A UE supporting mixed type traffic, such as eMBB and AR/VR, needs to report CQI corresponding to an MCS for 10-1 BLER and report CQI corresponding to an MCS for 10-5 BLER [1]. The UE can be provided separate CSI-ReportConfig for two corresponding CSI reports and each P/SP-CSI report has priority 0.

In Rel-16, a CSI report associated with traffic having priority 1 is multiplexed in a PUCCH having priority 0. As a consequence, it is deprioritized for power allocation and may be dropped whenever there are not sufficient PUCCH/PUSCH resources when multiplexed with HARQ-ACK, or when it overlaps with a PUCCH that includes UCI with priority 1. Although A-CSI can follow the priority of the PUSCH, the A-CSI multiplexing delay in the PUSCH (even if potentially somewhat reduced for a new UE capability relative to Rel-16) makes reliance on A-CSI inappropriate for URLLC application requiring short latency.

Proposal 1: Consider support of P/SP-CSI report with priority 1.

In Rel-16, A-CSI report triggering by DCI format 1_x or by a GC-DCI format was considered but not adopted. For URLLC services with periodic/deterministic traffic models, a periodic CSI report suffices. URLLC services with sporadic/FTP traffic models require very low latency and either very low BLER for small TBS (e.g. AR/VR with 1 ms air interface latency, 10-5 target BLER, and 32 bytes TBS) or somewhat larger BLER for somewhat larger TBS (e.g. power distribution or AR/VR with 1 ms air interface latency, 10-3 target BLER, and 4096 or 10K bytes TBS). The following are observed for link adaptation based on an A-CSI report:

- Throughput gains are marginal, if any, for small TBS and sporadic traffic

  - In TDD, considering A-CSI-RS and PUCCH overhead, the trade-off from using an A-CSI report is likely negative over using RSRP (with or without periodic CSI reports)

  - Link adaptation by DCI format 1_x can only benefit TB retransmissions. Given a target BLER of 10-2-10-3 for initial TB transmission, and without considering A-CSI-RS measurement/quantization errors or PUCCH overhead or CSI report decoding errors, it is clear that a throughput gain, if any, is marginal

- The latency for an A-CSI report is prohibitive for URLLC. For example, for UE processing capability 1, the latency for generating and multiplexing an A-CSI report is 3 msec for 15 kHz SCS and 1.374 msec for 120 kHz SCS. Further latency exists for when the PUCCH can be transmitted and for the PUCCH transmission duration.

- The specification impact is significant and includes new triggering of A-CSI-RS, A-CSI reporting in PUCCH, and addressing HARQ-ACK codebook errors when the UE misses the triggering DCI format (if A-CSI report and HARQ-ACK codebook are in a same PUCCH; if not, additional specifications and latency are required).

Observation: URLLC services requiring low latency or low target BLERs cannot benefit from A-CSI reports.

# Conclusions

This contribution considered CSI feedback enhancements for Rel-17 URLLC and proposes the following.

Proposal 1: Consider support of P/SP-CSI report with priority 1.

In addition, the following is observed.

Observation: URLLC services requiring low latency and low target BLERs cannot benefit from A-CSI reports.

# References:

- TS 38.214 v16.2.0, “NR; Physical layer procedures for data”