3GPP TSG RAN WG1 #102-e	R1-2006160

# e-Meeting, Aug. 17th – Aug. 28th, 2020

Agenda Item:		8.8.1.1

Source:				Samsung

Title:					Baseline coverage performance using LLS for FR1

Document for:	Discussion

Introduction

As listed in the study item objective [1], link-level simulations are to be used for the identification of baseline coverage performance for both DL and UL for the various scenarios and services. This contribution mainly focuses on FR1 preliminary results based on the simulation assumptions agreed in RAN1#101-e meeting. Simulation results for FR2 are given in [5].

Evaluation methodology

In Rel-11 LTE coverage enhancement [2], Maximum Coupling Loss (MCL) was chosen as the metric to evaluate coverage of a radio access technology. The coupling loss is defined as the total long-term channel loss over the link between the Tx antenna ports and the Rx antenna ports. The maximum coupling loss (MCL) is the limit value of the coupling loss at which the service can be delivered, and then MCL can be calculated as the difference between the maximum Tx power and RX sensitivity. Therefore, the coverage of the service can be expressed by MCL. However, MCL used in [2] does not include the antenna gains, shadowing, and penetration loss which depend on the scenario and channel.

In Rel-15 IMT2020 self-evaluation [3], available path loss in link budget is used in terms of coverage performance for a radio technology. Available path loss includes the loss in the signal path due to shadow fading and penetration loss depending on the scenarios and channel. In order to take into account the different scenarios, the link budget methodology used in IMT-2020 self-evaluation for 3GPP submission is preferred. The detailed evaluation procedure for available path loss and corresponding maximum range is provided in the Appendix in Table A-1. The calculated maximum range would provide evidence to identify the coverage bottlenecks of transmission channels in both UL and DL.

Proposal 1. The available path loss should be considered in the link budget calculation for coverage performance in FR1 to take into account the different scenarios and channel.

Simulation assumptions

In RAN1#101-e meeting, simulation parameters were agreed for several scenarios, services and channels. In this contribution, we give the simulation results for various scenarios, such as urban, rural, and rural with long distance scenarios, for  uplink and downlink data and control channels, such as PUSCH, PUCCH, PDSCH, and PDCCH. In detail, The values of the simulation parameters used in the evaluation for FR1 are summarized in the Appendix.

Preliminary results

Link budget results using the link budget template in Table A-1 in the Appendix have been performed based on the  simulation parameter values in the Appendix. In addition, the required SNR in the link budget template is calculated based on the link-level simulation results at the performance target for each channel. The baseline performance in dB and coverage distance in meter are defined as available path-loss (29) and maximum range (30) in link budget template in Table A-1, respectively. ISD in meter is evaluated based on the coverage distance in meter. The target performance uses the expected ISD for each scenario (e.g., 500m for urban, 1732m for rural, and 30km for rural with long distance) to compute the target path loss from the path-loss model in [Section 7.4.1, 4], and the corresponding performance gap is obtained as

Gap = target path loss – available path loss in baseline performance.

The following sections show the detailed results of the evaluation. Overall we observed that UL channels need improvement in most or all evaluated cases, while DL channels perform above the considered performance gap, with PDSCH as the best performing channel. Based on the evaluation results, we propose the following

Proposal 2. Coverage enhancements for PUSCH and PUCCH should be prioritized for FR1.

Urban scenario at 4GHz

Baseline performance is provided for urban scenario with 4GHz carrier frequency and TDD system. For urban scenario, the target ISD is 500m and thus the target performance for ISD=500m is calculated based on the path loss model in [Section 7.4.1, 4] as 121.75 dB in terms of path loss.

Table 1. Results for urban scenario with NLOS O-to-I channel at FR1

| Carrier | Service & data rate | Frame structure | Pathloss model | # of PRBs | Required SNR (dB) | Maximum path loss (dB) | Target Performance (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ISD=500m | Gap |
| 4GHz | PUSCH for eMBB | DDDSUDDSUU | NLOS (O-to-I) | 30 | -0.95 | 109.25 | 121.75 | 12.5 |
|  | VoIP | DDDSUDDSUU |  | 4 | -5.90 | 122.45 |  | -0.7 |
|  | PUCCH format 1 for UCI 2bits | - |  | 1 | -7.95 | 124.37 |  | -2.62 |
|  | PUCCH format 3 for UCI 4bits | - |  | 1 | -7.65 | 124.07 |  | -2.32 |
|  | PUCCH format 3 for UCI 11bits | - |  | 1 | -4.65 | 121.07 |  | 0.68 |
|  | PUCCH format 3 for UCI 22bits | - |  | 1 | -1.55 | 117.97 |  | 3.78 |
|  | PDSCH for eMBB | DDDSUDDSUU |  | 90 | -3.3 | 131.82 |  | -10.07 |
|  | PDCCH | - |  | 48 | -11.25 | 138.92 |  | -17.17 |

As observed in Table 1, there is 12.5 dB gap from target ISD=500m for PUSCH with 1Mbps eMBB service in case of TDD configuration with DDDSUDDSUU. For VoIP service, there is -0.7 dB gap for 0.02 target residual BLER. For PUCCH, there are -2.62, -2.32, 0.68, and 3.78 dB gap for format 1 with 2 UCI bits, format 3 with 4 UCI bits, format 3 with 11 UCI bits, and format 3 with 22 UCI bits, respectively. For downlink channel, there are -10.07 and -17.17 dB gap for PDSCH with eMBB service and PDCCH, respectively. Based on the results, it is observed that for urban scenario, PUSCH enhancement in eMBB service and PUCCH (including CSI) enhancement are needed with ISD=500m.

Observation 1. For urban scenario, PUSCH enhancement in eMBB is needed with ISD=500m.

Observation 2. For urban scenario, PUCCH including CSI enhancement is needed with ISD=500m.

Rural scenario at 4GHz

Baseline performance is provided for rural scenario with 4GHz carrier frequency and TDD system. We also consider the NLOS outdoor-to-indoor (O-to-I) and outdoor-to-outdoor (O-to-O) channel. For rural scenario, the target ISD is 1732m and thus the target performance for ISD=1732m is calculated based on the path loss model in [Section 7.4.1, 4] as 131.57 dB in terms of path loss.

Table 2. Results for rural scenario with NLOS O-to-I channel at FR1

| Carrier | Service & data rate | Frame structure | Pathloss model | # of PRBs | Required SNR (dB) | Maximum path loss (dB) | Target Performance (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ISD=1732m | Gap |
| 4GHz | PUSCH for eMBB | DDDSUDDSUU | NLOS (O-to-I) | 4 | -0.5 | 125.88 | 131.57 | 5.69 |
|  | VoIP | DDDSUDDSUU |  | 4 | -5.90 | 130.78 |  | 0.79 |
|  | PUCCH format 1 for UCI 2bits | - |  | 1 | -7.95 | 132.46 |  | -0.89 |
|  | PUCCH format 3 for UCI 4bits | - |  | 1 | -7.65 | 132.16 |  | -0.59 |
|  | PUCCH format 3 for UCI 11bits | - |  | 1 | -4.65 | 129.16 |  | 2.41 |
|  | PUCCH format 3 for UCI 22bits | - |  | 1 | -1.55 | 126.06 |  | 5.51 |
|  | PDSCH for eMBB | DDDSUDDSUU |  | 10 | -2.65 | 153.22 |  | -21.65 |
|  | PDCCH | - |  | 48 | -11.3 | 151.24 |  | -19.67 |

As observed in Table 2, there is 5.69 dB gap from target ISD=1732m for PUSCH with 100kbps eMBB service in case of TDD configuration with DDDSUDDSUU. For VoIP service, there is 0.79 dB gap for 0.02 target residual BLER. For PUCCH, there are -0.89, -0.59, 2.41, and 5.51 dB gap for format 1 with 2 UCI bits, format 3 with 4 UCI bits, format 3 with 11 UCI bits, and format 3 with 22 UCI bits, respectively. For downlink channel, there are -21.65 and -19.67 dB gap for PDSCH with eMBB service and PDCCH, respectively. Based on the results, it is observed that for rural scenario, PUSCH enhancement in both eMBB and VoIP service and PUCCH (including CSI) enhancement are needed with ISD=1732m.

Observation 3. For rural scenario at 4GHz with O-to-I channel, PUSCH enhancement in both eMBB and VoIP services is needed with ISD=1732m.

Observation 4. For rural scenario at 4GHz with O-to-I channel, PUCCH including CSI enhancement is needed with ISD=1732m.

Table 3. Results for rural scenario with NLOS O-to-O channel at FR1

| Carrier | Service & data rate | Frame structure | Pathloss model | # of PRBs | Required SNR (dB) | Maximum path loss (dB) | Target Performance (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ISD= 1732m | Gap |
| 4GHz | PUSCH for eMBB | DDDSUDDSUU | NLOS (O-to-O) | 4 | -0.85 | 128.25 | 131.57 | 3.32 |
|  | VoIP | DDDSUDDSUU |  | 4 | -7.60 | 134.50 |  | -2.93 |
|  | PUCCH format 1 for UCI 2bits | - |  | 1 | -7.95 | 133.96 |  | -2.39 |
|  | PUCCH format 3 for UCI 4bits | - |  | 1 | -7.65 | 133.66 |  | -2.09 |
|  | PUCCH format 3 for UCI 11bits | - |  | 1 | -4.65 | 130.66 |  | 0.91 |
|  | PUCCH format 3 for UCI 22bits | - |  | 1 | -1.55 | 127.56 |  | 4.01 |
|  | PDSCH for eMBB | DDDSUDDSUU |  | 10 | -2.95 | 155.54 |  | -23.97 |
|  | PDCCH | - |  | 48 | -11.2 | 152.64 |  | -21.07 |

As observed in Table 3, there is 3.32 dB gap from target ISD=1732m for PUSCH with 100kbps eMBB service in case of TDD configuration with DDDSUDDSUU. For VoIP service, there is -2.93 dB gap for 0.02 target residual BLER. For PUCCH, there are -2.39, -2.09, 0.91, and 4.01 dB gap for format 1 with 2 UCI bits, format 3 with 4 UCI bits, format 3 with 11 UCI bits, and format 3 with 22 UCI bits, respectively. For downlink channel, there are -23.97 and -21.07 dB gap for PDSCH with eMBB service and PDCCH, respectively. Based on the results, it is observed that for rural scenario, PUSCH enhancement in eMBB service and PUCCH (including CSI) enhancement are needed with ISD=1732m.

Observation 5. For rural scenario at 4GHz with O-to-O channel, PUSCH enhancement in eMBB service is needed with ISD=1732m.

Observation 6. For rural scenario at 4GHz with O-to-O channel, PUCCH including CSI enhancement is needed with ISD=1732m.

Rural scenario at 2GHz

Baseline performance is provided for rural scenario with 2GHz carrier frequency and FDD system. We also consider the NLOS outdoor-to-indoor (O-to-I) and outdoor-to-outdoor (O-to-O) channel. For rural scenario, the target ISD is 1732m and thus the target performance for ISD=1732m is calculated based on the path loss model in [Section 7.4.1, 4] as 125.55 dB in terms of path loss.

Table 4. Results for rural scenario with NLOS O-to-I channel at FR1

| Carrier | Service & data rate | Frame structure | Pathloss model | # of PRBs | Required SNR (dB) | Maximum path loss (dB) | Target Performance (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ISD=1732m | Gap |
| 2GHz | PUSCH for eMBB | FDD | NLOS (O-to-I) | 4 | -2.70 | 128.08 | 125.55 | -2.53 |
|  | VoIP |  |  | 4 | -4.55 | 129.43 |  | -3.88 |
|  | PUCCH format 1 for UCI 2bits |  |  | 1 | -7.95 | 132.46 |  | -6.91 |
|  | PUCCH format 3 for UCI 4bits |  |  | 1 | -7.65 | 132.16 |  | -6.61 |
|  | PUCCH format 3 for UCI 11bits |  |  | 1 | -4.65 | 129.16 |  | -3.61 |
|  | PUCCH format 3 for UCI 22bits |  |  | 1 | -1.55 | 126.06 |  | -0.51 |
|  | PDSCH for eMBB |  |  | 10 | -1.60 | 146.16 |  | -20.61 |
|  | PDCCH |  |  | 48 | -11.4 | 145.32 |  | -19.77 |

As observed in Table 4, there is -2.53 dB gap from target ISD=1732m for PUSCH with 100kbps eMBB service in case of TDD configuration with DDDSUDDSUU. For VoIP service, there is -3.88 dB gap for 0.02 target residual BLER. For PUCCH, there are -6.91, -6.61, -3.61, and -0.51 dB gap for format 1 with 2 UCI bits, format 3 with 4 UCI bits, format 3 with 11 UCI bits, and format 3 with 22 UCI bits, respectively. For downlink channel, there are -20.61 and -19.77 dB gap for PDSCH with eMBB service and PDCCH, respectively. Based on the results, it is observed that for rural scenario, coverage enhancement is not needed with ISD=1732m.

Observation 7. For rural scenario at 2GHz with O-to-I channel, coverage enhancement is not needed with ISD=1732m.

Table 5. Preliminary results for rural scenario with NLOS O-to-O channel at FR1

| Carrier | Service & data rate | Frame structure | Pathloss model | # of PRBs | Required SNR (dB) | Maximum path loss (dB) | Target Performance (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ISD=1732m | Gap |
| 2GHz | PUSCH for eMBB | FDD | NLOS (O-to-O) | 4 | -2.00 | 129.40 | 125.55 | -3.85 |
|  | VoIP |  |  | 4 | -8.70 | 135.60 |  | -10.05 |
|  | PUCCH format 1 for UCI 2bits |  |  | 1 | -7.95 | 133.96 |  | -8.41 |
|  | PUCCH format 3 for UCI 4bits |  |  | 1 | -7.65 | 133.66 |  | -8.11 |
|  | PUCCH format 3 for UCI 11bits |  |  | 1 | -4.65 | 130.66 |  | -5.11 |
|  | PUCCH format 3 for UCI 22bits |  |  | 1 | -1.55 | 127.56 |  | -2.01 |
|  | PDSCH for eMBB |  |  | 10 | -1.90 | 148.48 |  | -22.93 |
|  | PDCCH |  |  | 48 | -11.4 | 146.82 |  | -21.27 |

As observed in Table 5, there is -3.85 dB gap from target ISD=1732m for PUSCH with 100kbps eMBB service in case of TDD configuration with DDDSUDDSUU. For VoIP service, there is -10.05 dB gap for 0.02 target residual BLER. For PUCCH, there are -8.41, -8.11, -5.11, and -2.01 dB gap for format 1 with 2 UCI bits, format 3 with 4 UCI bits, format 3 with 11 UCI bits, and format 3 with 22 UCI bits, respectively. For downlink channel, there are -22.93 and -21.27 dB gap for PDSCH with eMBB service and PDCCH, respectively. Based on the results, it is observed that for rural scenario, coverage enhancement is not needed with ISD=1732m.

Observation 8. For rural scenario at 2GHz with O-to-O channel, coverage enhancement is not needed with ISD=1732m.

Rural scenario with long distance

Baseline performance is provided for rural scenario with long distance and FDD system. For rural scenario with long distance, the target ISD is 30km and thus the target performance for ISD=30km is calculated based on the path loss model in [Section 7.4.1, 4] as 142.91 dB in terms of path loss.

Table 6. Results for rural scenario with long distance at FR1

| Carrier | Service & data rate | Frame structure | Pathloss model | # of PRBs | Required SNR (dB) | Maximum path loss (dB) | Target Performance (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ISD=30km | Gap |
| 0.7GHz | PUSCH for eMBB | FDD | LOS | 4 | -7.80 | 134.01 | 142.9 | 8.89 |
|  | VoIP |  |  | 4 | -15.00 | 140.71 |  | 2.19 |
|  | PUCCH format 1 for UCI 2bits |  |  | 1 | -10.10 | 135.49 |  | 7.41 |
|  | PUCCH format 3 for UCI 4bits |  |  | 1 | -10.20 | 135.59 |  | 7.31 |
|  | PUCCH format 3 for UCI 11bits |  |  | 1 | -8.00 | 133.39 |  | 9.51 |
|  | PUCCH format 3 for UCI 22bits |  |  | 1 | -5.10 | 130.49 |  | 12.41 |
|  | PDSCH for eMBB |  |  | 10 | -4.90 | 144.27 |  | -1.37 |
|  | PDCCH |  |  | 48 | -12.30 | 141.08 |  | 1.82 |

As observed in Table 6, there is 8.89 dB gap from target ISD=30km for PUSCH with 100kbps eMBB service in case of TDD configuration with DDDSUDDSUU. For VoIP service, there is 2.19 dB gap for 0.02 target residual BLER. For PUCCH, there are 7.41, 7.31, 9.51, and 12.41 dB gap for format 1 with 2 UCI bits, format 3 with 4 UCI bits, format 3 with 11 UCI bits, and format 3 with 22 UCI bits, respectively. For downlink channel, there are -1.37 and 1.82 dB gap for PDSCH with eMBB service and PDCCH, respectively. Based on the results, it is observed that for rural scenario, PUSCH, PUCCH, PDCCH coverage enhancement are needed with ISD=500m.

Observation 9. For rural scenario with long distance, PUSCH enhancement in both eMBB and VoIP services is needed with ISD=30km.

Observation 10. For rural scenario with long distance, PUCCH enhancement is needed with ISD=30km.

Observation 11. For rural scenario with long distance, PDCCH enhancement is needed with ISD=30km.

Conclusion

This contribution presents FR1 simulation results to identify baseline coverage performance of several UL and DL channels in FR1. The following observations and proposals are made:

Proposal 1. The available path loss should be considered in the link budget calculation for coverage performance in FR1 to take into account the different scenarios and channel.

Proposal 2. Coverage enhancements for PUSCH and PUCCH should be prioritized for FR1.

Observation 1. For urban scenario, PUSCH enhancement in eMBB is needed with ISD=500m.

Observation 2. For urban scenario, PUCCH including CSI enhancement is needed with ISD=500m.

Observation 3. For rural scenario at 4GHz with O-to-I channel, PUSCH enhancement in both eMBB and VoIP services is needed with ISD=1732m.

Observation 4. For rural scenario at 4GHz with O-to-I channel, PUCCH including CSI enhancement is needed with ISD=1732m.

Observation 5. For rural scenario at 4GHz with O-to-O channel, PUSCH enhancement in eMBB service is needed with ISD=1732m.

Observation 6. For rural scenario at 4GHz with O-to-O channel, PUCCH including CSI enhancement is needed with ISD=1732m.

Observation 7. For rural scenario at 2GHz with O-to-I channel, coverage enhancement is not needed with ISD=1732m.

Observation 8. For rural scenario at 2GHz with O-to-O channel, coverage enhancement is not needed with ISD=1732m.

Observation 9. For rural scenario with long distance, PUSCH enhancement in both eMBB and VoIP services is needed with ISD=30km.

Observation 10. For rural scenario with long distance, PUCCH enhancement is needed with ISD=30km.

Observation 11. For rural scenario with long distance, PDCCH enhancement is needed with ISD=30km.

- References

- RP-200861, Revised SID on NR coverage enhancement, China Telecom

- TR 36.824, Evolved Universal Terrestrial Radio Access (E-UTRA); LTE coverage enhancements (Release 11).

- TR 37.910, 	Study on self-evaluation towards IMT-2020 submission (Release 16).

- TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz (Release 16).

- R1-2006161, Baseline coverage performance using LLS for FR2, Samsung

- Appendix

Table A-1. Link budget template in [3]

| Parameter | Values |
| --- | --- |
| Scenario |  |
| Frame structure |  |
| Carrier frequency (Hz) |  |
| BS antenna heights (m) |  |
| UT antenna heights (m) |  |
| Cell area reliability for control channel |  |
| Cell area reliability for data channel |  |
| Transmission bit rate for control channel (bit/s) |  |
| Transmission bit rate for data channel (bit/s) |  |
| Target packet error rate for the required SNR in item (19a) for control channel |  |
| Target packet error rate for the required SNR in item (19b) for data channel |  |
| Spectral efficiency (bit/s/Hz) |  |
| Pathloss model (select from LoS or NLoS) |  |
| UE speed (km/h) |  |
| Feeder loss (dB) |  |
| Transmitter |  |
| (1) Number of transmit antennas. (The number shall be within the indicated range in § 8.4 of Report ITU-R M.2412-0) |  |
| (2) Maximal transmit power per antenna (dBm) |  |
| (3) Total transmit power = function of (1) and (2) (dBm) (The value shall not exceed the indicated value in § 8.4 of Report ITU-R M.2412-0) |  |
| (4) Transmitter antenna gain (dBi) |  |
| (5) Transmitter array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, CDD (cyclic delay diversity), etc.) (dB) |  |
| (6) Control channel power boosting gain (dB) |  |
| (7) Data channel power loss due to pilot/control boosting (dB) |  |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) |  |
| (9a) Control channel EIRP = (3) + (4) + (5) + (6) – (8) dBm |  |
| (9b) Data channel EIRP = (3) + (4) + (5) – (7) – (8) dBm |  |
| Receiver |  |
| (10) Number of receive antennas (The number shall be within the indicated range in § 8.4 of Report ITU-R M.2412-0) |  |
| (11) Receiver antenna gain (dBi) |  |
| (11bis) Receiver array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, etc.) (dB) |  |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) |  |
| (13) Receiver noise figure (dB) |  |
| (14) Thermal noise density (dBm/Hz) |  |
| (15a) Receiver interference density for control channel (dBm/Hz) |  |
| (15b) Receiver interference density for data channel (dBm/Hz) |  |
| (16a) Total noise plus interference density for control channel = 10 log (10^(((13) + (14))/10) + 10^((15a)/10)) dBm/Hz |  |
| (16b) Total noise plus interference density for data channel = 10 log (10^(((13) + (14))/10) + 10^((15b)/10)) dBm/Hz |  |
| (17a) Occupied channel bandwidth for control channel (for meeting the requirements of the traffic type) (Hz) |  |
| (17b) Occupied channel bandwidth for data channel (for meeting the requirements of the traffic type) (Hz) |  |
| (18a) Effective noise power for control channel = (16a) + 10 log((17a)) dBm |  |
| (18b) Effective noise power for data channel = (16b) + 10 log((17b)) dBm |  |
| (19a) Required SNR for the control channel (dB) |  |
| (19b) Required SNR for the data channel (dB) |  |
| (20) Receiver implementation margin (dB) |  |
| (21a) H-ARQ gain for control channel (dB) |  |
| (21b) H-ARQ gain for data channel (dB) |  |
| (22a) Receiver sensitivity for control channel = (18a) ++ (19a) + (20) – (21a) dBm |  |
| (22b) Receiver sensitivity for data channel = (18b) ++ (19b) + (20) – (21b) dBm |  |
| (23a) Hardware link budget for control channel = (9a) + (11) + (11bis) − (22a) dB |  |
| (23b) Hardware link budget for data channel = (9b) + (11) + (11bis) − (22b) dB |  |
| Calculation of available pathloss |  |
| (24) Lognormal shadow fading std deviation (dB) |  |
| (25a) Shadow fading margin for control channel (function of the cell area reliability and (24)) (dB) |  |
| (25b) Shadow fading margin for data channel (function of the cell area reliability and (24)) (dB) |  |
| (26) BS selection/macro-diversity gain (dB) |  |
| (27) Penetration margin (dB) |  |
| (28) Other gains (dB) (if any please specify) |  |
| (29a) Available path loss for control channel = (23a) – (25a) + (26) – (27) + (28) – (12) dB |  |
| (29b) Available path loss for data channel = (23b) – (25b) + (26) – (27) + (28) – (12) dB |  |
| Range/coverage efficiency calculation |  |
| (30a) Maximum range for control channel (based on (29a) and according to the system configuration section of the link budget) (m) |  |
| (30b) Maximum range for data channel (based on (29b) and according to the system configuration section of the link budget) (m) |  |

Table A-2 Simulation parameters

| Parameters | Values |
| --- | --- |
| Scenario and frequency | Urban: 4GHz (TDD) Rural: 4GHz (TDD), 2GHz (FDD) Rural with long distance: 700MHz (FDD) |
| Frame structure for TDD | DDDSUDDSUU (S: 10D:2G:2U) only for 4GHz |
| Pathloss model (select from LoS or NLoS) | Urban: NLoS Rural: NLoS and LoS |
| BWP | 100MHz for 4GHz. 20MHz for 2GHz and 700MHz |
| SCS | 30kHz for TDD, 15kHz for FDD. |
| Channel model for link-level simulation | TDL-C for NLOS, TDL-D for LOS. |
| UE velocity | Urban: 3km/h for indoor Rural: 3km/h for indoor, 120km/h for outdoor |
| Number of receive antenna elements for BS | Urban: 192 antenna elements for 4GHz, Rural: 64 antenna elements for 4GHz 32 antenna elements for 2GHz 16 antenna elements for 700MHz |
| Number of TxRUs for BS | gNB modeling in LLS for TDL: 2 gNB receive chains in LLS. |
| Delay spread | Urban: 300ns Rural: 300ns Rural with long distance: 30ns |

Table A-3. Channel specific parameters for PUSCH

| Parameters | Values |
| --- | --- |
| BLER for PUSCH | For eMBB, w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Number of UE transmit chains for PUSCH | 1 |
| DMRS configuration for PUSCH | For 120km/h: Type I, 2 DMRS symbol, no multiplexing with data. For 3km/h: Type I, 1 DMRS symbol, no multiplexing with data. |
| Waveform for PUSCH | DFT-s-OFDM |
| Repetitions for PUSCH | For eMBB, w/o repetition as baseline, For VoIP, w/ repetition and # of actual repetitions = 5 (2/14/2/14/14 symbols in DDDSUDDSUU with repetition type B) |
| HARQ configuration for PUSCH | For eMBB, no HARQ retransmission. For VoIP, w/ HARQ and the maximum number of HARQ transmission = 4 due to latency requirements. |
| Latency requirements for voice | 50ms |
| PUSCH duration | 14 OS |
| PRBs/TBS/MCS for eMBB for PUSCH | 30 PRBs for 1Mbps and 4 PRBs for 100kbps QPSK modulation |
| PRBs/MCS for VoIP for PUSCH | 4 PRBs for VoIP QPSK modulation |
| Frequency hopping | w/o frequency hopping for PUSCH |

Table A-4. Channel specific parameters for PUCCH

| Parameters | Values |
| --- | --- |
| PUCCH format type | Format 1, 2bits UCI. Format 3, 4/11/22 bits UCI |
| BLER for PUCCH | For PUCCH format 1: ACK missed detection probability: 1%. For PUCCH format 3: BLER: 1% |
| Number of PRBs for PUCCH | 1 PRB |
| Number of UE transmit chains for PUCCH | 1 |
| Number of repetitions for PUCCH | w/o repetition for PUCCH. |
| PUCCH duration | 14 OS |
| DMRS configuration for PUCCH | Additional DMRS symbols for PUCCH Format 3. |
| Frequency hopping | w/ frequency hopping for PUCCH. |

Table A-5. Channel specific parameters for PDSCH

| Parameters | Values |
| --- | --- |
| Waveform | CP-OFDM |
| PRBs/MCS/TBS | 90 PRBs for urban scenario 10 PRBs for other scenarios QPSK modulation |
| PDSCH duration | 12 OS |
| Other parameters | FFS |

Table A-6. Channel specific parameters for PDCCH

| Parameters | Values |
| --- | --- |
| Aggregation level | 16 |
| Payload | 40 bits |
| CORESET size | 2 symbols, 48 PRBs |
| Tx Diversity | Precoder cycling |
| BLER for PDCCH | 1% BLER |