3GPP TSG RAN WG1 #102-e					    	                     R1-2006359

e-Meeting, August 17th – 28th, 2020

# Source:		     ETRI

# Title:		 Discussion on UL timing advance for NTN

# Agenda item:    8.4.2

# Document for:  Discussion

Introduction

In RAN#88e, the following has been agreed on the work item of NR NTN[1].

The work item aims to specify the enhancements identified for NR NTN (non-terrestrial networks) especially LEO and GEO with implicit compatibility to support HAPS (high altitude platform station) and ATG (air to ground) scenarios according to the following principles: - FDD is assumed for core specification work for NR-NTN. - NOTE: This does not imply that TDD cannot be used for relevant scenarios e.g. HAPS, ATG - Earth fixed Tracking area is assumed with Earth fixed and moving cells - UEs with GNSS capabilities are assumed. - Transparent payload is assumed.

With these elements and [2], we discuss UL timing advance in more detail.

Discussion

It is necessary to discuss the UE transmission timing with GNSS capabilities. Based on [3], the reference point for the UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus NTA+NTA offset×TC. The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell. NTA for PRACH is defined as 0. In addition, based on [4], uplink frame number  for transmission from the UE shall start TTA=NTA+NTA offsetTC before the start of the corresponding downlink frame at the UE where NTA offset is given by [5], except for msgA transmission on PUSCH where NTA=0 shall be used.

Considering long RTT, the above paragraphs of [3] and [4] can be changed as follows for NTN.

The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus NTA+N TA, NTN+NTA offset×TC. The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell. NTA for PRACH is defined as 0. In addition, uplink frame number  for transmission from the UE shall start TTA=NTA+N TA,NTN+NTA offsetTC before the start of the corresponding downlink frame at the UE where NTA offset is given by [5], except for msgA transmission on PUSCH where NTA=0 shall be used.

N TA,NTN can be one of {0, eTASL, cTASL, cTAFL, (eTASL+cTAFL), (cTASL+cTAFL)}. eTA and cTA denote estimated TA by UE and common TA indicated by gNB, respectively. Subscripts SL and FL are RTT of service link and feeder link, respectively. NTA,NTN may vary by types of NTN platforms. If the value of NTA,NTN is 0, NTA,NTN cannot be configured. If cTA is used for NTA,NTN, the corresponding cTA parameter(cTASL and/or cTAFL and/or (cTASL+cTAFL)) should be included in MIB or SIB. If eTA is used for NTA,NTN, a procedure  for gNB to acquire the RTT of service link can be required. For UEs with GNSS capabilities, NTA,NTN may include eTASL.

Proposal 1:	At least the RTT of service link can be reflected in the UL transmission timing. The reflected value for service link is a value estimated by UE or a common value indicated by gNB. If the estimated value is used, a procedure for gNB to acquire the RTT of service link can be required.

According to NTA,NTN and NTA offset, the design of PRACH, the configuration of RACH occasion, and the bit size of NTA are related. If eTA is used, it may not be necessary to design a new PRACH due to differential delay by UEs. However, the effect of eTA timing error on PRACH design, RACH occasion, and the bit size of NTA should be considered. If cTA is used, the effect of the maximum differential delay between UEs on PRACH design, RACH occasion, and the bit size of NTA should be considered.

Proposal 2:	For the design of PRACH, the configuration of RACH occasion, and the bit size of NTA, the requirements of the timing error of the estimated RTT by UE and the maximum differental delay between the UEs in a cell/beam should be determined.

Based on [5][6], the ra-ResponseWindow starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set that is at least one symbol, after the last symbol of the PRACH occasion corresponding to the PRACH transmission.

Considering long RTT, the above paragraphs of [5][6] can be changed as follows for NTN.

The ra-ResponseWindow starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set that is at least one symbol, no earlier than TNTN,  RAR after the last symbol of the PRACH occasion corresponding to the PRACH transmission. TNTN,  RAR should include at least the RTT of feeder link. In order to reflect the RTT of the feeder link to TNTN,  RAR, the corresponding cTA parameter(cTAFL and/or (cTASL+cTAFL)) should be included in MIB or SIB. For beam failure recovery, TNTN,  RAR can be NTA+NTA,NTN+NTA offsetTC.

Proposal 3:	At least the RTT of the feeder link should be reflected at the ra-ResponseWindow starting time. For this, the corresponding parameter should be included in MIB or SIB.

Based on [5], a timing advance command, TA, for a TAG indicates adjustment of a current NTA value, NTA_old, to the new NTA value, NTA_new, by index values of TA = 0, 1, 2,..., 63, where for a SCS of 2μ∙15 kHz, NTA_new=NTA_old+TA-31∙16∙64/2μ.

If a UE performs timing adjustment implicitly, it can be necessary to define NTA_old and a procedure for gNB to acquire the RTT of the service link. A timing drift rate or eTA may be used for implicit timing adjustment. If the timing draft rate is used, it can be cell specific and/or UE specific parameter. If implicit timing adjustment is used, it may be applied from PUSCH scheduled by RAR UL grant. In addition, if the UE considers the RTT of the feeder link, it is necessary to discuss how to reflect the timing adjustment of the RTT of the feeder link. This may be reflected in NTA_old or TA.

Proposal 4:	If a UE performs timing adjustment implicitly, it can be necessary to define NTA_old and a procedure for gNB to acquire the RTT of the service link.

Conclusions

In this contribution, we present our considerations for uplink timing advance in NTN scenarios.  Based on the discussion, we make the following proposal.

Proposal 1:	At least the RTT of service link can be reflected in the UL transmission timing. The reflected value for service link is a value estimated by UE or a common value indicated by gNB. If the estimated value is used, a procedure for gNB to acquire the RTT of service link can be required.

Proposal 2:	For the design of PRACH, the configuration of RACH occasion, and the bit size of NTA, the requirements of the timing error of the estimated RTT by UE and the maximum differental delay between the UEs in a cell/beam should be determined.

Proposal 3:	At least the RTT of the feeder link should be reflected at the ra-ResponseWindow starting time. For this, the corresponding parameter should be included in MIB or SIB.

Proposal 4:	If a UE performs timing adjustment implicitly, it can be necessary to define NTA_old and a procedure for the base station to acquire the RTT of the service link.

References

- RP-201256, “Solutions for NR to support non-terrestrial networks (NTN)”, 3GPP TSG RAN meeting #88e

- 3GPP TR 38.821 V16.0.0 (2019-12), “Solutions for NR to support non-terrestrial networks (NTN) (Release 16)”

- 3GPP TS 38.133 V16.4.0 (2020-06), “NR; Requirements for support of radio resource management (Release 16)”

- 3GPP TS 38.211 V16.2.0 (2020-06), “NR; Physical channels and modulation (Release 16)”

- 3GPP TS 38.213 V16.2.0 (2020-06), “NR; Physical layer procedures for control (Release 16)”

- 3GPP TS 38.321 V16.1.0 (2020-07), “NR; Medium Access Control (MAC) protocol specification (Release 16)”