3GPP TSG- RAN WG1 Meeting #102-e                                                   R1-2006501

# e-Meeting, August 17th – 28th, 2020

Agenda Item:	8.1.2.2

Source:	Apple Inc.

Title:	On Inter-Cell Multi-TRP Enhancement

Document for:	Discussion/Decision

# Introduction

In RP-193133, the following objectives on inter-cell multi-TRP enhancement has been agreed.

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Identify and specify features to improve reliability and robustness for channels other than PDSCH (that is, PDCCH, PUSCH, and PUCCH) using multi-TRP and/or multi-panel, with Rel.16 reliability features as the baseline - Identify and specify QCL/TCI-related enhancements to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception - Evaluate and, if needed, specify beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception - Enhancement to support HST-SFN deployment scenario: - Identify and specify solution(s) on QCL assumption for DMRS, e.g. multiple QCL assumptions for the same DMRS port(s), targeting DL-only transmission - Evaluate and, if the benefit over Rel.16 HST enhancement baseline is demonstrated, specify QCL/QCL-like relation (including applicable type(s) and the associated requirement) between DL and UL signal by reusing the unified TCI framework

In this contribution, we provide some discussion on inter-cell multi-TRP enhancement.

# Discussion

In Rel-16, intra-cell multi-TRP operation has been supported, where the UE may receive downlink signal from TRPs with the same physical cell ID. In Rel-17, the objective is to support inter-cell multi-TRP operation as shown in Figure 1.

Figure 1: Scenario for inter-cell multi-TRP operation

In Rel-17, L1/L2 centric inter-cell mobility is also included in one of the objectives. For L1/L2 centric inter-cell mobility, UE also needs to maintain the communication with multiple gNBs, and gets ready for TCI switching as shown in Figure 2.

Figure 2: Scenario for L1/L2 centric inter-cell mobility

Therefore, compared to L1/L2 centric inter-cell mobility, there is no fundamental difference from both gNB and UE behavior to support inter-cell multi-TRP operation. Thus, a common signaling framework should be used to support both L1/L2 centric mobility enhancement and inter-cell multi-TRP enhancement.

To support inter-cell operation, gNB should be able to provide the TCI indication based on SSB from a neighbor cell. Thus, the TCI can be divided into several groups, where each group is associated with a physical cell ID. In addition to the physical cell ID, some other SSB related configuration, e.g. transmission power, periodicity and so on, should be provided.

Proposal 1: A common framework should be supported for both L1/L2 centric mobility and inter-cell multi-TRP operation.

Proposal 2: Support to divide TCI states into N groups, where each group is associated with a physical cell ID.

- Support to configure the physical cell ID, SSB transmission power and periodicity for a TCI state group.

# Conclusion

In this contribution, we discussed inter-cell multi-TRP enhancement. Based on the discussion, the following proposals have been achieved.

Proposal 1: A unified framework should be supported for both L1/L2 centric mobility and inter-cell multi-TRP operation.

Proposal 2: Support to divide TCI states into N groups, where each group is associated with a physical cell ID.

- Support to configure the physical cell ID, SSB transmission power and periodicity for a TCI state group.