3GPP TSG RAN WG1 #102-e			R1-2006702

e-Meeting, August 17th – 28th, 2020

Source:	NTT DOCOMO, INC.

Title:	Maintenance for UE power saving

Agenda Item:	7.2.7

Document for: 	Discussion and Decision

- Introduction

At the RAN1#101 meeting, the remaining issues related to UE power saving were discussed and RAN1 made agreements [1]. In this contribution, we propose a correction on Rel-16 specification regarding UE power saving.

- RAN2 LS on DCP regarding the MAC-PHY interactions

RAN2 sent LS on DCP regarding the MAC-PHY interactions [2]. According to the LS, it has been already specified in MAC spec that DCP monitoring occasion overlapping with Active time, measurement gap and BWP switching period is invalid. Thus, corresponding text in RAN1 spec should be removed in order to avoid the duplication. The description for other invalid DCP monitoring is still necessary in RAN1 spec.

Proposal 1: Following text proposal is applied to section 10.3 in TS 38.213.

## 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells ~ If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE - is not required to monitor PDCCH for detection of DCI format 2_6, as described in Clauses 10, 11.1, 12, and in Clause 5.7 of [11, TS 38.321] for all corresponding PDCCH monitoring occasions outside Active Time prior to a next long DRX cycle, or - does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next long DRX cycle ~

- Conclusion

In this contribution, the maintenance for UE power saving was discussed. Based on the discussion, the following proposal was made:

Proposal 1: Following text proposal is applied to section 10.3 in TS 38.213.

## 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells ~ If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE - is not required to monitor PDCCH for detection of DCI format 2_6, as described in Clauses 10, 11.1, 12, and in Clause 5.7 of [11, TS 38.321] for all corresponding PDCCH monitoring occasions outside Active Time prior to a next long DRX cycle, or - does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next long DRX cycle ~

References

- 3GPP RAN1#101-e, Chairman’s note, June 2020.

- R1-2005210, ” Reply LS on DCP”, RAN2