3GPP TSG RAN WG1 #102-e			     R1-2006729

e-Meeting, August 17th – 28th, 2020

Source:	NTT DOCOMO, INC.

Title:	Discussion on CSI feedback enhancements for Rel.17 URLLC

Agenda Item:	8.3.1.2

Document for: 	Discussion and Decision

- Introduction

Following objectives are described for CSI feedback enhancements for Rel-17 URLLC in [1].

- Study, identify and specify if needed, required Physical Layer feedback enhancements for meeting URLLC requirements covering - UE feedback enhancements for HARQ-ACK [RAN1] - CSI feedback enhancements to allow for more accurate MCS selection [RAN1] Note: DMRS-based CSI feedback is not in scope of this WI

In this contribution, we share our views on enhancements for Rel-17 CSI feedback enhancements for URLLC.

- Discussions

  - A-CSI on PUCCH

Currently periodic CSI (P-CSI), semi-persistent CSI (SP-CSI), and aperiodic CSI (A-CSI) are supported in Rel-15 and Rel-16 as the CSI feedback mechanism to report channel-quality indicator used for MCS selection as well as rank indicator, precoder-matrix indicator, etc. P-CSI is reported on PUCCH by configuring PUCCH resource and transmission periodicity. SP-CSI is reported on PUCCH or PUSCH by configuring PUCCH resource and triggering by MAC CE or UL grant DCI, respectively. A-CSI is transmitted on PUSCH triggered by UL grant DCI. Considering that URLLC traffic is sporadic and low latency is required, A-CSI is reasonable among the CSI reporting types. For example, if CSI reporting needs to rely on P-CSI or SP-CSI, the reporting periodicity should be configured short to meet the low latency requirement, which leads to large uplink overhead and UE power consumption. Or, if the reporting periodicity is configured long, gNB makes scheduling conservative as there is few CSI reporting. Less CSI reporting would be contradict to demand as CSI for DL transmission is more often required for DL heavy scenarios. Based on A-CSI on PUCCH especially on short PUCCH transmission, fast CQI acquisition can be achieved. Therefore, this contribution discusses A-CSI on PUCCH as a CSI feedback enhancement for Rel-17 URLLC.

A-CSI on PUCCH triggering mechanism and PUCCH resource

As described above, A-CSI on PUCCH is a candidate for CSI feedback enhancement in Rel-17 URLLC. First we discuss how to trigger A-CSI on PUCCH especially taking the following points into account:

- Whether DL grant or UL grant is used for triggering?

  - What field is used to trigger A-CSI on PUCCH

  - Can data be scheduled by the DL grant or UL grant together with triggering A-CSI on PUCCH?

- What field(s) is used for triggering?

Currently A-CSI is triggered by UL grant such as DCI format 0_1/0_2. It would be difficult to trigger and report A-CSI timely in DL heavy scenarios. Also, if UL grant scheduling a PUSCH transmission triggers A-CSI on PUCCH in a self-contained slot, simultaneous PUCCH and PUSCH transmission with FDMed manner will be required. This is not supported in Rel-15 and Rel-16. In order to address the issues and enhance the scheduling flexibility, A-CSI on PUCCH triggered by DL grant would be beneficial. Regarding whether DCI can schedule data together with A-CSI on PUCCH, we think it is beneficial that one DL grant DCI can schedule PDSCH and indicate corresponding HARQ-ACK timing together with triggering A-CSI. It reduces UE blind decoding overhead compared to the case where UE needs to receive multiple DL grants to schedule PDSCH and to trigger A-CSI separately. In addition, it would reduce latency of A-CSI on PUCCH transmission since UE has more triggering occasion of A-CSI on PUCCH. It is considered to introduce a new field, e.g. CSI request field, for A-CSI on PUCCH in DL grant including 1_1/1_2 to trigger the A-CSI.

Fig.1 example of A-CSI on PUCCH triggered by DL grant with DL-SCH.

Proposal 1:

- Support A-CSI on PUCCH triggered by DL grant including DCI format 1_1 and 1_2.

  - A new field, e.g. CSI request field, is introduced to trigger A-CSI on PUCCH

  - The DL grant can schedule PDSCH and indicate corresponding HARQ-ACK timing together with A-CSI on PUCCH triggering

About PUCCH resource for A-CSI on PUCCH, the existing PUCCH resource indication can be baseline and thus the following four options can be considered:

- Opt.1: An A-CSI PUCCH resource is dynamically indicated by DCI (A-CSI on PUSCH concept)

- Opt.2: An A-CSI PUCCH resource is configured by RRC (P-/SP-CSI on PUCCH concept)

- Opt.3: Multiple A-CSI PUCCH resources are configured by RRC and indicated which to use by DCI (HARQ-ACK concept)

- Opt.4: Multiple joint A-CSI PUCCH resource with HARQ-ACK is configured by RRC and indicated which to use by DCI (new concept)

Opt.1 has no compatibility with triggering A-CSI by DL grant with PDSCH as TDRA and FDRA are used for scheduling PDSCH. Opt.2 has less scheduling flexibility as the resource cannot be dynamically changed, which does not match with the characteristic of A-CSI. Opt.3 has enough flexible PUCCH resources and gNB can dynamically indicate which PUCCH resource to be used for UE depending on scheduling condition. On the other hand, it requires additional bit(s) for DCI and would result in less reliability due to higher coding rate. Opt.4 has the same merit as Opt.3 and additional bits are not necessary. However, scheduling flexibility is less than Opt.3 if HARQ-ACK timing is prioritized in the scheduling since the transmission timing of A-CSI PUCCH needs to compromise at the HARQ-ACK timing. If A-CSI timing is prioritized in the scheduling, re-transmission latency would increase since the transmission timing of HARQ-ACK needs to compromise at the A-CSI timing then. In short, there is trade-off between A-CSI and HARQ-ACK timing in Opt.4. Based on the above analysis, we prefer Opt.3 as the first preference and Opt.4 as the second preference.

Proposal 2:

- PUCCH resources for A-CSI on PUCCH are configured by RRC and which PUCCH resource to use is dynamically indicated by DCI.

  - Separate and/or joint PUCCH resource configuration and indication of HARQ-ACK can be considered.

Intra-UE multiplexing/prioritization of A-CSI on PUCCH and other UL channels

In Rel-16, intra-UE UL multiplexing/prioritization with different priorities was introduced in order to improve the URLLC traffic performance. With the Rel-16 intra-UE UL multiplexing/prioritization, UE drops low priority UL channel/signal and transmits high priority UL channel/signal in case they are overlapping in time domain, i.e. at least one symbol overlapping. In case A-CSI on PUCCH is adopted in Rel-17, the priority determination of A-CSI on PUCCH needs to be discussed. Here the priority determination includes 1) priorities for the intra-UE multiplexing/prioritization described in clause 9 in 38.213 [2], and 2) priorities for overlapping CSIs in time domain described in clause 5.2.5 in 38.214 [3].

1) Priorities for the intra-UE multiplexing/prioritization described in clause 9 in 38.213

First how to determine the priority of A-CSI on PUCCH needs to be discussed. The following options can be considered:

- Opt.1: fixed priority; always high or low

- Opt.2: dynamic indication by DCI

  - Alt.1: Same priority indicator field for HARQ-ACK is used.

  - Alt.2: Separate priority indicator field from HARQ-ACK is used.

- Opt.3: semi-static configuration by RRC

  - Alt.1: Priority is configured in CSI-AperiodicTriggerState

  - Alt.2: Priority is configured in CSI-ReportConfig

Assuming that appropriate priority of A-CSI on PUCCH would change depending on scheduling condition, dynamic indication might be suitable. For example, if gNB really wants to trigger A-CSI on PUCCH in DL heavy scenarios to select accurate MCS for the upcoming DL transmission, the priority should be high. On the other hand, if there is other UCIs to be prioritized with limited PUCCH resource capacity, the priority of A-CSI on PUCCH could be low. Second, based on the priority determination, UE behavior on multiplexing/prioritization of A-CSI on PUCCH and other UL channels should be discussed including the intra-UE UL multiplexing/prioritization with different priorities to be discussed in another URLLC agenda for Rel-17.

2) Priorities for overlapping CSIs in time domain described in clause 5.2.5 in 38.214

In case different triggering types of CSIs are overlapping in at least one symbol and transmitted on the same carrier, CSI report with higher priority is not transmitted. The priority level is the order of A-CSI on PUSCH > SP-CSI on PUSCH > SP-CSI > P-CSI. Assuming that A-CSI on PUCCH is also dynamically triggered by DCI and substitute for A-CSI on PUSCH in DL heavy scenarios, the priority should be higher than A-CSI on PUSCH in order to ensure A-CSI on PUCCH to be transmitted.

Proposal 3:

- Support priority handling of A-CSI on PUCCH for the following UE behavior:

  - Intra-UE UL multiplexing/prioritization including Rel-17 multiplexing with different priorities

    - Priority level can be dynamically indicated by DCI

  - Overlapping CSIs in time domain in clause 5.2.5 in 38.214

    - Priority level is higher than A-CSI on PUSCH

A-CSI on PUCCH multiplexing on PUSCH repetition Type B

If A-CSI on PUCCH is supported, it would collide with PUSCH repetition Type B. In such a case, collision handling needs to be treated properly. It can be baseline that the following Rel-16 multiplexing behavior and timeline for PUCCH multiplexed on PUSCH repetition Type B and A-CSI on PUSCH multiplexed on PUSCH repetition Type B:

- In case PUCCH overlaps with multiple repetitions of PUSCH repetition Type B that satisfy the multiplexing timeline conditions, UCI is multiplexed on only the first actual repetition.

  - The multiplexing timeline condition is the one defined in clause 9.2.5 of TS38.213.

- A-CSI on PUSCH is transmitted on the first actual repetition if it is triggered by DCI on PUSCH repetition Type B.

Fig. 2 Multiplexing behavior of PUCCH vs. PUSCH repetition Type B.

Based on the above behavior, A-CSI on PUCCH should be multiplexed on the first actual PUSCH repetition that satisfies the multiplexing timeline condition in clause 9.2.5 of TS 38.213. Note that it would be useless to repeatedly transmit A-CSI on PUCCH on PUSCH repetitions over one or more slots in DL heavy scenarios because there are few UL symbols and other UL channels also needs to be transmitted on the few symbols. Besides, A-CSI will be outdated as time goes by.

Proposal 4:

- Support A-CSI on PUCCH multiplexed on PUSCH repetition Type B

  - A-CSI on PUCCH is multiplexed on the first actual PUSCH repetition that meets the multiplexing timeline.

- Conclusion

In this contribution, we proposed following for CSI feedback enhancements.

Proposal 1:

- Support A-CSI on PUCCH triggered by DL grant including DCI format 1_1 and 1_2.

  - A new field, e.g. CSI request field, is introduced to trigger A-CSI on PUCCH

  - The DL grant can schedule PDSCH and indicate corresponding HARQ-ACK timing together with A-CSI on PUCCH triggering

Proposal 2:

- PUCCH resources for A-CSI on PUCCH are configured by RRC and which PUCCH resource to use is dynamically indicated by DCI.

  - Separate and/or joint PUCCH resource configuration and indication of HARQ-ACK can be considered.

Proposal 3:

- Support priority handling of A-CSI on PUCCH for the following UE behavior:

  - Intra-UE UL multiplexing/prioritization including Rel-17 multiplexing with different priorities

    - Priority level can be dynamically indicated by DCI

  - Overlapping CSIs in time domain in clause 5.2.5 in 38.214

    - Priority level is higher than A-CSI on PUSCH

Proposal 4:

- Support A-CSI on PUCCH multiplexed on PUSCH repetition Type B

  - A-CSI on PUCCH is multiplexed on the first actual PUSCH repetition that meets the multiplexing timeline.

References

- RP-201310, ‘Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR WID,’ Nokia, Nokia Shanghai Bell

- TS 38.213, v16.2.0

- TS 38.214, v16.2.0