3GPP TSG-RAN WG1 Meeting #102-e	R1-2006852

# e-Meeting,  August 17 – 28, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | DRAFT | rev | - | Current version: | 15.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of PTRS port association for PUSCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The existing text cannot fully reflect update for PUSCH/DM-RS precedure in TS38.211/212. Based on the updated specification, PTRS port should be associated to DMRS port but one paragraph implemented incorrectly by associating PT-RS with PUSCH layer(s). Thus, it is clarified as DMRS port instead of layer. Also, there is undefined indication of layer [x] and [y] are desribed with brace not open. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the procedure for association of PTRS and DMRS ports, - Replace layer [x] and [y] by the correct description “one of the scheduled DM-RS port(s) corresponding to the UL layers” . - Missing part of PTRS-DMRS association for configured grant is updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PTRS procedure is incomplete in the sence that gNB an UE may have different interpretations of association of PTRS port with DMRS port. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: This CR is intended for correcting the existing text with the alignement with the related text in the other specification. No functional change exists. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.2.3	UE PT-RS transmission procedure

If a UE is not configured with the higher layer parameter phaseTrackingRS in DMRS-UplinkConfig, the UE shall not transmit PT-RS. The PTRS is only present on PUSCH scheduled by PDCCH with CRC scrambled by MCS-C-RNTI, C-RNTI, CS-RNTI, SP-CSI-RNTI and on PUSCH corresponding to a configured grant.

#### 6.2.3.1	UE PT-RS transmission procedure when transform precoding is not enabled

< unchanged part omitted>

For non-codebook based UL transmission, the actual number of UL PT-RS port(s) to transmit is determined based on SRI(s) in DCI format 0_1 or higher layer parameter sri-ResourceIndicator in rrc-ConfiguredUplinkGrant. A UE is configured with the PT-RS port index for each configured SRS resource by the higher layer parameter ptrs-PortIndex configured by SRS-Config if the UE is configured with the higher layer parameter phaseTrackingRS in DMRS-UplinkConfig. If the PT-RS port index associated with different SRIs are the same, the corresponding UL DM-RS ports are associated to the one UL PT-RS port.

For partial-coherent and non-coherent codebook based UL transmission, the actual number of UL PT-RS port(s) is determined based on TPMI and/or number of layers which are indicated by Precoding information and number of layers field in DCI format 0_1 or configured by higher layer parameter precodingAndNnumberOfLayers:

-	if the UE is configured with the higher layer parameter maxNrofPorts in PTRS-UplinkConfig set to 'n2', the actual number of UL PT-RS port(s) and the associated DM-RS port(s) are derived from indicated TPMI as:

-	UL PT-RS port 0 is associated with one of the scheduled DM-RS port(s) corresponding to the UL layers which are transmitted with PUSCH antenna port 1000 and/or PUSCH antenna port 1002 in indicated TPMI, and UL PT-RS port 1 is associated with one of the scheduled DM-RS port(s) corresponding to the UL layers which are transmitted with PUSCH antenna port 1001 and/or PUSCH antenna port 1003 in indicated TPMI.

-   The DM-RS port(s) associated with UL PT-RS port 0 and/or 1 are given by DCI parameter PTRS-DMRS association or, by the association between UL PT-RS port(s) and DM-RS port(s) defined by value “00” in Table 7.3.1.1.1.2-26 if the PUSCH corresponds to a configured grant Type 1 transmission.

< unchanged part omitted>