3GPP TSG RAN WG1 Meeting #102-e	R1-2007416

eMeeting, August 17 – 28, 2020

Agenda Item:	7.2.4.2.1

Source:	Moderator (Ericsson)

Title:	TPs for 38.213 for the agreements in [102-e-NR-5G_V2X_NRSL-Mode-1-01]

Document for:	Discussion, Decision

# 1	Introduction

This document captures the TPs for 38.213 endorsed in the following e-mail discussion:

[102-e-NR-5G_V2X_NRSL-Mode-1-01] Email discussion/approval covering:

- Remaining issues for configured grant

  - Whether clarifications for the formula determining the granted slots are necessary and whether the issue should be left to RAN2.

  - Clarifications on signalling for number of retransmissions

  - Editorial corrections and clarifications for configured grant (if any).

- DCI aspects

  - Alignment of DCI format 3_0 with other DCI formats.

  - Cells on which the UE monitors DCI formats 3_0 and 3_1, including discussion on PUCCH cell.

  - Editorial corrections and clarifications for DCI (if any).

By 8/20, followed by potential TPs by 8/25 – Ricardo (Ericsson)

The single text proposal in this document captures the following agreements:

Agreements:

- DCI formats 3-0 and 3-1 are monitored on PCell or a SCell.

  - Discuss further offline whether it’s a same-carrier scheduling case only or not

    - No RRC impact is expected

- PUCCH carrying SL HARQ-ACK reports is transmitted on PCell

  - Discuss further offline the applicability or not to PUCCH Scell in case of two PUCCH groups in CA

Agreements:

- Note: It is understood that the carrier transmitting DCI formats 3_0/3_1 is configurable per the current signalling

  - Only one carrier can be configured for a UE to monitor DCI formats 3_0/3_1

  - From RAN1 perspective, no additional RRC signaling is necessary.

  - It is subject to UE capability discussion for the case when the carrier transmitting DCI formats 3_0/3_1 is different from the SL carrier

- The PUCCH group containing the carrier transmitting DCI format 3_0 is also the PUCCH group carrying SL HARQ-ACK reports (either on PCell or PUCCH SCell)

  - It is assumed that if CG type 1 is configured for CG SL HARQ-ACK reporting, it there is always a configuration of a carrier transmitting DCI format 3_0

# 2	Text Proposals

#### TP 1.2.2-1

The motivation of the TP is as follows:

- Reasons for change: clarifications on monitoring of DCI formats 3_0 and 3_1 and SL HARQ-ACK reporting.

- Summary of changes:

  - Clarifications that monitoring of DCI format 3_0 or DCI format takes place in at most one cell.

  - Clarification on how the PUCCH group is selected for SL HARQ-ACK reporting to the gNB.

- Specs/Sections impacted: TS 38.213 Clause 16

- Consequences if not approved: SL Mode 1 does not work correctly.

-------------------------- Start of Text Proposal for TS 38.213 -------------------------- <Unchanged parts omitted> # 16 UE procedures for sidelink A UE is provided by locationAndBandwidth-SL a BWP for SL transmissions (SL BWP) with numerology and resource grid determined as described in [4, TS38.211]. For a resource pool within the SL BWP, the UE is provided by numSubchannel a number of sub-channels where each sub-channel includes a number of contiguous RBs provided by subchannelsize. The first RB of the first sub-channel in the SL BWP is indicated by startRB-Subchannel. Available slots for a resource pool are provided by timeresourcepool and occur with a periodicity provided by 'periodResourcePool'. For an available slot without S-SS/PSBCH blocks, SL transmissions can start from a first symbol indicated by startSLsymbols and be within a number of consecutive symbols indicated by lengthSLsymbols. For an available slot with S-SS/PSBCH blocks, the first symbol and the number of consecutive symbols is predetermined. The UE expects to use a same numerology in the SL BWP and in an active UL BWP in a same carrier of a same cell. If the active UL BWP numerology is different than the SL BWP numerology, the SL BWP is deactivated. A UE transmitting using a Mode-1 grant uses the corresponding fields in SCI to reserve the next resource(s) allocated by the same grant. A priority of a PSSCH according to NR radio access or according to E-UTRA radio access is indicated by a priority field in a respective scheduling SCI format. A priority of a PSSS/SSSS/PSBCH according to E-UTRA radio access is provided by LTESidelinkSSBPriority. A priority of an S-SS/PSBCH block is provided by sl-SSB-PriorityNR-r16. A priority of a PSFCH is same as the priority of a corresponding PSSCH. A UE does not expect to be configured to monitor DCI format 3_0 or DCI format 3_1 in a search space in more than one cell. <Unchanged parts omitted> ## 16.5 UE procedure for reporting HARQ-ACK on uplink A UE can be provided PUCCH resources or PUSCH resources [12, TS 38.331] to report HARQ-ACK information that the UE generates based on HARQ-ACK information that the UE obtains from PSFCH receptions, or from absence of PSFCH receptions. The UE reports HARQ-ACK information on PCell or PUCCH Scell of the PUCCH group to which the cell on which the UE monitors DCI format 3_0 belongs. For SL configured grant Type 1 or Type 2 PSSCH receptions by a UE within a time period provided by periodSlCG, the UE generates one HARQ-ACK information bit in response to the PSFCH receptions to multiplex in a PUCCH transmission occasion that is after a last time resource, in a set of time resources. For PSSCH transmissions scheduled by a DCI format 3_0, a UE generates HARQ-ACK information in response to PSFCH receptions to multiplex in a PUCCH transmission occasion that is after a last time resource in a set of time resources provided by the DCI format 3_0. <Unchanged parts omitted> ------------------------------------ End of Text Proposal ------------------------------------