### Maintenance of NR-based Access to Unlicensed Spectrum

#### Physical Layer Signals and Channels

##### Initial access signals/channels

R1-2005330	Remaining issues on initial access signals and channles	vivo

R1-2005597	Remaining issues on the initial access signals for NR-U	ZTE, Sanechips

R1-2005789	Maintenance on initial access signals and channels	Huawei, HiSilicon

R1-2005904	Remaining issues on Initial Access Signals and Channels for NR-U	Nokia, Nokia Shanghai Bell

R1-2005910	Initial access signals and channels	Ericsson

R1-2006017	Discussion on the remaining issues of initial access signal/channel	OPPO

R1-2006092	Initial access signals and channels for NR-U	Samsung

R1-2006288	Remaining issues on initial access signals	Spreadtrum Communications

R1-2006298	Remaining issues of initial access signals and channels for NR-U	LG Electronics

[102-e-NR-unlic-NRU-InitSignalsChannels] Email discussion/approval on the following until 8/20; if necessary, endorse associated TPs by 8/25 – Jing (Qualcomm)

- PRACH configuration in multiple RB sets (Issue 4.4 in FL summary)

- Msg A PUSCH RB-set determination (Issue 4.5 in FL summary)

##### DL signals and channels

R1-2006875	Feature lead summary for NR-U DL Signals and Channels	Moderator (Lenovo)

R1-2005331	Remaining issues on physical DL channel design in unlicensed spectrum	vivo

R1-2005598	Remaining issues on the DL channels for NR-U	ZTE, Sanechips

R1-2005807	Maintenance on DL signals and channels	Huawei, HiSilicon

R1-2005844	DL signals and channels for NR-unlicensed	Intel Corporation

R1-2005905	Remaining issues on DL signals and channels	Nokia, Nokia Shanghai Bell

R1-2005911	DL signals and channels	Ericsson

R1-2006018	Discussion on the remaining issues of DL signals and channels	OPPO

R1-2006093	DL signals and channels for NR-U	Samsung

R1-2006273	Remaining issues in DL signals and channels for NR-U	Spreadtrum Communications

R1-2006299	Remaining issues of DL signals and channels for NR-U	LG Electronics

R1-2006350	Remaining issues on DL signals and channels for NR-U	ETRI

R1-2006483	Remaining issues of DL signal and channels	Apple

R1-2006553	Remaining issues on DL signals/channels for NR-U	Sharp

R1-2006836	TP for DL signals and channels for NR-U	Qualcomm Incorporated

R1-2006862	Maintenance for DL signals and channels for NR-U	Panasonic Corporation

[102-e-NR-unlic-NRU-DL_Signals_and_Channels-01] Email discussion/approval on Search Space including the following issues from R1-2006875 until 8/20; if necessary, endorse associated TPs by 8/26 – Alex (Lenovo)

- SFI (+other fields) presence configurability in DCI format 2_0 (Issue B5)

- General Slot Format determination and corresponding UE behaviour (Issue B11)

- Special states/indications in "available RB set indication" (Issue B1)

[102-e-NR-unlic-NRU-DL_Signals_and_Channels-02] Email discussion/approval on Search Space including the following issues from R1-2006875 until 8/21; if necessary, endorse associated TPs by 8/27 – Alex (Lenovo)

- Search space set switching behaviour (Issue A5)

- CSI-RS transmission power, measurements, validity/presence of periodic/semi-persistent CSI-RS (Issue D1)

[102-e-NR-unlic-NRU-DL_Signals_and_Channels-03] Email discussion/approval of TPs to capture earlier agreements and align specifications on the following until 8/20– Alex (Lenovo)

- IE Names in DCI 2_0 (Issue Z1)

- Capturing RRC parameter subcarrierSpacing-r16 (Issue Z2)

- Parameter correction for CSI-RS reception (Issue Z3)

- Processing time for switching (Issue Z4)

Capture endorsement of TP#3 in R1-2005598 referring to Alex’s final document when it is ready along with cover page information.

Capture endorsement of TP#7 in R1-2006553 referring to Alex’s final document when it is ready along with cover page information.

[102-e-NR-unlic-NRU-DL_Signals_and_Channels-04] Email discussion/approval of reply LS for R1-2005220 by 08/20 – Alex (Lenovo)

Option 1: If UE cannot transmit HARQ-ACK on MAC-CE deactivation due to UL CCA failure, UE continues to be in its previous state, i.e., it should measure and report L1-RSRP until it successfully transmits HARQ-ACK

Option 4: For semi-persistent CSI reporting with PUCCH, if UE cannot transmit HARQ-ACK on the MAC CE deactivation due to the UL LBT failure, the UE performs deactivation at the original MAC action time.

##### UL signals and channels

R1-2005913	Feature Lead Summary for UL Signals and Channels	Ericsson

R1-2005332	Remaining issues on physical UL channel design in unlicensed spectrum	vivo

R1-2005599	Remaining issues on the UL channels for NR-U	ZTE, Sanechips

R1-2005808	Maintenance on UL signals and channels	Huawei, HiSilicon

R1-2005826	Text proposals for UL signals and channels for NR-U	Lenovo, Motorola Mobility

R1-2005912	UL signals and channels	Ericsson

R1-2006019	Discussion on the remaining issues of UL signals and channels	OPPO

R1-2006094	UL signals and channels for NR-U	Samsung

R1-2006300	Remaining issues of UL signals and channels for NR-U	LG Electronics

R1-2006371	Remaining Issues on UL Signals & Channels for NR-U 	Nokia, Nokia Shanghai Bell

R1-2006554	Remaining issues on UL signals/channels for NR-U	Sharp

R1-2007108

[102-e-NR-unlic-NRU-ULSignalsChannels] Email discussion/approval on the following from R1-2005913 until 8/19; if necessary, endorse remaining TPs by 8/25 – Steve (Ericsson)

- Issue #1: Corrections for frequency domain resource allocation for PUSCH scheduled by RAR UL grant and DCI 0_0 addressed to TC-RNTI when UL resource allocation Type 2 (Interlacing) is configured

- Issue #3: Addition of UL Resource Allocation Type 2 to definition of special values in DCI 0_1 for semi-persistent CSI deactivation

- Issue #5: Editorial corrections to 38.212, 38.213, 38.214

- Issue #9: Discuss clarifications on UCI multiplexing in PUSCH accounting for LBT outcome and whether or not a conclusion is needed

Agreement:

TP#1, TP#3a, TP#5a, TP#6 in R1-200xxxx (FL summary v021-Moderator) are endorsed for TS 38.214

Agreement:

TP#2a in R1-200xxxx (FL summary v021-Moderator) are endorsed for TS 38.211

Agreement:

TP#7 in R1-200xxxx (FL summary v021-Moderator) are endorsed for TS 38.213

Agreement:

TP#8 in R1-200xxxx (FL summary v021-Moderator) is endorsed for TS 38.212

Agreement:

TP#4 in R1-2007108 in Section 3 is endorsed for TS 38.213 with a change of “clause 7” to “clause 7 of TS38.214”

Conclusion:

For operation with shared spectrum channel access, it is a common understanding that when UE performs UCI multiplexing on PUSCH or PUCCH, that the multiplexing procedure is not dependent on the outcome of the channel access procedure corresponding to the PUSCH or PUCCH transmission.

#### Physical Layer Procedure

##### Channel access procedures

R1-2006675	Feature Lead Summary on Channel Access Procedures for NR-U	Moderator (Nokia)

R1-2005333	Remaining issues on the channel access procedures	vivo

R1-2005600	Remaining issues on the channel access procedure for NR-U	ZTE, Sanechips

R1-2005809	Maintenance on channel access procedures	Huawei, HiSilicon

R1-2005914	Channel access procedures	Ericsson

R1-2006020	Discussion on the remaining issues of channel access procedure	OPPO

R1-2006095	Channel access procedures for NR-U	Samsung

R1-2006301	Remaining issues of channel access procedure for NR-U	LG Electronics

R1-2006351	Remaining issues on channel access procedures for NR-U	ETRI

R1-2006370	Remaining Issues on Channel Access Procedures for NR-U	Nokia, Nokia Shanghai Bell

R1-2006763	TP for Channel access procedures for NR unlicensed	Qualcomm Incorporated

R1-2006881	Remaining issues on channel access procedure for NR-U	WILUS Inc.

[102-e-NR-unlic-NRU-ChAcc-01] Email discussion/approval on the following from R1-2006675 by 8/20; if necessary, endorse associated TPs by 8/26 – Timo (Nokia)

- Issue#1: Indication of LBT type, CP extension and CAPC; N1 timeline for UL transmissions with CP extension

- Issue#4: Clarifications to channel access for semi-static channel occupancy

- Issue#5: DL and UL Channel Access related

- Issue#6: Multi-channel Channel Access

Agreement:

The first one of the two non-consecutive UL transmissions scheduled with a single DCI (regardless of whether it is SRS or PUSCH/PUCCH) follows the indicated CP extension and the LBT type.

[102-e-NR-unlic-NRU-ChAcc-02] Email discussion/approval of TPs to capture earlier agreements and align specifications on clarifications to UL to DL COT sharing (Issue#3 in R1-2006675) until 8/19 – Timo (Nokia)

Agreement:

The text proposal in Section 3 of R1-2007291 for Clause 4.2.3 of TS 37.213 is endorsed.

Agreement:

The text proposal in Proposal#1 in Section 7 of R1-2007267 for Clause 4.2.1 of TS 37.213 is endorsed.

Agreement:

The text proposals in Proposal#2 and Proposal#3 in Section 7 of R1-2007267 for Clause 4.3 of TS 37.213 are endorsed.

Conclusion:

For a discovery burst, its duty cycle is defined as D_DB/P_DB, wherein D_DB is the duration of the discovery burst and determined according to the transmission starting instance of the discovery burst, and P_DB is determined as the periodicity of the discovery burst transmission window, as described in subclause 4.1 of TS 38.213.

Agreement:

The text proposal in Proposal#5 in Section 7 of R1-2007267 for Clause 4.1.4.2 of TS 37.213 is endorsed.

Agreement:

The text proposals in Proposal#6 and Proposal#7 in Section 7 of R1-2007267 for Clause 4.2.2.2 of TS 37.213 are endorsed.

Agreement:

The text proposal in Proposal#8 in Section 7 of R1-2007267 for Clauses 4.1.6.1.1 and 4.1.6.1.2 of TS 37.213 is endorsed.

Proposal:

Capture the statement “the absence of any other technology sharing a channel can be guaranteed on a long-term basis (e.g. by level of regulation)” in Section 4.3 of 37.213 or statements from the WID in the Chairman’s notes or CR cover sheet.

##### Enhancements to initial access procedure

R1-2006648	Feature lead summary of enhancements to initial access procedures	Moderator (Charter Communications)

R1-2005334	Remaining issues on initial access procedure for NR-U	vivo

R1-2005601	Remaining issues on the initial access procedure for NR-U	ZTE, Sanechips

R1-2005756	TPs on DCI reserved bits length in NR-U	NEC

R1-2005810	Maintenance on initial access procedures	Huawei, HiSilicon

R1-2005915	Enhancements to initial access procedures	Ericsson

R1-2006021	Discussion on the remaining issues of enhancements to initial access procedure	OPPO

R1-2006096	Initial access procedures for NR-U	Samsung

R1-2006449	On Enhancements to Initial Access Procedures for NR-U	Nokia, Nokia Shanghai Bell

R1-2006764	TP for Initial access and mobility procedures for NR-U	Qualcomm Incorporated

[102-e-NR-unlic-NRU-InitAccessProc-01] Email discussion/approval on the following from R1-2006648 by 8/20; if necessary, endorse associated TPs by 8/26 – Amitav (Charter)

- (#2.5) Timing reference cell update to 38.213.

- (#3.1, #3.3) DCI Format 1_0 bit-length alignment. Reflect LSB of SFN is not always in the DCI 1_0 when RAR/msgB window is smaller than 10ms.

- (#4.1) Further clarifications on CSI-RS measurement averaging.

- Proposed conclusions (no spec impact):

  - For FBE mode, idle UE may transmit PRACH only within FFPs for which SSB, SIBs or paging are detected.

  - There is no requirement on the difference between the first and last candidate SS/PBCH block index of the SS/PBCH block burst within a discovery burst transmission window.

    - Note: This implies gNB can transmit multiple bursts of SS/PBCH blocks within a discovery burst transmission window.

- If necessary, reply LS from RAN1 to RAN4 is necessary based on R1-2003271 and R1-2005213

Conclusion:

For semi-static channel access, an idle UE may transmit PRACH only within FFPs for which SSB, SIBs or paging transmissions are detected.

Conclusion:

There is no requirement on the difference between the first and last candidate SS/PBCH block index of the SS/PBCH block burst within a discovery burst transmission window.

- Note: This implies gNB can transmit multiple bursts of SS/PBCH blocks within a discovery burst transmission window.

TP#3c and TP#5a are agreed. Amitav to provide Tdoc number with cover page info.

##### HARQ enhancement

R1-2005335	Remaining issues on HARQ operation for NR-U	vivo

R1-2005602	Remaining issues on the HARQ for NR-U	ZTE, Sanechips

R1-2005811	Maintenance on HARQ-ACK enhancement	Huawei, HiSilicon

R1-2005827	Text proposals for HARQ enhancement for NR-U	Lenovo, Motorola Mobility

R1-2005845	Enhancements to HARQ for NR-unlicensed	Intel Corporation

R1-2005907	Remaining issues on NR-U HARQ scheduling and feedback	Nokia, Nokia Shanghai Bell

R1-2005916	HARQ enhancement	Ericsson

R1-2006022	Discussion on the remaining issues of HARQ enhancements	OPPO

R1-2006097	HARQ enhancement for NR-U	Samsung

R1-2006302	Remaining issues of HARQ procedure for NR-U	LG Electronics

R1-2006555	Remaining issues and corrections on HARQ enhancement for NR-U	Sharp

R1-2006765	TP for Enhancements to Scheduling and HARQ Operation for NR-U	Qualcomm Incorporated

R1-2006900	Text proposal for enhanced dynamic HARQ procedures	Google, Inc.

[102-e-NR-unlic-NRU-HARQ-01] Email discussion/approval on the following from R1-2006983 until 8/20; if necessary, endorse associated TPs by 8/26 – David (Huawei)

- Correction on Type-3 HARQ-ACK codebook (issue B18): Clarification of UCI multiplexing timeline based on a request for a Type-3 HARQ-ACK codebook report without scheduling a PDSCH in the UE procedure for reporting multiple UCI types.

Agreement:

The text proposal for TS38.213 in R1-2007101 is endorsed.

[102-e-NR-unlic-NRU-HARQ-02] TPs for the following from R1-2006983 until 8/19; if necessary, endorse associated TPs by 8/25 – David (Huawei)

- Correction on multi-PUSCH scheduling (issue D1): correct ambiguity on TDRA table for DCI format 0_1 when multiple PUSCH scheduling is configured to the UE. Correction to include pusch-TimeDomainAllocationListForMultiPUSCH-r16 in TS38.214 Table 6.1.2.1.1-1A, and potentially also a TP for 38.212 clause 7.3.1.1.2.

- Discussion on out-of-order PDSCH-to-HARQ issues (C3, C4): in case of HARQ-ACK information retransmission and in case of scheduling with NNK1, aim to at least clarify which cases (C3, C4-case1 and C4-case2) are OOO and which cases are not OOO. Cases may be redefined during the discussion. It is TBD whether any correction would be needed after that clarification

Agreement:

The text proposals in R1-2007102 for Clause 7.3.1.1.2 of TS 38.212 and Clause 6.1.2.1.1 of TS 38.214 are endorsed.

##### Configured grant enhancement

R1-2005337	Feature lead summary on NRU configured grant enhancement	vivo

R1-2005336	Remaining issues on the enhancements to configured grant	vivo

R1-2005603	Remaining issues on the configured grant for NR-U	ZTE, Sanechips

R1-2005812	Maintenance on the configured grant procedures	Huawei, HiSilicon

R1-2005828	Text proposals for configured grant enhancement for NR-U	Lenovo, Motorola Mobility

R1-2005917	Configured grant enhancement	Ericsson

R1-2006023	Discussion on the remaining issues of configured grant enhancements	OPPO

R1-2006098	Configured grant enhancement for NR-U	Samsung

R1-2006303	Remaining issues of configured grant for NR-U	LG Electronics

R1-2006766	TP for Enhancements to configured grants for NR-U	Qualcomm Incorporated

R1-2006866	Remaining issues for DFI in NR-U	ASUSTeK

[102-e-NR-unlic-NRU-CG] Email discussion/approval on the following from R1-2005337 until 8/20; if necessary, endorse any remaining TPs by 8/26 – Rakesh (Vivo)

- RV ID (Issue 1)

- CG-PUSCH repetition (Issue 3)

- TPs for DFI related issues (Issue 4)

- NDI field for validation activation/release (TP#1 of Issue 6)

Capture agreements for below once final Tdoc number is available:

On issue 1, I proposed to adopt option 1 as no one raised concern since last 24 hours.

On issue 3, I proposed to adopt revised TP from Ericsson as no one raised concern since last 24 hours.

On issue 4, the proposal is still to agree on option1. I see there are few comments to either support option2 or 4 however majority of companies support option1. I would encourage companies with concern to raise when this AI is treated.

On issue 6, I proposed to agree TP#1 as no one raised concern since last 24 hours

##### Wide-band operation

R1-2005538	Remaining issue on wideband operation	Fujitsu

R1-2005604	Remaining issues on the wideband operation for NR-U	ZTE, Sanechips

R1-2005813	Maintenance on the wideband operation procedures	Huawei, HiSilicon

R1-2005829	Text proposals for wideband operation for NR-U	Lenovo, Motorola Mobility

R1-2005906	Remaining issues on Wideband operation in NR-U	Nokia, Nokia Shanghai Bell

R1-2005918	Wideband operation	Ericsson

R1-2006024	Discussion on the remaining issues of wide-band operations	OPPO

R1-2006556	Remaining corrections for wideband operation for NR-U	Sharp

R1-2006767	TP for Wideband operation for NR-U operation	Qualcomm Incorporated

[102-e-NR-unlic-NRU-WB] Email discussion/approval on the following until 8/19; if necessary, endorse remaining TPs by 8/25 – Seonwook (LGE)

- Editorial changes and discussion on whether/how to reflect previous RAN1 conclusion for CORESET misconfiguration

  - RRC parameter name alignment (freqMonitorLocations-r16, intraCellGuardBandsDL-r16, intraCellGuardBandsUL-r16) (Issue 8 in FL summary)

  - Whether/how to clarify indices of RB set and GB in TS 38.214 Section 7 (Issue 8 in FL summary)

  - Whether/how to reflect previous RAN1 conclusion for CORESET mis-configuration (Issue 7 in FL summary)

Agreement:

TP#1 in Section 4 of R1-200xxxx for Clause 10.1 of TS 38.213 is endorsed.

Agreement:

TP#2 in Section 4 of R1-200xxxx for Clause 7 of TS 38.214 is endorsed.

#### Others

R1-2005919	A harmonized energy detection threshold for 6 GHz license-exempt spectrum	Ericsson

R1-2006025	Discussion on the other remaining issues	OPPO

R1-2006409	Discussion on the basic feature groups for NRU	Huawei, HiSilicon