3GPP TSG RAN WG1 Meeting #102-e

e-Meeting, August 17th – 28th, 2020

Title:	Session notes for 7.2.5

(Maintenance of Physical Layer Enhancements for NR URLLC)

Source:	Ad-hoc chair (Samsung)

### Maintenance of Physical Layer Enhancements for NR URLLC

Including maintenance for IIoT

#### PDCCH enhancements

R1-2005347	PDCCH enhancements for URLLC	vivo

R1-2005413	Remaining issues on PDCCH enhancements for NR URLLC	ZTE

R1-2005506	Remaining Issue of PDCCH Enhancements for NR URLLC	Ericsson

R1-2005672	Remaining issues on PDCCH enhancements	CATT

R1-2005790	Corrections on PDCCH enhancement for URLLC	Huawei, HiSilicon

R1-2005850	Remaining corrections for PDCCH enhancements for URLLC	Intel Corporation

R1-2006051	PDCCH enhancements for URLLC	OPPO

R1-2006109	Maintenance on PDCCH enhancements	Samsung

R1-2006278	Remaining issues of PDCCH enhancements for URLLC	Spreadtrum Communications

R1-2006487	Remaining issues on  PDCCH enhancements	Apple

R1-2006549	Remaining Issues on PDCCH Enhancements for Rel-16 URLLC	Quectel

R1-2006563	Remaining issues on PDCCH enhancements for NR URLLC	Sharp

R1-2006774	Remaining issues on PDCCH Enhancements for URLLC	Qualcomm Incorporated

R1-2006865	Remaining issue for TCI field	ASUSTeK

R1-2006882	Remaining issues on PDCCH for NR URLLC	WILUS Inc.

R1-2006968	Feature lead summary#1 on PDCCH enhancements	Moderator (Huawei)

R1-2007004	Feature lead summary#2 on PDCCH enhancements	Moderator (Huawei)

[102-e-NR-L1enh-URLLC-PDCCH enhancements-01] Email discussion/approval on remaining issues on DCI format design – Chengyan (Huawei)

- Issue A-1: Remaining issue on DCI size alignment in TS 38.212

- Issue A-2: Type2 HARQ-ACK codebook construction related to DAI bit width

- Discussion/Agreement by 8/21 and TPs by 8/28

[102-e-NR-L1enh-URLLC-PDCCH enhancements-02] Email discussion/approval on remaining issues on enhanced PDCCH monitoring capability – Chengyan (Huawei)

- Issue B-1: Corrections on span definition

- Issue B-2: Corrections on “aligned spans” case

- Issue B-5-3 & B-5-4 & B-5-6: Miscellaneous editorial corrections

- Issue B-5-2: PDCCH monitoring within a slot

- Discussion/Agreement by 8/21 and TPs by 8/28

R1-2007066	Summary #1 of email discussion [102-e-NR-L1enh-URLLC-PDCCH enhancements-01] on remaining issues on enhanced PDCCH monitoring capability	Moderator (Huawei)

Comeback

Proposal 2-2 (in section 3) for R1-2007066: Endorse the following text proposal in R1-2xxxxxx for TS 38.212 Section 7.3.1.0.

- Note: Qualcomm expressed concern on taking this alternative

R1-2007067	Summary #1 of email discussion [102-e-NR-L1enh-URLLC-PDCCH enhancements-02] on remaining issues on enhanced PDCCH monitoring capability	Moderator (Huawei)

Comeback

(Revised Proposal 2.1-1) TP for Issue B-1: Corrections on span definition

#### UCI enhancements

R1-2005348	UCI enhancements for URLLC	vivo

R1-2005414	Remaining issues on UL control enhancements for NR URLLC	ZTE

R1-2005507	Remaining Issue of UCI Enhancements for NR URLLC	Ericsson

R1-2005541	Remaining issues on UCI enhancements for URLLC	Fujitsu

R1-2005549	Maintenance of Rel-16 URLLC UCI enhancements	Nokia, Nokia Shanghai Bell

R1-2005673	Remaining issues on UCI enhancements	CATT

R1-2005792	Corrections on UCI enhancement	Huawei, HiSilicon

R1-2005851	Remaining details on UCI enhancements for URLLC	Intel Corporation

R1-2006052	UCI enhancements for URLLC	OPPO

R1-2006110	Maintenance on UCI enhancements	Samsung

R1-2006279	Remaining issues of UCI enhancements for URLLC	Spreadtrum Communications

R1-2006472	UCI enhancements for URLLC	InterDigital, Inc.

R1-2006488	Remaining Issues on UCI Enhancements for eURLLC	Apple

R1-2006653	Remaining issue on the HARQ-ACK/PUSCH priority	ITRI

R1-2006697	Maintainance for UCI enhancement for Rel-16 URLLC	NTT DOCOMO, INC.

R1-2006775	Remaining issues on UCI Enhancements for URLLC	Qualcomm Incorporated

[102-e-NR-L1enh-URLLC-UCI_Enh-01] PHY priority in special cases – Jia (OPPO)

- Issue 4.2.1-4.2.4 in FL’s summary (For 4.2.1, not to repeat discussions from last meeting. Try to find a simple resolution quickly.)

- Discussion/Agreement by 8/21, TPs by 8/28

[102-e-NR-L1enh-URLLC-UCI_Enh-02] Misc corrections/clarifications – Jia (OPPO)

- Corrections reflecting previous agreements: Issue 2.3.1, 3.3.1, 5.2 from FL’s summary

- Clarify if Type 1 HARQ-ACK codebook or PUCCH repetition is supported for sub-slot: Issue 2.2.1, 2.3.4 from FL’s summary

- Other corrections: Issue 2.3.3(incl. SR) from FL’s summary

- Discussion/Agreement by 8/21, TPs by 8/28

R1-2007053	Summary#1 of email thread [102-e-NR-L1enh_URLLC-UCI_Enh-01]	Moderator (OPPO)

Conclusion

- A UE not indicating the support of [FG11-4b] and being configured for monitoring of DCI format 1_1 and 1_2 in a BWP is not expected to be configured with neither of PriorityIndicator-ForDCIFormat1_1 and PriorityIndicator-ForDCIFormat1_2 for that BWP of a serving cell.

- A UE not indicating the support of [FG12-1a] and being configured for monitoring of DCI format 0_1 and 0_2 in a BWP is not expected to be configured with neither of PriorityIndicator-ForDCIFormat0_1 and PriorityIndicator-ForDCIFormat0_2 for that BWP of a serving cell.

- No specification impacts.

Conclusion

Other than what is currently specified in TS38.213, there is no consensus on the default priority if priority field in DCI is not configured.

R1-2007054	Summary#1 of email thread [102-e-NR-L1enh_URLLC-UCI_Enh-02]	Moderator (OPPO)

Possible Agreement

The following text proposal is endorsed as part of editor’s CR on TS38.213.

---------------------------------------- Start of TP 38.213 V16.2.0 section 9-----------------------------------

9             UE procedure for reporting control information

<Unchanged parts are omitted>

A PUSCH or a PUCCH transmission, including repetitions if any, can be of priority index 0 or of priority index 1. For a configured grant PUSCH transmission, a UE determines a priority index from priority, if provided. For a PUCCH transmission with HARQ-ACK information corresponding to a SPS PDSCH reception or a SPS PDSCH release, a UE determines a priority index from harq-CodebookID, if provided. For a PUSCH transmission with semi-persistent CSI reporting, a UE determines a priority index from a priority indicator field, if provided, in a DCI format 0_1 or DCI format 0_2 which activates the semi-persistent CSI reporting. If a priority index is not provided to a UE for a PUSCH or a PUCCH transmission, the priority index is 0.

If in an active DL BWP a UE monitors PDCCH either for detection of DCI format 0_1 and DCI format 1_1 or for detection of DCI format 0_2 and DCI format 1_2, a priority index can be provided by a priority indicator field. If a UE indicates a capability to monitor, in an active DL BWP, PDCCH for detection of DCI format 0_1 and DCI format 1_1 and for detection of DCI format 0_2 and DCI format 1_2, a DCI format 0_1 or a DCI format 0_2 can schedule a PUSCH transmission of any priority and a DCI format 1_1 or a DCI format 1_2 can schedule a PDSCH reception and trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority. When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes, the UE first resolves the overlapping for PUCCH and/or PUSCH transmissions of a same priority index. Then, if the UE determines to transmit

---------------------------------------- End of TP 38.213 V16.2.0 section 9-----------------------------------

R1-2007054

Comeback: Text refinement might be necessary

-------------------------------------------------- Start of text proposal ------------------------------------------------------

9.2.1 PUCCH Resource Sets

*** Unchanged text is omitted ***

If the format indicates PUCCH-format0, the PUCCH format configured for a PUCCH resource is PUCCH format 0, where the PUCCH resource also includes an index for an initial cyclic shift provided by initialCyclicShift, a number of symbols for a PUCCH transmission provided by nrofSymbols, a first symbol for the PUCCH transmission provided by startingSymbolIndex.

If the format indicates PUCCH-format1, the PUCCH format configured for a PUCCH resource is PUCCH format 1, where the PUCCH resource also includes an index for an initial cyclic shift provided by initialCyclicShift, a number of symbols for a PUCCH transmission provided by nrofSymbols, a first symbol for the PUCCH transmission provided by startingSymbolIndex, and an index for an orthogonal cover code by timeDomainOCC.

If the format indicates PUCCH-format2 or PUCCH-format3, the PUCCH format configured for a PUCCH resource is PUCCH format 2 or PUCCH format 3, respectively, where the PUCCH resource also includes a number of PRBs provided by nrofPRBs, a number of symbols for a PUCCH transmission provided by nrofSymbols, and a first symbol for the PUCCH transmission provided by startingSymbolIndex. If a UE is provided by useInterlacePUCCH-PUSCH in BWP-UplinkDedicated, and the format indicates PUCCH-format2-r16 or PUCCH-format3-r16, the PUCCH resource also includes, if provided, an index of a second interlace by interlace1. If format indicatesPUCCH-format2-r16 or PUCCH-format3-r16 and interlace1 is not provided, the PUCCH resource also includes, if provided, an orthogonal cover code length by OCC-Length-r16 and an orthogonal cover code index by OCC-Index-r16. If the format indicates PUCCH-format3-r16, the UE assumes that the  [4, TS38.211] PRBs with the lowest indexes within the first, and if configured, second interlace are used for PUCCH transmission.

If the format indicates PUCCH-format4, the PUCCH format configured for a PUCCH resource is PUCCH format 4, where the PUCCH resource also includes a number of symbols for a PUCCH transmission provided by nrofSymbols, an orthogonal cover code length by occ-Length, an orthogonal cover code index by occ-Index, and a first symbol for the PUCCH transmission provided by startingSymbolIndex.

If a UE is provided subslotLength-ForPUCCH, the first symbol of a PUCCH resource for CSI or SR is relative to the first symbol of a slot with  symbols.

A UE can be configured up to four sets of PUCCH resources. A PUCCH resource set is provided by PUCCH-ResourceSet and is associated with a PUCCH resource set index provided by pucch-ResourceSetId, with a set of PUCCH resource indexes provided by resourceList that provides a set of pucch-ResourceId used in the PUCCH resource set, and with a maximum number of UCI information bits the UE can transmit using a PUCCH resource in the PUCCH resource set provided by maxPayloadSize. For the first PUCCH resource set, the maximum number of UCI information bits is 2. A maximum number of PUCCH resource indexes for a set of PUCCH resources is provided by maxNrofPUCCH-ResourcesPerSet. The maximum number of PUCCH resources in the first PUCCH resource set is 32 and the maximum number of PUCCH resources in the other PUCCH resource sets is 8.

----------------------------------------------------- End of text proposal ------------------------------------------------------

Agreement

The following text proposal is endorsed as part of editor’s CR on TS38.213.

--------------------------------- Start of Proposed Text 1 for TS 38.213 -------------------------------

9	UE procedure for reporting control information

A PUSCH or a PUCCH transmission, including repetitions if any, can be of priority index 0 or of priority index 1. For a configured grant PUSCH transmission, a UE determines a priority index from priority, if provided. For a PUCCH transmission with HARQ-ACK information corresponding to a SPS PDSCH reception or a SPS PDSCH release, a UE determines a priority index from harq-CodebookID, if provided. For a PUCCH transmission with SR, a UE determines the corresponding priority as described in subclause 9.2.4. If a priority index is not provided to a UE for a PUSCH or a PUCCH transmission, the priority index is 0.

If a PUCCH-Config is provided with a subslotLengthFor PUCCH, the PUCCH resource corresponding to any SR or CSI configuration with the same priority as the PUCCH-Config, should be confined within the sub-slot associated to the PUCCH-Config.

--------------------------------- End of Proposed Text 1 for TS 38.213 -------------------------------

#### PUSCH enhancements

R1-2006490	Feature lead summary #1 on PUSCH enhancements for NR eURLLC (AI 7.2.5.3)	Moderator (Apple)

R1-2005349	PUSCH enhancements for URLLC	vivo

R1-2005415	Remaining issues on PUSCH enhancements for NR URLLC	ZTE

R1-2005508	Remaining Issue of PUSCH Enhancements for NR URLLC	Ericsson

R1-2005674	Remaining issues on PUSCH enhancements	CATT

R1-2005793	Corrections on PUSCH enhancement	Huawei, HiSilicon

R1-2006053	PUSCH enhancements for URLLC	OPPO

R1-2006111	Remaining issues for PUSCH enhancement	Samsung

R1-2006489	Remaining Issues on PUSCH enhancements for eURLLC	Apple

R1-2006776	Remaining issues on PUSCH enhancements for URLLC	Qualcomm Incorporated

R1-2006883	Remaining issues on PUSCH enhancement for NR URLLC	WILUS Inc.

R1-2006992	Feature lead summary #2 on PUSCH enhancements for NR eURLLC (AI 7.2.5.3)	Moderator (Apple)

[102-e-NR-L1enh-URLLC-PUSCH_Enh-01] Remaining issues on URLLC PUSCH – Sigen (Apple)

- Issue #1: Optimization regarding numberInvallidSymbolsForDL-UL-Switching

- Issue #2: Peak rate restriction

- Issue #3: Number of MIMO layers

- Discussions/Agreements by 8/21, TPs by 8/28

R1-2007025	Summary #1 of Email Discussion [102-e-NR-L1enh-URLLC-PUSCH_Enh-01]	Moderator (Apple)

Conclusion

No further optimization regarding the handling of the invalid symbols provided by numberInvallidSymbolsForDL-UL-Switching is considered in Rel-16.

Conclusion

For PUSCH repetition Type B, there is no restriction on the number of transmission layers supported.

#### Enhancements to scheduling/HARQ

Including aspects related to intra-UE prioritization/cancellation

R1-2005350	Enhancement for Scheduling/HARQ	vivo

R1-2005416	Remaining issues on scheduling/HARQ enhancements for NR URLLC	ZTE

R1-2005509	Remaining Issue of Scheduling/HARQ for NR URLLC	Ericsson

R1-2005550	Maintenance of Rel-16 URLLC intra-UE prioritization enhancements	Nokia, Nokia Shanghai Bell

R1-2005632	On the retransmission of a cancelled UL TB	MediaTek Inc.

R1-2005675	Remaining issues on intra-UE prioritization	CATT

R1-2005852	Remaining details on Scheduling & HARQ enhancements for URLLC	Intel Corporation

R1-2006054	Enhancements to scheduling and HARQ	OPPO

R1-2006112	Maintenance on scheduling/HARQ enhancements	Samsung

R1-2006280	Remaining issues of enhancements to scheduling/HARQ	Spreadtrum Communications

R1-2006294	Remaining issues of Enhancements to scheduling/HARQ for NR URLLC	LG Electronics

R1-2006389	Corrections on operation of HARQ	Huawei, HiSilicon

R1-2006491	TB CRC Handling in case of Cancellation of Initial Transmission in eURLLC	Apple

R1-2006633	Remaining issues on scheduling/HARQ enhancements	Asia Pacific Telecom co. Ltd

R1-2006698	Maintenance for scheduling/HARQ for Rel-16 URLLC	NTT DOCOMO, INC.

R1-2006777	Remaining issues on HARQ and scheduling for URLLC	Qualcomm Incorporated

[102-e-NR-L1enh-URLLC-Scheduling and HARQ-01] Prioritization order involving semi-static DL symbols, SSB and dynamic SFI – Kianoush (Qualcomm)

- Processing order between semi-static TDD configuration and intra-UE prioritization

  - Order of intra-UE prioritization and cancellation due to collision with configured DL symbols and SSB

  - Clarification on the 3-step UE behavior and UE behavior in case there is collision after step 3.

- Processing order between dynamic SFI and intra-UE prioritization

  - Proposed agreement:

    - UE behavior of handling intra-UE prioritization/multiplexing for overlapping UL transmissions on semi-static flexible symbols is not affected by UL cancellation due to dynamic SFI or DL grant.

    - Note that the proposed agreement is to start the discussions.

- Discussions/Agreements by 8/21, TPs by 8/28

[102-e-NR-L1enh-URLLC-Scheduling and HARQ-02] Processing order of intra-UE prioritization and multiplexing and handling CBG-based PUSCH retransmission – Kianoush (Qualcomm)

- Whether/how to perform multiplexing/PUCCH overriding for the LP channels in case of collision.

  - Starting point for the discussion:

    - LP multiplexing is performed similar to Rel. 15 as if HP channels do not exist; this means that LP operations, multiplexing/overriding/etc, are performed before cancellation.

    - HP multiplexing is performed independently.

    - Similar to the current spec., any HP DCI can trigger a cancellation.

- Re-transmission of the CBG-based PUSCH with cancellation

  - Option 1: the UE is not expected to be scheduled for a re-transmission of the TB including the last CBG if each of the other CBGs (except for the last one) have either not been transmitted at least once before or are not scheduled for a re-transmission in the same UL grant as the last CBG.

  - Option 1a: The UE is not expected to be scheduled for a re-transmission of a CBG #N in a given TB unless CBG #N-1 has been transmitted

  - before or is scheduled in the same UL grant that includes CBG#N.

  - Option 2: the TB CRC for the retransmission of the same TB is set to all zeros.

  - Option 3: It is up to UE implementation to determine which values to use as the TB CRC (which may not be the actual TB CRC) for the retransmission of the same TB.

  - Option 4: the minimum processing time for PUSCH scheduled for re-transmission is extended by D symbols.

  - Option 5: The UE is not expected to be scheduled with partial TB for the retransmission.

- Discussions/Agreements by 8/21, TPs by 8/28

R1-2007034

R1-2007035

Working Assumption

- UE behavior of handling intra-UE prioritization/multiplexing for overlapping UL transmissions on semi-static flexible symbols is not affected by UL cancellation due to dynamic SFI or [DL grant]

- Note: The UE performs prioritization/multiplexing first and once done applies dynamic SFI

#### Enhanced inter UE Tx prioritization/multiplexing

R1-2005351	UL inter UE Tx prioritization for URLLC	vivo

R1-2005417	Remaining issues on UL inter-UE multiplexing between eMBB and URLLC	ZTE

R1-2005510	Remaining Issue of Inter-UE Prioritization and Multiplexing of  UL Transmissions	Ericsson

R1-2005676	Remaining issues on inter-UE UL multiplexing	CATT

R1-2005791	Corrections on UL inter-UE multiplexing	Huawei, HiSilicon

R1-2006055	Inter UE Tx prioritization and multiplexing	OPPO

R1-2006113	Maintenance on inter-UE multiplexing	Samsung

R1-2006355	Enhanced inter UE Tx prioritization/multiplexing	ETRI

R1-2006492	Remaining Issues on Inter-UE Cancellation for eURLLC	Apple

R1-2006660	Inter-UE prioritization/multiplexing	InterDigital, Inc.

R1-2006778	Remaining issues on uplink Inter-UE Tx Multiplexing and Prioritization	Qualcomm Incorporated

R1-2006980	Summary of UL inter UE Tx prioritization – Preparing phase	Moderator (vivo)

[102-e-NR-L1enh-URLLC-InterUE-01] Remaining issues on inter-UE prioritization/multiplexing (including issues #1, #7, #8, #9 from FL’s summary) - Xueming (vivo)

- Discussion and agreement by 8/21, TPs by 8/28

R1-2007077	Summary of [102-e-NR-L1enh-URLLC-InterUE-01] Remaining issues on inter-UE prioritization/multiplexing	Moderator (vivo)

Conclusion

Earlier cancellation relaxation for UL CI is not supported. No spec update.

Conclusion

It is up to gNB scheduler to avoid cancellation of RACH related PUSCH transmissions, no spec update.

Conclusion

In case of PUSCH repetitions, it is understood that the open-loop power control parameter set indication in the UL grant applies to all the PUSCH repetitions. No spec update.

For futher discussion

Discussion point 2: Please share your view on UE processing order between PHR calculation and UL cancellation due to UL CI, and what is the required spec impact?

#### Enhanced UL configured grant transmission

R1-2005418	Remaining issues on enhancements for UL configured grant transmission	ZTE

R1-2005511	Remaining Issue of Enhancements to UL Configured Grant Transmission for NR URLLC			Ericsson

R1-2005677	Corrections on Enhanced UL configured grant transmission	CATT

R1-2006056	Configured grant enhancements for URLLC	OPPO

R1-2006493	Remaining issues on enhanced UL configured grant transmission	Apple

R1-2006657	Corrections for configured grant PUSCH	vivo

R1-2006969	Feature lead summary on URLLC enhanced configured grant transmission	Moderator (vivo)

[102-e-NR-L1enh-URLLC-eCG-01] Remaining issues on URLLC eCG – Lihui (vivo)

- Cancellation and initial transmission occasion of RV0

- Clarification the intra-UE prioritization is per actual or per nominal repetition for PUSCH with repetition Type B

- RRC parameter corrections

- Discussion/Agreements by 8/21, TPs by 8/28

#### Others

R1-2006296	Summary on maintenance of other aspects for URLLC/IIOT	Moderator (LG Electronics)

R1-2005352	Other issues for URLLC	vivo

R1-2005419	Remaining issues on SPS enhancements	ZTE

R1-2005512	Remaining Issue of Other Enhancements for NR URLLC/IIoT	Ericsson

R1-2005551	Maintenance of Rel-16 URLLC/IIoT SPS enhancements	Nokia, Nokia Shanghai Bell

R1-2005678	Remaining issues on SPS enhancements	CATT

R1-2005775	Remaining issues on DL SPS enhancement for URLLC	NEC

R1-2006057	DL SPS enhancement	OPPO

R1-2006114	Maintenance on IIoT SPS enhancements	Samsung

R1-2006295	Remaining issues of other aspects for URLLC/IIOT	LG Electronics

R1-2006390	Corrections on other aspects for URLLC/IIOT enhancements	Huawei, HiSilicon

R1-2006564	PUCCH resource determination for SPS PDSCH receptions for NR URLLC	Sharp

R1-2006634	Remaining issues on DL SPS enhancements	Asia Pacific Telecom co. Ltd

R1-2006699	Maintainance for SPS enhancement for Rel-16 URLLC	NTT DOCOMO, INC.

R1-2006779	Remaining issues on uplink collision handling and SPS for URLLC	Qualcomm Incorporated

R1-2006884	Remaining Issues on SPS PDSCH for NR URLLC	WILUS Inc.

[102-e-NR-L1enh-URLLC-IIoTenh-01] HARQ-ACK for SPS release and reception – Duckhyun (LG Electronics)

- Issues 3.1, 3.5, 4.4 from feature lead’s summary

- Discussion and agreement by 8/21 and TPs by 8/28

R1-2007124

Email discussion to finalize the following (baseline)

a SPS release PDCCH in a slot can be received only if the SPS release reception meets the timeline condition between single SPS PDSCH release and single PDSCH reception with all of the SPS PDSCH required to be received in the slot with the indicated SPS configurations. (Option 1)