3GPP TSG RAN WG1 Meeting #102-e

e-Meeting, August 17th – 28th, 2020

Title:	Session notes for 7.2.6 (	Maintenance of Enhancements on MIMO for NR)

Source:	Ad-hoc chair (Samsung)

### Maintenance of Enhancements on MIMO for NR

For this agenda, no more than 3 contribution per company/organization/university

R1-2006116	Feature lead summary for MU-MIMO CSI	Samsung

R1-2006497	FL Summary on SCell BFR and L1-SINR	Apple

R1-2005353	Remaining issues on Multi beam operation	vivo

R1-2005354	Remaining issues on Multi TRP operation	vivo

R1-2005355	Miscelleneous corrections on Rel-16  MIMO	vivo

R1-2005449	Maintenance of multi-beam operation	ZTE

R1-2005450	Maintenance of full power UL transmission	ZTE

R1-2005451	Maintenance of Multi-TRP enhancements	ZTE

R1-2005554	Remaining issues on NR MIMO	InterDigital, Inc.

R1-2005559	Remaining issues on multi-beam operation	Sony

R1-2005624	Remaining issues on multi-beam operation	MediaTek Inc.

R1-2005679	Remaining issues on multi-beam operation enhancement	CATT

R1-2005819	Maintenance on multi-TRP operation	Lenovo, Motorola Mobility

R1-2005853	Corrections to multi TRP	Intel Corporation

R1-2005924	Maintenance on MU-CSI Enhancements	Lenovo, Motorola Mobility

R1-2005975	Text proposals for enhancements on multi-TRP and panel Transmission	OPPO

R1-2005976	Text proposals for Multi-beam Operation Enhancement	OPPO

R1-2005977	Discussion and Text proposals for full TX power UL transmission	OPPO

R1-2006115	On maintenance of Rel.16 MU CSI enhancements and multi-beam operation	Samsung

R1-2006117	On Rel.16 multi-TRP/panel transmission	Samsung

R1-2006118	On UL full power transmission	Samsung

R1-2006249	Remaining issues on CSI enhancement for MU-MIMO	Spreadtrum Communications

R1-2006256	Discussion on remaining issues on UL full power transmission	Spreadtrum Communications

R1-2006257	Discussion on remaining issues for multi-TRP operation	Spreadtrum Communications

R1-2006395	Remaining issues for Multi-TRP in Rel-16	Huawei, HiSilicon

R1-2006396	Remaining issues on multi-beam enhancements in R16	Huawei, HiSilicon

R1-2006397	Remaining issues on UL full power transmission in R16	Huawei, HiSilicon

R1-2006494	Remaining issues on Multi-TRP enhancement	Apple

R1-2006495	Remaining issues on Beam Management	Apple

R1-2006496	Remaining issues on Rel-16 full power transmission	Apple

R1-2006551	Corrections for enhancements on MIMO for NR	Sharp

R1-2006588	Discussion on remaining issues of multi-TRP/panel transmission	CATT

R1-2006593	Text proposals on enhancements on multi-TRP/panel transmission	LG Electronics

R1-2006594	Remaining issues on multi beam operation	LG Electronics

R1-2006595	Discussions on full Tx power UL transmission	LG Electronics

R1-2006608	Corrections for Full Power UL Transmission	Ericsson

R1-2006635	Maintenance of SCell Beam Failure Recovery	Asia Pacific Telecom co. Ltd

R1-2006688	Remaining issues on single-DCI based Multi-TRP	Ericsson

R1-2006689	Remaining issues on multi-DCI based Multi-TRP	Ericsson

R1-2006700	Remaining issues on multi-TRP/panel transmission	NTT DOCOMO, INC.

R1-2006701	PUCCH/PDCCH beam after CBRA-BFR in Rel.16	NTT DOCOMO, INC., Nokia, Nokia Shanghai Bell, Ericsson

R1-2006780	Remaining issues on MU-CSI enhancement, UL full power, and low PAPR RS	Qualcomm Incorporated

R1-2006781	Multi-TRP Enhancements	Qualcomm Incorporated

R1-2006782	Enhancements on Multi-beam Operation	Qualcomm Incorporated

R1-2006840	Correction of RRC parameter name for AP-TRS	Nokia, Nokia Shanghai Bell

R1-2006841	Maintenance of Rel-16 CSI enhancement	Nokia, Nokia Shanghai Bell

R1-2006842	Maintenance of Rel-16 Multi-TRP operation	Nokia, Nokia Shanghai Bell

R1-2006979	Summary#2 for Rel.16 NR eMIMO maintenance	Moderator (Samsung)

[102-e-NR-eMIMO-01] PLRS for DCI 0_2 and RRC parameter alignment (including TP for MB.3) – Jiwon (LG Electronics)

- Discussion and agreements by Aug 19, TPs by Aug 21

Conclusion

For PUSCH, UE feature 16-1c is applicable to DCI format 0_0, 0_1, and 0_2.

Agreement

- The 7 text proposals in R1-2007005 are endorsed for the editor’s CR on TS38.213, TS38.214

- The text proposal in R1-2007092 is endorsed for the editor’s CR on TS38.213 (clause 7.1.1)

[102-e-NR-eMIMO-02] Simultaneous TCI state activation – Jiwon (LG Electronics)

- Discussion and agreements by Aug 21, TPs by Aug 28

Conclusion

By RRC configuration, each CC list cannot include a CC/BWP in which two CORESET pools are configured.

Agreement

Send an LS to RAN2 with following question

- Q) whether the following understanding is correct

- In current RAN2 specification, it is not precluded to enable single DCI based multi-TRP operation in one or more CC(s)/BWP(s) included in simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2 by using Enhanced TCI States Activation/Deactivation for UE-specific PDSCH MAC CE.

LS is endorsed in R1-2007203

R1-2007087	Outcome of email thread [102-e-NR-eMIMO-02]	Moderator (LG Electronics)

[102-e-NR-eMIMO-03] Email thread#3 Control beam after CBRA-BFR and editorial TPs (including TP for MB.10) – Yushu (Apple)

- Discussion and agreements by Aug 21, TPs by Aug 28

Agreement

All the TPs in R1-2007288 are endorsed for the editor’s CR.

[102-e-NR-eMIMO-04] Email thread#4 Processing time for URLLC scheme 3 – Li (OPPO)

- Discussion and agreements by Aug 21, TPs by Aug 28

Agreement

The text proposal in R1-2007120 for TS 38.214 is endorsed for the editor’s CR

[102-e-NR-eMIMO-05] Email thread#5 Editorial TPs for mTRP and MU-CSI (including TP for MU.1) – Li (OPPO)

- Discussion and agreements by Aug 19, TPs by Aug 21

Agreement

The text proposal in R1-2006984 is endorsed for editors’ CRs on TS38.214

Agreement

The text proposals in the following tdocs are endorsed for the editors’ CRs.

- R1-2007121     Text Proposal for TS 38.211 in [102-e-NR-eMIMO-05]

- R1-2007122     Text Proposal for TS 38.213 in [102-e-NR-eMIMO-05]

- R1-2007123     Text Proposal for TS 38.214 in [102-e-NR-eMIMO-05]

[102-e-NR-eMIMO-06] Default QCL for AP CSI-RS – Li (OPPO)

- Start email discussion on Aug 24, discussion and agreements by Aug 26, TPs by Aug 28

Agreement

In multi-DCI based multi-TRP system, if the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is less than threshold beamSwitchTiming, the UE determines the QCL assumption for AP CSI-RS resource as follows:

- if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS.

  - When a UE is configured with enableDefaultTCIStatePerCoresetPoolIndex and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet , the other DL signal refers to PDSCH  associated with the same CORESETPoolIndex  as the PDCCH triggering the AP CSI-RS and scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS associated with the same CORESETPoolIndex  as the PDCCH triggering the AP CSI-RS and scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming  when the reported value is one of the values {14,28,48} and enableBeamSwitchTiming-r16 is not provided, aperiodic CSI-RS associated with the same CORESETPoolIndex  as the PDCCH triggering the AP CSI-RS and scheduled with offset larger than or equal to 48 when the reported value of beamSwitchTiming  is one of the values {224, 336} and enableBeamSwitchTiming-r16 is not provided, periodic CSI-RS, semi-persistent CSI-RS;

- If there is no other DL signal with an indicated TCI state in the same symbols as the CSI-RS,

  - When a UE is configured with enableDefaultTCIStatePerCoresetPoolIndex and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet , the UE applies the QCL parameter(s) of the CORESET associated with a monitored search space with the lowest controlResourceSetId among CORESETs , which are configured with the same value of CORESETPoolIndex as the PDCCH triggering that AP CSI-RS, in the latest slot in which one or more CORESETs associated with the same value of CORESETPoolIndex as the PDCCH triggering that AP CSI-RS.

- UE is not expected to receive AP CSI -RS and PDSCH /AP-CSI-RS associated with different CORESETPoolIndex in overlapped symbol(s).

- UE is not expected to receive AP CSI-RS and SP/P CSI-RS with different QCL type D in overlapped symbol(s)

Note: The above behavior is applied at least for the same carrier scheduling case.

Agreement

In single-DCI based multi-TRP system if the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is less than threshold beamSwitchTiming , the UE determines the QCL assumption for AP CSI-RS resource as follows

- If there is any other DL signal with indicated TCI state in the same symbols as AP CSI-RS, The UE applies QCL assumption of the other signal.

  - If a UE is configured with enableTwoDefaultTCIStates and at least one TCI codepoint indicates two TCI states, when the other signal is a PDSCH indicated with two TCI states, the UE applies the first TCI state of those two TCI states.

- Otherwise,

  - If a UE is configured with enableTwoDefaultTCIStates and at least one TCI codepoint indicates two TCI states, the UE applies the first one of two TCI states corresponding to the lowest DCI codepoint among those mapped to two TCI states

- UE is not expected to receive AP CSI-RS and SP/P CSI-RS with different QCL type D in overlapped symbol(s)

Note: The above behavior is applied at least for the same carrier scheduling case.

[102-e-NR-eMIMO-07] TPMI grouping for mode 2 – Rakesh (vivo)

- Start email discussion on Aug 24, discussion and agreements by Aug 26, TPs by Aug 28

- Discussion and agreements by Aug 21, TPs by Aug 28

[102-e-NR-eMIMO-08] 2-port PTRS for mode 1 and RRC parameter alignment (including TP for UL.3) – Rakesh (vivo)

- Discussion and agreements by Aug 21, TPs by Aug 28

- Start email discussion on Aug 24, discussion and agreements by Aug 26, TPs by Aug 28

Agreement

The text proposals in section 2.2 (alignment of RRC parameter names) of R1-2007345 are endorsed for the editors’ CRs.

[102-e-NR-eMIMO-09] Response LS to RAN4 on LS on feasibility of ULFPTx modes and transparent TxD for certain UE implementations by 8/20 – Leiming (Huawei)

Agreement

Capture the following responses in reply LS to RAN4 on feasibility of ULFPTx modes and transparent TxD. The LS is endorsed in R1-2007245.

Question 1: Whether the two mentioned methods are both feasible to transmit the full output power? Answer 1: From RAN1’s perspective, both Method-1 and Method-2 can be supported for UL full power transmission. For two SRS port transmission in method-2, the UE will need to transmit two SRS ports on each PA in a symbol, while for method-1 each PA carries one SRS port. Question 2: If answer is yes, which ULFPTx modes can be supported for these two methods? Answer 2: From RAN1’s perspective, Method-1 can be supported for any UL full power transmission modes, and Method-2 can be supported at least for Mode-2 and Mode-full power (i.e., the other mode). Question 3: Whether the ULFPTx mode-2 and the other ULPFTx mode are feasible for FR2 UE? Answer 3: From RAN1’s perspective, UL full power modes {Mode-1, Mode-2 and Mode-full power (i.e., the other mode)} can be supported for FR2.