3GPP TSG RAN WG1 Meeting #102-e

e-Meeting, August 17th – 28th, 2020

Title:	Session notes for 8.1 (Further enhancements on MIMO for NR)

Source:	Ad-hoc chair (Samsung)

## Further enhancements on MIMO for NR

Please refer to RP-193133 for detailed scope of the WI

R1-2006125	High-level work plan for Rel.17 FeMIMO WI	Moderator (Samsung)

R1-2006126	Moderator summary for Rel.17 FeMIMO EVM offline	Moderator (Samsung)

### Enhancements on Multi-beam Operation

Mainly targeting FR2 while also applicable to FR1

R1-2005560	Considerations on the enhancement of multi-beam operation	Sony

R1-2005842	Enhancements to multi-beam operation	Ericsson

R1-2006128	Multi-Beam Enhancements	Samsung

R1-2006991	Multi-Beam Enhancements	Samsung

Update of R1-2006128

R1-2006540	Enhancements on multi-beam operation	Beijing Xiaomi Electronics

R1-2006636	Discussion on Enhancements for Multi-beam Operation	Asia Pacific Telecom co. Ltd

[102-e-NR-feMIMO-01] Email discussion on enhancements on multi-beam operation by 8/28 – Eko (Samsung)

- Prioritize topics to be resolved in RAN1#102-e by 8/19 (EVM should be highest priority)

R1-2005246	Enhancements on multi-beam operation in Rel-17	Huawei, HiSilicon

R1-2005290	Enhancement on multi-beam operation	FUTUREWEI

R1-2005363	Discussion on multi beam enhancement	vivo

R1-2005454	Enhancements on Multi-beam Operation	ZTE

R1-2005482	Enhancements on Multi-beam Operation	InterDigital, Inc.

R1-2005858	Enhancements on Multi-Beam Operation	Intel Corporation

R1-2005619	Enhancement on multi-beam operation	MediaTek Inc.

R1-2005683	Discussion on enhancement on multi-beam operation	CATT

R1-2005750	Discussion on multi-beam operation	NEC

R1-2005782	Enhancements on multi-beam operation	Fraunhofer IIS, Fraunhofer HHI

R1-2005820	Enhancements on Multi-beam Operation	Lenovo, Motorola Mobility

R1-2005858	Enhancements on Multi-Beam Operation	Intel Corporation

R1-2005954	Enhancements on Multi-Beam Operation	AT&T

R1-2005983	Enhancements on Multi-beam Operation	OPPO

R1-2006200	Enhancements on multi-beam operation	CMCC

R1-2006248	Enhancements on Multi-beam Operation	Spreadtrum Communications

R1-2006499	On Beam Management enhancement	Apple

R1-2006565	Enhancement on multi-beam operation for UE with multi-panel	Sharp

R1-2006596	Enhancements on Multi-beam Operation	LG Electronics

R1-2006717	Discussion on multi-beam operation	NTT DOCOMO, INC.

R1-2006790	Enhancements on Multi-beam Operation	Qualcomm Incorporated

R1-2006843	Enhancements on Multi-beam Operation	Nokia, Nokia Shanghai Bell

R1-2006950	Enhancements on Multi-Beam Operation	Intel Corporation

Late submission

R1-2006951	Discussion on multi-beam operation	NTT DOCOMO, INC.

Late submission

### Enhancements for Multi-TRP Deployment

R1-2006718	Discussion on UL dense deployment	NTT DOCOMO, INC.

#### Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH

R1-2005542	Enhancements on Multi-TRP for PUCCH and PUSCH	Fujitsu

R1-2005728	Discussion on multi-TRP enhancement	China Telecom

R1-2006791	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	Qualcomm Incorporated

R1-2006844	Enhancements for Multi-TRP URLLC schemes	Nokia, Nokia Shanghai Bell

R1-2006868	Discussion on enhancement on M-TRP	ASUSTeK

[102-e-NR-feMIMO-02] Email discussion on enhancements on multi-TRP for PDCCH by 8/28 – Mostafa (Qualcomm)

- Prioritize topics to be resolved in RAN1#102-e by 8/19 (EVM should be highest priority)

R1-2007179	Summary of EVM discussion for mTRP PDCCH reliability enhancements	Qualcomm

Agreement

The following s agreed for evaluation of PDCCH

- According to the evaluation scenario (e.g., at FR1 in urban macro / at FR1 in indoor hotspot / at FR2 in indoor hotspot), one of three Tables (Table A.3-1 ~ A.3-3) of 38.824 can be a baseline of EVM for Rel-17 FeMIMO item 2a.

  - System bandwidth other than those mentioned in the Tables can be considered and reported by the companies.

- In addition, the following table is used for EVM for Rel-17 FeMIMO item 2a (Common assumptions for PDCCH/PUCCH/PUSCH)

| Parameters | Values |
| --- | --- |
| The number of TRPs | 2 |
| Channel model | TDL for FR1 (CDL for FR1 can be optionally used) CDL for FR2 (TDL for FR2 can be optionally used) |
| Path-loss modeling | {0,3,6} dB gap between TRPs |
| Blockage | Blockage model from Rel-16 (x dB power offset with probability p): Companies to report x and p, and other assumptions, if any. |
| Target BLER | [10^-3, 10^-4, 10^-5]: BLER values shown in plots should be based on enough number of samples, e.g., ~100/BLER samples |

- The following table is used for detailed assumptions for PDCCH

| Parameters | Values |
| --- | --- |
| Baseline schemes | Option 1: Rel-15 PDCCH Option 2: Spec transparent SFN For FR1: Both options 1 and 2 can be considered For FR2: Option 1. |
| AL | 8 as baseline. Companies are encouraged to simulate other AL’s additionally for different code rate regimes. |
| # of RBs/symbols | 1 or 2 symbols. Companies to report # of RBs. |
| DCI payload | 40+24(CRC)=64 as baseline. Other payload values are not precluded. |
| CCE-to-REG mapping | Both Interleaved and non-interleaved can be considered. Companies to report the assumptions including interleaverSize in the case of interleaved. |
| REG bundling size | 6 and 2 as baseline. |
| Precoding assumptions | Precoding cycling, precoder granularity=REG bundle as baseline. Closed-loop precoding can be used optionally |
| Schemes | Details of the schemes used (including TDM,FDM, etc.) to be reported by companies. |
| Receiver assumption | Up to companies to report |

[102-e-NR-feMIMO-03] Email discussion on enhancements on multi-TRP for PUSCH, PUCCH by 8/28– Keeth (Nokia)

- Prioritize topics to be resolved in RAN1#102-e by 8/19 (EVM should be highest priority)

R1-2007182	Summary of AI:8.1.2.1 Enhancements for Multi-TRP URLLC for PUCCH and PUSCH	Moderator (Nokia, Nokia Shanghai Bell)

Agreement

- Detailed assumptions for PUCCH evaluation:

| Parameters | Potential values |
| --- | --- |
| Baseline scheme | Rel-15 PUCCH repetition |
| PUCCH format | Format 1 and 3. Other PUCCH Formats can be optionally considered. |
| # of RBs/symbols | PUCCH Format 1: 4 symbols, 1 RB PUCCH Format 3: 4 and 8 symbols, 1 RB Other combinations are not precluded. |
| UCI payload | 2 bits for PUCCH Format 1 (and Format 0, if considered). Companies to report assumptions on other PUCCH Formats |
| Frequency hopping | Reported by companies |
| Number of repetitions (when applicable) | 2, 4, 8 |
| Schemes | TDM Details to be reported by companies |
| Receiver assumption | Reported by companies |

- Detailed assumptions for PUSCH evaluation:

| Parameters | Potential values |
| --- | --- |
| Baseline scheme | Rel-15/-16 PUSCH repetition |
| # of RBs/symbols | Companies to Report. |
| DMRS pattern | DM-RS configuration type 1 DM-RS Configuration type 2 (optional) |
| # of layers | 1, 2 (optional) |
| Code rates | Low (<0.2) and moderate (<0.4) |
| Frequency hopping | Reported by companies |
| UL transmission scheme | Codebook based UL transmission is baseline. Non-codebook based can be optional. |
| Redundancy Version | Reported by companies |
| Number of repetitions (when applicable) | 2, 4, 8 Other numbers are not precluded |
| Schemes | TDM Details to be reported by companies |
| Receiver assumption | Reported by companies |

Agreement

To improve reliability and robustness for PUCCH using multi-TRP and/or multi-panel, consider all PUCCH formats.

Agreement

To enable TDMed PUCCH repetitiontransmission with different beams, support configuring/activating of multiple PUCCH Spatial Relation Info. RAN1 shall further study the exact schemes considering the following aspects,

- Method of configuration/activation of multiple spatial relation info

- Use of the same PUCCH resource or different PUCCH resource for PUCCH repetitiontransmission

- Mapping between PUCCH repetition/symbol and spatial relation info among multiple PUCCH repetitions / multiple PUCCH symbols.

Agreement

For configuration/indication of the number of PUCCH repetitions, RAN1 shall further study the following,

- Alt.1: Use Rel-15 like framework

- Alt.2: Dynamic indication of the number of PUCCH repetitions

Agreement

For multi-TRP PUCCH transmission, further investigate required power control enhancement.

Agreement

Further study M-TRP CG PUSCH reliability enhancements in Rel-17.

R1-2005285	Multi-TRP/panel for non-PDSCH	FUTUREWEI

R1-2005364	Discussion on enhancement on PDCCH, PUCCH, PUSCH in MTRP scenario	vivo

R1-2005455	Multi-TRP enhancements for PDCCH, PUCCH and PUSCH	ZTE

R1-2005483	Discussion on Multi-TRP Physical Channel Enhancements	InterDigital, Inc.

R1-2005561	Considerations on Multi-TRP for PDCCH, PUCCH, PUSCH	Sony

R1-2005621	Enhancements on Multi-TRP for PDCCH, PUSCH and PUCCH	MediaTek Inc.

R1-2005684	Discussion on enhancements on multi-TRP/panel for PDCCH, PUCCH and PUSCH	CATT

R1-2005751	Discussion on multi-TRP for PDCCH, PUCCH and PUSCH	NEC

R1-2005783	On multi-TRP enhancements for PDCCH and PUSCH	Fraunhofer IIS, Fraunhofer HHI

R1-2005821	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	Lenovo, Motorola Mobility

R1-2005859	Multi-TRP enhancements for PDCCH, PUCCH and PUSCH	Intel Corporation

R1-2005984	Enhancements on Multi-TRP based enhancement for PDCCH, PUCCH and PUSCH	OPPO

R1-2006129	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	Samsung

R1-2006201	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	CMCC

R1-2006258	Discussion on enhancements on multi-TRP for PDCCH, PUCCH and PUSCH	Spreadtrum Communications

R1-2006365	Discussion on Multi-TRP	TCL Communication Ltd.

Late submission

R1-2006367	On PDCCH, PUCCH and PUSCH robustness	Ericsson

R1-2006391	Enhancements on Multi-TRP for reliability and robustness in Rel-17	Huawei, HiSilicon

R1-2006500	On multi-TRP reliability enhancement	Apple

R1-2006543	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	Beijing Xiaomi Electronics

R1-2006566	Enhancement on multi-TRP operation for PDCCH and PUSCH	Sharp

R1-2006597	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	LG Electronics

R1-2006627	Multi-TRP Enhancements for PDCCH, PUCCH and PUSCH	Convida Wireless

R1-2006637	Discussion on enhancements on multi-TRP for uplink channels	Asia Pacific Telecom co. Ltd

R1-2006719	Discussion on MTRP for reliability	NTT DOCOMO, INC.

R1-2006901	Discussion on multi-TRP/multi-panel transmission	TCL Communication Ltd.

#### Enhancements on Multi-TRP inter-cell operation

R1-2005286	Inter-cell multi-TRP operation	FUTUREWEI

R1-2005365	Discussion on inter-cell MTRP operation	vivo

R1-2005985	Enhancement on inter-cell multi-TRP operation	OPPO

[102-e-NR-feMIMO-04] Email discussion on enhancements on multi-TRP intercell operation – Rakesh (vivo)

- Prioritize topics to be resolved in RAN1#102-e by 8/19 (EVM should be highest priority if there is any new EVM assumptions needed)

R1-2005456	Discussion on Multi-TRP inter-cell operation	ZTE

R1-2005484	TCI/QCL Enhancements for M-TRP Inter-cell Operation	InterDigital, Inc.

R1-2005562	Considerations on inter-cell operation	Sony

R1-2005685	Discussion on multi-TRP/panel inter-cell operation	CATT

R1-2005822	Enhancements on Multi-TRP inter-cell operation	Lenovo, Motorola Mobility

R1-2005860	Multi-TRP enhancements for inter-cell operation	Intel Corporation

R1-2006130	Enhancements on Multi-TRP inter-cell operation	Samsung

R1-2006202	Enhancements on Multi-TRP inter-cell operation	CMCC

R1-2006259	Discussion on enhancement multi-TRP inter-cell operation	Spreadtrum Communications

R1-2006368	On inter-cell operation for mTRP	Ericsson

R1-2006392	Enhancements on inter-cell Multi-TRP operations in Rel-17	Huawei, HiSilicon

R1-2006501	On Inter-cell multi-TRP operation	Apple

R1-2006545	Enhancement on Inter-cell Multi-TRP operations	Beijing Xiaomi Electronics

R1-2006567	Enhancement on inter-cell multi-TRP operation	Sharp

R1-2006598	Enhancements on Multi-TRP inter-cell operation	LG Electronics

R1-2006720	Discussion on inter-cell multi-TRP operations	NTT DOCOMO, INC.

R1-2006792	Enhancements on Multi-TRP inter-cell operation	Qualcomm Incorporated

R1-2006845	Enhancements to enable inter-cell multi-TRP operations	Nokia, Nokia Shanghai Bell

#### Enhancements on beam management for multi-TRP

R1-2005686	Discussion on enhancements on beam management for multi-TRP	CATT

R1-2006260	Discussion on enhancements on beam management for multi-TRP	Spreadtrum Communications

R1-2006502	On multi-TRP BM enhancement	Apple

R1-2006654	Discussion on beam management for Multi-TRP	ITRI

[102-e-NR-feMIMO-05] Email discussion on enhancements on beam management for multi-TRP – Runhua (CATT)

- Prioritize topics to be resolved in RAN1#102-e by 8/19 (EVM should be highest priority if there is any new EVM assumptions needed)

R1-2005287	Beam management for simultaneous multi-TRP transmission with multi-panel reception				FUTUREWEI

R1-2005366	Discussion on MTRP multibeam enhancement	vivo

R1-2005457	Enhancements on beam management for Multi-TRP	ZTE

R1-2005485	Discussion on Beam Management Enhancements for Multi-TRP	InterDigital, Inc.

R1-2005563	Considerations on beam management for multi-TRP	Sony

R1-2005620	Enhancements on beam management for multi-TRP	MediaTek Inc.

R1-2005752	Discussion on beam management for multi-TRP	NEC

R1-2005784	On beam management enhancements for multi-TRP	Fraunhofer IIS, Fraunhofer HHI

R1-2005823	Enhancements on beam management for multi-TRP	Lenovo, Motorola Mobility

R1-2005861	Multi-TRP enhancements for beam management	Intel Corporation

R1-2005955	Views on beam management enhancement for multi-TRP	AT&T

R1-2005986	Enhancements on beam management for multi-TRP	OPPO

R1-2006131	Enhancements on beam management for multi-TRP	Samsung

R1-2006203	Enhancements on beam management for multi-TRP	CMCC

R1-2006393	Enhancements on multi-TRP for multi-panel reception in Rel-17	Huawei, HiSilicon

R1-2006546	Enhancement on beam management for Multi-TRP	Beijing Xiaomi Electronics

R1-2006599	Enhancements on beam management for multi-TRP	LG Electronics

R1-2006638	Discussion of enhancements on beam management for multi-TRP	Asia Pacific Telecom co. Ltd

R1-2006681	On beam management enhancements for simultaneous multi-TRP transmission with multi-panel reception	Ericsson

R1-2006721	Discussion on beam management for MTRP	NTT DOCOMO, INC.

R1-2006793	Enhancements on beam management for multi-TRP	Qualcomm Incorporated

R1-2006846	Enhancements on Beam Management for Multi-TRP/Panel Transmission	Nokia, Nokia Shanghai Bell

#### Enhancements on HST-SFN deployment

R1-2005486	Enhanced M-TRP for HST-SFN 	InterDigital, Inc.

R1-2005862	On HST SFN enhancements	Intel Corporation

R1-2005753	Discussion on HST-SFN deployment	NEC

R1-2006204	Enhancements on HST-SFN deployment	CMCC

[102-e-NR-feMIMO-06] Email discussion on enhancements on HST-SFN deployment by 8/28 – Alexei (Intel)

- Prioritize topics to be resolved in RAN1#102-e by 8/19 (EVM should be highest priority)

Agreement

Proposal on evaluation methodology in section 2.1 of FL’s summary (to be updated with tdoc#) with following modifications (yellow part of proposal is not agreed)

- FFS: Propagation condition for FR1, FR2 whether CDL extension is optional or baseline

- FFS: UE height for both FR1, FR2

Agreement

For HST evaluation in FR2, Alt 2-3 is mandatory, other alternatives, i.e. Alt 2-4 and Alt. 2-1, are optional.

- Alt 2-1: Ds=700m, Dmin=150m

- Alt 2-3: Ds=200-300m, Dmin=30-50m

- Alt 2-4: Ds=580m, Dmin=5m

R1-2005367	Evaluation and discussion on HST-SFN schemes	vivo

R1-2005458	Discussion on Multi-TRP HST enhancements	ZTE

R1-2005564	Considerations on HST-SFN operation for multi-TRP	Sony

R1-2005592	Enhancement to support HST-SFN deployment scenario	FUTUREWEI

R1-2005687	Discussion on enhancements on HST-SFN deployment	CATT

R1-2005925	Enhancements for HST-SFN deployment	Lenovo, Motorola Mobility

R1-2005987	Enhancements on HST-SFN deployment	OPPO

R1-2006132	Enhancements on HST-SFN	Samsung

R1-2006261	Discussion on enhancements on HST-SFN deployment	Spreadtrum Communications

R1-2006394	Enhancements on Multi-TRP for high speed train in Rel-17	Huawei, HiSilicon

R1-2006475	Enhancement on HST-SFN deployment	Ericsson

R1-2006503	Views on Rel-17 HST enhancement	Apple

R1-2006600	Enhancements on HST-SFN deployment	LG Electronics

R1-2006722	Discussion on HST-SFN deployment	NTT DOCOMO, INC.

R1-2006794	Enhancements on HST-SFN deployment	Qualcomm Incorporated

R1-2006847	Enhancements for HST-SFN deployment	Nokia, Nokia Shanghai Bell

### Enhancements on SRS flexibility, coverage and capacity

R1-2005459	Enhancements on SRS flexibility, coverage and capacity	ZTE

R1-2006963	Enhancements on SRS flexibility, coverage and capacity	ZTE

Update of R1-2005459

R1-2005622	Enhancements on SRS flexibility, coverage and capacity	MediaTek Inc.

R1-2006364	Discussion on enhancement of SRS in Rel. 17 further enhanced MIMO 	CEWiT

R1-2006568	Enhancement on SRS	Sharp

R1-2006723	Discussion on SRS enhancement	NTT DOCOMO, INC.

[102-e-NR-feMIMO-07] Email discussion on SRS flexibility, coverage, and capacity by 8/28 – Hao (ZTE)

- Prioritize topics to be resolved in RAN1#102-e by 8/20 (EVM should be highest priority)

R1-2005247	Enhancements on SRS for Rel-17	Huawei, HiSilicon

R1-2005288	Enhancements on SRS flexibility, coverage and capacity	FUTUREWEI

R1-2005368	Discussion on SRS enhancement	vivo

R1-2005487	Discussion on SRS Enhancements	InterDigital, Inc.

R1-2005565	Considerations on SRS flexibility, coverage and capacity	Sony

R1-2005688	Discussion on enhancements on SRS  flexibility, coverage and capacity	CATT

R1-2005754	Discussion on SRS enhancement	NEC

R1-2005824	Enhancements on SRS	Lenovo, Motorola Mobility

R1-2005863	Discussion on SRS enhancements	Intel Corporation

R1-2005988	Enhancements on SRS flexibility, coverage and capacity	OPPO

R1-2006133	Enhancements on SRS	Samsung

R1-2006205	Enhancements on SRS flexibility, coverage and capacity	CMCC

R1-2006255	Considerations on SRS enhancement	Spreadtrum Communications

R1-2006504	Views on Rel-17 SRS enhancement	Apple

R1-2006601	Enhancements on SRS flexibility, coverage and capacity	LG Electronics

R1-2006610	SRS Performance and Potential Enhancements	Ericsson

R1-2006795	Enhancements on SRS flexibility, coverage and capacity	Qualcomm Incorporated

R1-2006848	Enhancements on SRS in Rel-17	Nokia, Nokia Shanghai Bell

### CSI enhancements: MTRP and FR1 FDD reciprocity

R1-2005248	Enhancements on CSI for Rel-17	Huawei, HiSilicon

R1-2005926	CSI enhancements for mTRP and FDD reciprocity	Lenovo, Motorola Mobility

R1-2005956	CSI enhancements: FDD reciprocity and multi-TRP	AT&T

R1-2006602	CSI enhancements for Rel-17	LG Electronics

R1-2005785	On FDD channel reciprocity in real-world scenarios	Fraunhofer IIS, Fraunhofer HHI

R1-2006998	On FDD channel reciprocity in real-world scenarios	Fraunhofer IIS, Fraunhofer HHI

Update of R1-2005785

[102-e-NR-feMIMO-08] Email discussion on CSI enhancements: MTRP and FR1 FDD reciprocity by 8/28 – Min (Huawei)

- Prioritize topics to be resolved in RAN1#102-e by 8/20 (EVM should be highest priority)

R1-2005289	CSI enhancement for multi-TRP and FDD	FUTUREWEI

R1-2005369	Evaluation on MTRP CSI and Partial reciprocity	vivo

R1-2005460	CSI enhancements for Multi-TRP and FR1 FDD reciprocity	ZTE

R1-2005488	On Type II Port Selection Codebook Enhancement	InterDigital, Inc.

R1-2005566	Considerations on CSI enhancements	Sony

R1-2005623	CSI enhancement for NCJT	MediaTek Inc.

R1-2005689	CSI enhancements on for MTRP and FR1 FDD with partial reciprocity	CATT

R1-2005755	Discussion on CSI enhancement for multi-TRP	NEC

R1-2005864	On CSI enhancements for MTRP and FDD reciprocity	Intel Corporation

R1-2005989	CSI enhancement for M-TRP and FDD reciprocity	OPPO

R1-2006134	Views on Rel. 17 CSI enhancements	Samsung

R1-2006206	Enhancements on CSI reporting for Multi-TRP	CMCC

R1-2006262	Discussion on CSI enhancement for multiple TRP/Panel transmission	Spreadtrum Communications

R1-2006505	Views on Rel-17 CSI enhancement	Apple

R1-2006569	Enhancement on CSI measurement and reporting	Sharp

R1-2006685	On CSI enhancements in Rel-17 feMIMO	Ericsson

R1-2006724	Discussion on CSI enhancements	NTT DOCOMO, INC.

R1-2006796	CSI enhancements for MTRP and FR1 FDD reciprocity	Qualcomm Incorporated

R1-2006849	Enhancement on CSI measurement and reporting	Nokia, Nokia Shanghai Bell

### Others

R1-2005291	Sounding enhancement for interference probing in TDD cooperative MIMO	FUTUREWEI

R1-2005370	Discussion on higher layer aspects for multi-TRP enhancement	vivo

R1-2005461	Further details on Multi-beam and Multi-TRP operation	ZTE

R1-2005489	Evaluation of Multi-TRP MIMO Deployment with Multi-panel UEs	InterDigital, Inc.

R1-2005825	HARQ feedback of SPS PDSCH reception in multi-DCI based multiple TRPs	Lenovo, Motorola Mobility

R1-2006135	Simulation results for multi-beam enhancements	Samsung

R1-2006369	Critical MIMO issue from observations in field	Ericsson

Withdrawn

R1-2006414	Discussion on field measurement and evaluation assumptions for FDD CSI enhancements in Rel-17		Huawei, HiSilicon

R1-2006570	Considerations on TRS for HST-SFN scenario	Sharp

R1-2006838	Discussion of further enhancements on MIMO for NR	Qualcomm Incorporated

Withdrawn

R1-2006949	Critical MIMO issue from observations in field	Ericsson LM