3GPP TSG RAN WG1 Meeting #102-e

e-Meeting, August 17th – 28th, 2020

Title:	Session notes for 8.9 (Rel-17 enhancements for NB-IoT and LTE-MTC)

Source:	Ad-hoc chair (Samsung)

## Rel-17 enhancements for NB-IoT and LTE-MTC

Please refer to RP-201306 for detailed scope of the WI

R1-2006447	Work plan of Rel-17 enhancements for NB-IoT and LTE-MTC	Huawei, Ericsson

### Support of 16-QAM for unicast in UL and DL for NB-IoT

R1-2005304	Support of 16QAM for unicast in UL and DL in NB-IoT	Huawei, HiSilicon

R1-2005479	Discussion on UL and DL 16QAM for NB-IoT	ZTE

R1-2005529	Support of 16-QAM for NB-IoT	Nokia, Nokia Shanghai Bell

R1-2005557	Support of 16-QAM for unicast in UL and DL in NB-IoT	Ericsson

R1-2005648	Considerations on support of 16QAM for NB-IOT	MediaTek Inc.

R1-2005837	Support 16QAM for NBIoT	Lenovo, Motorola Mobility

R1-2005941	Design consideration to support 16-QAM for NB-IOT	Sierra Wireless, S.A.

R1-2005974	Initial discussion on support of 16 QAM for NB-IoT	Beijing Xiaomi Software Tech

R1-2006192	Support of 16-QAM for NB-IoT	Qualcomm Incorporated

[102-e-LTE-Rel17_NB_IoT_eMTC-01] Email discussion on support of 16-QAM for unicast in UL and DL for NB-IoT by 8/28 – Yubo (Huawei)

- Prioritize topics to be resolved in RAN1#102-e by 8/19

R1-2007239	Feature lead summary on 102-e-LTE-Rel17_NB_IoT_eMTC-01	Moderator (Huawei)

Agreement

At least for standalone and guard-band deployments, the maximum TBS to support 16-QAM for unicast in DL is select one option from following:

- Option 1: 4968 bits with ISF=7

- Option 2: 5072 bits with ISF=7

- Option 3: 5736 bits with ISF=7

- FFS on ISF>7 for this maximum TBS

FFS for inband deployments

Agreement

Further study on TBS/MCS table design, resource assignment and TBS allocation to support 16QAM in DL considering at least:

- MCS field size

- Achievable code rates

- Avoidance of link-adaptation issues (i.e., large SINR differences between different entries within one TBS row or between different entries in adjacent TBS rows)

- The break point between different modulation schemes

- Impacts of deployment modes

- Indication of modulation scheme for retransmissions

- Applicability of repetitions

- UE data rate

Agreement

Further study on TBS/MCS table design, resource assignment and TBS allocation to support 16QAM in UL based at least on the following:

- MCS field size

- Achievable code rates

- Avoidance of link-adaptation issues (i.e., large SINR differences between different entries within one TBS row or between different entries in adjacent TBS rows)

- Throughput/UE data rate increase while keeping the max TBS from Rel-16

- The break point between different modulation schemes

- Indication of modulation scheme for retransmissions

- Applicability of repetitions

- Applicability to different number of subcarriers

Agreement

For DL power allocation, support signaling the ratio of NPDSCH EPRE to NRS EPRE. FFS signaling details, including how/whether to signal the ratio for the following cases

- NPDSCH in symbols without NRS and CRS

- NPDSCH in symbols with CRS (only for “In-band” deployment)

- NPDSCH in symbols with NRS

Agreement

Adopt the following evaluation assumptions for support of 16QAM in DL and UL for NB-IoT

<Simulation assumptions for DL>

| Parameter | Value/Description |
| --- | --- |
| Operation mode for DL | Stand-alone, Guard-band, and In-band with 2 or 4 CRS ports |
| Number of antennas | 1T or 2T, 1R |
| Channel model | AWGN |
| Frequency Resource | 1 PRB |
| Number of repetitions | Baseline number of repetitions = 1 (Companies can provide results for other repetition) |
| Modulation Order | QPSK, 16-QAM |
| Noise Estimation | Ideal |
| Channel Estimation | Realistic |
| Frequency Offset | 0 |
| Time Offset | 0 |

<Simulation assumptions for UL>

| Parameter | Value/Description |
| --- | --- |
| Number of antennas | 1T, 2R |
| Channel model | AWGN |
| Frequency Resource | 12-tone |
| Number of repetitions | Baseline number of repetitions = 1 (Companies can provide results for other repetition) |
| Modulation Order | QPSK, 16-QAM |
| Noise Estimation | Ideal |
| Channel Estimation | Realistic |
| Frequency Offset | 0 |
| Time Offset | 0 |

### Support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL for eMTC

R1-2005305	Support of 14-HARQ processes in DL for HD-FDD MTC UEs	Huawei, HiSilicon

R1-2005480	Support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL for eMTC		ZTE

R1-2005530	Support of 14-HARQ processes in DL for eMTC	Nokia, Nokia Shanghai Bell

R1-2005558	Support of 14 HARQ processes in DL in LTE-MTC	Ericsson

R1-2005940	Design considerations to support 14-HARQ for LTE-M	Sierra Wireless, S.A.

R1-2005973	Initial discussion on support of additional PDSCH scheduling delay for introduction of 14 HARQ processes in DL for eMTC	Beijing Xiaomi Software Tech

R1-2006193	Support of 14 HARQ processes and scheduling delay	Qualcomm Incorporated

[102-e-LTE-Rel17_NB_IoT_eMTC-02] Email discussion on support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL for eMTC by 8/28– Gerardo (Ericsson)

- Prioritize topics to be resolved in RAN1#102-e by 8/19

R1-2007265	Feature Lead Summary: [102-e-LTE-Rel17_NB_IoT_eMTC-02]	Moderator (Ericsson)

Agreement

Introduce a new RRC configuration parameter to enable 14 HARQ processes.

Agreement

For a UE configured with 14 HARQ processes, a PDSCH scheduling delay of 2 BL/CE DL subframes and 7 [FFS subframes type(s)] is supported at least in the PUCCH non-repetition case:

- FFS details of signaling.

- FFS other delay values to account for the presence of non-BL/CE subframes in the PUCCH non-repetition case.

- FFS if the 14 HARQ processes feature is supported in PUCCH repetition case.

Working Assumption

Introduce a new optional UE capability to support 14 HARQ processes

### Others

R1-2005481	DL TBS increase for eMTC	ZTE

R1-2006448	Channel quality reporting in NB-IoT to support 16QAM	Huawei, HiSilicon

R1-2006463	On the support of a maximum DL TBS of 1736 bits in LTE-MTC	Ericsson