3GPP TSG RAN WG1 #102-e	R1-200xxxx

e-Meeting, 17th - 28th August, 2020

Agenda item:	8.12

Source: 		Moderator (CMCC)

Title: 	Phase 2 moderator summary on NR Multicast and Broadcast Services

Document for:	Discussion/decision

Introduction

As announced by chairman, one email thread is planned to discuss high-level aspects for NR MBS in this meeting.

[102-e-NR-MBS-01] Email discussion/approval using R1-2007001 as a starting point, focusing on high-level aspects – Fei (CMCC)

- By 8/19 – Classification of high priority/medium priority items for this e-Meeting

- By 8/24 – high priority items

- By 8/27 -  medium priority items

The initial moderator summary is provided in R1-2007001 which can be found in the inbox. The issues in the summary are classified into two tiers.

The first tier issues are the ones for the high level concept and can be considered as starting point for this meeting, which are summarized in table 1 below and the details can be found in section 2 in R1-2007001.

The second tier issues are low priority issues and are mainly the ones for details or further step issues set up on the conclusion of the first tier issues, which are targeted to be discussed in the following meeting but some of them can also be discussed in this meeting upon the first tier issues are concluded. The summary and the details for the second tier issues can be found in section 3 in R1-2007001.

Table 1: The first tier issues

| Sub-agenda | Issues (summary in section 2 in R1-2007001) |
| --- | --- |
| Group scheduling for RRC_CONNECTED UEs | Issue 1 (Question 1 in R1-2007001): Regarding the two high level group scheduling mechanisms, i.e., group-common PDCCH based group scheduling and UE-specific PDCCH based group scheduling, whether down selection is needed or both of them can be considered for MBS for RRC_CONNECTED UEs? |
|  | Issue 2 (Question 2 in R1-2007001): Please share your views on the following two alternatives for frequency resource configuration for MBS for RRC_CONNECTED UEs. • Alternative 1: Introduce a MBS specific BWP • Alternative 2: Define a MBS common frequency resource confined within UE’s active BWP. |
|  | Issue 3 (Question 3 in R1-2007001): Whether the simultaneous operation with unicast reception in the WID means a UE is required to receive multicast PDSCH and unicast PDSCH simultaneously in one slot? If the answer is YES, which multiplexing type(s) of simultaneous reception of unicast PDSCH and multicast PDSCH in a slot can be supported in NR MBS? e.g., TDM, FDM, SDM. |
| Reliability improvement for RRC_CONNECTED UEs | Issue 4 (Proposal 1 in R1-2007001, with little update): For RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast without additional evaluation for it, i.e., no evaluation is needed to justify whether HARQ-ACK feedback is needed. |
|  | Issue 5 (Proposal 2 in R1-2007001, with little update): For RRC_CONNECTED UEs, consider following reliability improvement mechanisms for MBS. • CSI feedback, FFS whether modification is needed on top of existing CSI feedback mechanism for unicast • PDSCH repetition, FFS whether spec impact is implied |
|  | Issue 6 (Question 4 in R1-2007001, with little update): Whether a common evaluation methodology and assumptions are necessary for NR MBS? If the answer is YES, what’s the purpose of the evaluation? And what’s your suggestion on the common evaluation methodology and assumptions? |
| Basic functions for MBS for RRC_IDLE/RRC_INACTIVE UEs | Issue 7 (Proposal 3 in R1-2007001): For UE in IDLE/INACTIVE state, the frequency resource for PTM transmission is • Alt 1: Initial BWP • Alt 2: Configured with larger size to cover initial BWP • Alt 3: Configured to be within initial BWP |
|  | Issue 8 (Proposal 4 in R1-2007001): Multi-beam/beam-sweeping operation is supported for PTM in IDLE/INACTIVE state. |

According to Chairman’s guidance, this email thread will be organized in three phases:

- Phase 1: by 8/19, classification of high priority/medium priority items for this e-Meeting based on the summarized first tier issues.

- Phase 2: by 8/24, discuss and conclude the high priority items.

- Phase 3: by 8/27, discuss and conclude the medium priority items.

In phase 1, based on companies’ views, the following high/medium priority items have been classified for this meeting:

- High priority:

  - Issue 1/4/6

- Medium priority:

  - Issue 2/3/5

For phase 2, companies are invited to provide their views on the high priority issues for this e-Meeting in section 2. Moderator will provide the observation and proposal on the high priority issues based on companies’ inputs.

For phase 3, companies are invited to provide their views on the medium priority issues for this e-Meeting in section 3. Moderator will provide the observation and proposal on the medium priority issues based on companies’ inputs.

Email discussion on high priority issues (Phase 2)

Company Views (1st round of email discussion)

Group scheduling mechanisms for RRC_CONNECTED UEs

Based on companies’ submitted contributions, two group scheduling mechanisms were proposed. The first is group-common PDCCH based group scheduling, which is similar to LTE SC-PTM transmission. In this mechanism, CRC of PDCCH is scrambled by a common RNTI (e.g., G-RNIT) and the PDSCH is also scrambled by the common RNTI. Nine companies proposed to consider this mechanism for MBS.

The second is UE-specific PDCCH based group scheduling. In this mechanism, CRC of PDCCH is scrambled by C-RNTI, but different PDCCHs schedule a group common PDSCH for a group of UEs. From UE’s perspective, it is the same as unicast transmission. Three companies proposed to also consider this mechanism.

One company proposed to also consider sub-G-RNTI PDCCH based group scheduling, in which sub-G-RNTI can be used to scramble a sub-group common PDCCH for a small group scheduling. This mechanism seems to fall into group-common PDCCH based group scheduling in high level, if any difference, the details can be considered in later stage.

| group scheduling mechanisms | Companies |
| --- | --- |
| Option 1: group-common PDCCH based group scheduling | Huawei, Nokia, QC, Convida, ZTE, Intel, CATT, CMCC, LG |
| Option 2: UE-specific PDCCH based group scheduling | CMCC, vivo, CATT |

[High priority] Issue 1 (Question 1 in R1-2007001): Regarding the two high level group scheduling mechanisms, i.e., group-common PDCCH based group scheduling and UE-specific PDCCH based group scheduling, whether down selection is needed or both of them can be considered for MBS for RRC_CONNECTED UEs?

Please share your views and comments in the table.

| Company | Comment |
| --- | --- |
| TD Tech, Chengdu TD Tech | Support Option 1 : group-common PDCCH based group scheduling Comments on option 2: The PDSCH for unicast is bit-scrambled by C-RNTI. The group common PDSCH is impossible to be entirely same as the PDSCH for unicast. When the number of UEs is large, the PDCCH resource consumed becomes larger than the PDCCH resource used for beam-sweeping in Option 1. |
| vivo | We think both of them can be considered for MBS for RRC_CONNECTED UEs for the following reasons: - group-common PDCCH based group scheduling can be a basic scheduling mechanism, considering there may be quite lots of UEs in an MBS group, this mechanism is beneficial for PDCCH overhead reduction. But this mechanism will introduce many efforts when HARQ-ACK feedback is supported. - UE-specific PDCCH based group scheduling can also be used in some cases, for example, the case that number of MBS UEs is medium and the MBS service reliability requirement is high, i.e., HARQ-ACK is needed for the service. In this case, there is no has no PDCCH blocking issue. Then UE-specific PDCCH can benefit from scheduling and HARQ-ACK feedback perspective as summarized in the following table. In our view, group-common PDCCH mechanism will have large spec impact / standardization effort for HARQ-ACK feedback, multiplexing / prioritization between multicast and unicast, etc. It is more suitable for an MBS with no HARQ-ACK feedback or with only group-specific NACK only feedback. UE-specific PDCCH mechanism can the minimize spec impact / standardization effort for these aspects and is a good scheduling scheme for MBS PDSCH retransmission. Therefore, no down select is needed. Table 1 Comparisons of groupcast PDCCH and unicast PDCCH group-common PDCCH; UE-specific PDCCH. PDCCH overhead; Low; high. Search space configuration; Larger spec impact; No spec impact. DCI size alignment; If DCI 1_1/1_2 is used, it may have impact on DCI size alignment If DCI 2_x is used, other group common DCI need to align the DCI payload size with it, which may reduce the PDCCH performance; Same as unicast PDSCH, no additional impact. Impact on HARQ-ACK feedback; PUCCH resource; Hard to indicate orthogonal PUCCH resources due to the same PRI value.. HARQ-ACK feedback timing; Same timing, all UEs in an MBS group will feed back HARQ-ACK in the same slot, resulting PUCCH overload and collision. TPC; Difficult to indicate different UEs’ TPC using one single DC. Impacts when considering simultaneous receptions of MBS PDSCH and unicast PDSCH; DAI; Separating DAI counting for groupcast PDSCH and uncast PDSCH. HARQ-ACK Codebook; Separate or joint HARQ-ACK codebook needs to be discussed/specified. if separate codebook for MBS PDSCH and unicast PDSCH, multiplexing/prioritization of different codebooks needs to be discussed. RNTI; if UE is interested in multiple MBS services, UE needs to monitor a PDCCH with multiple g-RNTIs. |
| CMCC | Both group-common PDCCH based group scheduling and UE-specific PDCCH based group scheduling can be supported for MBS for RRC_CONNECTED UEs. It is noteworthy that the decision on which group scheduling mechanism should be supported should not only consider the overhead of PDCCH, but also the other aspects, e.g., the standization effort or spec impact to support HARQ-ACK feedback if it is supported, the PUCCH resource utilization efficiency for HARQ-ACK feedback, etc. From the comparison in the following table, we can see that, on the one hand, the PDCCH overhead of UE-specific PDCCH based scheme is larger than that of group common PDCCH based scheme, on the other hand, the standardization efforts for UE-specific PDCCH based scheme to support ACK-NACK based HARQ feedback is much less than group common PDCCH based scheme. Group common PDCCH based scheme; UE-specific PDCCH based scheme. PDCCH overhead; Small; Larger. PUCCH resource configuration for HARQ-ACK; Less configuration flexibility, less PUCCH resource utilization efficiency and larger spec impact - For ACK/NACK feedback, network needs to configure orthogonal PUCCH resources for UEs in the same group by RRC so that a single PRI field in DCI can optionally be used to select separate PUCCH resource for each UE. This will cause that NW has to reserve a certain amount of dedicated PUCCH resources for multicast transmission which cannot be shared with unicast transmission. - Separate PUCCH resource configurations for multicast and unicast may be needed, which will introduce additional spec impact.; More Flexible, more efficiency and little spec impact - The same as for unicast.. HARQ-ACK multiplexing for multicast and unicast / HARQ-ACK codebook construction; More complicated and large spec impact - If HARQ-ACK multiplexing for multicast and unicast in a slot is supported, the joint codebook construction procedure is much more complicated and will induce more spec impact whether for semi-static or dynamic HARQ-ACK codebook. - If HARQ-ACK multiplexing for multicast and unicast in a slot is not supported, HARQ-ACK for multicast or unicast has to be dropped which may impact the performance, or it has to be based on network implementation to avoid collision of HARQ-ACK feedback for multicast and unicast, which will introduce additional scheduling restriction.; NO spec impact - Can reuse Rel-15/Rel-16 HARQ codebook construction procedure.. Considering the pros and cons of these two group scheduling schemes, we think they can be used for different use cases. - For the case with relatively less number of UEs in a MBS group (but multicast still has advantage on spectral efficiency compared with unicast transmission), it is more suitable to use UE-specific PDCCH group scheduling together with ACK/NACK based HARQ-ACK feedback to improve reliability with little spec impact and managable PDCCH / PUCCH overhead. - For the case with large number of UEs in a MBS group, it is more suitable to use group common PDCCH scheduling for which NACK only based HARQ-ACK feedback can be used to improve reliability with small PDCCH / PUCCH overhead. Therefore, we think both two group scheudling mechanisms can be supported. |
| LG | We think that group scheduling with G-RNTI is beneficial for the newtork especially when multiple UEs interested in a service can receive same TBs of the service via same time/frequency resources. However, it would not be always possible for certain UEs. Thus, it is also good to consider UE specific scheduling with C-RNTI. |
| Nokia | For RRC Connected mode UEs, in general we prefer option 1, because of the lower downlink signaling overhead. However, we can envisage of some scenarios where option 2 may also be beneficial to support additional feedback from certain UEs. In those scenarios, the UE could be configured with an additional USS to support additional UE specific reliability information. |
| ZTE | We suggest to support group-common PDCCH only. Compared with UE-specific PDCCH, group-common PDCCH can save lots of resources as all UEs in one group share the same PDCCH. Besides, for broadcast for RRC_IDLE/RRC_INACTIVE UEs, only group-common PDCCH (or more specifically, cell-common PDCCH) can be applied. It is preferred to keep commonality between broadcast and multicast. That is, group-common PDCCH needs to be supported anyway considering broadcast service scheduling, and more standardization work will be needed for additional support for UE-specific PDCCH. We would also like to mention that the UE-specific PDCCH based solution may have RAN2 impact. Based on our understanding, UE MAC layer needs to know the G-RNTI (corresponding to a type of service/logical channel) of each PDSCH. If UE-specific PDCCH based solution is introduced, then how can the network indicate the G-RNTI to the UE? |
| OPPO | Both options should be considered. Option 1 is beneficial in terms of PDCCH overhead reduction, however, there are some cases where gNB cannot transmit group-common PDCCH to a specific UE, e.g. UE active BWP is different from MBS BWP, or to multiplex HARQ-ACK for unicast and MBS in a single PUCCH, therefore UE-specific PDCCH is also needed. Furthermore, it is preferable to make the definition of group-common PDCCH and UE-specific PDCCH clearer, from our point of view, group-common PDCCH should be transmitted in CSS and scrambled by a RNTI shared by multiple UEs (e.g. G-RNTI), while UE-specific PDCCH should be at least transmitted in USS, as to the RNTI for scrambling the PDCCH, it should not be restricted to C-RNTI only at this stage. |
| Qualcomm | At least Option 1 should be supported, since Option 1 is more flexible to schedule a common PDSCH for a small group or a large group of UEs with the same PDCCH overhead. Option 2 does not scale for a very large number of UEs. FFS Option 2, which may be useful to schedule PDSCH for a small group of UEs or to schedule the retransmission of multicast data. |
| Ericsson | We prefer option 1 (group-common PDCCH based group scheduling), and see no need to further study “UE-specific PDCCH based group scheduling” |
| BBC | At this stage we propose to consider both options. Various companies mention that Option 1 has benefits for a large group of UEs while Option 2 has benefits for reduced number of UEs. However, it is not clear to us the number of UEs best supported by each option. Therefore, down-selection or support of both options could be confirmed after more studies have been conducted. |
| Intel | Both CSS and USS based scheduling should be supported. - Group common PDCCH based scheduling can be considered as the baseline. As mentioned before, this option is good for the cases when large number of UEs are grouped to receive multicast PDSCH. Type 3 CSS can be used with addition of at least a G-RNTI to the set of RNTIs which can scramble the DCI of the monitored PDCCH. Alternately a new CSS type can also be defined. This mechanism has the advantage of lower overhead, but support of HARQ will be limited to possibly NACK-only on a shared PUCCH resource. This can still be ok since network only needs to detect at least one NACK in order to trigger retransmission. - USS can be used when the number of UEs is lower and the overhead due PDCCH is acceptable. With this mechanism ACK/NACK based HARQ can also be supported. Furthermore, USS can also be used for potentially scheduling retransmission to a sub-group of users. The RNTI for monitoring a multicast PDCCH should not however be limited to only C-RNTI. |
| Convida | We support to do down selection between option1 and option2. We support option 1 Group-common PDCCH based group scheduling. Group common PDCCH has advantage of lower signalling overhead. In addition, group common PDCCH can be considered as a universal approach which can be applied to both multicast and broadcast use cases. |
| Spreadtrum | Both options have advantages respectively, Group-common PDCCH based group scheduling is beneficial for PDCCH overhead reduction, but needs more standardization work for HARQ feedback; UE-specific PDCCH based group scheduling has little spec impact, but has more PDCCH overhead. Based on the latest agreements from RAN2: Focus initially on NR SA, TBD to what extent other scenarios NR DC, NE DC can be supported. Confirm Will support PTM transmission in a cell. Confirm that We will, for multicast services introduce support for PTP and PTM transmission of shared traffic delivered by 5GC, at least for connected mode (does not exclude other cases) For a UE, gNB dynamically decides whether to deliver multicast data by PTM or PTP (Shared delivery) It seems that both options from RAN1 should not be excluded to support PTP and PTM transmission. |
| Huawei/HiSilicon | Both options are discussed as above. Option 1 in our opinion should be the essence of PTM so should be at least supported. On top of option 1, whether option 2 is supported needs justification. Therefore, we suggest we can conclude at least option 1 is supported and FFS on option 2. |
| CATT | Both options can be supported in NR MBS for different use cases. The pros and cons are anaylyzed in many contributions for both options, and each option has its own advantages in specific scenarios. There is no need to limit just one scheduling mechanism for NR MBS, and flexibility should be considered by supporting both options. For sub-group-common PDCCH based group scheduling, as FL mentioned that it seems to be defined as group-common PDCCH scheduling in high level. There is some difference between sub-group-common PDCCH and group-common PDCCH, e.g. use cases and RNTI definitions. To make sure that any other potential scheduling mechanism is not precluded, the sub-group scheduling mechanism should be added. While considering the two options, how to indicate/utilize PUCCH resources for HARQ-ACK feedback should be also taken into account. Different cases can be classified as: - Group-common PDCCH to schedule PDSCH + Group-common PUCCH - Group-common PDCCH to schedule PDSCH + UE-specific PUCCH - UE-specific PDCCH to schedule PDSCH + UE-specific PUCCH - UE-specific PDCCH to schedule PDSCH + Group-common PUCCH |
| Samsung | Group-common PDCCH should be supported. I think we can more focus on the essential scheduling for multicast. |

Reliability improvement mechanisms for RRC_CONNECTED UEs

Based on companies’ submitted contributions, three reliability improvement mechanisms have more supporters than others as illustrated in the following table, including HARQ-ACK feedback, CSI feedback and PDSCH repetition.

Regarding HARQ-ACK feedback, nine companies suggested to support it for at least multicast for RRC_CONNECTED state, some companies [Huawei, HiSilicon] have submitted some simulation results for justifying the benefits of HARQ-ACK feedback, one company proposed to study the potential gain and standardization impact for HARQ-ACK feedback,

Regarding CSI-feedback, six companies proposed that CSI feedback can be supported to improve reliability. Four of them [CMCC][VIVO][CATT][ZTE] think the existing CSI-RS configuration and CSI feedback mechanism for unicast can be directly used for MBS without additional spec impact. Two of them [QC][E///] think some modifications may be needed.

Regarding PDSCH repetition, some of the proponents think the existing PDSCH repetition mechanism for unicast can be reused for MBS.

Regarding multi-beam/beam sweeping operation, two companies [Sony, CATT] mentioned it in the sub-agenda for reliability improvement. Beam sweeping was also raised in some contributions for RRC_IDLE/INACTIVE UEs and two other companies [ZTE, LG] also raised similar issue for group scheduling, it can be discussed later in which sub-agenda it should be discussed.

Each of other potential mechanisms only have one proponent, including conservative scheduling based on network implementation, multi-DCI based M-TRP transmission and HARQ-based time-interleaving.

| Reliability improvement mechanisms | Companies |
| --- | --- |
| HARQ-ACK feedback | CMCC, Huawei, OPPO, vivo, CATT, Convida, QC, E///, Samsung, Nokia |
| CSI feedback | CMCC, E///, CATT, vivo, QC, ZTE |
| PDSCH repetition | CMCC, ZTE, Intel, vivo, LG, Nokia |
| Multi-beam/beam sweeping operation | CATT, Sony |
| Conservative scheduling (network implementation) | ZTE, Nokia |
| Multi-DCI based M-TRP transmission | LG |
| HARQ-based time-interleaving | BBC |

[High priority] Issue 4 (Proposal 1 in R1-2007001, with little update): For RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast without additional evaluation for it, i.e., no evaluation is needed to justify whether HARQ-ACK feedback is needed.

Please share your views and comments in the table.

| Company | Comment |
| --- | --- |
| TD Tech and Chengdu TD Tech | The views and comments of ours are listed in the table below. Reliability improvement mechanisms; Companies: TD Tech and Chengdu TD Tech. HARQ-ACK feedback; supported. CSI feedback; Comments: The further discussion and simulation are needed to make the decision.. PDSCH repetition; Supported. Multi-beam/beam sweeping operation; Supported. Conservative scheduling (network implementation); Comments: The further discussion and simulation results are needed to make the decision.. Multi-DCI based M-TRP transmission; Comments: The further discussion and simulation results are needed to make the decision.. HARQ-based time-interleaving; Comments: The further discussion and simulation results are needed to make the decision.. |
| vivo | We support the proposal. Diffentent from LTE, NR MBS support many use cases, and some case, such as V2X or Industry applications have very high reliabiility requirement, e.g. 99.9999%. It is impossible to meet this requirement simply using link adaption based on CSI feedback only or automatic repetition. Considering this perspective, HARQ-ACK is anyway needs to be supported. No evaluation is needed to justify this considering the limited TU for MBS WI. Table 1. Requirements for different MBS use cases MBS use cases; Latency; Reliability. V2X; 5-100ms; 90% to 99.9999%. Live Video; 150ms; 99.9%. IOT Software update; Latency Tolerant; Higher reliability is beneficial. Industry applications; 0.5ms; 99.9999%. |
| CMCC | Support the proposal. - In Rel-13 LTE SC-PTM, simulations had been carried out to evaluate the gain of HARQ-ACK feedback and justify that HARQ-ACK can improve the reliability. The evaluations for SC-PTM aimed at supporting multicast/broadcast service for group communications as defined in 3GPP TS 22.468 and mission critical push to talk as defined in 3GPP TS 22.179, which are consistent with the requirement mentioned in the WID of NR MBS. In addition, considering both R17 NR MBS and LTE SC-PTM aim to support single cell multicast/broadcast, we think the evaluation results and observation for HARQ-ACK feedback in LTE SC-PTM are also applicable for R17 NR MBS. - In addition, NR MBS also aims to support more services which may require much higher reliability, e.g., V2X applications. The higher reliability the service requires, the more adavatages can be expected from HARQ-ACK feedback. Therefore, we think the HARQ-ACK feedback should be supported for CONNECTED UEs, and no additional evaluation is needed to justify whether to support it. |
| LG | If legacy UE specific scheduling is used for a multicast TB, it can already support HARQ-ACK feedback. Thus, HARQ-ACK feedback can be supported for multicast without additional evaluation at least for UE specific scheduling. |
| Nokia | Additional evaluation for HARQ-ACK is desirable, preferably with some minimum reliability to target. In our view, there are many variants of HARQ-ACK feedback that could be supported. We would like to see evaluations of these techniques compared against each other (justifying a common set of simulation assumptions) and against some minimum reliability targets. Subject to those evaluations, RAN1 can then make an informed decision about the support of HARQ-ACK. |
| ZTE | We suggest to evaluate the potential gain for HARQ-ACK for NR MBS first. In LTE SC-PTM, some preliminary evaluations on spectrum efficiency between ‘SC-PTM with CQI and HARQ ACK/NACK feedback’ and ‘SC-PTM with CQI feedback only and eOLLA’ had been done. Conclusions were made in TR 36.890 as follows. - Conclusions ... SC-PTM performance in terms of spectral efficiency was evaluated by the simulations. The performance analysis results in the following conclusions: - Link adaptation allowed by the availability of UL feedback provides significant gains when the number of receiving UEs is rather small and decreases with increasing group size. HARQ with retransmissions can further improve the spectral efficiency in some scenarios but these improvements are small. It has not been concluded whether the gains provided by HARQ and retransmission are worth of the increased complexity of the system.. For NR MBS, the deployment scenarios have changed compared with LTE SC-PTM. The simulation assumptions, e.g., frequency, bandwidth, traffic model and antenna configuration, etc., should be updated accordingly. It is questionable on whether/under which conditions/how much gain can be achieved by supporting HARQ-ACK feedback under the new simulation assumptions. So some more careful evaluations on the gain of supporting HARQ-ACK feedback in NR MBS can be done prior to specific mechanism design. With more careful evaluation, companies can even compare the potential performance gain between different HARQ-ACK solutions, which is also beneficial to subsequent technical discussion if HARQ-ACK is supported. |
| OPPO | Support. HARQ feedback has already been supported in groupcast of NR sidelink as an effective mechanism to improve sidelink reliability, this can be used as baseline for HARQ feedback design in MBS. |
| Qualcomm | For multicast service with high reliability requirement (e.g., smart grid control for group communications and IoT applications, V2X applications, etc.), it is necessary to support L1 retransmission based on HARQ-ACK feedback to receive multicast transmission in RRC_CONNECTED state. We believe it is not sufficient to rely on link adaptation using CQI feedback only to combat bursty interference. Based on the previous LTE study and the proposals so far, we think HARQ-ACK feedback should be supported for NR multicast (no need further simulation-based evaluation just for this). We can further discuss the details in upcoming meetings, e.g. whether to support NAK-based or ACK/NAK-based. |
| Ericsson | We agree that HARQ-ACK should be supported without additional evaluation to justify this. The particular solution for HARQ-ACK is however FFS |
| BBC | We also agree that HARQ-ACK can be supported for RRC_CONNECTED without additional evaluation. However, the specific HARQ-ACK solution needs to be selected based on further studies. |
| Intel | Configurable HARQ/ACK feedback can be supported for RRC_CONNECTED UEs i.e., it may be switched off by configuration. This can be useful for cases when RRC_IDLE UEs are also supported in the group or repetition is used as a reliability mechanism. Potential gains for HARQ/ACK, as well as the specific HARQ/ACK technique to be used can be further studied and evaluated. |
| Convida | Support the proposal. HARQ-ACK feedback can be assumed as the baseline in support of reliability enhancement. No additional evaluation may be needed. However, it is not clear if HARQ-ACK feedback will be suitable for all reliability vs latency combinations of MBS use cases, since HARQ-ACK feedback may have some impacts on latency in some cases although reliability could be improved. For reliability enhancement, additional methods such as PDSCH repetitions could also be considered. |
| Spreadtrum | We also agree that HARQ-ACK can be assumed as the baseline for RRC_CONNECTED without additional evaluation. HARQ-ACK solution needs be further studied. |
| Huawei/HiSilicon | We support the proposal. |
| CATT | We agree with the proposal. - To improve the reliability, HARQ-ACK and SCI feedback should be considered. There is no additional evaluation needed. - The gain of HARQ-ACK feedback is depending on the number of UEs and channel condition. It is suggested that gNB decide the enable/disable of UEs based on CSI reporting. |
| Samsugn | We support FL’s proposal. |

Evaluation

Regarding evaluation, four companies have contributions on evaluation in the “Others” sub-agenda. One company [ZTE] provided the evaluation results to support CSI feedback, one company [Huawei] provided the evaluation results to support HARQ-ACK feedback, one company [Nokia] proposed the methodology and assumptions for evaluation of different UL feedback schemes, and one company [E///] proposed the methodology and assumptions for evaluation of different PTM features.

Before we discuss a common evaluation methodology and assumptions, we need to first determine the purpose of the evaluation campaign.

[High priority] Issue 6 (Question 4 in R1-2007001, with little update): Whether a common evaluation methodology and assumptions are necessary for NR MBS? If the answer is YES, what’s the purpose of the evaluation? And what’s your suggestion on the common evaluation methodology and assumptions?

Please share your views and comments in the table.

| Company | Comment |
| --- | --- |
| TD Tech and Chengdu TD Tech | The related simulaiton is needed to prove the corresponding method is nessesary for the NR MBS. The simulation assumptions shall be discussed to ensure that the simulation results from the different companies can be compared with each other. |
| vivo | From our understanding, a common evaluation methodology and assumptions are not necessary for NR MBS. As above proposed, HARQ-ACK for MBS should be supported without evaluation. Then, the motivation to spend much time to do evaluation is not clear to us. |
| CMCC | Not necessary. As the FL summary about reliability improvement mechanisms for RRC_CONNECTED UEs above, HARQ-ACK feedback, CSI feedback and PDSCH repetition are three major mechanisms. - For HARQ-ACK feedback, as our comment in issue 4, no evaluation is needed to justify whether HARQ-ACK feedback is needed. - For CSI feedback and PDSCH repetition, they are also efficient ways to improve reliability, and we think the spec impact is small and even none. Therefore, we think there is no strong motivation to do any evaluation for NR MBS, and all these three reliability improvement mechanisms can be supported. |
| LG | Some assumptions for evaluations are already provided by a few companies in AI 8.12.5. Thus, it seems good to have common evaluation methodology and assumptions to justify a certain solution, if we cannot easily draw the benefit of the solution or reach an agreement on support of it. |
| Nokia | Yes, to enable a fair, efficient and meaningful comparison of different reliability techniques from different companies. Note, as part of these assumptions/methodology, we would like to see a baseline unicast scenario defined, to allow differences between different simulations to be more readily identified. |
| ZTE | Yes. As mentioned in our tdoc x5437, there are different candidate mechanisms to improve the reliability of NR MBS, e.g., HARQ-ACK, CSI feedback and repetition. The simulation results can further guide the subsequent discussion on whether/how to support reliability improvement. Further, the simulation results can provide some useful information for the operators who are interested in the MBS, e.g., which mechanism to deploy in the practical network if multiple mechanisms are specified for reliability improvement. Regarding the common evaluation methodology and assumptions, we have the following comments. - Using system-level simulations - The Rural and Dense-Urban scenarios can be evaluated considering the main target use cases of Public safety and Mission critical. - Full buffer or periodic deterministic traffic model can be used. - Spectral efficiency or user experienced data rate can be considered as the performance metric. - The details of the simulation assumptions can be fully discussed in the following e-mail discussion. |
| OPPO | Although common evaluation methodology and assumptions are helpful for performance comparison, from procedure point of view, the evaluation methodology work is not part of the WI and it will take up a lot of effort in RAN1, especially evaluation work is not considered when allocating TUs for this WI in RAN. In our view, evaluation results can be provided by companies with evaluation assumptions elaborated as some companies already done, it is not appropriate to go into detailed evaluation methodology and assumptions discussion. |
| Qualcomm | At current stage, we are not sure what the common simulation methodology and assumption will be used for. The evaluation may be needed to compare detailed mechanisms for potential enhancement. It would be preferable to first list up the options with clear description and then to discuss the assumption for further comparison, e.g. if there is a proposed new MIMO codebook for multicast, then we need evaluation to see the relative gain to the existing codebook. Depending on the scheme, evaluation methodology may be based on LLS or SLS. The evaluation assumption may be also variant for different RRC states. For example, if we want to consider implementation-SFN deployments for IDLE mode reception, the assumption would be different than the ones that are being proposed, and probably closer to the methodology used in TR 36.776. |
| Ericsson | We think most aspects of this WI can be agreed on without computer simulation-based evaluations, since most functionality is expected to be legacy PTP functionality that is adapted to the PTM case, or can be decided based on other type of evaluation. However, there may be cases where such computer simulations are necessary. In these cases it is important to have a common methodology and assumptions. There is however no urgency in agreeing this. Such agreements may be made when the need arises. |
| BBC | As in our response to Issue 4, we do not think simulations are needed to confirm the gains of HARQ-ACK. However, simulations may be needed to select a specific solution of HARQ-ACK. In which case, we think a common evaluation methodology and assumptions are necessary for NR MBS. |
| Intel | Baseline simulation assumptions are good to have. For evaluations for reliability improvements, especially CQI feedback, repetition and HARQ/ACK schemes, it might be advantageous to agree on baseline SLS assumptions with respect to traffic models, deployment scenarios etc., to align results from companies for fair comparison. |
| Convida | As discussed in issue 4, we think HARQ-ACK feedback should be supported without further evaluation. However, whether HARQ-ACK feedback alone is sufficient in order to meet reliability versus latency requirements for all NR MBS use cases might need some further evaluation. |
| Spreadtrum | For HARQ-ACK feedback, major companies think no further evaluation is need. For other reliability mechanisms, if supported, we also think there is no strong motivation to do any evaluation. |
| Huawei/HiSilicon | As commented to issue 4, we don’t see necessity of evaluations for HARQ-ACK nor other techniques so far. If later on, evaluation is needed for some potential enhancement within the WID scope, proponents can report the evaluations. |
| CATT | We do not observe the necessity of evaluation for NR MBS, e.g. HARQ-ACK feedback. |
| Samsung | We think RAN1 may not need evaluation in some topics. If evaluation is needed, then the common assumption is needed. However, before that, we need to have the common understanding of the purpose of the evaluation, e.g., in order to compare what. |

Initial Proposals (2nd round of email discussion)

18 companies have provided their views on high priority issue 1/4/6 in 1st round input, the following observation is made based on these inputs:

Observation:

- For issue 1:

  - 9 companies [vivo, CMCC, LG, Nokia, OPPO, BBC, Intel, Spreadtrum, CATT] think both option 1 and option 2 can be considered for RRC_CONNECTED UEs. One of them [OPPO] suggests to make the definition of two group scheduling schemes clearer.

  - 5 company [QC, Huawei, HiSilicon, MTK, Samsung] thinks at least option 1 should be supported, and FFS for option 2 to keep it open for consideration.

  - 4 companies [TD Tech, ZTE, Ericsson, Convida] support option 1 only. Three of them [ZTE, Ericsson, Convida] are also fine to at least support option 1 and FFS for option 2.

- For issue 4:

  - 15 companies support the proposal.

  - 1 company [ZTE] suggests to evaluate the potential gain of HARQ-ACK feedback. 1 company [Nokia] thinks additional evaluation is desirable to compare different HARQ-ACK schemes with some minimum reliability targets. 1 company [Intel] thinks potential gains for HARQ-ACK feedback and the specific HARQ-ACK schemes to be used can be further studied and evaluated.

- For issue 6:

  - 10 companies [vivo, CMCC, OPPO, QC, Spreadtrum, Huawei, HiSilicon, MTK, CATT, Samsung] think the motivation to agree on common evaluation methodology and assumptions at current stage is not clear. One of them mentioned the evaluation methodology and assumptions may be different for different proposed schemes.

  - 1 company [E///] thinks it is not urgent to agree on common evaluation methodology and assumptions at current stage, and such agreements may be made when the need arises.

  - 7 companies [TD Tech, LG, Convida, Nokia, ZTE, Intel, BBC] think it is good to have common evaluation methodology and assumptions. The purpose could be, e.g., to verify whether other reliability improvement schemes except HARQ-ACK feedback are needed [TD Tech, LG, Convida], or to enable meaningful comparisons of different reliability improvement schemes from different companies [Nokia, ZTE, Intel], or to select a specific solution for HARQ-ACK feedback [BBC].

Based on the above observation, the following initial proposals are made:

- Potential Proposal 1 for issue 1: Both group-common PDCCH based group scheduling and UE-specific PDCCH based group scheduling can be considered for MBS fFor RRC_CONNECTED UEs

  - The general description of two group scheduling mechanisms are clarified as follows:

    - Group-common PDCCH based group scheduling:

      - For RRC_CONNECTED UEs in the same MBS group, the PDSCH of a MBS TB is common for the group of UEs and it is scheduled by a group-common PDCCH with CRC scrambled by a common RNTI (e.g., G-RNTI, sub-G-RNTI).

    - UE-specific PDCCH based group scheduling:

      - For RRC_CONNECTED UEs in the same MBS group, the PDSCH for a MBS TB is common for the group of UEs, and it is scheduled by each UE-specific PDCCH with CRC scrambled by UE-specific RNTI (e.g., C-RNTI, MCS-C-RNTI, etc.) for each UE.

- Potential Proposal 2 for issue 4: For RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast and no additional evaluation is needed to justify this.

- Potential Proposal 3 for issue 6:  Take the following high level evaluation methodology and assumptions as starting point for potential evaluations in MBS.

  - System-level simulation is recommended

  - Evaluation scenarios: Rural and Dense-Urban scenarios for FR1 defined in TR38.901.

  - FFS: Which traffic model is used

    - Option 1: CBR traffic model

    - Option 2: Periodic deterministic traffic model

    - Option 3: Full buffer

  - FFS: Performance metrics

  - FFS: The details of the simulation assumptions

  - FFS: Which reliability improvement scheme(s) needs evaluation

    - Note: No evaluation is needed to justify the support of HARQ-ACK feedback for RRC_CONNECTED UEs

Companies can comments directly in the email thread or in the table below.

| Company | Comment |
| --- | --- |
| Huawei/HiSilicon | For proposal 1, if option 1 is supported, I fail to see the necessity to additionally support option 2 at this stage. Since no companies object option 1, at least we can agree on option 1 and FFS on option 2. (Note: there are some typos in the proposal 1 basically for RNTIs..) Ok with proposals 2&3. |
| MTK | For issue 1: For NR MBS, group-common PDCCH based group scheduling can be a basic mechanism, which can reduce PDCCH resource overhead and access more UEs. So, we share the same view with QC/HW, at least optional 1 is supported and option 2 is FFS. For issue 4: In R16 V2X groupcast communication, it had already introduced the HARQ-ACK feedback mechanism to ensure the reliability. So, it is best to reuse HARQ-ACK feedback mechanism directly and no need to spend extra time to evaluate it. We can mainly focus on how to design the feedback mechanism in NR MBS. For issue 6: As our mentioned in Phase 1 stage and issue 4 above, we think there is no need to do any simulation evaluation for MBS. So, I suggest to revise proposal 3 based on newest comments. |
| CATT | For proposal 1: - Both options can be supported based on different scenarios, e.g. different number of UE receiving the same MBS. Both options have its benefit in terms of specific scenarios, e.g. HARQ resource indication, signaling overhead. The network can decide which kind of scheduling mechanism can be used. Additionally, both options should also consider about PUCCH resources for HARQ feedback. - Group-common PDCCH to schedule PDSCH + Group-common PUCCH - Group-common PDCCH to schedule PDSCH + UE-specific PUCCH - UE-specific PDCCH to schedule PDSCH + UE-specific PUCCH - UE-specific PDCCH to schedule PDSCH + Group-common PUCCH - For Group common PDCCH, we also think sub-group-common PDCCH can be applied when the number of UEs in the same MBS group is too large and the locations of UEs are scattered. So we would like to suggest to add sub-G-RNTI in the e.g. in the bracket. We are fine of proposal 2 and proposal 3. |
| vivo | We are fine with proposals 1&2. For proposal 1, it is saying both group scheduling mechanism “can be considered”, rather than “are supported”, there is no need to put option 2 in FFS. As commented by many companies, the decision on which group scheduling mechanism should be supported should not only consider the overhead of PDCCH, but also the other aspects, e.g., the standardization effort or spec impact to support HARQ-ACK feedback if it is supported, etc. Now, it seems everybody is ok to support HARQ-ACK for MBS, but there are many variants of HARQ-ACK feedback, we haven’t made decision on which one(s) to be supported. If only group-specific NACK feedback is supported, I admit option 1 may be enough. But if UE-specific HARQ-ACK feedback is supported, the pros for option 2 is very clear. In that case, option 2 should be supported. So, no need to do down selection now. For proposal 3, we still don’t find the reason to have this evaluation methodology and assumptions based on the proposal. In our view, the first thing we should be clear is the purpose of evaluation. If there is no clear purpose now, I think we don’t need to take time on it and there are so many FFS need to be solved if we support this. If proponents think evaluation is needed for some potential enhancements later, they can report the evaluations. |
| ZTE | For proposal 1, considering that group common PDCCH is supported by all companies. We agree with Huawei/MTK that we could first agree on supporting Option1 (group-common PDCCH) and FFS for Option2 (UE-specific PDCCH). For proposal 2, we are generally fine. But we want to clarify the following two aspects: 1. HARQ-ACK feedback contains both ACK/NACK feedback and NACK-only feedback; 2. HARQ-ACK feedback should be supported at least for group-common PDCCH based group scheduling because all the companies agree to support the group-common PDCCH based group scheduling. With this, we would like to propose the following updated proposal. - Potential Proposal 2 for issue 4: For RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast and no additional evaluation is needed to justify this. - HARQ-ACK feedback is supported at least for group-common PDCCH based group scheduling. - FFS ACK-NACK HARQ or NACK-only HARQ For proposal 3, it seems the last Note is not needed as proposal 2 has already clarifies that HARQ-ACK will be supported. Besides, it is not clear whether HARQ-ACK feedback contains both ACK-NACK HARQ and NACK-only HARQ, thus we propose to delete the last Note. |
| Ericsson | For proposal 1, we agree with Huawei. Every company that commented has expressed support for option 1, so we can go ahead with a proposal to support option 1. For option 2, we are OK with keeping the option open for consideration and having it as an FFS. For proposal 2, we agree with the proposal with the understanding that this means that the use of HARQ-ACK as such is agreed without further evaluation, but that different proposals for HARQ-ACK solutions may later need to be evaluated. For Proposal 3 we can agree to that. We wish however to re-iterate that we think most aspects of this WI can be agreed on without computer simulation-based evaluations and we see no urgency. |
| OPPO | - Proposal 1 is general fine for us except some comments: - If this proposal is agreed later, the meaning of group-common PDCCH based group scheduling and UE-specific PDCCH based group should be defined, “general description” reads like only for informative purpose. - For the 2 group scheduling schemes, the current description is from MBS group perspective, however, it seems that in PHY layer there is no such definition for now, maybe to describe from one RRC_CONNECTED UE perspective would be better. - For UE-specific PDCCH based group scheduling, which RNTI is used for scrambling is dependent on further design, we prefer to keep it open now. - In summary, we suggest following changes for Proposal 1: - Potential Proposal 1 for issue 1: Both group-common PDCCH based group scheduling and UE-specific PDCCH based group scheduling can be considered for MBS for RRC_CONNECTED UEs. - The general description of two group scheduling mechanisms are clarified as follows: - Group-common PDCCH based group scheduling: - For an RRC_CONNECTED UEs in the same MBS group, the PDSCH of an MBS TB is common for the group of UEs and it is scheduled by a group-common PDCCH in CSS with CRC scrambled by a common RNTI (e.g., G-RNIT). - UE-specific PDCCH based group scheduling: - For an RRC_CONNECTED UEs in the same MBS group, the PDSCH for an MBS TB is common for the group of UEs, and it is scheduled by each UE-specific PDCCH in USS with CRC scrambled by UE-specific RNTI (e.g., C-RNIT, MCS-C-RSNTI, etc.) for each UE. - Proposal 2: Agree. - Proposal 3: Based on the summary of issue 6 from the moderator, there is no clear majority that a common evaluation methodology is needed. Among the companies who support to do this work, their motivation and purpose are not aligned. Furthermore, there are a number of FFS items listed, what is worse, many aspects and models are not included in the list. All these imply that a significant amount of work is required to come up with a common evaluation methodology in RAN1. This is a work item where study on performance gain is not really a top priority, can we simply rely on company contributions to bring evaluation results to justify the performance to get the work completed? |
| BBC | We are fine with Proposal 1. For Proposal 2, we think at the moment is not clear which specific HARQ-ACK solutions will be supported and this needs further study and discussion. Hence, we suggest the following addition to the Proposal 2 (highlighted in yellow): - Potential Proposal 2 for issue 4: For RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast and no additional evaluation is needed to justify this. FFS specific HARQ-ACK solutions to be supported. We are also fine with the initial assumptions of Proposal 3. |
| Qualcomm | For proposal 1, our observation is as follows: - There is no objection to support Option 1 but no clear consensus on Option 2. - For Option 2, the current description seems only allow using UE-specific PDCCH to schedule a common PDSCH for a group of UEs. We would like to keep other possibilities open, e.g., using UE-specific PDCCH for unicast retransmission of an MBS TB. - Better not to include the details of RNTI and CSS/USS, since they have not been fully discussed yet. Therefore, we recommend the proposal 1 is updated as: Potential Proposal 1 for issue 1: At least support group-common PDCCH with CRC scrambled by a common RNTI to schedule an MBS PDSCH for a group of RRC_CONNECTED UEs. - FFS UE-specific PDCCH for scheduling an MBS PDSCH For proposal 2, we support it. For the guidance of next meeting, we can add ‘FFS’ to further consider the options of HARQ feedback proposed by majority companies. Potential Proposal 2 for issue 4: For RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast and no additional evaluation is needed to justify this. - FFS ACK/NACK-based and NACK-based HARQ feedback For proposal 3, it seems not mature for companies to have consensus in this meeting. We would like to revise the main bullet as Potential Conclusion for issue 6: Companies are recommended to take the following high level evaluation methodology and assumptions as starting point for potential evaluations in MBS. |
| Convida | For proposal 1 for issue 1, we share the same view with HW/MTK/ZTE that we can agree on supporting option 1 first and have option 2 for FFS. For proposal 2, we are fine with the proposal for issue 4. Regarding the comments provided by ZTE and QC, we think that the details on HARQ-ACK feedback mechanism should be further studied and discussed in the future meetings without putting any constrain on it at the early stage. For proposal 3, we are fine with the proposal for issue 6. We are also OK with the approach suggested by Ericsson, e.g., discuss the evaluation methodology and assumptions later when there is a need. |
| Intel | Proposal 1: We are ok to support group common scheduling and have FFS for UE specific scheduling. Proposal 2: We would like to add the following FFS points: - FFS: The details of HARQ feedback schemes that will be supported - FFS: HARQ can be optionally disabled by DCI or RRC configuration We think that while HARQ is useful, there should be an option to configure it ON or OFF. Proposal 3: We are ok with Proposal 3 |
| Moderator | After received more companies’ inputs in 1st round, and also some inputs for the initial proposal in the 2nd round, I updated the proposals for the three issues below this table. For issue 1: It seems everyone agrees to at least support group-common PDCCH scheduling group-common MBS PDSCH. For UE-specific PDCCH based scheduling, most companies can accept to keep it open for consideration, some companies suggest to keep it open for UE-specific PDCCH to schedule group-common MBS PDSCH or UE-specific MBS PDSCH. @Huawei/MTK/ZTE/Ericsson/Convida/Intel: Your concern should be addressed in the updated proposal, please see if it is OK. @QC: Your concern and suggestions are reflected in the updated proposal, please see if it is OK. @CATT: For the PUCCH resource indication scheme for HARQ-ACK feedback, I think it can be discussed later and separately with the current proposal. For sub-G-RNTI, I didn’t directly capture it in the updated proposal since people may don’t know the exact meaning of it and it seems it wasn’t used before, but I think it also belong to the common RNTI in the updated proposal, so hope the updated proposal is OK for you. @vivo/BBC: As I explained, it seems many companies want to first agree the group-common PDCCH based scheme, and the UE-specific PDCCH based scheme is still open with the updated proposal. @OPPO: Regarding the meaning of the group-common PDCCH based group scheduling and UE-specific PDCCH based group scheduling, please see if it is clear with the updated proposal. Regarding the CSS and USS in your suggestion, I didn’t capture them now, since it seems companies have different views on the SS type, and I think it can be discussed later and separately with the current proposal. For issue 2: It seems generally the proposal is OK for most companies. To address some companies’ comments, the proposal was updated to give some guidance for discussion in the next meeting. @QC/ZTE/BBC/Intel: Your comments are reflected in the FFS part. @Ericsson: Yes, I think this proposal didn’t preclude that different proposals for HARQ-ACK feedback solutions may later need to be evaluated. For issue 3: Firstly, based on more companies’ inputs, it seems the majority view becomes that there is no urgency or no clear motivation to agree the common evaluation methodology and assumptions in this meeting. Secondly, few companies provided contributions on the common evaluation methodology and assumptions in this meeting, it seems it is not mature to have a stable agreement on this, since you can see even we try to take a lot of time to discuss in this meeting there may still be many FFS parts. Thirdly, companies who think it is good to have common evaluation methodology and assumptions may not mean to have a stable proposal in this meeting. Although the purpose of the evaluation mentioned by companies are generally related to reliability improvement, but it seems they are not converged enough. Based on the above, I suggest not to spend time on discussing the common evaluation methodology and assumptions in this meeting, and we can move on to discussion the other parts. We still can further discuss the common evaluation methodology and assumptions later after the motivation of the evaluation is clear enough and the group has a good consensus on that. So I deleted the proposal for the time being and we don’t need to have an agreement on this in this meeting. Please see if this is OK for everyone. |

Updated Proposals (3rd round of email discussion)

Based on the above observation and 2nd round input, the proposals are updated (The reason for the update and the response to companies’ comments can be found in the table above):

- Updated Proposal 1 for issue 1: For RRC_CONNECTED UEs, at least support group-common PDCCH with CRC scrambled by a common RNTI to schedule an MBS PDSCH.

  - FFS: whether to support UE-specific PDCCH to schedule an MBS PDSCH which could be UE-specific or common for a group of UEs.

- Updated Proposal 2 for issue 4: For RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast and no additional evaluation is needed to justify this.

  - FFS: The detailed HARQ-ACK feedback solutions, e.g., ACK/NACK based, NACK-only based.

  - FFS: HARQ-ACK feedback can be optionally disabled.

- Potential Proposal 3 for issue 6:  Take the following high level evaluation methodology and assumptions as starting point for potential evaluations in MBS.

  - System-level simulation is recommended

  - Evaluation scenarios: Rural and Dense-Urban scenarios for FR1 defined in TR38.901.

  - FFS: Which traffic model is used

    - Option 1: CBR traffic model

    - Option 2: Periodic deterministic traffic model

    - Option 3: Full buffer

  - FFS: Performance metrics

  - FFS: The details of the simulation assumptions

  - FFS: Which reliability improvement scheme(s) needs evaluation

    - Note: No evaluation is needed to justify the support of HARQ-ACK feedback for RRC_CONNECTED UEs

Companies can provide comments directly in the email thread or in the table below for the updated proposals.

| Company | Comment |
| --- | --- |
| LG | Regarding Proposal 1, the meaning of the MBS PDSCH is not clear to us, epecially with UE specific PDCCH. We think that PDSCH transmssion of a MBS data can be either specific to a single UE (i.e. UE specific PDSCH) or common to a group of UEs (i.e. group common PDSCH). Accordingly, we propose to clarify the Proposal 1 as follows : - Updated Proposal 1 for issue 1: For RRC_CONNECTED UEs, at least support group-common PDCCH with CRC scrambled by a common RNTI to schedule an MBS PDSCH. - FFS: whether to support UE-specific PDCCH to schedule an MBS PDSCH which could be UE-specific or common for a group of UEs for transmission of MBS data. Regarding Proposal 2, if HARQ-ACK feedback can be optionally disabled, it can be optionally enabled. Thus, we propose to clarify the proposal 2 as follows: - Updated Proposal 2 for issue 4: For RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast and no additional evaluation is needed to justify this. - FFS: The detailed HARQ-ACK feedback solutions, e.g., ACK/NACK based, NACK-only based. - FFS: HARQ-ACK feedback can be optionally disabled and/or enabled. |
| Nokia | For proposal 1, we like the LG suggestion but would like to support an additional FFS to support the modification of PUCCH resources (similar to @CATT). - Updated Proposal 1 for issue 1: For RRC_CONNECTED UEs, at least support group-common PDCCH with CRC scrambled by a common RNTI to schedule an MBS PDSCH. - FFS: whether to support UE-specific PDCCH to schedule an MBS PDSCH which could be UE-specific or common for a group of UEs for the transmission of MBS data. - FFS: whether to support UE-specific PDCCH to modify the PUCCH resources used to support the transmission of MBS data. For proposal 2, we support the LG « and/or enabled » suggestion For proposal 3, we are a little surprised this has been completely deleted. We would at least prefer a working assumption, given that : - 8 companies have shown an interest. - Potentially some of these companies can co-author a joint proposal before the next meeting. - In the various LTE releases where Broadcast enhancements were developed, a standard evaulation model was developed. |
| ZTE | Regarding proposal 1, we support the FL proposal. LG and Nokia’s version is also fine, we are ok to add the second FFS point raised by Nokia/CATT if this helps to converge. Regarding proposal 2, we are fine with the current formulation. Regarding proposal 3, as also commented by Nokia, it may be better to keep the proposal (or working assumption) considering there are at least 8 companies showing their interests. Maybe keeping it as a working assumption can be the middle ground. |
| OPPO | We have following comments on updated Proposal 1: - We also prefer the wording proposed by LG, i.e. replace “MBS PDSCH” with “PDSCH for the transmission of MBS data”. - As to the PDSCH scheduled by UE specific PDCCH, whether the PDSCH is UE specific or group common seems a next step issue, to our understanding, whether the PDSCH is UE specific or group common may be up to gNB, and it may be also transparent to UE. What matters here for now is whether a PDSCH for the transmission of MBS data scheduled by UE-specific PDCCH is supported or not. - Updated Proposal 1 for issue 1: For RRC_CONNECTED UEs, at least support group-common PDCCH with CRC scrambled by a common RNTI to schedule an MBS PDSCH for the transmission of MBS data. - FFS: whether to support UE-specific PDCCH to schedule an MBS PDSCH which could be UE-specific or common for a group of UEs for the transmission of MBS data. We are fine with updated proposal 2, to add “and/or enabled” at the end of the second FFS is also fine for us. We support to remove Proposal 3. |
| CATT | For proposal 1 of issue 1, it is okay for us in principle. The indication of PUCCH is also an important aspect when designing the HARQ-ACK feedback, and both ACK/NACK and NACK-only based mechanism needs UL resource indication. I think it is better to have an FFS in the proposal, to say that PUCCH indication can be discussed later. It would be clear to guide companies when they are preparing the designs in contributions. (similar to @Nokia). Because it is high level discussion in this meeting, and detailed mechanism can be discussed later, we can generally have an FFS about PUCCH indication. - Updated Proposal 1 for issue 1: For RRC_CONNECTED UEs, at least support group-common PDCCH with CRC scrambled by a common RNTI to schedule an MBS PDSCH. - FFS: whether to support UE-specific PDCCH to schedule an MBS PDSCH which could be UE-specific or common for a group of UEs. - FFS: How to indicate PUCCH resources used for HARQ-ACK feedback.. For proposal 2 of issue 4, we are fine with LGE’s suggestion that HARQ-ACK can be enable/disabled. For proposal 3, we are also fine to remove it. |
| Huawei/HiSilicon | Updated proposal 1 is ok in principle, and we like the change from OPPO and keep it more generic under FFS. The FFS suggested by Nokia is unclear to us by “modifying the PUCCH resources”. If the intention is bringing up a new factor to consider UE-specific PDCCH, I don’t see the need to keep this FFS because we have FFS already to UE-specific PDCCH in general. CATT and Nokia echo each other for this point, but the changes are totally different. The change from CATT is not specific to UE-specific PDCCH. HARQ-ACK feedback resources have to be fixed at later anyway regardless of which option adopted, so I failed to see the necessity of the FFS from CATT. Updated proposal is ok and fine with the change of “and/or enabled”. We are ok with deleting proposal 3. |
| vivo | For proposal 1, we like Nokia’s additional FFS suggestion but would like to change “modify” to “indicate/modify”, i.e., FFS: whether to support UE-specific PDCCH to indicate/modify the PUCCH resources used to support the transmission of MBS data. For proposal 2, we support the LG’s “and/or enabled” suggestion We support to remove Proposal 3. |
| Ericsson | Regarding Proposal 1, since « PTP » already exists in NR, the variant of a UE-specific PDCCH scheduling a UE-specific PDSCH does not need to be mentioned. Further, the acronym « MBS » is more related to the IP Multicast service as such and does not have any relation to the physical layer. We prefer instead to talk about « PTP and « PTM » or « individual transmissions » and « group transmission ». This leads us to the following proposed reformulation of Proposal 1 : - For RRC_CONNECTED UEs, at least support group-common PDCCH with CRC scrambled by a common RNTI to schedule a group-common PDSCH, using the same common RNTI. . - FFS: whether to support UE-specific PDCCH to schedule a group-common PDSCH. - NOTE: The use of a UE-specific PDCCH to schedule a UE-specific PDSCH is already supported by NR and does not need agreement. Regarding Proposal 2 we are fine with LG’s updated proposal. Regarding Proposal 3 we are fine with the moderator’s proposal and think it may be difficult to reach consensus at this meeting. We prefer to concentrate on other topics for the remaining part of this meeting. |
| CMCC | For proposal 1, we are fine with the LG’s version about proposal 1. Regarding Nokia and CATT’s comments, it seems they are not the same thing. For “support UE-specific PDCCH to modify the PUCCH resources” added by Nokia, it is not clear what does that mean, especially considering that it hasn’t been discussed how to indicate the PUCCH resources for the HARQ-ACK feedback for group-common PDCCH based group scheduling. It seems a little bit earlier to discuss how to modify the PUCCH resources, and it may not easy to converge now. Regarding CATT’s comments, we tend to agree with Huawei, anyhow we will discuss how to indicated PUCCH resource for HARQ-ACK feedback in the next step, and it seems not necessary to add the FFS “How to indicate PUCCH resources used for HARQ-ACK feedback”. Regarding OPPO’s second comment for proposal 1, for UE-specific PDCCH, we think it is not transparent to UE whether the PDSCH is UE-specific or group-common, because the scrambling initialization for UE-specific PDSCH and group-common PDSCH is not the same. Companies have explained the benefit of UE-specific PDCCH scheduling group-common PDSCH, especially the less spec impact on support of ACK/NACK based HARQ-ACK feedback, so we think it is better to keep “PDSCH which could be UE-specific or common for a group of UEs” in the second FFS to guide the discussion in next meeting. For proposal 2, we are fine with the change of “and/or enabled”. For proposal 3, we are fine to delete it. |
| Nokia | Proposal 1: We would be happy with CATT’s latest proposal. FFS: How to indicate PUCCH resources used for HARQ-ACK feedback. |
| MTK | For Proposal 1, we are fine with moderator’s proposal. For Proposal 2, we prefer LG’s modification, adding “and/or enabled” for HARQ feedback mechanism. The HARQ ACK/NACK “disabled/enabled” indication is used in NR V2X broadcast or group cast communication. It also can be reused in NR MBS. Thus, LG’s updated proposal is more reasonable. For Proposal 3, we support to remove this proposal. |
| Qualcomm | For updated Proposal 1, we are ok to clarify the MBS PDSCH suggested by LGE. For the main bullet, it’s better to add ‘a PDSCH common for a group of UEs for transmission of MBS data’. Regarding other suggestions, we think how to support HARQ-ACK feedback by using PDCCH has been included in the FFS of Proposal 2. No need to add more FFS here. - Updated Proposal 1 for issue 1: For RRC_CONNECTED UEs, at least support group-common PDCCH with CRC scrambled by a common RNTI to schedule a PDSCH common for a group of UEs for transmission of MBS data. - FFS: whether to support UE-specific PDCCH to schedule a PDSCH which could be UE-specific or common for a group of UEs for transmission of MBS data. For updated Proposal 2, the LGE’s updates look good to us. For the issue 6, we are fine with moderator’s suggestion since it seems lack of common understanding to have any working assumption or agreement in this meeting. |
| Moderator | For issue 1 : - Regarding the suggestion from LG/Nokia/ZTE/OPPO/Huawei/Qualcomm to replace “MBS PDSCH” with ”PDSCH with transmission of MBS data”, It was incorporated in the updated proposal. - Regarding the suggestion from Nokia/CATT to add another FFS, i.e., “FFS: How to indicate PUCCH resources used for HARQ-ACK feedback”, there are different views from Huawei/CMCC/Qualcomm who think it is not necessary since anyhow the PUCCH resource for HARQ-ACK feedback will be discussed in next step, so I didn’t capture it in the updated version. - Regarding the suggestion from OPPO/Huawei to keep it generic as “UE-specific PDCCH to schedule a PDSCH“ instead of “UE-specific PDCCH to schedule a UE-specific PDSCH or a group-common PDSCH“, I think it would be good to provide companies some guide for the next step discussion. This is also related to Ericsson’s comment. - Regarding Ericsson’s comments, I try to reflect them in the latest version. I understand the use of a UE-specific PDCCH to schedule a UE-specific PDSCH is already supported by NR, however, as mentioned by Qualcomm in the 2nd round of inputs, it seems they are thinking using UE-specific PDCCH for unicast retransmission of an MBS TB. I provide two options here on proposal 1 for companies to select. From my point of view, both of them are OK. Please share your views on them. For issue 2 : It seems everyone is ok with LG’s revision, so it was incorporated in the latest version. For issue 3 : Two companies proposed to keep the proposal as an working assumption, and the other companies are fine to delete it. Although I don’t think we can easily have consensus on this, but I can give a last try to see if companies can accept it as an working assumption. I also deleted some of the FFS parts, since it seems some companies have concern on so many FFS parts. If there is objection to keep it as working assumption, we will not spend time on this discussion any more in this meeting. |

Updated Proposals (4th round of email discussion)

Based on the 3rd round of inputs, the proposals are updated (The reason for the update and the response to companies’ comments can be found in the table above):

- Updated Proposal 1 for issue 1:

- Option 1: For RRC_CONNECTED UEs, at least support group-common PDCCH with CRC scrambled by a common RNTI to schedule a group-common PDSCH, using the same common RNTI, for transmission of MBS data.

  - FFS: whether to support UE-specific PDCCH to schedule a UE-specific PDSCH or group-common PDSCH  for transmission of MBS data.

- Option 2: For RRC_CONNECTED UEs, at least support group-common PDCCH with CRC scrambled by a common RNTI to schedule a group-common PDSCH, using the same common RNTI.

  - FFS: whether to support UE-specific PDCCH to schedule a group-common PDSCH.

- Updated Proposal 2 for issue 4: For RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast and no additional evaluation is needed to justify this.

  - FFS: The detailed HARQ-ACK feedback solutions, e.g., ACK/NACK based, NACK-only based.

  - FFS: HARQ-ACK feedback can be optionally disabled and/or enabled.

- Potential Proposal 3 for issue 6:  (Working assumption) Companies are recommended to take the following high level evaluation methodology and assumptions as starting point if evaluations in MBS are needed.

  - System-level simulation is recommended

  - Evaluation scenarios: Rural and Dense-Urban scenarios for FR1 defined in TR38.901.

  - FFS: The details of the simulation assumptions

  - FFS: Which reliability improvement scheme(s) needs evaluation

Companies can provide comments directly in the email thread or in the table below for the updated proposals.

| Company | Comment |
| --- | --- |
| Convida | For updated proposal 1 for issue 1, we are fine with both options. Regarding the FFS suggested by Nokia and CATT, we agree with QC/HW/CMCC that it has already been covered by the FFS in the updated proposal 2 by moderator. Also, the motivation and meaning of the FFS proposed by Nokia is not clear to us. We agree with the moderator that those FFS should not be captured in the updated proposal. For updated proposal 2 for issue 4, we are fine with the moderator’s proposal. For updated proposal 3 for issue 6, we slightly prefer the previous proposal by moderator, i.e., removing and not keeping the entire proposal for proposal 3. |
| Intel | For proposal 1, we ok with Option 1 since it’s a little more general at this stage of discussion. We are ok with Proposal 2. We are also ok with Working assumption for proposal 3, since we think harmonized assumptions might be useful for aligning evaluation results. |
| Lenovo/Motorola Mobility | For Proposal 1, it seems both the main bullets of option 1 and option 2 are same and the difference is only in the FFS part, right? Proposal 2 is fine with us. For Proposal 3, we tend to remove it, i.e., keep previous proposals by moderator. |
| Qualcomm | For Updated Proposal 1, we prefer Option 1. For Updated Proposal 2, we support it. For Potential Proposal 3, we prefer to remove it. If companies want to have something to guide further discussion in next meeting, we prefer to take it as a Conclusion rather than a WA. |
| Spreadtrum | For updated proposal 1, the FFS proposed in these two options are not clear to us, for option1, it means that only one of them will be considered or both of them can be considered? For option2, UE-specific PDSCH is default to be supported and only group common PDSCH is FFS? For updated proposal 2, we are ok. For updated proposal 3, we also prefer the previous proposal by moderator, i.e., removing it, |

Email discussion on medium priority issues (Phase 3)

Company Views (1st round of email discussion)

High level issue related to BWP

For RRC_CONNECTED UEs, different UEs may have different active BWPs for unicast transmission, while for MBS different UEs in the same MBS group need to receive the same group common PDSCH, so how to configure the frequency resources for MBS needs to be discussed.

Two options are proposed by companies for frequency resource configuration for NR MBS for RRC_CONNECTED UEs.

- Option 1: Introduce a MBS specific BWP.

- Option 2: Define a common frequency resource for MBS confined within UE’s active BWP. The common frequency resource allocated to a group of UEs should be within the intersection of the active BWPs of all the UEs within the group. The common frequency resource could be configured per BWP.

The proponents for option 1 think it will be hard to find a common numerology and option 1 has better forward compatibility towards Objective B, while the opponents for option 1 think UE will be required to support two active BWPs for simultaneous operation with unicast reception, and if UE does not support two active BWPs, UE has to switch BWPs back and forth for receiving MBS or unicast, which is usually not desirable due to unnecessary latency caused by BWP switching. One company support option 1. Three or four companies support option 2. Another two companies [OPPO] [LG] also have some BWP related discussions and proposals.

| Options | Companies |
| --- | --- |
| Option 1: Introduce a MBS specific BWP | ZTE |
| Option 2: Define a MBS common frequency resource confined within UE’s active BWP | CMCC, Huawei, Samsung, [vivo] |

[Medium priority] Issue 2 (Question 2 in R1-2007001): Please share your views on the following two alternatives for frequency resource configuration for MBS for RRC_CONNECTED UEs.

- Alternative 1: Introduce a MBS specific BWP

- Alternative 2: Define a MBS common frequency resource confined within UE’s active BWP.

Please share your views and comments in the table.

| Company | Comment |
| --- | --- |
| Qualcomm | We think a common frequency resource need to be defined for MBS reception (in both Alt1 and Alt2). But the key discussion here is whether to reuse the BWP framework or a different framework. In NR, the pdsch-Config is within BWP-DownlinkDedicated. The group-common PDSCH requires many parameter to be configured in a common way, such as PDSCH time/freq resource allocation, scrambling index, MBS table, RBG size, VRB-to-PRB interleaver, rate matching patterns, etc.. So, we can first define a common frequency resource for group-common PDSCH and further discuss whether to reuse the BWP framework or something new. In addition, a UE may belong to different UE groups receiving different multicast services. Therefore, one or more than one common frequency resource can be configured per UE. Therefore, we propose: - Define common frequency resource for group-common PDSCH. - FFS whether to reuse the BWP framework or not - FFS one or more than one common frequency resource configured per UE |

Clarification on simultaneous operation with unicast reception in the WID

The WID of NR MBS mentioned to specify necessary enhancements that are required to enable simultaneous operation with unicast reception. One company [vivo] proposed to clarify whether the simultaneous operation with unicast reception means a UE is required to receive multicast PDSCH and unicast PDSCH simultaneously in one slot. Three companies [CMCC] [vivo] [Intel] proposed to discuss whether TDM/FDM/SDM can be supported for simultaneous reception of unicast PDSCH and multicast PDSCH in one slot.

Since the clarification and discussion on this issue has great impact on the detailed design for simultaneous operation with unicast reception, it is proposed to discuss this issue in this meeting.

[Medium priority] Issue 3 (Question 3 in R1-2007001): Whether the simultaneous operation with unicast reception in the WID means a UE is required to receive multicast PDSCH and unicast PDSCH simultaneously in one slot? If the answer is YES, which multiplexing type(s) of simultaneous reception of unicast PDSCH and multicast PDSCH in a slot can be supported in NR MBS? e.g., TDM, FDM, SDM.

Please share your views and comments in the table.

| Company | Comment |
| --- | --- |
| Qualcomm | We think simultaneous operation with unicast reception in the WID means a UE is required to receive multicast PDSCH and unicast PDSCH simultaneously in one slot. LTE SC-PTM supports FDM between unicast and multicast PDSCH, based on the UE capability. It could be the starting point for NR multicast reception. Therefore, we propose - Support FDM between unicast PDSCH and multicast PDSCH in a slot based on UE capability. - FFS TDM or SDM in a slot. |

[Medium priority] Issue 5 (Proposal 2 in R1-2007001, with little update): For RRC_CONNECTED UEs, whether the following reliability improvement mechanisms can be supported for MBS:

- CSI feedback

  - FFS: whether modification is needed on top of existing CSI feedback mechanism for unicast

- PDSCH repetition

  - FFS: whether spec impact is implied

Please share your views and comments in the table.

| Company | Comment |
| --- | --- |
| Qualcomm | Firstly, we support the CSI feedback to improve the reliability of NR multicast reception for RRC_CONNECTED UEs. The multicast beam for group-common PDCCH/PDSCH is not same as that of unicast. If the multicast beam fails, the RRC_CONNECTED UEs can make use of unicast connection to assist the beam management for multicast. Whether to use P/SP/A-CSI-RS for multicast CSI feedback need further study. Another approach is SRS-based CSI measurement for DL transmission in TDD band. The SRS for multicast transmission may need different configuration than that of unicast due to different frequency resource and user multiplexing. Secondly, regarding PDSCH repetition, the unicast PDSCH support slot aggregation. The PDSCH aggregation factor is configured in pdsch-Config per BWP-DownlinkDedicated. The repetition can be supported to improve the multicast coverage. But if we support PDSCH slot aggregation for multicast, the configuration for group-common PDSCH could be different from that of unicast. Therefore, we propose: - For RRC_CONNECTED UEs, support CSI feedback for group-common PDCCH/PDSCH. - FFS the configuration of TRS/CSI-RS for multicast transmission - FFS the configuration of SRS for multicast transmission - For RRC_CONNECTED UEs, support repetition for group-common PDSCH. - FFS the configuration of group-common PDSCH repetition |

Initial Proposals (2nd round of email discussion)

To be added

Appendix A: Second tier issues summarized in R1-2007001

Table A.1: Summary of second tier issues of group scheduling for RRC_CONNECTED UEs

| Second tier issues | Possible questions or proposals |
| --- | --- |
| 3.1.1 Configuration of group scheduling for multicast/broadcast | Question: Whether broadcast and multicast need to be differentiated for RRC_CONNECTED UEs? If the answer is YES, whether the same configuration mechanism of group scheduling for Broadcast is applied for both RRC_CONNECTED UEs and RRC_IDLE/INACTIVE UEs? |
|  | Proposal: For RRC_CONNECTED UEs, at least part of the parameters for multicast configuration is received by dedicated RRC signaling. |
| 3.1.2.1 CORESET configuration for MBS | Proposal: For group common PDCCH based group scheduling and a MBS common frequency resource configured with in UE’s active DL BWP, the CORESET is configured within the MBS common frequency resource. |
|  | Proposal: For group common PDCCH based group scheduling and a MBS common frequency resource configured with in UE’s active DL BWP, the CORESET for MBS is configured per BWP. |
| 3.1.2.2 Search space configuration for MBS | Proposal: Consider the following options for search space configuration for MBS for RRC_CONNECTED UEs: • Option 1: CSS (existing CSS type or new defined CSS type) • Option 2: USS |
| 3.1.2.3 DCI format for MBS | Proposal: Consider the following options for DCI format for MBS for RRC_CONNECTED UEs: • Option 1: DCI format 1_0 • Option 2: DCI format 1_1 • Option 3: DCI format 2_x • Option 4: New DCI format |
| 3.1.2.4 Blind decoding related issues | Proposal: The maximum number of monitored PDCCH candidates and non-overlapped CCEs per slot are not increased for MBS. |
|  | Proposal: Keep the “3+1” DCI size budget for MBS with group common PDCCH based group scheduling. |
| 3.1.2.5 Multi-beam/beam sweeping operation | Proposal: Support multi-beam/beam sweeping operation for MBS PDCCH/PDSCH for RRC_CONNECTED UEs. |
| 3.1.3 Simultaneous operation with unicast reception | Proposal: The UE is expected to process maximum two transport blocks for simultaneous reception of unicast PDSCH and multicast PDSCH. |
| 3.1.4 Other issues | Proposal: Support DL SPS for MBS for RRC_CONNECTED UEs. |
|  | Proposal: Support multi-layer MIMO for MBS PDSCH for RRC_CONNECTED UEs. |
|  | Question: Whether modifications are needed for QCL framework in order to support MBS transmission? |
|  | Proposal: Introduce a new reception type of PDCCH and PDSCH for MBS for RRC_CONNECTED UEs, if group common PDCCH based group scheduling is supported. |
|  | Question: Whether to support receiving MBS service on a Scell? |
|  | Question: Whether to support SFBC for MBS? |

Table A.2: Summary of second tier issues of reliability improvement for RRC_CONNECTED UEs

| Second tier issues | Possible questions or proposals |
| --- | --- |
| 3.2.1 HARQ-ACK feedback | Proposal: Consider the following two alternatives for HARQ-ACK feedback for RRC_CONNECTED UEs: - Alternative 1: ACK/NACK based HARQ-ACK feedback - Alternative 2: NACK-only based HARQ-ACK feedback |
|  | Proposal: HARQ-ACK feedback for NR MBS should be RRC configurable if it is supported for RRC_CONNECTED UEs. |
|  | Proposal: Both PTM-based and PTP-based retransmissions can be supported for NR MBS for RRC_CONNECTED UEs. |
|  | Question: Whether multiplexing of HARQ-ACK of unicast and multicast transmission should be supported for UEs receiving both unicast and multicast service. |
|  | Question: Whether prioritization of HARQ-ACK of unicast and multicast transmission should be supported for UEs receiving both unicast and multicast service. |
| 3.2.2 CSI feedback | Proposal: Consider to support following schemes for NR MBS: - Option 1: Single port transmission - Option 2: Open-loop spatial multiplexing - Option 3: Closed-loop spatial multiplexing |
| 3.2.3 Other issues | Question: Whether it is needed to discuss the reliability requirements for NR MBS from RAN1 perspective? If the answer is YES, then whether the reliability requirements are RRC state dependent and whether the reliability requirements for multicast and broadcast traffic are the same or not? |
|  | Question: If it is decided to discuss the reliability requirements for NR MBS in RAN1, then how to define the reliability requirements? |

Table A.3: Summary of second tier issues of MBS for RRC_IDLE/RRC_INACTIVE UEs

| Second tier issues | Possible proposals |
| --- | --- |
| CORESET for PTM | For UE in IDLE/INACTIVE state, the CORESET for PTM is - Alt 1: CORESET0 - Alt 2: Configurable |
| Search space for PTM | For UE in IDLE/INACTIVE state, the search space for PTM is - Alt 1: One(s) of existing common search space - Alt 2: A new type of CSS set |
| Multi-beam operation | Monitoring occasions for PTM is associated with SSB. |
| HARQ-ACK | Whether HARQ-ACK feedback is supported for PTM for UE in IDLE/INACTIVE state: Alt 1: Supported but NACK only Alt 2: Not supported |
| Search spaces for SIBx/MCCH if defined | Search spaces for SIBx/MCCH needs to be discussed. |
| MCS table and number of layers | MCS table to be used and how many layers are used should be configured |

Appendix B: Summary of Phase 1 discussion

Seventeen companies have provided their views on the classification of priorities for this meeting in phase 1 discussion. The statistics are shown in the table below.

|  | Issue 1 (Question 1 in R1-2007001) | Issue 2 (Question 2 in R1-2007001) | Issue 3 (Question 3 in R1-2007001) | Issue 4 (Proposal 1 in R1-2007001) | Issue 5 (Proposal 2 in R1-2007001) | Issue 6 (Question 4 in R1-2007001) | Issue 7 (Proposal 3 in R1-2007001) | Issue 8 (Proposal 4 in R1-2007001) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # High | 17 | 8 | 7 | 14 | 6 | 9 | 2 | 2 |
| # Medium |  | 8 | 10 | 3 | 11 | 7 | 7 | 6 |
| # Low |  | 1 |  |  |  | 1 | 8 | 9 |

The following observations can be drawn from companies’ views:

- Issue 1/4/6: More than half of the companies think these three issues should be high priority items.

- Issue 2/3/5: Less than half of the companies think these three issues should be high priority items, but almost all of the companies think they should be at least medium priority items.

- Issue 7/8: About half of the companies think these issues should be low priority. It is explicitly mentioned in Chairman’s notes that no plan to treat 8.12.3 in this meeting.

Based on the above observation, the following proposal is made on the classification of high/medium priority items for this meeting:

Proposal: The following high/medium priority items are classified for this meeting:

- High priority:

  - Issue 1/4/6

- Medium priority:

  - Issue 2/3/5

References

- R1-2007001	FL summary on NR Multicast and Broadcast Services	Moderator (CMCC)

- RP-193248	New WID proposal: NR Multicast and Broadcast Services

- RP-201038	Revised WID: Core part: NR multicast and broadcast services

- R1-2005249	Resource configuration and group scheduling for RRC_CONNECTED UEs	Huawei, HiSilicon

- R1-2005406	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs	vivo

- R1-2005436	Mechanisms to Support Group Scheduling for RRC_CONNECTED UEs	ZTE

- R1-2005531	Group Scheduling Mechanisms to Support 5G Multicast / Broadcast Services for RRC_CONNECTED Ues	Nokia, Nokia Shanghai Bell

- R1-2005589	Considerations on MBMS group scheduling for RRC_CONNECTED UEs	Sony

- R1-2005693	Discussion on group scheduling mechanism for RRC_CONNECTED UEs in MBS	CATT

- R1-2005898	Group Scheduling for NR-MBS	Intel Corporation

- R1-2006013	Group scheduling for NR Multicast and Broadcast Services	OPPO

- R1-2006173	On Mechanisms to support group scheduling for RRC_CONNECTED UEs	Samsung

- R1-2006233	Discussion on group scheduling mechanisms in NR MBS	CMCC

- R1-2006320	Support of group scheduling for RRC_CONNECTED UEs	LG Electronics

- R1-2006631	On group scheduling mechanism for NR multicast and broadcast	Convida Wireless

- R1-2006830	Views on group scheduling for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

- R1-2006918	Mechanism for group scheduling of RRC_CONNECTED UEs in NR	Ericsson

- R1-2005250	Mechanisms to improve reliablity for RRC_CONNECTED UEs	Huawei, HiSilicon

- R1-2005407	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs	vivo

- R1-2005437	Mechanisms to Improve Reliability for RRC_CONNECTED UEs	ZTE

- R1-2005532	Mechanisms for 5G Multicast / Broadcast Reliability Improvements for RRC_CONNECTED Ues			Nokia, Nokia Shanghai Bell

- R1-2005590	Considerations on MBMS reliability for RRC_CONNECTED UEs	Sony

- R1-2005694	Discussion on reliability improvement mechanism for RRC_CONNECTED UEs in MBS	CATT

- R1-2005899	Mechanisms to Improve Reliability for NR-MBS	Intel Corporation

- R1-2006014	UL feedback for RRC-CONNECTED UEs in MBMS	OPPO

- R1-2006174	On Mechanisms to improve reliability for RRC_CONNECTED Ues	Samsung

- R1-2006234	Discussion on reliability improvement in NR MBS	CMCC

- R1-2006321	Mechanisms to improve reliability of Broadcast/Multicast service	LG Electronics

- R1-2006632	On reliability enhancement for NR multicast and broadcast	Convida Wireless

- R1-2006831	Views on UE feedback for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

- R1-2006863	HARQ-based time-interleaving for NR Multicast/Broadcast	BBC

- R1-2006919	Mechanisms to improve reliability for RRC_CONNECTED UEs receiving PTM transmission			Ericsson

- R1-2005272	Discussion on multicast support for IDLE/INACTIVE UEs	Huawei, HiSilicon

- R1-2005408	Discussion on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs			vivo

- R1-2005438	Basic Functions for Broadcast or Multicast for RRC_IDLE or RRC_INACTIVE UEs	ZTE

- R1-2005533	Basic Functions for Broadcast / Multicast for  RRC_IDLE / RRC_INACTIVE Ues	Nokia, Nokia Shanghai Bell

- R1-2005695	Discussion on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs			CATT

- R1-2006015	Discussion on enhancements for IDLE and INACTIVE state UEs	OPPO

- R1-2006175	On Basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs	Samsung

- R1-2006235	Discussion on NR MBS in RRC_IDLE RRC_INACTIVE states	CMCC

- R1-2006322	Basic function for broadcast/multicast	LG Electronics

- R1-2006832	Views on group scheduling for Multicast RRC_IDLE/INACTIVE UEs	Qualcomm Incorporated

- R1-2006920	Basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs	Ericsson

- R1-2005439	Preliminary Simulation Results of Rel-17 MBS	ZTE

- R1-2005534	Simulation assumptions and evaluation scenarios for 5G Multicast Services	Nokia, Nokia Shanghai Bell

- R1-2006016	PUCCH resource allocation for UL feedback in MBMS	OPPO

- R1-2006236	On R17 NR MBS WI	CMCC

- R1-2006410	Performance evaluation of HARQ for NR multicast	Huawei, HiSilicon

- R1-2006658	Other issues for Rel-17 MBS	vivo

- R1-2006861	MIMO support in NR Multicast/Broadcast	BBC

- R1-2006921	Assumptions for Performance Evaluations of NR-MBS	Ericsson