3GPP TSG RAN WG1 #102-e			R1-2005347

e-Meeting, August 17th – 28th, 2020

Source:	vivo

Title:	PDCCH enhancements for URLLC

Agenda Item:	7.2.5.1

Document for:	Discussion and Decision

- Introduction

In the previous meeting, PHY priority is discussed. Companies have the different understanding and no agreements are achieved [1][2]:

PHY priority when both DCI format 0_1/1_1 and DCI format 0_2/1_2 are configured to be monitored per BWP If a UE is NOT capable of supporting dynamic switching of HARQ-ACK/PUSCH priority via both DCI format 0_1/1_1 and 0_2/1_2, and the UE is configured with DCI format 0_1 / 1_1 and 0_2/1_2, down-select from the belows: - Alt-1 (based on Interpretation 1): The UE is expected to assume fixed priority by DCI format (i.e., low priority for DCI format 0_1/1_1, high priority for DCI format 0_2/1_2). - Alt-1d (based on Interpretation 1): The UE is expected to assume a low priority for any DCI format from the DCI formats 0_1/1_1/0_2/1_2. - Alt-2 (based on Interpretation 2): The UE is expected to assume low priority for DCI format 0_1/1_1, and to follow the indicated priority (low or high), if configured, in the scheduling DCI format for DCI format 0_2/1_2. - Note: If the indicated priority field is not configured in DCI format 0_2/1_2, follow the solution for "Default priority".

In this contribution, we share our view on remaining issues of PDCCH enhancements for URLLC.

- DCI format for URLLC

  - How to determine the priority for UE without priority indication capability

In RAN1#99 meeting, the following working assumption and agreement are achieved

Working assumption: When a single PDSCH/PUSCH processing timeline is configured in the carrier, at least when only DCI format 0_1/1_1 is configured or only DCI format 0_2/1_2 is configured in USS per BWP, a DCI format (from the formats 0_1/1_1/0_2/1_2) can be used to schedule PDSCH with different HARQ-ACK priorities or PUSCH with different priorities. - 1-bit field in DCI can be configured as the PHY identification of the priority - No indication of different priorities by DCI formats 0_0/1_0 Agreement : When both DCI format 0_1/1_1 and DCI format 0_2/1_2 are configured to be monitored per BWP, a DCI format (from the formats 0_1/1_1/0_2/1_2) can be used to schedule PDSCH with different HARQ-ACK priorities or PUSCH with different priorities. - This feature is UE optional

Per the working assumption above, when UE is configured with only DCI format 0_1/1_1 or only DCI format 0_2/1_2 in USS, the UE can be scheduled with a PDSCH with different HARQ-ACK priorities or a PUSCH with different priorities by the PHY identification of the priority in the DCI.

According to the agreement, when UE is configured with both DCI format 0_1/1_1 and DCI format 0_2/1_2 and the UE supports this optional capability, the UE can be scheduled with a PDSCH with different HARQ-ACK priorities or a PUSCH with different priorities by the PHY identification of the priority in any of the DCI formats.

Observation 1: If a UE is configured with both DCI format 0_1/1_1 and DCI format 0_2/1_2 and supports the capability of dynamic indication of priorities via both DCI format 0_1/1_1 and DCI format 0_2/1_2, the UE can be scheduled with a PDSCH with different HARQ-ACK priorities or a PUSCH with different priorities by the PHY identification of the priority in any of the DCI formats.

For a UE configured with both DCI format 0_1/1_1 and DCI format 0_2/1_2 which does not support the capability in the above agreement, the UE is not capable of dynamic indication of priorities of HARQ-ACK/PUSCH via both DCI format 0_1/1_1 and DCI format 0_2/1_2. In such case, the UE is capable of supporting dynamic indication in either DCI format 0_1/1_1 or DCI format 0_2/1_2, since the capability in the agreement should be based on the capability in the working assumption. If a UE supports dynamic indication of priorities in either DCI format 0_1/1_1 or DCI format 0_2/1_2 according to the working assumption, there is no reason to assume that the UE can support dynamic indication of priorities in neither DCI format 0_1/1_1 nor DCI format 0_2/1_2, when the UE is configured with both DCI format 0_1/1_1 and DCI format 0_2/1_2 but without the capability of supporting dynamic indication of priorities according to the agreement.

In case of a UE configured with both DCI format 0_1/1_1 and DCI format 0_2/1_2 but without the capability of supporting dynamic indication of priorities according to the agreement, one simple solution is that the priority of HARQ-ACK or PUSCH is fixed as low priority if it is scheduled by DCI format 0_1/1_1, while different priorities for HARQ-ACK or PUSCH can be scheduled by the PHY priority indication in DCI format 0_2/1_2, e.g. PHY priority determination bases on Alt-2 (based on Interpretation 2).

Proposal 1: If a UE is NOT capable of supporting dynamic switching of HARQ-ACK/PUSCH priority via both DCI format 0_1/1_1 and 0_2/1_2, and the UE is configured with DCI format 0_1 / 1_1 and 0_2/1_2,

- PHY priority determination bases on Alt-2 (based on Interpretation 2), i.e., the UE is expected to assume low priority for DCI format 0_1/1_1, and to follow the indicated priority (low or high), if configured, in the scheduling DCI format for DCI format 0_2/1_2. If the indicated priority field is not configured in DCI format 0_2/1_2, follow the solution for "Default priority".

- Conclusion

In the contribution, we have some investigations on URLLC PDCCH enhancement, and propose that,

Observation 1: If a UE is configured with both DCI format 0_1/1_1 and DCI format 0_2/1_2 and supports the capability of dynamic indication of priorities via both DCI format 0_1/1_1 and DCI format 0_2/1_2, the UE can be scheduled with a PDSCH with different HARQ-ACK priorities or a PUSCH with different priorities by the PHY identification of the priority in any of the DCI formats.

Proposal 1: If a UE is NOT capable of supporting dynamic switching of HARQ-ACK/PUSCH priority via both DCI format 0_1/1_1 and 0_2/1_2, and the UE is configured with DCI format 0_1 / 1_1 and 0_2/1_2,

- PHY priority determination bases on Alt-2 (based on Interpretation 2), i.e., the UE is expected to assume low priority for DCI format 0_1/1_1, and to follow the indicated priority (low or high), if configured, in the scheduling DCI format for DCI format 0_2/1_2. If the indicated priority field is not configured in DCI format 0_2/1_2, follow the solution for "Default priority".

------------------------------------------------------------------------------------------------------------------------------------

4. References

[1] Chairman notes RAN1 #99 meeting

[2] Chairman notes RAN1 #101 e-meeting