3GPP TSG RAN WG1 #102-e			R1-2005577

e-Meeting, August 17th – 28th, 2020

Agenda Item: 	8.5.1

Source:	Sony

Title:	Remaining issues on Scenarios for Evaluations of NR Positioning

Document for:	Discussion and Decision

# Introduction

A study item on NR positioning enhancements in Rel-17 has been approved with the main motivation to address the higher accuracy location estimation and low latency requirements resulting from new applications and industry verticals [1]. One of the objectives is to study enhancements and solutions necessary to support the high accuracy (horizontal and vertical), low latency, network efficiency (scalability, RS overhead, etc.), and device efficiency (power consumption, complexity, etc.) requirements for commercial uses cases (incl. general commercial use cases and specifically (I)IoT use cases.

In RAN1 #101-e, the following agreements related to positioning requirements were made [2]:

Agreement:

- In Rel-17 target positioning requirements for commercial use cases are defined as follows:

  - Horizontal position accuracy (< 1 m) for [90%] of UEs

  - Vertical position accuracy (< [2 or 3] m) for [90%] of UEs

  - End-to-end latency for position estimation of UE (< [100 ms])

  - FFS: Physical layer latency for position estimation of UE (< [10 ms])

- In Rel-17 target positioning requirements for IIoT use cases are defined as follows:

  - Horizontal position accuracy (< X m) for [90%] of UEs

    - X = [0.2 or 0.5] m

  - Vertical position accuracy (< Y m) for [90%] of UEs

    - Y = [0.2 or 1] m

  - End-to-end latency for position estimation of UE (< [10ms, 20ms, or 100ms])

  - FFS: Physical layer latency for position estimation of UE (< [10ms])

- Note: Target positioning requirements may not necessarily be reached for all scenarios

Agreement:

It will be left to companies to define the configurations for DL PRS and UL SRS for the evaluation of positioning performance.

- Note: Configurations of DL PRS and UL SRS supported by Rel-16 specifications are used for evaluation of the achievable performance based on Rel-16 positioning technologies.

Further agreements on baseline simulation scenarios, baseline parameters to be used for performance evaluations of NR positioning enhancements were also made.

In this contribution, we discuss our views on remaining issues on scenarios for evaluation of NR positioning.

# Discussions

For IIoT use-cases, both InF-SH and InF-DH models as described in TR 38.901 were adopted as the baseline scenarios for defining the channel models, parameters, and modelling techniques for performance evaluations in the Rel.17 positioning enhancements [2].

The baseline parameters for all scenarios in the evaluation of the positioning performance in Rel-17 as shown in the Appendix A are inherited from TR 38.855. It covers both FR1 and FR2. It is observed some baseline parameters may not be practically deployed. The bandwidth for FR1 and FR2 are 100 MHz and 400 MHz, respectively. These are basically the entire bandwidth for both frequency ranges. It may not be practical to allocate Positioning Reference Signal (PRS) in the entire bandwidth. However, the simulation results would still be useful as it can be considered as the upper-bound results.

Another aspect is on PRS configuration. It was agreed that the companies have a freedom to define the configuration of DL PRS and UL SRS. We expect companies will provide evaluation results together with simulation assumptions during RAN1 102e meeting. The results and the simulation assumptions can be analysed and whether common practical parameters can be identified.

We consider that using impractical simulation assumptions / parameters may lead to imprecise conclusions of the study item. Furthermore, by using practical simulation assumptions / parameters may expose more positioning techniques in order to meet the stringent positioning requirements.  We have observed that the positioning estimation accuracy is heavily affected by the PRS bandwidth configuration as shown in our companion contribution [7].

Observation 1: Some of the scenario parameters may not be practically implemented (e.g. bandwidth for FR1 and FR2 are 100 MHz, and 400MHz, respectively). In practice, it may not be possible to achieve the required positioning accuracy.

Proposal 1: Define the new set of practical scenario parameters (e.g. basic parameters (smaller bandwidth) and DL PRS and UL SRS configuration) for evaluation of positioning techniques.

In RAN1 #101e meeting, the modelling of mobility aspect was discussed. Eventually, it was agreed that UE mobility can be considered in evaluation with the consideration of the spatial consistency procedure defined in TR 38.901. During e-mail discussion, it was clear that UE mobility model requires numerous considerations, such as:

- Track model

- Velocity and acceleration

- Positioning update rate

- Direction

The above considerations have not reached consensus and triggered further discussions, such as the error model of the velocity, acceleration, random bearing for the trajectory, etc. Considering the modelling of mobility aspect is not part of the study item descriptions (SID) and we have extremely limited number of meetings and time unit, we propose to down prioritize the discussion on UE mobility model during the study item. It would be wise to discuss on other important items that are described in the SID. However, it does not prohibit the proponent to present the simulation results with their own modelling/assumption.

Proposal 2: Down prioritize UE mobility model and let interested proponent to use their own model/assumption.

Another issue to study is the impact of hand-blockage on the accuracy of positioning estimates at FR2 frequencies, e.g., for handheld tools in indoor factory scenarios, or for general handheld devices in commercial uses cases. Power reductions due to hand-blockage depend on the direction of the incoming/outgoing signals. Basically, a UE having one or more panels blocked by the user’s hand will receive a smaller number of high-quality PRS signals from the surrounding gNBs. Hand-blockage may therefore have an adverse effect on the accuracy and/or latency of positioning estimates. Additionally, power reductions due to blockage by the body of the user or other users are also possible, and might be as high as 20 dB, and even 30 dB, at FR2 frequencies.

Observation 2: Power reductions due to hand- and body-blockage may have an adverse effect on the accuracy and/or latency of positioning estimates at FR2 frequencies.

Proposal 3: To model the effect of hand- and body-blockage a loss of [10] dB is applied to a randomly selected UE panel; larger values, e.g. [20] dB or [30] dB can also be considered. This power reduction is applicable to handheld UEs at FR2 frequencies, such as tools in indoor factory scenarios.

Furthermore, handheld UEs which detect, e.g., the proximity of the user´s head might be required to apply a transmit power reductions, typically in the range from 3 to 12 dB, to comply with applicable MPE regulations. This further impact accuracy and/or latency of positioning estimates methods, in particular those relying on uplink transmissions.

Observation 3: Transmit power reductions to comply with MPE regulations at FR2 frequencies may have an adverse effect on the accuracy and/or latency of uplink-based positioning estimation methods.

Proposal 4: To model the effect of the MPE issue a transmit power reduction of [10] dB is applied to a randomly selected UE panel. This transmit power reduction is applicable to handheld UEs when performing UL-based positioning at FR2 frequencies, such as tools in indoor factory (InF) scenarios.

# Summary

# In this contribution we have expressed our view on the remaining issues in scenarios and parameters for the evaluation of NR positioning. Our observation and proposals are listed below:

Observation 1: Some of the scenario parameters may not be practically implemented (e.g. bandwidth for FR1 and FR2 are 100 MHz, and 400MHz, respectively). In practice, it may not be possible to achieve the required positioning accuracy.

Proposal 1: Define the new set of practical scenario parameters (e.g. basic parameters (smaller bandwidth) and DL PRS and UL SRS configuration) for evaluation of positioning techniques.

Proposal 2: Down prioritize UE mobility model and let interested proponent to use their own model/assumption.

Observation 2: Power reductions due to hand- and body-blockage may have an adverse effect on the accuracy and/or latency of positioning estimates at FR2 frequencies.

Proposal 3: To model the effect of hand- and body-blockage a loss of [10] dB is applied to a randomly selected UE panel; larger values, e.g. [20] dB or [30] dB can also be considered. This power reduction is applicable to handheld UEs at FR2 frequencies, such as tools in indoor factory scenarios.

Observation 3: Transmit power reductions to comply with MPE regulations at FR2 frequencies may an adverse effect on the accuracy and/or latency of uplink-based positioning estimation methods.

Proposal 4: To model the effect of the MPE issue a transmit power reduction of [10] dB is applied to a randomly selected UE panel. This transmit power reduction is applicable to handheld UEs when performing UL-based positioning at FR2 frequencies, such as tools in indoor factory scenarios.

# References

- RP-193237 SID: NR Positioning Enhancements

- Chairman notes RAN1#101e meeting, May-June 2020.

- TR 38.901 Study on channel model for frequencies from 0.5 to 100 GHz v16.0

- TR 38.855 Study on NR positioning support

- TS 22.261 “Service requirements for the 5G system”

- TR 22.804 “Study on Communication for Automation in Vertical Domains”

- R1-1005578, “Initial Views on Evaluation of Positioning Accuracy and Latency”, Sony, RAN1#102e, August 2020

# Appendix A

Table: Common scenario parameters applicable for all scenarios

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 3.5GHz | 28GHz |
| Bandwidth, MHz | 100MHz | 400MHz |
| Subcarrier spacing, kHz | 30kHz for 100MHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Baseline: Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU Optional: FFS |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 in TR 38.855 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns (Optional) |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |