3GPP TSG RAN WG1 Meeting #102	R1-2005603

e-Meeting, August 17th – 28th, 2020

Source: 	ZTE, Sanechips

Title: 	Remaining issues on the configured grant for NR-U

Agenda item:	7.2.2.2.4

Document for:	Discussion/Decision

# Introduction

In the last RAN1 #101 e-meeting, some remaining issues of configured grant were further discussed. Several agreements were reached and the corresponding CRs were approved. Regarding to the configured grant for Rel-16 NR-U, we find some remaining issues still need to be discussed. In this contribution, we provide our views on configured grant for Rel-16 NR-U.

# Discussion

## Offset value determination for CG-UCI

In Rel-16 discussion, the beta offset for CG-UCI and HARQ-ACK use the same Table 9.3-1 of TS 38.213 [1], which is used to determine the number of REs for multiplexing CG-UCI and HARQ-ACK on CG-PUSCH. Meanwhile, the mechanism of HARQ-ACK multiplexing on CG-PUSCH is reused to CG-UCI multiplexing on CG-PUSCH. The beta offset used to determine the number of REs for HARQ-ACK multiplexing on CG-PUSCH can be configured as dynamic or semi-static. Currently, beta offset for CG-UCI can only be configured a semi-static value. For the flexibility, beta offset for CG-UCI also should be configured as dynamic and we propose to capture this in section 9.3 of TS 38.213.

From the perspective of simplified specification description, “betaOffsetCG-UCI-r16” can be moved from “ConfiguredGrantConfig” to “BetaOffsets” as the following description.

BetaOffsets ::= SEQUENCE { betaOffsetACK-Index1 INTEGER(0..31) OPTIONAL, -- Need S betaOffsetACK-Index2 INTEGER(0..31) OPTIONAL, -- Need S betaOffsetACK-Index3 INTEGER(0..31) OPTIONAL, -- Need S betaOffsetCSI-Part1-Index1 INTEGER(0..31) OPTIONAL, -- Need S betaOffsetCSI-Part1-Index2 INTEGER(0..31) OPTIONAL, -- Need S betaOffsetCSI-Part2-Index1 INTEGER(0..31) OPTIONAL, -- Need S betaOffsetCSI-Part2-Index2 INTEGER(0..31) OPTIONAL, -- Need S betaOffsetCG-UCI-r16 INTEGER (1..ffsValue) OPTIONAL, -- Need R }

ConfiguredGrantConfig ::= SEQUENCE { < Unchanged parts are omitted > cg-UCI-Multiplexing ENUMERATED {enabled} OPTIONAL, -- Need R cg-COT-SharingOffset-r16 INTEGER (1..ffsValue) OPTIONAL, -- Need R cg-COT-SharingList-r16 SEQUENCE (SIZE (1..ffsValue)) OF CG-COT-Sharing-r16 OPTIONAL, -- Need R < Unchanged parts are omitted > CG-UCI-OnPUSCH ::= CHOICE { dynamic SEQUENCE (SIZE (1..4)) OF BetaOffsets, semiStatic BetaOffsets < Unchanged parts are omitted > }

Proposal 2: Adopt the TP#1 to support dynamic instruction the beta offset for CG-UCI multiplexing on CG-PUSCH in TS38.213.

< Start of text proposal 1> ## 9.3 UCI reporting in physical uplink shared channel Offset values are defined for a UE to determine a number of resources for multiplexing HARQ-ACK information and for multiplexing CSI reports in a PUSCH. Offset values are also defined for multiplexing CG-UCI [5, TS 38.212] in a CG-PUSCH. The offset values are signalled to a UE either by a DCI format scheduling the PUSCH transmission or by higher layers. If a DCI format that does not include a beta_offset indicator field schedules the PUSCH transmission from the UE and the UE is provided betaOffsets = 'semiStatic', the UE applies the , , and values that are provided by betaOffsets = 'semiStatic' for the corresponding HARQ-ACK information, Part 1 CSI reports and Part 2 CSI reports. If the PUSCH transmission is with a configured grant and the UE is provided CG-UCI-OnPUSCH= 'semiStatic', the UE applies the , βoffsetCG-UCI, , and values that are provided by CG-UCI-OnPUSCH = 'semiStatic' for the corresponding HARQ-ACK information, CG-UCI, Part 1 CSI reports and Part 2 CSI reports. If the PUSCH is scheduled by DCI format 0_0 and the UE is provided betaOffsets = 'dynamic', the UE applies the , , and values that are determined from the first value of betaOffsets = 'dynamic'. If the PUSCH is a configured grant Type 2 PUSCH activated by DCI format 0_0 and the UE is provided CG-UCI-OnPUSCH ='dynamic', the UE applies the , βoffsetCG-UCI, , and values that are determined from the first value of CG-UCI-OnPUSCH = 'dynamic'. HARQ-ACK information offsets are configured to values according to Table 9.3-1. The betaOffsetACK-Index1, betaOffsetACK-Index2, and betaOffsetACK-Index3 respectively provide indexes , , and for the UE to use if the UE multiplexes up to 2 HARQ-ACK information bits, more than 2 and up to 11 HARQ-ACK information bits, and more than 11 bits in the PUSCH, respectively. CG-UCI offsets βoffsetCG-UCI are configured to values according to Table 9.3-1. Part 1 CSI report and Part 2 CSI report offsets and , respectively, are configured to values according to Table 9.3-2. The betaOffsetCSI-Part1-Index1 and betaOffsetCSI-Part2-Index1 respectively provide indexes and for the UE to use if the UE multiplexes up to 11 bits for Part 1 CSI reports or Part 2 CSI reports in the PUSCH. The betaOffsetCSI-Part1-Index2 and betaOffsetCSI-Part2-Index2 respectively provide indexes or for the UE to use if the UE multiplexes more than 11 bits for Part 1 CSI reports or Part 2 CSI reports in the PUSCH. If a DCI format that includes a beta_offset indicator field with one bit or two bits, as configured by uci-OnPUSCH, schedules the PUSCH transmission from the UE, the UE is provided by each of {betaOffsetACK-Index1, betaOffsetACK-Index2, betaOffsetACK-Index3} a set of two or four indexes, by each of {betaOffsetCG-UCI-r16} a set of four IoffsetCG-UCI indexes, by each of {betaOffsetCSI-Part1-Index1, betaOffsetCSI-Part1-Index2} a set of two or four indexes, and by each of {betaOffsetCSI-Part2-Index1, betaOffsetCSI-Part2-Index2} a set of two or four indexes from Tables 9.3-1 and 9.3-2, respectively, for multiplexing HARQ-ACK information, CG-UCI, Part 1 CSI reports, and Part 2 CSI reports, respectively, in the PUSCH transmission. The beta_offset indicator field indicates a value, a IoffsetCG-UCI value, a value and a value from the respective sets of values, with the mapping defined in Table 9.3-3 and in Table 9.3-3A. For a PUSCH transmission that is configured by a ConfiguredGrantConfig and includes CG-UCI, the UE multiplexes CG-UCI in the PUSCH transmission if the UE is provided by betaOffsetCG-UCI-r16 a IoffsetCG-UCI value, from a set of values, with the mapping defined in Table 9.3-1. If the UE multiplexes HARQ-ACK information in the PUSCH transmission, as described in Clause 9.2.5, the UE jointly encodes the HARQ-ACK information and the CG-UCI [5, TS 38.212] and determines a number of resources for multiplexing the combined information in a PUSCH using βoffsetHARQ-ACK which provides indexes Ioffset,1HARQ-ACK and Ioffset,2HARQ-ACK for the UE to use if the UE multiplexes up to 11, and more than 11 combined information bits, respectively. < End of text proposal 1>

# Conclusion

In this contribution, we discuss the configured grant aspects for NR operation in unlicensed spectrum, and have the following proposals.

Proposal 1: Adopt the TP#1 to support dynamic instruction the beta offset for CG-UCI multiplexing on CG-PUSCH in TS38.213.

# Reference

3GPP TS 38.213, NR, Physical layer procedures for control