3GPP TSG RAN WG1 #102-e	R1-2005608

e-Meeting, August 17th - 28th , 2020

Title:	Discussion on the channel access mechanism for above 52.6GHz

Source: 	ZTE, Sanechips

Agenda Item:	8.2.2

Document for:	Discussion and Decision

# Introduction

In the RAN #86 meeting, a new SID on supporting NR from 52.6 GHz to 71 GHz was agreed [1] and this study item will include the following objectives for channel access mechanism in RAN1:

- Study of channel access mechanism, considering potential interference to/from other nodes, assuming beam based operation, in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6 GHz and 71 GHz [RAN1].

  - Note: It is clarified that potential interference impact, if identified, may require interference mitigation solutions as part of channel access mechanism.

In this contribution, we will share our views on channel access mechanism for frequencies between 52.6 GHz and 71 GHz in different aspects.

# Discussion

## Regulation rules

NR operation in unlicensed spectrum should target fair and friendly coexistence with incumbent systems (e.g. WiFi/NR-U) and intra-RAT services. To satisfy fair coexistence target, NR-U should fulfill regulatory requirements for above 52.6 GHz as following:

Channel access mechanism

Channel access procedure is a mechanism by which NR equipment performs a clear channel assessment (CCA) detection before transmission. In ETSI EN 302 567 [2], LBT (Listen Before Talk) is mandatory to facilitate spectrum sharing in Europe area. Thus, LBT should be supported in Europe in order to comply with regulatory requirements applicable to frequencies between 57 GHz and 66 GHz. But such mechanism is not required in other countries, e.g., ITU region 2 and 3. Even so, we still need to consider LBT regulatory of other RAT devices (e.g., Wi-Fi 802.11 ad/ay ) operated in the same frequency spectrum. Wherein, 802.11 ad/ay nodes need to use LBT to assess the state (busy or idle) of the current channel. From this point of view, in order to ensure friendly and fair coexistence with Wi-Fi, LBT should be considered and enhanced for NR above 52.6 GHz.

Occupied Channel Bandwidth (OCB)

In ETSI EN 302 567 [2], the Occupied Channel Bandwidth is the bandwidth containing 99 % of the power of the signal, which shall be between 70 % and 100 % of the declared Nominal Channel Bandwidth (NCB). Wherein, The Nominal Channel Bandwidth is the widest band of frequencies, inclusive of guard bands, assigned to a single channel. However, such restriction is not required in the US, China, Japan, South Korea, Australia and Singapore. In this regard, some studies should be made for the constraints of OCB requirements on BWP or larger bandwidth.

Channel Occupancy Time (COT)

In unlicensed spectrum, channel availability cannot always be guaranteed. In addition, certain regions such as Europe and Japan prohibit continuous transmission and impose limits on the maximum Channel Occupancy Time (COT) in the unlicensed spectrum. In ETSI EN 302 567 [2], the total time that an equipment makes use of an Operating Channel is defined as the Channel Occupancy Time, which shall be less than 9 ms, after which the device shall perform a LBT.

Equivalent Isotropically Radiated Power (EIRP) and Power Spectral Density (PSD)

In TR 38.807 [3], the following regulatory requirements in different regions were provided:

| Country | Frequency band (GHz) | EIRP | PSD |
| --- | --- | --- | --- |
| Europe | 57-66 | Mean EIRP = 40dBm | 13 dBm/MHz |
| South Africa | 57-64 | Max EIRP 55dBm Max Tx power 10dBm. Min antenna gain 30dBi. | / |
|  | 57-66 | Max EIRP 40dBm | / |
| USA | 57-71 | Indoor: Max avg, EIRP 40dBm Max peak EIRP 43dBm | / |
|  |  | Outdoor: Max avg. EIRP (82 – 2N) dBm Max peak EIRP (85 – 2N) dBm. N = max(0, 51 dBi – antenna-gain) |  |
| Canada | 57-64 | Indoor: 40dBm avg/43dBm Peak | / |
|  |  | Outdoor PtP: 82dBm when Gant>51dBi; 82-2× (51-Gant) when Gant≤51dBi |  |
| Brazil |  | Total Peak Power at transmitter output ≤ 27dBm (500mW) | Max avg. Power Density: 9 microW/cm^2 @3m Max peak Power Density: 18 microW/cm^2 @3m |
| China | 59-64 | Output power of antenna port: ≤ 10 dBm Peak EIRP: 47dBm Mean EIRP: ≤ 44 dBm | / |
| Korea | 57 – 66 | Max Tx power 500mW (beamformed antenna) Max Tx power 100mW (omni-direction antenna). Max PSD 13dBm/MHz Max EIRP 57dBm [Note 1] | / |

Based on the above requirement in different areas, for wideband operation with the limitation of maximum EIRP and PSD, the PSD may be very low due to its total transmission power should be less than or equal to the maximum EIRP limitation, e.g., 40dBm for 57 GHz - 66 GHz band in South Africa. Too lower PSD will impact on NR-U coverage and system performance. Therefore, the challenge is how to solve the bandwidth and PSD problems related to LBT mechanisms for wider bandwidth.

In summary, the above regulatory requirements should be supported for above 52.6 GHz. Further, some enhancement mechanism should be also studied deeply to achieve good spectrum sharing with other systems..

Proposal 1: The following regulation rules for above 52.6 GHz, including channel access, OCB, COT, EIRP and PSD should be supported and enhanced to achieve good spectrum sharing with other systems.

## Channel access mechanism for above 52.6 GHz

### Omni-directional LBT

In low frequency (e.g., below 7 GHz), in order to guarantee fair coexistence with the incumbent Wi-Fi system such as 802.11 ac/ax in the unlicensed carrier, omni-directional LBT with energy detection considering no array gain was introduced in LTE-LAA/eLAA/FeLAA/NR-U.

In high frequency, similar omni-directional LBT is used in IEEE 802.11 ad/ay for 60 GHz frequency band. However, for release 17 NR-U, if such omni-directional LBT is simply copied in NR-U above 52.6 GHz frequency band, this will be able to cause a sensing inaccuracy problem, or “over protection” issue.

For example, high interference detected on the omni-directional beam range could block the transmission on narrow directional beam range even if the transmission does not interfere with the transmission of the other nodes in the other beam directions.

Based on the above analysis, it is necessary to study directional LBT mechanism for directional transmission and reception mode in order to improve the probability of channel access and spatial reuse

### Directional LBT

Based on the analysis of Section 2.2.1, we can know that directional LBT may be a good choice to increase the opportunities of channel access and achieve better spatial reuse compared to omni-directional LBT, wherein, for directional LBT, the transmitting node performs energy detection with directional beam instead of omni beam. Furthermore, directional LBT may accentuate the hidden/exposed node problem compared to onmi-directional LBT and the reason is that it can make the difference between the observed interference at transmitter side and actual interference at the receiver side much larger compared to onmi-directional LBT. In order to alleviate the impact of the hidden/exposed nodes problem, some methods may be considered and studied, e.g., the receiving node performs a LBT mechanism and sends an indication signal to alleviate hidden node problem. Besides, the transmitter performs sensing operation on the transmission beam range to reduce exposed node problem or mismatch sensing beam and transmission beam.

We can see from the analysis of section 2.2.1 and 2.2.2 that channel access scheme should be studied taking into account the characteristic of beam-based transmission and reception mode for above 52.6 GHz, especially for directional LBT to improve the probability of channel access and spatial reuse.

In this part, we provide system-level simulations to evaluate the impact of LBT schemes on the coexistence between NR-U and Wi-Fi systems, where one operator using omni-directional LBT mimics the behavior of Wi-Fi system and the other operator is assumed as NR-U can choose between omni-directional LBT and directional LBT. The detailed simulation assumptions are listed in Appendix. Besides, more SLS results for above 52.6 GHz NR-U can be found in our companion contribution [4].

Figure 1 plots mean, 5%, 50% and 95% user-perceived throughput performance for Case 1 and Case 2, herein,

- Case 1 corresponds to omni-directional LBT for Operator 1 (mimic an existed Wi-Fi operator) vs omni-directional LBT for Operator 2 (mimic a new NR-U operator) ;

- Case 2 corresponds to omni-directional LBT for Operator 1(mimic an existed Wi-Fi operator) vs directional LBT for Operator 2 (mimic a new NR-U operator).

We can observe the following phenomenon from Figure 1:

- In mean and 5% and 95% UPT, it is shown from Case1 that Operator 1 has a relatively high throughput with omni-directional LBT compared to the same LBT mechanism used for Operator 2. The performance difference between the two operators is mainly due to the random distribution of the nodes in this simulation drop which is not friendly to operator 2. However, we can see from Case 2 that such phenomenon is mitigated when directional LBT is used for NR-U instead of omni-directional LBT.

- In 50% UPT, it can be observed that Operaptr 1 and Operaptr 1 show very similar performance and means that the performance of Operator 1 assuming omni-directional LBT will not be significantly affected even if directional LBT is used for Operator 2.

Therefore, we note based on the following simulation result and the above analysis that these results are consistent with our analysis mentioned in Section 2.2.1 and 2.2.2 and the principle of friendly and fair coexistence, that is, directional LBT is benefit for directional beam transmission mode and the impact on the performance of existing Wi-Fi system is negligible.

Figure 1 Mean, 5%, 50% and 95% user-perceived throughput for Case1 and Case 2

Observation 1: According to regulation of ETSI EN 302 567, at least LBT operation should be supported in Europe area for above 52.6 GHz.

Observation 2: Compared to omni-directional LBT, directional LBT is beneficial to increase the probability of channel access and the spatial reuse efficiency for NR-U, and the impact on the performance of the existed Wi-Fi system is negligible.

### No LBT

For COT sharing case in NR-U low frequency band such as below 7 GHz, No LBT is allowed to use in a case that the gap between DL and UL is less than or equal to 16us. Wherein, at least such rule can be used for a case that same beam direction is used in the DL or UL transmissions situation.

Another a straightforward use case is that No LBT can be used for some areas such as ITU region 2 and 3. That is to say, in these areas, the node cannot perform any LBT operation and directly transmit transmission before transmission. Wherein, in Europe area, LBT is mandatory to be used for unlicensed carrier in order to comply with regulatory requirements applicable to frequencies between 57 GHz and 66 GHz.

Another use case that is easy to think of is interference controlled environment, that is, the node preparing to transmit does not interfere with the ongoing transmission node.

A common case that NR-U and NR-U coexistence scenario if the absence of any other systems can be guaranteed. For instance, the node1 belongs to operator 1 while the node2 is served for operator 2. If the transmitted beams of node 1 and node 2 do not overlap or transmission of two nodes is not interfered each other, then the transmission for the node preparing to transmit will not affect that for another node even if LBT is not performed for the node preparing to transmit. However, once one node falls into the beam range of another node, and LBT is not performed for the node preparing to transmit, then it may cause seriously impact on the performance of the node that is already transmitting.

Observation 3: At least No LBT can be considered to be used in COT sharing case and interference controlled environment.

Proposal 2: Release 17 NR-U should consider supporting different channel access modes for above 52.6 GHz, e.g., directional LBT and No LBT.

## LBT for MCOT Sharing between DL and UL

In LTE-LAA, if the nodes perform Type1 channel access mechanism with omni-directional beam successfully, it is allowed to transmit within a MCOT. Further, if this MCOT can be shared with other nodes (e.g., eNB or UE), the other nodes need to perform Type 2 channel access mechanism with omni-directional beam.

Different from LTE-LAA, NR takes advantage of beamforming technology and TRP-based topology to expand cell coverage and also enhance edge user performance. With these features, LBT for MCOT in NR-U high frequency band may be different compared with LTE-LAA. Owing to the beamforming transmission mode, in order to transmit message to the serving UEs in a certain beam range, gNB needs to transmit different beams toward different directions, which can be within the same or different time units. Therefore, for this situation, if LTE-LAA LBT for MOCT is simply reused, gNB only performs a omni-directional LBT at the beginning of the MCOT and does not perform any LBT operation for the remaining DL transmission within MCOT, it may cause a problem that within MCOT some beams experience an unoccupied channel and the other beams encounter a channel with higher interference from other coexisting nodes. In order to avoid this issue, same beam direction can be used for consecutive transmission, or enhancement channel access schemes for COT sharing case or introduce gap for LBT between DL/UL consecutive transmissions with different beams within COT.

Proposal 3: For multiple transmission(s) with different beams case, channel condition difference for different beams should be considered when designing the channel access schemes for COT sharing in NR unlicensed spectrum.

# Conclusion

In this contribution, we share some our views on channel access mechanism for frequencies between 52.6 GHz and 71 GHz in different aspects and provide the following proposal:

Proposal 1: The following regulation rules for above 52.6 GHz, including channel access, OCB, COT, EIRP and PSD should be supported and enhanced to achieve good spectrum sharing with other systems.

Observation 1: According to regulation of ETSI EN 302 567, at least LBT operation should be supported in Europe area for above 52.6 GHz.

Observation 2: Compared to omni-directional LBT, directional LBT is beneficial to increase the probability of channel access and the spatial reuse efficiency for NR-U, and the impact on the performance of the existed Wi-Fi system is negligible.

Observation 3: At least No LBT can be considered to be used in COT sharing case and interference controlled environment.

Proposal 2: Release 17 NR-U should consider supporting different channel access modes for above 52.6 GHz, e.g., directional LBT and No LBT.

Proposal 3: For multiple transmission(s) with different beams case, channel condition difference for different beams should be considered when designing the channel access schemes for COT sharing in NR unlicensed spectrum.

# Reference

- RP-193259, “New SID: Study on supporting NR from 52.6GHz to 71GHz”, Intel

- ETSI EN 302 567 V2.1.1, Harmonized European Standard, Multiple-Gigabit/s radio equipment operating in the 60GHz band, 2017-07

- 3GPP TR 38.807-100: “Study on requirements for NR beyond 52.6 GHz”

- R1-2005609, “Preliminary simulation results for above 52.6GHz”, ZTE, Sanechip

- RP-172021, “Revised SID on NR-based Access to Unlicensed Spectrum”, Qualcomm

- 3GPP TR 38.889-001: “Study on NR-based Access to Unlicensed Spectrum”

# Appendix

## SLS assumptions

| Parameters | Values or Assumptions |
| --- | --- |
| Carrier Frequency | 60 GHz |
| Channel Bandwidth | 2 GHz |
| Subcarrier spacing | 960 kHz |
| Scenario | Indoor A |
| LBT schemes | Omni-directional LBT Directional LBT |
| CCA threshold | -92 dBm |
| Channel Model | InH Open office model in TR 38.901 Chapter 7.4.1 |
| BS Tx Power | 14 dBm |
| UE Tx Power | 21 dBm |
| BS Antenna gain | 5 dBi |
| UE Antenna gain | 5 dBi |
| BS Noise Figure | 7 dB |
| UE Receiver Noise Figure | 10 dB |
| UE receiver | MMSE-IRC |
| BS antenna Array configuration | (M, N, P, Mg, Ng) = (8, 16, 2, 1, 1), dH = dV = 0.5 λ |
| UE antenna Array configuration | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | FTP Model 3 (27Mbyte file) |