3GPP TSG RAN WG1 Meeting #102-e		                                    R1-2005696

e-Meeting, August 17th – 28th, 2020

Source:                    CATT

Title:	Discussion on cross-carrier scheduling from Scell to Pcell

Agenda Item:	8.13.1

Document for:	Discussion and Decision

Introduction

At RAN#86 meeting, the new WID on support of NR dynamic spectrum sharing was approved [1]. The work item is limited to FR1 and focus on the following two objectives

- PDCCH enhancements for cross-carrier scheduling including [RAN1, RAN2]

  - PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell

  - Study, and if agreed specify PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI

    - The number of cells can be scheduled at once is limited to 2

    - The increase in DCI size should be minimized

In this contribution, we provide our views on the relevant issues of PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell.

Discussion

Scheduling restriction for cross-carrier scheduling

Currently, gNB can only configure a P(s)Cell as scheduling cell once cross carrier scheduling is configured via higher layer parameter CrossCarrierSchedulingConfig, which is included in a root IE ServingCellConfig.  In the other words, only self-scheduling is available for P(s)Cell. The IE structure of CrossCarrierSchedulingConfig is shown below.

CrossCarrierSchedulingConfig information element

-- ASN1START

-- TAG-CrossCarrierSchedulingConfig-START

CrossCarrierSchedulingConfig ::=        SEQUENCE {

schedulingCellInfo                      CHOICE {

own                                     SEQUENCE {        -- Cross carrier scheduling: scheduling cell

cif-Presence                            BOOLEAN

},

other                                   SEQUENCE {         -- Cross carrier scheduling: scheduled cell

schedulingCellId                        ServCellIndex,

cif-InSchedulingCell                    INTEGER (1..7)

}

},

...

}

other Parameters for cross-carrier scheduling, i.e., a serving cell is scheduled by a PDCCH on another (scheduling) cell. The network configures this field only for SCells.

There is a configuring restriction that the field ‘other’ can only be configured for a secondary cell. In the other words, there is no way to schedule a P(s)Cell through a DCI transmitted on the SCell. From the perspective of configuration, the only thing needed to be fixed is to remove the restriction that ‘other’ can only be applied to SCells. It should be noted that the above configuration can only be applied to USS.

Proposal 1: The restriction on ‘other’ included in CrossCarrierSchedulingConfig, i.e. ‘the network configures this field only for SCells’, should be removed.

Scheduling functionality for cross-carrier scheduling from SCell to PCell

As mentioned above, the cross-carrier scheduling from SCell to P(s)Cell can be easily realized by removing the restriction that P(s)Cell cannot be a scheduled cell. From the RAN1 perspective, the following functionality is already supported for the cross-carrier scheduling from P(s)Cell/SCell to SCell:

- UE monitors UE-specific search space only on the scheduling cell

- Both PDSCH and PUSCH can be scheduled on the scheduled cell from a DCI transmitted on the scheduled cell

- Mixed numerology is supported

- Type0/0A/1/2-PDCCH CSS set can only be configured on the primary cell

- Total BD/non-overlapping CCE calculation

- UE capability reporting for carrier aggregation

- Hash function determination for cross-carrier scheduling

From our perspective, we think there is no impact on the above functionality or procedure if the P(s)Cell is scheduled by a SCell.

Observation: There is no RAN1 impact on cross-carrier scheduling functionality if the P(s)Cell is scheduled by SCell.

Furthermore, if multi-cell PDSCH scheduling (multiple PDSCHs on multiple cells) via a single DCI is supported, the scheduled cells can be any combination of the configured serving cells. The target cells can either include the scheduling cell or any two cells except the scheduling cell. It will be friendly to flexibility and the migration from LTE to NR.

Figure1: Illustration of cross-carrier scheduling from SCell to PCell

Proposal 2: The scheduled cells can be any two cells out of the configured serving cells if multi-cell PDSCH scheduling via a single DCI is supported.

Common search space maintained on the P(s)Cell

Generally speaking, all the defined search space sets can be categorized into three types, i.e. common search space, group common search space and UE-specific search space.  The common search space sets can only be configured on the P(s)Cell and schedule all the UEs staying in the same cell. The detail difference among different search space types can be summarized in Table 1 as shown below.

Table 1: Summary for different type of search space sets

| Search space set type | Staying cell | Traffic type |
| --- | --- | --- |
| Type0-PDCCH common search space set | Primary cell of the MCG | DCI format with CRC scrambled by a SI-RNTI; DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, the MCS-C-RNTI, or the CS-RNTI |
| Type0A-PDCCH common search space set | Primary cell of the MCG | DCI format with CRC scrambled by a SI-RNTI; DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, the MCS-C-RNTI, or the CS-RNTI |
| Type1-PDCCH common search space set | Primary cell of MCG and SCG | DCI format with CRC scrambled by a SI-RNTI; DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, the MCS-C-RNTI, or the CS-RNTI |
| Type2-PDCCH common search space set | Primary cell of the MCG | DCI format with CRC scrambled by a SI-RNTI; DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, the MCS-C-RNTI, or the CS-RNTI |
| Type3-PDCCH common search space set | No restrictions | DCI formats with CRC scrambled by INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, or TPC-SRS-RNTI and, only for the primary cell, C-RNTI, MCS-C-RNTI, or CS-RNTI(s) |
| UE-specific search space set | No restrictions | DCI formats with CRC scrambled by C-RNTI, MCS-C-RNTI, SP-CSI-RNTI, or CS-RNTI(s) |

If the cross-carrier scheduling from SCell to P(s)Cell is supported, further clarification on how to handle the common search space may be needed.

- Whether a UE still monitors common search spaces on the P(s)Cell if the P(s)Cell  is scheduled by a DCI transmitted on the SCell?

The key point is whether a UE monitors the common search spaces configured on the P(s)Cell which is not the scheduling cell. It is actually pretty similar to the case where cross-carrier scheduling from SCell to SCell is configured. The scheduling cell is a SCell without Type0/0A/1/2 common search space sets configuration on it. UE has to monitor them on the P(s)Cell. We don’t identify any problems with reusing the same solutions if the P(s)Cell is configured as a scheduled cell.

- Whether the DCI formats with CRC scrambled by C-RNTI, MCS-C-RNTI, SP-CSI-RNTI, or CS-RNTI(s) can still be monitored in the common search spaces on the P(s)Cell if the UE is scheduled by a DCI transmitted on the SCell?

In the current specification, the following behavior on unicast DCI monitoring in CSS is captured.

< --for type 0/0A/1/2-PDCCH common search space sets-->

If a UE is provided

-	one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, and

-	a C-RNTI, an MCS-C-RNTI, or a CS-RNTI,

the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, the MCS-C-RNTI, or the CS-RNTI in the one or more search space sets in a slot where the UE monitors PDCCH candidates for at least a DCI format 0_0 or a DCI format 1_0 with CRC scrambled by SI-RNTI, RA-RNTI or P-RNTI.

< --for type 3-PDCCH common search space sets-->

a Type3-PDCCH CSS set configured by SearchSpace in PDCCH-Config with searchSpaceType = common for DCI formats with CRC scrambled by INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, or TPC-SRS-RNTI and, only for the primary cell, C-RNTI, MCS-C-RNTI, or CS-RNTI(s), and

UE can monitor DCI formats with CRC scrambled by C-RNTI, an MCS-C-RNTI, or a CS-RNTI in Type0/0A/1/2 common search space sets if the UE monitors PDCCH candidates for at least a DCI format 0_0 or a DCI format 1_0 with CRC scrambled by SI-RNTI, RA-RNTI or P-RNTI. It should be noticed that the text in specification is general and can be applied to any case where the Type0/0A/1/2 common search spaces sets are configured. Considering a unified DCI payload size can be guaranteed for the fallback DCIs in a CSS, there are no additional efforts to monitor a fallback DCI with CRC scrambled by unicast RNTI value. The same behavior should be maintained in the case where cross-carrier scheduling from SCell to P(s)Cell is supported.

For Type3-PDCCH CSS set, gNB can freely configure the DCI formats need to be monitored on the P(s)Cell. We do not see the motivation to abandon the configuration flexibility for cross-carrier scheduling from SCell to P(s)Cell.

Proposal 3: DCI format 0_0 and DCI format 1_0 with CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI(s) can be monitored in the common search space sets on the scheduled P(s)Cell.

Conclusion

This contribution discusses possible PDCCH monitoring reduction solutions and raises some issues related to capability reduction and bandwidth reduction. We have the following observation and proposals:

Observation: There is no RAN1 impact on cross-carrier scheduling functionality if the P(s)Cell is scheduled by SCell.

Proposal 1: The restriction on ‘other’ included in CrossCarrierSchedulingConfig, i.e. ‘the network configures this field only for SCells’, should be removed.

Proposal 2: The scheduled cells can be any two cells out of the configured serving cells if multi-cell PDSCH scheduling via a single DCI is supported.

Proposal 3: DCI format 0_0 and DCI format 1_0 with CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI(s) can be monitored in the common search space sets on the scheduled P(s)Cell.

References

  - RP-193260, “New WID on NR Dynamic spectrum sharing (DSS)”, Ericsson, RAN#86

  - TS38.213, V15.9.0