3GPP TSG RAN WG1 Meeting #102-e	R1-2006080

August 17th – August 28th, 2020.

# Title:	[Draft] Reply LS on clarification of transparent diversity feasibility

# Response to:	LS (R4-2009171) on feasibility of ULFPTx modes and transparent TxD for certain UE implementations

# Release:	Rel-16

# Work Item:	NR_eMIMO-Core

Source:	RAN1 (Samsung)

To:	RAN4

Cc:

Contact Person:

Name:

Tel. Number:

E-mail Address:

# Attachments:	-

1. Overall Description:

In RAN4 study of ULFPTx under eMIMO WI [1], enabling transparent Tx diversity (TxD) was agreed at least from Rel-16, and the applicability of transparent TxD is NOT related to UE supporting or not supporting Rel-16 ULFPTx. Two possible cases were identified in RAN4 to use transparent TxD to achieve the required transmission power, i.e. for a FR1 UE having two TX branches/antennae,

- First case: Transparent TxD for UE configured with single SRS port (either with DCI_0_0 or single SRS port with DCI_0_1);

- Second case: Transparent TxD for UE configured with 2 SRS ports (FFS whether TxD is feasible in this case).

For the second case, two possible methods to transmit a multi-port SRS resource (i.e. 2Tx ports) with two PAs (PA1 and PA2) were considered, i.e.

- Method-1: SRS port-1 maps to PA1, SRS port-2 maps to PA2

- Method-2: SRS port-1 maps to PA1+PA2, SRS port-2 maps to PA1+PA2

RAN1 would like to provide replies on the following questions from RAN4 in [1]:

Question 1: Whether the two mentioned methods are both feasible to transmit the full output power? Question 2: If answer is yes, which ULFPTx modes can be supported for these two methods? Question 3: Whether the ULFPTx mode-2 and the other ULPFTx mode are feasible for FR2 UE?

Reply to Question 1: Method 1 is feasible for all ULFPTx modes (subject to UE capability and PA architectures). Method 2 on the other hand can be feasible but not for all ULFPTx modes (cf. reply to Question 2 below). In addition, Method 1 is simpler of the two methods since it doesn’t rely on virtualizing (combing) multiple PAs.

Reply to Question 2:

- Method 1: all ULFPTx modes can be supported depending on UE PA architecture.

  - Mode 0 can be supported if each PA is full-rated.

  - Mode 1 can be supported since full power can be achieved with TPMI = 2, i.e., [1 1]/sqrt(2).

  - Mode 2 can be supported if at least one PA is full-rated and the UE reports at least one full power TPMI (0 or 1), or the UE can be configured with a SRS resource with 1 port.

- Method 2

  - Mode 0 can be supported if each SRS port can achieve full power.

  - Mode 1: not usable, since if it is used, then both SRS ports (i.e. TPMI = 0,1) can achieve full power, which contradicts the fact that only TPMI = 2, i.e., [1 1]/sqrt(2) can achieve full power for mode 1.

  - Mode 2:

    - For a SRS resource with 1 port, full power can be supported.

    - For a SRS resource with 2 ports, full power can be supported for the full power TPMI(s) reported by the UE.

In short, Method 1 can support all modes and Method 2 can support mode 0 and 2 but not mode 1.

Reply to Question 3: Considering that mode 1 for FR2 UE has been confirmed to be feasible, there is no technical reason behind not allowing (or declaring infeasible) mode 0 or mode 2 in FR2, i.e., if a FR2 UE is allowed to support mode 1, then another FR2 UE, if it can, should be allowed to support mode 0 or mode 2. Hence, all modes should be feasible for FR2.

2. Actions:

To RAN4

ACTION: 	RAN1 respectfully asks RAN4 to take the above information into account in their further work

3. Date of Next Meetings:

TSG RAN1 Meeting #103	-e	26th October – 13th November 2020			Online.

TSG RAN1 Meeting #104 	1st – 5th March 2021					Athens, GR.

# References

- R4-2009171, draft LS on feasibility of ULFPTx modes and transparent TxD for certain UE implementations, Huawei