3GPP TSG-RAN WG1 Meeting #102-e	R1-2006438

e-Meeting, August 17th – 28th, 2020

Agenda Item:	7.2.4.4

Source:	Ericsson

Title:	Correction for in-device coexistence in SL

Document for:	Discussion, Decision

# 1		Discussion

The procedures for simultaneous NR and E-UTRA transmission/reception, as captured in TS 38.213, includes a parameter T msec which is not clearly defined. According to the following agreement, T is subject to processing time restriction of the UE.

Agreements RAN1#98: Unless packet priorities of both LTE and NR sidelink are known to both RATs prior to time of collision (subject to processing time restriction), then - It is up to UE implementation to handle LTE Tx/NR Rx overlap. - It is up to UE implementation to handle NR Tx and LTE Rx overlap. Agreements RAN1 #98bis: - For Tx/Rx overlap, - If packet priorities of both LTE and NR sidelinks are known to both RATs prior to time of transmission/reception (subject to processing time restrictions), then the packet with a higher relative priority is transmitted/received - In case the priorities of LTE and NR sidelink packets are the same, then it is up to UE implementation as to which packet is transmitted/received.

We therefore propose the following correction to the spec to clarify T.

---------------------------------------------Start of text proposal for TS 38.213--------------------------------------- #### 16.2.4.1 Simultaneous NR and E-UTRA transmission/reception If a UE - would transmit a first channel/signal using E-UTRA radio access and a second channel/signal using NR radio access, and - a transmission of the first channel/signal would overlap in time with a transmission of the second channel/signal, and - the priorities of the two channels/signals are known to the UE T msec prior to the start of the earlier of the two transmissions, where the value of T is determined by UE implementation means, the UE transmits only the channel/signal with the higher priority as determined by the SCI formats scheduling the transmissions or, in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers or, in case of PSFCH, equal to the priority of the corresponding PSSCH. If a UE - would respectively transmit or receive a first channel/signal using E-UTRA radio access and receive or transmit a second channel/signal using NR radio access, and - a transmission or reception of the first channel/signal would respectively overlap in time with a reception or transmission of the second channel/signal, and - the priorities of the two channels/signals are known to the UE T msec prior to the start of the earlier transmission or reception, where the value of T is determined by UE implementation means, the UE transmits or receives only the channel/signal with the higher priority as determined by the SCI formats scheduling the transmissions or, in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers or, in case of PSFCH, equal to the priority of the corresponding PSSCH. ----------------------------------------------------end text proposal for 38.213---------------------------------------------------