3GPP TSG RAN WG1 #102	R1-2006566

e-Meeting, August 17th – 28th, 2020

Source:	Sharp

Title:	Enhancement on multi-TRP operation for PDCCH and PUSCH

Agenda Item:	8.1.2.1

Document for:	Discussion and Decision

# Background

At RAN#86, a new work item on “Further enhancements on MIMO for NR” was agreed. For enhancement on multi-TRP operations, the following objectives were agreed [1].

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Identify and specify features to improve reliability and robustness for channels other than PDSCH (that is, PDCCH, PUSCH, and PUCCH) using multi-TRP and/or multi-panel, with Rel.16 reliability features as the baseline - Identify and specify QCL/TCI-related enhancements to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception - Evaluate and, if needed, specify beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception - Enhancement to support HST-SFN deployment scenario: - Identify and specify solution(s) on QCL assumption for DMRS, e.g. multiple QCL assumptions for the same DMRS port(s), targeting DL-only transmission - Evaluate and, if the benefit over Rel.16 HST enhancement baseline is demonstrated, specify QCL/QCL-like relation (including applicable type(s) and the associated requirement) between DL and UL signal by reusing the unified TCI framework

In this contribution, we share our initial views on enhancement of multi-TRP operation for PDCCH and PUSCH.

# Discussion

## Multi-TRP/panel PDCCH

In Rel-16, for PDSCH scheduling, PDSCHs transmission from multiple TRPs by multi-DCI has been introduced. Specifically, a UE can receive PDCCHs from multiple TRPs by configuring CORESET pool. For PDCCH, we can discuss this operation as a baseline.

Proposal 1: RAN1 should discuss PDCCH reception in multi-DCI based PDSCH scheduling as a baseline.

## Multi-TRP/panel PUSCH

Multi-TRP/panel PDSCH, the repeated transmission has been introduced and each repetition can be transmitted from each TRP/panel. Therefore, we think the same mechanisms as multi-TRP/panel based PDSCH is applied to PUSCH, i.e. PUSCH repetitions is used for multi-TRP/panel transmission. We already have PUSCH repetitions, e.g. repetition type B for URLLC. We should also consider this mechanism can be a baseline repetition scheme.

Proposal 2: PUSCH repetition mechanism specified in Rel-16 URLLC should be reused.

Regarding the multiplexing scheme, TDM should be the baseline scheme considering the UE complexity and power consumption. In addition, for SDM and FDM, it is required to implement simultaneous multi-beam transmission. Therefore,

Proposal 3: For multi-TRP PUSCH transmission, TDM scheme is the baseline.

# Conclusion

In this contribution, we have the following proposals:

Proposal 1: RAN1 should discuss PDCCH reception in multi-DCI based PDSCH scheduling as a baseline.

Proposal 2: PUSCH repetition mechanism specified in Rel-16 URLLC should be reused.

Proposal 3: For multi-TRP PUSCH transmission, TDM scheme is the baseline.

# References

- RP-193133, “New WID: Further enhancements on MIMO for NR”, RAN1#86, December 2019