3GPP TSG RAN WG1 #102-e		  R1-2006586

e-Meeting, August 17th – 28th, 2020

Agenda Item:	7.2.4.5

Source: 	ASUSTeK

Title:  	Remaining issues on sidelink physical layer procedure on NR V2X

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss about remaining issues on sidelink physical layer procedure.

- Discussion

2.1 Remaining issues on Power Control

Agreements:

For open-loop power control based on DL pathloss, the UE calculates DL pathloss using

- The RS resource used for power control for PUSCH transmission scheduled by DCI format 0_0 when the UE monitors DCI format 0_0.

- A RS resource from the SS/PBCH block that the UE uses to obtain MIB when the UE does not monitor DCI format 0_0.

In RAN1 #101-E meeting [1], it is agreed that a UE can be configured to use DL pathloss for sidelink power control. When the UE is configured to monitor PDCCH for detection of DCI format 0_0, the DL pathloss is derived using RS for PUSCH power control scheduled by DCI format 0_0. When the UE is not configured to monitor PDCCH for detection of DCI format 0_0, the DL pathloss is derived using RS resource from the SS/PBCH block that the UE uses to obtain MIB.

RAN2 agrees that mode-1 UE is allowed to continue using the configured SL grant type 1 when beam failure in NR Uu occur or T310 is running [2][3]. In case these NR Uu L1 problems happens, the mode-1 UE may not detect some DL RSs normally, especially the DL RS for PUSCH power control scheduled by DCI format 0_0. In other words, DL pathloss for sidelink transmission calculated from DCI format 0_0 may be not accurate and meaningless. Thus, for a UE configured to monitor PDCCH for detection of DCI format 0_0, one simple way to handle during NR Uu L1 problems is changed or fallback to use DL RS resource from the SS/PBCH block that the UE uses to obtain MIB.

Observation 1: In case of beam failure recovery or T310 in NR Uu is running, DL pathloss seems unavailable from the RS resource used for PUSCH power control scheduled by DCI format 0_0.

Proposal 1:  When beam failure recovery procedure or T310 in NR Uu is running, a RS resource from the SS/PBCH block that the UE uses to obtain MIB is used for DL pathloss-based sidelink power control.

2.2 Correction for 38.213

2.2.1 Correction for PSSCH sidelink power control

For section 16.2.1 power control of 38.213 [4], there are some consideration I~IV for correction, as in the following:

- According to RRC [5], as quoted in Appendix I, DL pathloss based power control is disabled if P0DL is not configured, and SL pathloss based power control is disabled if P0SL is not configured. The judgement and derivation for PPSSCH,D(i) and PPSSCH,SL(i) should be aligned.

- In case of DL pathloss based power control is disabled, PPSSCH,D(i) should be set to PCMAX to avoid specification complexity on maximumtransmitPower-SL consideration

In case of SL pathloss based power control is disabled, PPSSCH,SL(i) should be set to PCMAX to avoid specification complexity on maximumtransmitPower-SL consideration.

- In RRC, SL power configuration is included in resource pool configuration. SinceSL pathloss based power control is not supported for groupcast and broadcast, the corresponding specification seems missing.

- Align the parameter names between 38.331 and 38.213.

According text proposal for sidelink power control is provided below.

<   Text Proposal 1 for 38.213 >

### 16.2.1 PSSCH A UE determines a power PPSSCH,b,c(i) for a PSSCH transmission on a resource pool in symbols where a corresponding PSCCH is not transmitted in PSCCH-PSSCH transmission occasion i as: - if maximumtransmitPower-SL is provided then PPSSCH(i)=minPCMAX,PMAX,CBR,minPPSSCH,Di,PPSSCH,SL(i) [dBm] - else PPSSCH(i)=minPCMAX,minPPSSCH,Di,PPSSCH,SL(i) [dBm] where - PCMAX is defined in [8-1, TS 38.101-1] - PMAX,CBR is determined by a value of maximumtransmitPower-SL based on a priority level of the PSSCH transmission and a CBR range that includes a CBR measured in slot i-N [6, TS 38.214] - if dl-P0-PSSCH-PSCCH is provided - PPSSCH,D(i)=PO,D+10log102μ⋅MRBPSSCHi+αD⋅PLD [dBm] PPSSCH,D(i)=minPCMAX,PMAX,CBR,PPSSCH,SL(i) - else - PPSSCH,D(i)=PCMAXminPCMAX,PMAX,CBR [dBm] where - PO,D is a value of dl-P0-PSSCH-PSCCH if provided - αD is a value of dl-Alpha-PSSCH-PSCCH, if provided; else, αD=1 - PLD=PLb,f,c(qd) as described in Clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 when the UE is configured to monitor PDCCH for detection of DCI format 0_0 - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 - MRBPSSCH(i) is a number of resource blocks for the PSSCH transmission occasion i and μ is a SCS configuration - if sl-P0-PSSCH-PSCCH is provided and if cast type of the PSSCH transmission is Unicast - PPSSCH,SL(i)=PO,SL+10log102μ⋅MRBPSSCHi+αSL⋅PLSL [dBm] - else - PPSSCH,SL(i)=PCMAXminPCMAX,PPSSCH,D(i) [dBm] where - PO,SL is a value of sl-P0-PSSCH-PSCCH, if provided - αSL is a value of sl-Alpha-PSSCH-PSCCH, if provided; else, αSL=1 - PLSL= referenceSignalPower – higher layer filtered RSRP, where

2.2.2 Correction for PSFCH sidelink power control

In section 16.3 of 38.213 [4], when DL pathloss based power control is disable for PSFCH, UE will autonomously determine a number NTx,PSFCH≥1 of simultaneous PSFCH transmissions. The upper bound of  NTx,PSFCH is missing. Besides, the parameter names between 38.331 and 38.213 need to be aligned.

<   Text Proposal 2 for 38.213  >

### 16.2.3 PSFCH A UE with Nsch,Tx,PSFCH scheduled PSFCH transmissions, and capable of transmitting a maximum of Nmax,PSFCH PSFCHs, determines a number NTx,PSFCH of simultaneous PSFCH transmissions and a power PPSFCH,k(i) for a PSFCH transmission k, 1≤k≤NTx,PSFCH, on a resource pool in PSFCH transmission occasion i as […] - if dl-Alpha-PSFCH is provided, PPSFCH,one=PO,PSFCH+10log102μ+αPSFCH⋅PL [dBm] where - PO,PSFCH is a value of dl-Alpha-PSFCH - αPFSCH is a value of dl-P0-PSFCH , if provided; else, αPFSCH=1 - PL=PLb,f,c(qd) as described in Clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 when the UE is configured to monitor PDCCH for detection of DCI format 0_0 - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 - […] - else PPSFCH,k(i)=PCMAX-10log10(NTx,PSFCH) [dBm] where the UE autonomously determines NTx,PSFCH PSFCH transmissions with ascending priority order as described in Clause 16.2.4.2 such that Nmax,PSFCH≥NTx,PSFCH≥1 and where PCMAX is determined for the NTx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1]

2.2.3 Correction for reporting HARQ-ACK on Sidelink

In section 16.3 of 38.213 [4], the association to PSFCH PRBs from PSSCH slot and sub-channel is determined by pool configuration, instead of UE autonomously allocates.

Besides, for groupcast option 2, the PSFCH resource will depend on member ID in the sidelink group. Current TS 38.213 description, ”MID is the identity of the UE receiving the PSSCH as indicated by higher layers”, seems ambiguous to associate MID with which identity in higher layer. It is not clear to know MID means member ID. Thus, clarification is needed based on the terminology defined in TS 23.287.

<   Text Proposal 3 for 38.213 >

## 16.3 UE procedure for reporting HARQ-ACK on sidelink <omited> A UE is provided by rbSetPSFCH a set of MPRB, setPSFCH PRBs in a resource pool for PSFCH transmission in a PRB of the resource pool. For a number of Nsubch sub-channels for the resource pool, provided by numSubchannel, and a number of NPSSCHPSFCH PSSCH slots associated with a PSFCH slot, provided by periodPSFCHresource, the i+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH, i+1+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH-1 PRBs from the MPRB, setPSFCH PRBs are associated with slot i and sub-channel j, where Msubch, slotPSFCH=MPRB, setPSFCHNsubch⋅NPSSCHPSFCH, 0≤i<NPSSCHPSFCH, 0≤j<Nsubch, and the association starts in an ascending order of i and continues in an ascending order of j. The UE expects that MPRB, setPSFCH is a multiple of Nsubch∙NPSSCHPSFCH. <omited> A UE determines an index of a PSFCH resource for a PSFCH transmission in response to a PSSCH reception as PID+MIDmodRPRB, CSPSFCH where PID is a physical layer source ID provided by SCI format 2-A or 2-B [5, TS 38.212] scheduling the PSSCH reception, and MID is the member ID of the UE receiving the PSSCH as indicated by higher layers [X, TS 23.287] if the UE detects a SCI format 2-A with Cast type indicator field value of "01"; otherwise, MID is zero.

Proposal 2: 	Adopt the text proposal 1 in updating of TS38.213 section 16.2.1.

Proposal 3: 	Adopt the text proposal 2 in updating of TS38.213 section 16.2.3.

Proposal 4: 	Adopt the text proposal 3 in updating of TS38.213 section 16.3.

- Conclusion

In this contribution, we have following proposals:

Observation 1: In case of beam failure recovery or T310 in NR Uu is running, DL pathloss seems unavailable from the RS resource used for PUSCH power control scheduled by DCI format 0_0.

Proposal 1:  When beam failure recovery procedure or T310 in NR Uu is running, a RS resource from the SS/PBCH block that the UE uses to obtain MIB is used for DL pathloss-based sidelink power control.

Proposal 2: 	Adopt the text proposal 1 in updating of TS38.213 section 16.2.1.

Proposal 3: 	Adopt the text proposal 2 in updating of TS38.213 section 16.2.3.

Proposal 4: 	Adopt the text proposal 3 in updating of TS38.213 section 16.3.

- Reference

- 3GPP TSG RAN WG1 #101-E Chairman’s Notes

- Report of 3GPP TSG RAN2#107 meeting

- Report of 3GPP TSG RAN2#108 meeting

- 3GPP TS 38.213 V16.2.0 (2020-06), “NR; Physical layer procedures for control (Release 16)”

- 3GPP TS 38.331 V16.1.0 (2020-07), “NR; Radio Resource Control (RRC) protocol specification (Release 16)”