- 3GPP TSG RAN WG1 #102-e		                                       R1-2006659

- e-Meeting, Aug 17th – 28th, 2020

Source:	ZTE

Title:	Preliminary views on IoT over NTN

Agenda Item:	8.14

Document for:   Discussion

# Introduction

The first objective of this Study is to identify scenarios applicable to NB-IoT/eMTC [RAN1, RAN2], including:

- Bands of interest in sub 6 GHz

- Device type with PC3 or PC5 (LEO and GEO)

- Satellite constellation orbit LEO and GEO

- Transparent payload.

- Link budget

- NOTE 1: This first objective will be based on the scenarios documented in TR 38.821.

- NOTE 2: UE mobility assumptions follow terrestrial NB-IoT/eMTC assumptions.

The second objective is, for the above identified scenarios, to study and recommend necessary changes to support NB-IoT and eMTC over satellite, reusing as much as possible the conclusions of the studies performed for NR NTN in TR38.821. This objective will address the following items:

- Aspects related to random access procedure/signals [RAN1, RAN2]

- Mechanisms for time/frequency adjustment including Timing Advance, and UL frequency compensation indication [RAN1, RAN2]

- Timing offset related to scheduling and HARQ-ACK feedback [RAN1, RAN2]

- Aspects related to HARQ operation [RAN2, RAN1]

- General aspects related to timers (e.g. SR, DRX, etc.) [RAN2]

- RAN2 aspects related to idle mode and connected mode mobility [RAN2]

  - RLF-based for NB-IoT

  - Handover-based for eMTC

- System information enhancements [RAN2]

- Tracking area enhancements [RAN2]

- NOTE 3: GNSS capability in the UE is taken as a working assumption in this study for both NB-IoT and eMTC devices. With this assumption, UE can estimate and pre-compensate timing and frequency offset with sufficient accuracy for UL transmission. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed.

In RAN#86 meeting [1], the NTN WI has been endorsed with following scope:

In this contribution, preliminary views on the aspects including scenarios, assumption, synchronization, etc, are elaborated.

# Views on the scenarios/assumption

In order to conduct the thoroughly investigation on the feasibility and performance for IoT over NTN, at least typical scenarios and corresponding UE assumption should be defined:

- Scenarios:

During the SI for NR-NTN, all evaluations, e.g., link budget, are conducted by assuming that all terminals are in outdoor scenarios without additional loss, e.g., penetration loss or vegetation loss. However, with consideration on the typical use case of IoT, such assumption is not suitable. More specifically, following typical scenarios with corresponding characteristics can be considered:

- Indoor UE: As typical use case of terrestrial IoT, massive terminals to support service in indoor scenarios, e.g., sensor, meters, are widely deployed. For the evaluation of such case in NTN, except for the path loss, additional O2I penetration loss [2] needs to be considered. Moreover, other impacts, e.g., positioning accuracy should also be taken into account for solution evaluation.

- Outdoor UE: Different as NR NTN, even for some typical use case with for terminal in outdoor scenarios, e.g., logistic, forest monitoring, the assumption with pure free space loss only will not be available due to the blockage from vegetation [3] or others. Therefore, before the performance evaluation, typical use case should be identified with corresponding assumption on the channel condition. For example, for the logistic case, if the direct connection between IoT terminal and satellite is assumed even for the cargo within container, additional penetration loss similar as the O2I car penetration defined in [4] should be considered.

- UE density and traffic assumption:

As elaborated in [5], the terrestrial IoT is able to fulfill the requirement of ITU on 5G, e.g., 1,000,000 device/km2 by assuming the BS deployment with ISD equaling to 500 and 1732 m for different evaluation configurations. In NTN case, as the parameters listed in Table 1, the beam diameter is up to hundreds km even in case of nadir pointing of satellite. According to such assumption, the number of overall UE served by single beam will be extremely large and investigation on the capacity of existing design, e.g., PRACH/paging, should be conducted.

Table 1 Typical satellite configuration for large coverage [2]

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | S-band (i.e. 2 GHz) | 12 m | 1 m | 1 m |
| Satellite EIRP density |  | 53.5 dBW/MHz | 34 dBW/MHz | 28 dBW/MHz |
| Satellite Tx max Gain |  | 45.5 dBi | 24 dBi | 24 dBi |
| 3dB beamwidth |  | 0.7353 deg | 8.8320 deg | 8.8320 deg |
| Satellite beam diameter (Note 2) |  | 450 km | 190 km | 90 km |
| Note 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. Note 2: This beam size refers to the Nadir pointing of the satellite Note 3: All these satellite parameters are applied per beam. Note 4: The EIRP density values are considered identical for all frequency re-use factor options. |  |  |  |  |

Moreover, in case of performance evaluation for UL (e.g., IoT), the interference level (i.e., CIR [2]) is also critical, which is determined by number of simultaneously transmitting UEs randomly distributed over the reference beam. For NR-NTN evaluation, 10 UEs per beam, are used as the baseline with consideration on the typical use case of NR-NTN (e.g., normal mobile UE in remote area or vehicle-mounted with less density). W.r.t IoT case, determination of such value should be well investigated with consideration on the overall density and traffic load.

Proposal 1: Clarification on the target scenarios and UE-density/traffic load is needed before the evaluation

# Views on the potential enhancements

As mentioned in the above, some aspects for potential enhancements are listed in the SI scope. According to the preliminary investigation, following aspects are highlighted:

- XDD mode

In the existing design, both FDD and TDD are supported for terrestrial LTE-M/NB-IoT. W.r.t the NTN scenario, due to large RTT and fixed frame structure, lower system efficiency is foreseen in TDD mode and significant restriction on the scheduling is unavoidable with consideration on the “repetition”, which is used to ensure performance for DL/UL transmission. Therefore, similar as NR-NTN WI [6], only the FDD is considered for satellite scenarios.

- Synchronization

As basic assumption for this SI listed in Note 3 above, pre-compensation on the timing and frequency offset with sufficient accuracy for UL transmission is proposed. However, even in this way, at least following aspect should also be considered in addition to the accuracy:

- Impacts on the timing drift: as typical characteristic of IoT transmission, the large number of repetition should be considered to ensure the coverage, which will lead to longer duration for each transmission, e.g., up to hundreds millisecond. In this case, instead of correction of timing and frequency offset at the initial instant for transmission, impacts of timing drift should also be considered along with the proper assumption for the terminal oscillator.

- Power consumption

Comparing to the terrestrial network, the large path loss will be suffered with the direct connection to the space-bore platform, especially for GEO. In this way, with more repetition, increase of power assumption can be foreseen. Moreover, in order to tracking the variation of timing and frequency offset due to the high mobility of satellite, additional reception for GNSS signal with corresponding pre-compensation is also needed. In this way, evaluation on the power consumption and battery life should be considered.

Proposal 2: Only FDD is considered for the IoT over NTN.

Proposal 3: Impacts on the timing drift should be considered for the synchronization evaluation.

Proposal 4: Evaluation on power consumption should be considered for IoT over NTN.

# Conclusions

In this contribution, discussion on assumption and potential enhancement w.r.t the IoT over NTN has been conducted with following proposals:

Proposal 1: Clarification on the target scenarios and UE-density/traffic load is needed before the evaluation

Proposal 2: Only FDD is considered for the IoT over NTN.

Proposal 3: Impacts on the timing drift should be considered for the synchronization evaluation.

Proposal 4: Evaluation on power consumption should be considered for IoT over NTN.

# Reference

- RP-193235 New Study WID on NB-IoT/eTMC support for NTN

- TR38.821 Solution for NR to support NTN (Release 16)

- ITU-R P.833-8 Attenuation in vegetation

- TR38.901 Study on channel model for frequencies from 0.5 to 100 GHz

- RP-191524 3GPP ITU-R Contribution - Final IMT-2020 Submission Overview

- RP-193234 WI for solutions for NR to support non-terrestrial networks (NTN), RAN#86