3GPP TSG RAN WG1 #102		R1- 2007010

e-Meeting, August 17th – 28th, 2020

Source:	CATT

Title:	 Baseline coverage performance for FR1

Agenda Item:	8.8.1.1

Document for:	Discussion and Decision

Introduction

At RAN1#101 e-meeting, the link level simulation assumptions and the link budget method were discussed for FR1, some agreements were made as below.

Agreements:

- Adopt the following target data rates for eMBB performance evaluation for FR1.

- Urban scenario: DL 10Mbps, UL 1Mbps

- Rural scenario: DL 1Mbps, UL 100kbps

- Rural with long distance scenario: DL 1Mbps, UL 100kbps, [30kbps] (optional)

Agreements:

- For link level simulation, adopt following TBS for Msg3 for FR1

- 56 bits

Agreements:

- For link level simulation, for PDSCH of Msg.4 for FR1.

  - Reuse the following simulation assumption for PDSCH

    - Waveform, [PDSCH duration]

  - FFS: Payload size: [3000bits].

  - Other parameters: Reported by companies.

Agreements:

- For link level simulation, for SSB, PDCCH, PDSCH and PDCCH of Msg.2, PDSCH of Msg.4 and PDSCH for FR1.

  - Reuse following simulation assumptions agreed for PUSCH.

    - Scenario and frequency, frame structure, SCS, pathloss model, channel model, delay spread, UE velocity, number of antenna elements and TxRUs for BS.

  - The number of UE receive chains: is 2.

    - 4 for 4GHz/2.6GHz

    - 2 or 4 for 2GHz

    - 2 for 700MHz

  - For PDSCH, reuse DM-RS configuration, BLER, HARQ, Latency requirements for voice agreed for PUSCH.

    - Reuse DM-RS configuration agreed for PUSCH except that 3 DMRS symbols is used for Msg2.

- For link level simulation, for PRACH and Msg.3 for FR1.

  - Reuse following simulation assumptions agreed for PUSCH

    - Scenario and frequency, frame structure, pathloss model, channel model, delay spread, UE velocity, number of antenna elements and TxRUs for BS and Number of UE transmit chains.

  - For Msg.3, reuse SCS, HARQ configuration, frequency hopping agreed for PUSCH.

In this contribution, we provide simulation results of PDCCH/PDSCH/PUCCH/PUSCH for FR1 in different scenarios.

Discussion

The baseline performance in FR1 is defined as the MPL (Maximum Path Loss) from the link budget template used for IMT-2020 self-evaluation based on the given target data rate. The related target data rate for both downlink and uplink transmission in Urban, Rural and Rural with long distance scenarios are summarized in Table 1 according to the agreements [1]. Accordingly, the target performance is the MPL (Maximum Path Loss) at the cell edge corresponding to the target ISD also shown in table 1. The detailed link level simulation assumptions can be found in Appendix.

Table 1: the target data rate and ISD requirements for different scenarios

| Frequency band | Scenarios | Duplexing scheme | Target data rate | Target ISD | Target performance |
| --- | --- | --- | --- | --- | --- |
| FR1 | Urban | FDD/TDD | DL 10Mbps, UL: 1 Mbps for eMBB TBS 320bits for VoIP | ISD 1: 400m | NLOS 117.97dB for 4GHz; 114.22dB for 2.6GHz |
|  |  |  |  | ISD 2: 500m | NLOS 121.75dB for 4GHz; 118.01dB for 2.6GHz |
|  | Rural | FDD/TDD | DL 1Mbps, UL: 100 kbps for eMBB TBS 320bits for VoIP | ISD 1: 1732m | NLOS 131.57 dB for 4GHz; 127.83 dB for 2.6GHz 125.55 dB for 2GHz 116.43 dB for 700MHz |
|  |  |  |  |  | LOS 98.81dB for 4GHz; 96.56 dB for 2.6GHz 95.59dB for 2GHz 93.37 dB for 700MHz |
|  | Rural with long distance | FDD/TDD | DL 1Mbps, UL 100kbps, TBS 320bits for VoIP | ISD1: 12Km | LOS 132.43dB for 4Ghz 127dB for 700MHz |
|  |  |  |  | ISD2: 30Km | LOS 148.35dB for 4Ghz 142.91dB for 700MHz |

- Urban

Target MPL corresponding to target ISD of 400m and 500m are compared. NLOS outdoor-to-indoor case with 3km/h UE speed is considered in the evaluation. Required SINR for LLS with TDL-C channel model are evaluated.

Figure 1: the link budget results of Urban

In Urban scenario, the coverage of downlink channel, PRACH and PUCCH can meet the requirement.

For PUSCH, some cases cannot meet the requirement especially with target ISD 500m @4GHz as shown in Table 2.

Table 2: the bottleneck channels for urban

| Scneario | Bottleneck channel | Gap (dB) with target ISD 400m | Gap (dB) with target ISD 500m |
| --- | --- | --- | --- |
| 4GHz DDDSU | Msg 3 PUSCH | - | -2.71 |
|  | eMBB PUSCH | -7.28 | -11.06 |
|  | VoIP PUSCH w/o rep | -3.63 | -7.41 |
|  | VoIP PUSCH w/ repetition | - | -0.91 |
| 4GHz DDDSUDDSUU | Msg 3 PUSCH | - | -2.71 |
|  | eMBB PUSCH | -6.08 | -9.86 |
|  | VoIP PUSCH w/o rep | -3.63 | -7.41 |
|  | VoIP PUSCH w/ repetition | - | -0.21 |
| 2.6GHz DDDDDDDSUU | eMBB PUSCH | -3.43 | -7.22 |
|  | VoIP PUSCH w/o rep | - | -3.67 |

Observation 1: In Urban scenario, the PDCCH, SSB, PDSCH, PRACH and PUCCH are not bottleneck. The coverage of PUSCH is the bottleneck.

- eMBB PUSCH is the bottleneck channel in urban scenario for both 2.6 GHz and 4 GHz, up to 11 dB performance gap needs to be compensated

- The performance gap for VoIP PUSCH in urban scenario can be compensated via repetition

- Msg3 PUSCH is problematic if the target ISD=500m

- Rural

Target MPL corresponding to target ISD of 1732m is assumed. NLOS rural case with 3km/h UE speed and 120km/h are considered in the evaluation. Required SINR for LLS with TDL-C channel model and TDL-D model are evaluated.

Figure 2: the link budget results of Rural-TDLC-3kmph

Table 3: the bottleneck channels for Rural-TDLC-3kmph

| Scenario | Bottleneck channel | Gap (dB) with target ISD 1732m |
| --- | --- | --- |
| 4GHz DDDSU | PUCCH F1 | -6.00 |
|  | PF3/11bits/rep1 | -7.95 |
|  | PF3/11bits/rep2 | -5.67 |
|  | PF3/22bits/rep1 | -9.60 |
|  | PF3/22bits/rep2 | -7.71 |
|  | Msg 3 PUSCH | -4.20 |
|  | eMBB PUSCH | -5.50 |
|  | VoIP PUSCH w/o rep | -8.90 |
|  | VoIP PUSCH w/ repetition | -2.60 |
| 2.6GHz DDDDDDDSUU | PF3/11bits/rep1 | -3.74 |
|  | PF3/11bits/rep2 | -1.61 |
|  | PF3/22bits/rep1 | -5.33 |
|  | PF3/22bits/rep2 | -3.50 |
|  | Msg 3 PUSCH | -0.06 |
|  | eMBB PUSCH | -1.66 |
|  | VoIP PUSCH w/o rep | -5.36 |
| 2GHz FDD | PUCCH F1 | -7.80 |
|  | PF3/11bits/rep1 | -6.14 |
|  | PF3/11bits/rep2 | -2.15 |
|  | PF3/22bits/rep1 | -8.10 |
|  | PF3/22bits/rep2 | -6.10 |
|  | Msg 3 PUSCH | -0.39 |
|  | VoIP PUSCH w/o rep | -5.59 |

Figure 3: the link budget results of Rural-TDLC-120kmph

Table 4: the bottleneck channels for Rural-TDLC-120kmph

| Scenario | Bottleneck channel | Gap (dB) with target ISD 1732m |
| --- | --- | --- |
| 4GHz DDDSU | Msg 3 PUSCH | -2.18 |
|  | eMBB PUSCH | -2.88 |
|  | VoIP PUSCH w/o rep | -6.28 |
| 2.6GHz DDDDDDDSUU | VoIP PUSCH w/o rep | -2.64 |
| 2GHz FDD | VoIP PUSCH w/o rep | -3.37 |

Figure 4: the link budget results of Rural-TDLD-3kmph

Figure 5: the link budget results of Rural-TDLD-120kmph

In rural scenario, the coverage of PDCCH, SSB, PDSCH and PRACH can meet the requirement.

For PUCCH, in NLOS O2I (3kmph) scenarios, some cases cannot meet the coverage requirement with target ISD 1732m, especially in 4GHz, 2.6GHz and 2GHz. In NLOS O2O (120kmph) scenarios, the coverage of PUCCH can meet the requirement.

For PUSCH, the coverage cannot meet the requirement in some limited case as shown in Table 3~4.

All the evaluated channels in LOS scenarios can meet the requirement.

Observation 2: In Rural scenario, the PDCCH, SSB, PDSCH, PRACH are not bottleneck. The coverage of PUCCH and PUSCH are bottleneck in the scenarios with NLOS.

- The PUCCH format 3, eMBB PUSCH, VoIP PUSCH, Msg3 PUSCH is problematic if only 1 DMRS symbol is configured

- The performance gap for VoIP PUSCH can be compensated via repetition.

- Rural with long distance

Target MPL corresponding to target ISD of 12Km and 30Km are compared. NLOS outdoor-to-outdoor case with 120km/h UE speed is considered in the evaluation. Required SINR for LLS with TDL-D channel model is evaluated.

Figure 6: the link budget results of Rural with long distance

Table 5: the bottleneck channels for Rural with long distance

| Scenario | Bottleneck channel | Gap (dB) with target ISD 12km | Gap (dB) with target ISD 30km |
| --- | --- | --- | --- |

| 4GHz DDDSU | PUCCH PF1 | - | -0.48 |
| --- | --- | --- | --- |
|  | PUCCH/PF3/11bits/rep=1 | - | -2.45 |
|  | PUCCH/PF3/11bits/rep=2 | - | -0.38 |
|  | PUCCH/PF3/22bits/rep=1 | - | -4.1 |
|  | PUCCH/PF3/22bits/rep=2 | - | -2.06 |
|  | Msg 3 PUSCH | - | -14.8386 |
|  | eMBB PUSCH | - | -15.5386 |
|  | VoIP PUSCH w/o rep | -2.2186 | -18.1386 |
|  | VoIP PUSCH w/ repetition | - | -9.9386 |
| 4GHz DDDSUDDSUU | PUCCH PF1 | - | -0.48 |
|  | PUCCH/PF3/11bits/rep=1 | - | -2.45 |
|  | PUCCH/PF3/11bits/rep=2 | - | -0.38 |
|  | PUCCH/PF3/22bits/rep=1 | - | -4.1 |
|  | PUCCH/PF3/22bits/rep=2 | - | -2.06 |
|  | Msg 3 PUSCH | - | -14.8386 |
|  | eMBB PUSCH | - | -14.7386 |
|  | VoIP PUSCH w/o rep | -2.2186 | -18.1386 |
|  | VoIP PUSCH w/ repetition | - | -9.0386 |
| 700MHz FDD | PDCCH | - | -1.47 |
|  | Msg 3 PUSCH | - | -15.2192 |
|  | eMBB PUSCH | - | -13.9192 |
|  | VoIP PUSCH w/o rep | -2.8092 | -18.7192 |
|  | VoIP PUSCH w/ repetition | - | -9.1192 |

In rural with long distance scenario, it can be seen that in most cases, the coverage of DL channels can meet the requirements.

The coverage of most of UL channels can meet the requirements with target ISD 12km except the PUSCH VOIP without repetition. For the target ISD 30km, the coverage of PUCCH cannot meet requirements at 4GHz, while the PUSCH cannot meet the requirements at 4GHz and 700MHz.

Observation 3: In Rural with long distance scenario, the PUSCH and PUCCH channels become bottleneck with target ISD 30km:

- PUCCH cannot meet requirements at 4GHz

- PUSCH cannot meet the requirements at 4GHz and 700MHz, up to 18 dB performance gap needs to be compensated

Conclusion

This contribution discussed the preliminary simulation results for PDCCH/SSB/PDSCH/PRACH/ PUCCH/PUSCH in FR1, we have the following observations.

Observation 1: In Urban scenario, the PDCCH, SSB, PDSCH, PRACH and PUCCH are not bottleneck. The coverage of PUSCH is the bottleneck.

- eMBB PUSCH is the bottleneck channel in urban scenario for both 2.6 GHz and 4 GHz, up to 11 dB performance gap needs to be compensated

- The performance gap for VoIP PUSCH in urban scenario can be compensated via repetition

- Msg3 PUSCH is problematic if the target ISD=500m

Observation 2: In Rural scenario, the PDCCH, SSB, PDSCH, PRACH are not bottleneck in NLOS Scenarios. The coverage of PUCCH and PUSCH are bottleneck in some limited scenarios.

- The PUCCH format 3, eMBB PUSCH, VoIP PUSCH, Msg3 PUSCH is problematic if only 1 DMRS symbol is configured

- The performance gap for VoIP PUSCH can be compensated via repetition.

Observation 3: In Rural with long distance scenario, the PUSCH and PUCCH channels become bottleneck with target ISD 30km:

- PUCCH cannot meet requirements at 4GHz

- PUSCH cannot meet the requirements at 4GHz and 700MHz, up to 18 dB performance gap needs to be compensated

References

  - RP-193240, New SID on NR coverage enhancement, China Telecom, RAN Meeting #86.

Appendix: the LLS assumptions

Table 3: simulation assumptions of PUSCH

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenario | Urban | Rural | Rural with long distance |
| Carrier | TDD for 4GHz, 2.6GHz | TDD for 4GHz, 2.6GHz FDD for2GHz. 700MHz | TDD for 4GHz FDD for 700MHz |
| TDD configuration | TDD config 1: DDDSU (S: 10D:2G:2U) TDD confg 2: DDDSUDDSUU (S: 10D:2G:2U) TDD config 3: DDDDDDDSUU (S: 6D:4G:4U) | TDD config 1: DDDSU (S: 10D:2G:2U) TDD confg 2: DDDSUDDSUU (S: 10D:2G:2U) TDD config 3: DDDDDDDSUU (S: 6D:4G:4U) | TDD config 1: DDDSU (S: 10D:2G:2U) TDD confg 2: DDDSUDDSUU (S: 10D:2G:2U) |
| SCS | 30kHz for TDD | 30kHz for TDD, 15kHz for FDD. | 30kHz for TDD, 15kHz for FDD. |
| Number of antenna elements for BS | 192 antenna elements, (M,N,P,Mg,Ng) = (8,4,2,1,1) | 64 antenna elements for 4GHz and 2.6GHz, (M,N,P,Mg,Ng) = (8,4,2,1,1) 32 antenna elements for 2GHz, (M,N,P,Mg,Ng) = (8,2,2,1,1) 16 antenna elements for 700MHz, (M,N,P,Mg,Ng) = (4,2,2,1,1) | 64 antenna elements for 4GHz, (M,N,P,Mg,Ng) = (8,4,2,1,1) 16 antenna elements for 700MHz, (M,N,P,Mg,Ng) = (4,2,2,1,1) |
| Number of TxRUs for BS | 64 TXRUs | 64 antenna elements for 4GHz and 2.6GHz, (M,N,P,Mg,Ng) = (8,4,2,1,1) 32 antenna elements for 2GHz, (M,N,P,Mg,Ng) = (8,2,2,1,1) 16 antenna elements for 700MHz, (M,N,P,Mg,Ng) = (4,2,2,1,1) | 2 or 4 TXRUs for 700 MHz 64TxRUs for 4 GHz |
| Number of Tx chains for BS | 4 | 4 | 4 |
| Number of UE Tx chains | 1 | 1 | 1 |
| Duration of PUSCH | 14 OS | 14 OS | 14 OS |
| Number of DMRS | For eMBB/VoIP: 1 OS For Msg 3: 3 OS | For eMBB/VoIP: 3km/h: 1 OS 120km/h: 2 OS For Msg 3: 3 OS | For eMBB/VoIP: 3km/h: 1 OS 120km/h: 2 OS For Msg 3: 3 OS |
| Occupied channel bandwidth | 30 PRBs for eMBB 4 PRBs for VoIP 2 PRBs for Msg 3 | 4 PRBs for eMBB 4 PRBs for VoIP 2 PRBs for Msg 3 | 4 PRBs for eMBB 4 PRBs for VoIP 2 PRBs for Msg 3 |
| Modulation | QPSK | QPSK | QPSK |
| Coding Rate | TDD config 1: eMBB: CR=251/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; TDD config 2: eMBB: CR=193/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; TDD config 3: eMBB: CR=251/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; | TDD config 1: 3km/h: eMBB: CR=193/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; 120km/h: eMBB: CR=251/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; TDD config 2: 3km/h: eMBB: CR=157/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; 120km/h: eMBB: CR=157/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; TDD config 3: 3km/h: eMBB: CR=193/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; 120km/h: eMBB: CR=251/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; FDD: 3km/h: eMBB: CR=99/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; 120km/h: eMBB: CR=99/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; | TDD config 1: 3km/h: eMBB: CR=30/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; 30km/h: eMBB: CR=30/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; TDD config 2: 3km/h: eMBB: CR=30/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; 30km/h: eMBB: CR=30/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; |
| Channel Coding | LDPC | LDPC | LDPC |
| Channel Mode | TDL-C, 300ns | TDL-C, 300ns TDL-D, 300ns | TDL-D, 30ns |
| UE Mobility: | 3km/h | 3km/h for indoor, 120km/h for outdoor | 3km/h for indoor, 120km/h for outdoor |
| Channel estimation | True | True | True |
| Target BLER | 10% BLER for eMBB 2% BLER for VoIP 10% BLER for Msg3 | 10% BLER for eMBB 2% BLER for VoIP 10% BLER for Msg3 | 10% BLER for eMBB 2% BLER for VoIP 10% BLER for Msg3 |
| PUSCH repetition | w/o repetition for eMBB/Msg 3 w/o repetition and w/ repetition for VoIP: 8 repetitions for TDD config 1 12 repetitions for TDD config 2 8 repetitions for TDD config 3 | w/o repetition for eMBB/Msg 3 w/o repetition and w/ repetition for VoIP: 8 repetitions for TDD config 1 12 repetitions for TDD config 2 8 repetitions for TDD config 3 20 repetitions for FDD | w/o repetition for eMBB/Msg 3 w/o repetition and w/ repetition for VoIP: 8 repetitions for TDD config 1 12 repetitions for TDD config 2 20 repetitions for FDD |
| Waveform | DFT-s-OFDM | DFT-s-OFDM | DFT-s-OFDM |

Table 4: simulation assumptions of PUCCH

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenario | Urban | Rural | Rural with long distance |
| Carrier | 4GHz, 2.6GHz | TDD for 4GHz, 2.6GHz FDD for2GHz. 700MHz | TDD for 4GHz FDD for 700MHz |
| Format type | PUCCH format 1 with 2bits PUCCH format 3 with 11/22 bits | PUCCH format 1 with 2bits PUCCH format 3 with 11/22 bits | PUCCH format 1 with 2bits PUCCH format 3 with 11/22 bits |
| BWP | 100MHz | 100MHz for TDD 20MHz for FDD | 20MHz |
| SCS | 30kHz | 30kHz for TDD 15kHz for FDD | 15kHz |
| Number of antenna elements for BS | 192, (M,N,P,Mg,Ng) = (12,8,2,1,1) | 64, (M,N,P,Mg,Ng) = (8,4,2,1,1) 32, (M,N,P,Mg,Ng) = (8,2,2,1,1) | 16, (M,N,P,Mg,Ng) = (4,2,2,1,1) |
| Number of TxRUs for BS | 4 | 4 | 4 |
| Number of UE Tx chains | 1 | 1 | 1 |
| Duration of PUSCH | 14 OS | 14 OS | 14 OS |
| DMRS configuration for PUCCH | w/o additional DMRS | w/o additional DMRS | w/o additional DMRS |
| Occupied channel bandwidth | 1 PRB | 1 PRB | 1 PRB |
| Channel Mode | TDL-C, 300ns | TDL-D, 300ns for O2O TDL-C, 300ns for O2I | TDL-D, 30ns |
| UE Mobility: | 3 km/h | 120 km/h for outdoor 3 km/h for indoor | 120 km/h |
| Channel estimation | True | True | True |
| PUCCH repetition | 1/2 | 1/2 | 1/2 |
| Frequency hopping | On | On | On |

Table 5: simulation assumptions of PRACH

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenario | Urban | Rural | Rural with long distance |
| Carrier | 4GHz, 2.6GHz | TDD for 4GHz, 2.6GHz FDD for2GHz. 700MHz | TDD for 4GHz FDD for 700MHz |
| Format type | PRACH format 0 PRACH format B4 | PRACH format 0 PRACH format B4 | PRACH format 0 PRACH format B4 |
| SCS | 30kHz | 30kHz for TDD 15kHz for FDD | 15kHz |
| Number of TxRUs for BS | 4 | 4 | 4 |
| Number of UE Tx chains | 1 | 1 | 1 |
| Duration of PUSCH | 14 OS | 14 OS | 14 OS |
| Occupied channel bandwidth | 12 PRB | 12 PRB | 12 PRB |
| Channel Mode | TDL-C, 300ns | TDL-D, 300ns for O2O TDL-C, 300ns for O2I | TDL-D, 30ns |
| UE Mobility: | 3 km/h | 120 km/h for outdoor 3 km/h for indoor | 120 km/h |
| PRACH BLER | 1% missed detection at 0.1% false alarm probability | 1% missed detection at 0.1% false alarm probability | 1% missed detection at 0.1% false alarm probability |

Table 6: simulation assumptions of PDSCH

| Parameters | Values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | Urban |  |  |  | Rural |  | Rural with long distance |  |
|  | eMBB |  | VoIP |  | eMBB | VoIP | eMBB | VoIP |
| Carrier: | 4GHz (TDD), 2.6GHz (TDD) |  |  |  | 4GHz (TDD), 2.6GHz (TDD) 2GHz (FDD), 700MHz (FDD) |  | 700MHz (FDD), 4GHz (TDD) |  |
| BWP | 100MHz for 4GHz and 2.6GHz. |  |  |  | 100MHz for 4GHz and 2.6GHz. 20MHz for 2GHz 20MHz for 700MHz. |  | 100MHz for 4GHz. 20MHz for 700MHz. |  |
| SCS | 30kHz |  |  |  | 30kHz for TDD, 15kHz for FDD. |  | 30kHz for TDD, 15kHz for FDD. |  |
| Number of antenna elements for BS | 192 antenna elements for 4GHz and 2.6GHz, (M,N,P,Mg,Ng) = (12,8,2,1,1) (optional) 128 antenna elements for 4GHz, (M,N,P,Mg,Ng) = (8,8,2,1,1) |  |  |  | 64 antenna elements for 4GHz and 2.6GHz (M,N,P,Mg,Ng) = (8,4,2,1,1) 32 antenna elements for 2GHz (M,N,P,Mg,Ng) = (8,2,2,1,1) 16 antenna elements for 700MHz (M,N,P,Mg,Ng) = (4,2,2,1,1) |  | 64 antenna elements for 4GHz and 2.6GHz (M,N,P,Mg,Ng) = (8,4,2,1,1) 32 antenna elements for 2GHz (M,N,P,Mg,Ng) = (8,2,2,1,1) 16 antenna elements for 700MHz (M,N,P,Mg,Ng) = (4,2,2,1,1) |  |
| Number of TxRUs for BS | 64TxRUs for 2.6 and 4 GHz. |  |  |  | 2 or 4 TXRUs for 2GHz, 700 MHz 64TxRUs for 2.6 and 4 GHz. Optional: 32 TXRUs at 2 GHz |  | 2 or 4 TXRUs for 700 MHz 64TxRUs for 4 GHz. |  |
| UE receive chains | 4 for 4GHz/2.6GHz |  |  |  | 4 for 4GHz/2.6GHz 4 for 2GHz 2 for 700MHz |  | 4 for 4GHz; 2 for 700MHz |  |
| length of PDSCH | 12 OS |  |  |  | 12 OS |  | 12 OS |  |
| Configuration of DMRS | 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. |  |  |  | 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. |  | For 120km/h, (Optional: 30km/h): Type I, 2 or 3 DMRS symbol, no multiplexing with data. |  |
| Data rate | 10Mbps | [320] bits with 20ms data arriving |  |  | 1Mbps | [320] bits with 20ms data arriving | 1Mbps | [320] bits with 20ms data arriving |
| PRBs/TBS/MCS [TBD] | full bandwidth | [320] bits with 20ms data arriving |  |  | full bandwidth | [320] bits with 20ms data arriving | full bandwidth | [320] bits with 20ms data arriving |
| Channel Coding | LDPC |  |  |  | LDPC |  | LDPC |  |
| Channel Model | TDL-C for NLOS, |  |  |  | TDL-C for NLOS, TDL-D for LOS. |  | TDL-C for NLOS, TDL-D for LOS. |  |
| DS | 300ns |  |  |  | 300ns |  | 30ns |  |
| UE velocity: | 3km/h |  |  |  | 3km/h |  | 120km/h |  |
| Channel estimation | True |  |  |  | True |  | True |  |
| Target BLER | w/o HARQ, 10% iBLER. |  |  | 2% rBLER. | w/o HARQ, 10% iBLER. | 2% rBLER. | w/o HARQ, 10% iBLER. | 2% rBLER. |
| UL DL configuration if TDD is assumed: | DDDSU (S: 10D:2G:2U) only for 4GHz DDDSUDDSUU (S: 10D:2G:2U) only for 4GHz DDDDDDDSUU (S: 6D:4G:4U) only for 2.6GHz |  |  |  | DDDSU (S: 10D:2G:2U) only for 4GHz DDDSUDDSUU (S: 10D:2G:2U) only for 4GHz DDDDDDDSUU (S: 6D:4G:4U) only for 2.6GHz |  | DDDSU (S: 10D:2G:2U) only for 4GHz DDDSUDDSUU (S: 10D:2G:2U) only for 4GHz DDDDDDDSUU (S: 6D:4G:4U) only for 2.6GHz |  |

Table 7: simulation assumptions of PDCCH

| Parameters | Values |
| --- | --- |
| Carrier | 2GHz/2.6GHz/4GHz/700MHz |
| SCS | 30kHz, 15kHz |
| Aggregation level | 16 |
| Payload | 40 bits + 24CRC bits |
| CORESET size | 2 symbols, 48 PRBs |
| Tx Diversity | 1-port precoder cycling |
| BLER for PDCCH | 1% BLER |
| Number of SSB for broadcast PDCCH of Msg.2 | 4 |

Table 7: simulation assumptions of SSB

| Parameters | Values |
| --- | --- |
| Carrier | 2GHz/2.6GHz/4GHz/700MHz |
| Periodicity | 20ms |
| SCS | 15KHz, 30KHz |
| BLER for SSB | 1% BLER |
| Performance metric | Combination of 4 SSBs in 80ms. Note: UE is not assumed to know the SS/PBCH block index |