3GPP TSG RAN WG1 #102-e		R1-2007121

e-Meeting, August 17th – 28th, 2020

Source:	Moderator (OPPO)

Title:	Text Proposal for TS 38.211 in [102-e-NR-eMIMO-05]

Agenda Item:	7.2.6

Document for:	Discussion and Decision

- Text Proposal

Reason for changes:

The RRC parameter AdditionaldataScramblingIdentityPDSCH in TS 38.211 does not align with the RRC parameter name in TS 38.331.

Summary of changes:

In TS 38.211, section 7.3.1.1, correct the RRC parameter name AdditionaldataScramblingIdentityPDSCH to align with RRC parameter name in TS 38.331.

Specs/Sections impacted:

TS 38.211 V16.2.0 /7.3.1.1

Consequences if not approved:

Misalignment between TS 38.211 and TS 38.331.

The text proposal for TS 38.211 V16.2.0 is:

### 7.3.1 Physical downlink shared channel #### 7.3.1.1 Scrambling Up to two codewords can be transmitted. In case of single-codeword transmission, . For each codeword , the UE shall assume the block of bits bq0, …, bq(Mbitq-1), where Mbit(q) is the number of bits in codeword transmitted on the physical channel, are scrambled prior to modulation, resulting in a block of scrambled bits bq0, …, bq(Mbitq-1)according to b(q)i=b(q)i+c(q)(i) mod 2 where the scrambling sequence cq(i) is given by clause 5.2.1. The scrambling sequence generator shall be initialized with cinit=nRNTI⋅215+q⋅214+nID where - equals the higher-layer parameter dataScramblingIdentityPDSCH if configured and the RNTI equals the C-RNTI, MCS-C-RNTI, or CS-RNTI, and the transmission is not scheduled using DCI format 1_0 in a common search space; - nID∈0,1,…,1023 equals - the higher-layer parameter dataScramblingIdentityPDSCH if the codeword is scheduled using a CORESET with CORESETPoolIndex equal to 0; - the higher-layer parameter dataScramblingIdentityPDSCH2-r16 if the codeword is scheduled using a CORESET with CORESETPoolIndex equal to 1; if the higher-layer parameters dataScramblingIdentityPDSCH and dataScramblingIdentityPDSCH2-r16 are configured together with the higher-layer parameter CORESETPoolIndex containing two different values, and the RNTI equals the C-RNTI, MCS-C-RNTI, or CS-RNTI, and the transmission is not scheduled using DCI format 1_0 in a common search space; - nID=NIDcell otherwise and where corresponds to the RNTI associated with the PDSCH transmission as described in clause 5.1 of [6, TS 38.214]. < Unchanged parts are omitted >