3GPP TSG-RAN WG1 Meeting #102-e	R1-2007129

# e-Meeting, August 17th – 28th, 2020

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1340 | rev | - | Current version: | 14.15.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on conditions for selecting resources when the number of HARQ transmissions is two in sidelink transmission mode 4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sharp |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_V2X-Core |  |  |  |  |  | Date: |  |  | 2020-08-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In LTE sidelink transmission mode 4, when the number of HARQ transmissions is two, with current conditions that -15≤k≤15 and k≠0 as specified in section 14.1.1.7, one subframe in the set tn+j×Prsvp_TX'SL can be the same as one subframe in the set tn+k+j×Prsvp_TX'SL if k mod Prsvp_TX'=0. In this case, UE may transmit PSSCHs overlapping in time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that k mod Prsvp_TX'≠0 in the conditions when the number of HARQ transmissions is two in mode 4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE could be required to transmit PSSCHs overlapping in time. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.1.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 14.1.1.7	Conditions for selecting resources when the number of HARQ transmissions is two in sidelink transmission mode 4

When a set of subframes  for  have been selected for a set of transmission opportunities of PSSCH, a set of subframes  for  for another set of transmission opportunities of PSSCH shall meet the conditions ,  and k mod Prsvp_TX'≠0 where  and  is the maximum number of transmission opportunities of PSSCH in a selected subframe set. Here,  is the resource reservation interval provided by higher layers.