3GPP TSG-RAN WG1 Meeting #102-e	R1-2007409

# E-Meeting, August 17 – 28, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1360 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PUR-RNTI for NB-IoT in 36.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Huawei) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh3-Core |  |  |  |  |  | Date: |  |  | 2020-08-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN1 and RAN2 specifications are not following a common terminology with respect to the PUR RNTI. RAN1 specifications are using “PUR C-RNTI” while RAN2 specifications are using “PUR-RNTI” (e.g. see TS 36.302). |  |  |  |  |  |  |  |  |
| Summary of change: |  | PUR C-RNTI is corrected to be PUR-RNTI to be aligned with RAN2 specifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There may be ambiguity when referring to the PUR-RNTI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.4.1, 16.5.1, 16.5.1.1, 16.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

    - 16.4.1	UE procedure for receiving the narrowband physical downlink shared channel

<Unchanged parts omitted>

If a UE is configured by higher layers to decode NPDCCH with CRC scrambled by the PUR-RNTI, the UE shall decode the NPDCCH and the corresponding NPDSCH according to any of the combination defined in Table 16.4.1-9. The scrambling initialization of the NPDSCH corresponding to these NPDCCHs is by PUR-RNTI.

Table 16.4.1-9: NPDCCH and NPDSCH configured by PUR-RNTI

| DCI format | Search Space |
| --- | --- |
| DCI format N1 | UE specific by PUR-RNTI |

<Unchanged parts omitted>

    - 16.5.1	UE procedure for transmitting format 1 narrowband physical uplink shared channel

<Unchanged parts omitted>

A UE may transmit NPUSCH on preconfigured uplink resources as configured by higher layers. The scrambling initialization of NPUSCH transmission using preconfigured uplink resource is by PUR-RNTI.

If a UE is configured by higher layers to decode NPDCCHs with the CRC scrambled by the PUR-RNTI, the UE shall decode the NPDCCH according to the combination defined in Table 16.5.1-6 and in case the indication in the DCI corresponds to the retransmission of a transport block transmitted using preconfigured uplink resource, transmit a corresponding NPUSCH. The scrambling initialization of this NPUSCH corresponding to these NPDCCHs and the NPUSCH retransmission for the same transport block is by PUR-RNTI.

Table 16.5.1-6: NPDCCH and NPUSCH configured by PUR-RNTI

| DCI format | Search Space |
| --- | --- |
| DCI format N0 | UE specific by PUR-RNTI |

      - 16.5.1.1	Resource allocation

The resource allocation information in uplink DCI format N0 for NPUSCH transmission or configured by higher layers for NPUSCH transmission using preconfigured uplink resource indicates to a scheduled UE

- a set of contiguously allocated subcarriers () of a resource unit determined by the Subcarrier indication field,

- a number of resource units () determined by the resource assignment field according to Table 16.5.1.1-2,

- a repetition number () determined by the repetition number field according to Table 16.5.1.1-3. For a NPUSCH transmission using preconfigured uplink resource, the UE shall use the repetition number determined by the NPUSCH repetition adjustment field according to Table 16.5.1.1-3 from the most recent NPDCCH DCI format N0 with CRC scrambled by PUR-RNTI with the value of "modulation and coding scheme" field () set to '14' if detected, configured by higher layers otherwise.

<Unchanged parts omitted>

  - 16.6	Narrowband physical downlink control channel related procedures

<Unchanged parts omitted>

For NPDCCH UE-specific search space, the aggregation and repetition levels defining the search spaces and the corresponding NPDCCH candidates are listed in Table 16.6-1 by substituting the value of with the higher layer configured parameter npdcch-NumRepetitions, except for NPDCCH candidates associated with PUR-RNTI in which case it is given by higher layer parameter npdcch-NumRepetitions in PUR-Config-NB.

<Unchanged parts omitted>

-	for NPDCCH UE-specific search space,

-	is given by the higher layer parameter npdcch-StartSF-USS, except for NPDCCH candidates associated with PUR-RNTI in which case it is given by higher layer parameter npdcch-StartSF-USS in PUR-Config-NB,

-	is given by the higher layer parameter npdcch-Offset-USS, except for NPDCCH candidates associated with PUR-RNTI in which case it is given by higher layer parameter npdcch-Offset-USS in PUR-Config-NB,

<Unchanged parts omitted>

If the UE has initiated a NPUSCH transmission using preconfigured uplink resource ending in subframe n, the UE shall monitor the NPDCCH UE-specific search space in a search space window starting in subframe n+4 with duration given by higher layer parameter pur-SS-window-duration. Upon detection of a NPDCCH with DCI format N0 with CRC scrambled by PUR-RNTI intended for the UE within the search space window and the value of "modulation and coding scheme" field () in the corresponding DCI is set to '14', the UE is not required to monitor the NPDCCH UE-specific search space for the remaining search space window duration.