3GPP TSG RAN WG1 Meeting #102-e	R1-2007418

eMeeting, August 17 – 28, 2020

Agenda Item:	7.2.4.2.1

Source:	Moderator (Ericsson)

Title:	TPs for 38.214 for the agreements in [102-e-NR-5G_V2X_NRSL-Mode-1-02]

Document for:	Discussion, Decision

# 1	List of issues for discussion

This document captures the TPs for 38.214 endorsed in the following e-mail discussion

[102-e-NR-5G_V2X_NRSL-Mode-1-02] Email discussion/approval covering:

- HARQ reporting to gNB

  - Details in the WA from RAN#100-e for the case of reaching the maximum number of HARQ re-transmissions for a TB.

  - Whether there are other exceptional reports to the gNB (e.g., nothing to transmit for DG, etc.) and, if so, how to address them.

  - Editorial corrections and clarifications for HARQ reporting to gNB (if any).

- Processing times

  - Processing time for SL CG type-2.

  - Whether the gNB needs to be aware of SL HARQ RTT (Z = a + b) or alternative assumptions or behaviour, if necessary.

  - Editorial corrections and clarifications for processing times (if any).

By 8/20, followed by potential TPs by 8/25 – Ricardo (Ericsson)

The single text proposal in this document captures the following agreement:

Agreements:

- For SL configured grant type-2 activation, the UE processing time is equal to Tproc (agreed in RAN1#101-e).

# Text Proposals

## TP 1.4.1-1

The motivation of the TP is as follows:

- Reasons for change: introduction of processing time for SL CG type-2 (new agreement)

- Summary of changes:

  - Introduction of processing time for SL CG type-2.

- Specs/Sections impacted: TS 38.214 Clause 8.6

- Consequences if not approved: the processing time for CG type-2 is not defined.

-------------------------- Start of Text Proposal for TS 38.214 -------------------------- <Unchanged parts omitted> ## 8.6 UE PSSCH preparation procedure time For sidelink dynamic grant and for SL configured grant type 2 activation, if the first sidelink symbol in the sidelink allocation for a PSSCH for a transport block and the associated PSCCH, including the DM-RS and the duplicated symbol, as defined by the slot offset KSL of the scheduling DCI for dynamic grant or the activating DCI for SL configured grant type 2, is no earlier than at symbol L, where L is defined as the next sidelink symbol with its CP starting Tproc=(N2+d2,1)(2048+144)⋅κ2-μ⋅TC after the end of the reception of the last symbol of the PDCCH carrying the DCI scheduling the sidelink transmissions for dynamic grant or activating the SL configured grant type 2, then the UE shall transmit the PSSCH and the associated PSCCH. - N2 is based on µ of Table 8.6-1, where µ corresponds to the one of (µDL, µSL) resulting with the largest Tproc, where the µDL corresponds to the subcarrier spacing of the downlink with which the PDCCH carrying the DCI scheduling the PSSCH for dynamic grant or activating the SL configured grant type 2 was transmitted and µSL corresponds to the subcarrier spacing of the sidelink channel with which the PSSCH and the associated PSCCH are to be transmitted, and κ is defined in Clause 4.1 of [4, TS 38.211]. - d2,1 = 1. Otherwise the UE may ignore the scheduling DCI for dynamic grant or the activating DCI for SL configured grant type 2. <Unchanged parts omitted> ------------------------------------ End of Text Proposal ------------------------------------