3GPP TSG RAN WG1 Meeting #102-e R1-2007451 e-Meeting, 17th – 28th August, 2020 CR-Form-v12.0. Draft CHANGE REQUEST. 38.202; CR; 0018; rev; -; Current version:; 16.1.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Corrections on 5G V2X sidelink features. Source to WG:; Qualcomm. Source to TSG:. Work item code:; 5G_V2X_NRSL-Core; Date:; 2020-09-03. Category:; F; Release:; Rel-16. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; The following agreements was made according to the email discussion [102-e-NR-5G_V2X_NRSL-Mode-1-01] as summarized in R1-2007416 which allow monitoring PDCCH with SL-RNTI, SL-CS-RNTI, and SL Semi-Persistent Scheduling V-RNTI on one of PCell or SCell, but not on PSCell. The agreements also resolve remaining issues on DCI alignment of DCI format 3-0 with other DCI formats. Agreements: - DCI formats 3-0 and 3-1 are monitored on PCell or a SCell. - Discuss further offline whether it’s a same-carrier scheduling case only or not - No RRC impact is expected - PUCCH carrying SL HARQ-ACK reports is transmitted on PCell - Discuss further offline the applicability or not to PUCCH Scell in case of two PUCCH groups in CA Agreements: - If the DCI size budget is not exceeded, no alignment of DCI format 3_0 / 3_1 with other NR DCI formats is performed. - If the DCI size budget is exceeded, DCI format 3_0 / 3_1 is zero-padded until the size is equal to that of the next large Uu DCI format (in size). - The UE does not expect that the following two conditions happen simultaneously: - The DCI size budget is exhausted - DCI format 3_0 / 3_1 is larger than all other configured DCI formats. - Note: the DCI size budget is performed for Uu DCI formats first, before the considerations for DCI format 3_0/3_1 as listed in the above bullets Agreements: - In the preceding agreement, the DCI size budget refers to the budget of the cell on which the DCI format 3_0 or DCI format 3_1 is received. Agreements: - Note: It is understood that the carrier transmitting DCI formats 3_0/3_1 is configurable per the current signalling - Only one carrier can be configured for a UE to monitor DCI formats 3_0/3_1 - From RAN1 perspective, no additional RRC signaling is necessary. - It is subject to UE capability discussion for the case when the carrier transmitting DCI formats 3_0/3_1 is different from the SL carrier - The PUCCH group containing the carrier transmitting DCI format 3_0 is also the PUCCH group carrying SL HARQ-ACK reports (either on PCell or PUCCH SCell) - It is assumed that if CG type 1 is configured for CG SL HARQ-ACK reporting, it there is always a configuration of a carrier transmitting DCI format 3_0. -. Summary of change:; The L0, L1, M Downlink “Reception Types” from Table 6.2-2 are removed from the column corresponding to PSCell because the monitoring of PDCCH with SL-RNTI, SL-CS-RNTI, and SL Semi-Persistent Scheduling V-RNTI is happening on one of PCell or SCell and not on PSCell. In PCell and SCell columns of Table 6.2-2, remove the brackets from the L0+L1+M to enable simultaneous processing of DCI format 3_0 and 3_1 with other DCI formats.. Consequences if not approved:; Unclear behavior of which cells can be used to monitor DCI format 3_0 and 3_1 and whether they can be simultaneously monitored with other DCI formats.. Clauses affected:; 6.2. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:; Isolated impact analysis: The CR captures which cells can be used to monitor DCI format 3_0 and 3_1 and whether they can be simultaneously monitored with other DCI formats.. This CR's revision history:.

## 6.2 Downlink

The tables 6.2-1, 6.2-2 describe the possible combinations of physical channels that can be received simultaneously in the downlink by one UE. Table 6.2-1 introduces notation for a "Reception Type" which represents a physical channel and any associated transport channel. Table 6.2-2 describes the combinations of these "Reception Types" which are supported by the UE depending on capabilities [8, TS 38.306], and enumerates how many of each can be received simultaneously. The UE shall be able to receive all TBs according to the indication on PDCCH. Any subset of the combinations specified in table 6.2-2 is also supported.

Table 6.2-1: Downlink "Reception Types"

| "Reception Type" | Physical Channel(s) | Monitored RNTI | Associated Transport Channel | Comment |
| --- | --- | --- | --- | --- |
| A | PBCH | N/A | BCH |  |
| B | PDCCH+PDSCH | SI-RNTI | DL-SCH | Note 1 |
| C0 | PDCCH | P-RNTI | N/A | Note 1, Note 2 |
| C1 | PDCCH+PDSCH | P-RNTI | PCH | Note 1 |
| D0 | PDCCH+PDSCH | RA-RNTI or Temporary C-RNTI or MsgB-RNTI | DL-SCH | Note 3 |
| D1 | PDCCH+PDSCH | C-RNTI, CS-RNTI, MCS-C-RNTI | DL-SCH |  |
| D2 | PDCCH | C-RNTI, CS-RNTI, MCS-C-RNTI | DL-SCH |  |
| E | PDCCH | C-RNTI | N/A | Note 4 |
| F0 | PDCCH | Temporary C-RNTI | UL-SCH | Note 3 |
| F1 | PDCCH | C-RNTI, CS-RNTI, MCS-C-RNTI | UL-SCH |  |
| G | PDCCH | SFI-RNTI | N/A |  |
| H | PDCCH | INT-RNTI | N/A |  |
| J0 | PDCCH | TPC-PUSCH-RNTI | N/A |  |
| J1 | PDCCH | TPC-PUCCH-RNTI | N/A |  |
| J2 | PDCCH | TPC-SRS-RNTI | N/A |  |
| K | PDCCH | SP-CSI-RNTI | N/A |  |
| L0 | PDCCH | SL-RNTI | SL-SCH |  |
| L1 | PDCCH | SL-CS-RNTI | SL-SCH |  |
| M | PDCCH | SL Semi-Persistent Scheduling V-RNTI | SL-SCH | Note 5 |
| N | PDCCH | PS-RNTI | N/A |  |
| O | PDCCH | AI-RNTI | N/A |  |
| Note 1: These are received from PCell only. Note 2: In some cases UE is only required to monitor the short message within the DCI for P-RNTI. Note 3: These are received from PCell or PSCell. Note 4: This corresponds to PDCCH-ordered PRACH. Note 5: This corresponds to PDCCH scheduling LTE PC5. |  |  |  |  |

Table 6.2-2: Downlink "Reception Type" combinations

| Supported Combinations |  |  | Comment |
| --- | --- | --- | --- |
| PCell | PSCell | SCell |  |
| 1. RRC_IDLE |  |  |  |
| A + (B and/or C1 and/or D0) + F0 |  |  | Note 1 |
| 2. RRC_INACTIVE |  |  |  |
| A + (B and/or C1 and/or D0) + F0 |  |  | Note 1 |
| 3. RRC_CONNECTED |  |  |  |
| (A + C0 + (B and/or (D0 or (m1*D1+m2*D2))) + E + F0 + n*F1 + G + H + J0 + J1 + J2 + K + O + L0 + L1 + M) or ((A+B+C0+[D0]) [and/or] N) | (A + (D0 or (m1*D1+m2*D2)) + E + F0 + n*F1 + G + H + J0 + J1 + J2 + K + O) or ((A+B+C0+[D0]) [and/or] N) | m1*D1 + m2*D2 + E + n*F1 + G + H + J0 + J1 + J2 + K + O + L0 + L1 + M | Note 2, Note 3, Note 4, Note 5, Note 6, Note 7, Note 8 |
| Note 1: UE is not required to decode more than two PDSCH simultaneously, and decoding prioritization when more than two are received is up to UE implementation. Note 2: For PCell, UE is not required to decode SI-RNTI PDSCH simultaneously with C-RNTI PDSCH, unless in FR1. Note 3: Supported combinations are subject to UE capabilities for dual connectivity, carrier aggregation, receiving of group TPC commands, pre-emption indication and dynamic SFI monitoring. Note 4: The values of m2 ≥ 0 and n≥ 0 in the supported combinations are subject to the UE capability. Note 5: Support of monitoring PDCCH with SL-RNTI, SL-CS-RNTI, SL Semi-Persistent Scheduling V-RNTI are subject to UE capability. Note 6: The values of m1 ≥ 1 in the supported combinations are subject to the UE capability. Note 7: In Active time, a UE is not expected to monitor the DCI format for the PDCCH scrambled by PS-RNTI. Note 8: The PDCCH scrambled by PS-RNTI can only be configured on the PCell and PSCell. |  |  |  |