3GPP TSG RAN WG1 Meeting #103-e	R1-2007563

E-meeting, October 26-November 13, 2020

Agenda Item:	8.12.2

Source:	Huawei, HiSilicon

Title:	Mechanisms to improve reliability for RRC_CONNECTED UEs

Document for:	Discussion and Decision

# Introduction

During RAN#88-e, a new work item focused on developing technical solutions for enabling multicast and broadcast services in NR has been discussed and agreed [1]. One of the objectives in WID as mentioned above is that specify RAN basic functions for broadcast/multicast services (MBS) for UEs in RRC_CONNECTED state.

- Specify RAN basic functions for broadcast/multicast for UEs in RRC_CONNECTED state [RAN1, RAN2, RAN3]:

  - Specify required changes to improve reliability of Broadcast/Multicast service, e.g. by UL feedback. The level of reliability should be based on the requirements of the application/service provided.[RAN1, RAN2]

The following agreements about improving the reliability for RRC_CONNECTED UEs in NR MBS were made during the RAN1#102-e meeting [2].

Agreements:

- For RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast and no additional evaluation is needed to justify this.

  - FFS: The detailed HARQ-ACK feedback solutions, e.g., ACK/NACK based, NACK-only based.

  - FFS: HARQ-ACK feedback can be optionally disabled and/or enabled.

Agreements:

- For RRC_CONNECTED UEs, at least support slot-level repetition for group-common PDSCH.

  - FFS: whether enhancement is needed

Agreements:

- For RRC_CONNECTED UEs, existing CSI feedback can be used for multicast transmission.

  - FFS: whether enhancement is needed

In this contribution, we mainly discuss some details of HARQ feedback of NR MBS, including ACK/NACK based solution and NACK-only based solution, disabling and/or enabling of HARQ feedback. Then we discuss the prioritization and multiplexing rules for NR multicast services based on the priority based framework in URLLC Rel-16 and potential enhancements in URLLC Rel-17 [3]. Scheduling and resource configuration for NR MBS are discussed in another contribution [4].

# HARQ feedback

This section discusses two options of detailed HARQ feedback solutions for NR MBS, and discuss the disabling and enabling of HARQ feedback for NR MBS.

## Option 1: ACK/NACK feedback mode

In HARQ feedback for NR unicast, a UE chooses a PUCCH resource to feedback ACK/NACK. In HARQ feedback for NR multicast, one option is that the HARQ feedback for NR multicast takes the HARQ feedback procedure for NR unicast as a baseline.

When a UE in RRC_CONNECTED state receives a PDSCH of MBS, UE will feedback an ACK for receiving PDSCH successfully or a NACK for receiving PDSCH unsuccessfully in form of UCI information. Before feedback, UE will choose a PUCCH resource set according to the size of UCI information. By the PUCCH resource indicator in the DCI which schedules the PDSCH of MBS, UE chooses a specific PUCCH resource from the resource set chosen before to carry the UCI information. In addition, gNB can perform unicast based retransmissions based on the feedbacks on each UE’s PUCCH resource, or gNB will perform a multicast based retransmission as long as it receives at least one NACK feedback.

From the mechanism described above, HARQ feedback for NR MBS with option 1 can almost reuse the framework of the HARQ feedback procedure and configuration for NR unicast. HARQ codebook multiplexing with unicast can also be straightforward. One potential drawback is that large HARQ feedback overhead is consumed in case of a large number of RRC CONNECTED UEs receiving NR MBS, but mechanisms to reduce the HARQ overhead could be considered for option 1.

Observation 1: The ACK/NACK feedback mode for NR MBS can reuse the procedure and configuration of HARQ feedback for NR unicast, but may lead to large feedback overhead.

## Option 2: NACK only feedback mode

Another mode that multiple UEs share a common NACK resource can be considered for HARQ feedback of NR MBS.

With NACK only feedback mode (option 2), UE feedbacks NACK if it receives the corresponding PDSCH of MBS unsuccessfully, and it feedbacks nothing otherwise which saves resources of HARQ feedback. Therefore, gNB could configure a PUCCH resource which only carries NACK sequence to support this option. In addition, when gNB receives at least one NACK feedback at the NACK resource, e.g., by energy detection, gNB will perform a multicast based retransmission, since gNB cannot know which UE feedbacks the NACK.

From the mechanism described above, HARQ feedback for NR MBS with option 2 can save HARQ feedback overhead compared to option 1 by configuring a common NACK resource to multiple UEs. However, gNB cannot differentiate the UE reporting NACK from these multiple UEs. Moreover, it may be not easy to reuse unicast HARQ framework directly either, and the multiplexing among unicast and multicast HARQ feedbacks needs to be also considered.

Observation 2: NACK only feedback mode can save feedback overhead by letting multiple UEs share a common NACK resource, but may be not able to reuse unicast HARQ framework directly.

To summarize, both ACK/NACK feedback mode and NACK only feedback mode have pros and cons compared with each other.  Option 1 is more compatible with the unicast mechanism and could be adopted for NR MBS as the baseline solution. Option 2 is more appropriate for use cases with a large number of UEs receiving MBS, and is also applicable to RRC IDLE/INACTIVE UEs. Option 2 can be a complementary solution besides option 1.

Proposal 1: ACK/NACK feedback mode should be adopted to NR MBS as baseline.

Proposal 2: NACK only feedback mode could apply to use cases for a large number of UEs and even IDLE/ INACTIVE states.

## Disabling and enabling for HARQ feedback

In NR MBS which is dynamic changing from the perspectives of the number of UEs and channel conditions, there is a complicated trade-off between the HARQ feedback overhead and reliability.

From perspective of gNB, for use cases that the number of UEs receiving MBS is too large, gNB needs to disable HARQ feedback proactively for some UEs to reduce the HARQ feedback overhead. And when the amount of the UEs receiving the MBS becomes smaller, gNB needs to enable or re-enable HARQ feedback for some UEs to maintain a good reliability. Consider the trade-off between the HARQ feedback overhead and reliability, both enabling and disabling for HARQ feedback in NR MBS are important.

Proposal 3: Disabling and enabling HARQ feedback should be introduced to NR MBS which can help with a good trade-off between HARQ feedback overhead and system reliability for NR MBS.

One straightforward way to indicate disabling and enabling of HARQ feedback is via high layers e.g. RRC signaling. However, a long interval between two RRC signalings could result in low flexibility to change HARQ feedback states. From perspective of flexibility, DCI indicating is more preferable with enabling/disabling HARQ-ACK feedback as an optional function as the FFS captured in the chair notes [2].

HARQ-ACK feedback should be enabled by default because it is the basic mechanism to improve reliability comparing to SC-PTM supported in LTE. If the function of enabling/disabling HARQ-ACK feedback is configured, then DCI can further indicate how to enable/disable the HARQ-ACK feedback.

Proposal 4: Disabling and enabling HARQ feedback is indicated by DCI.

Proposal 5: The function of enabling/disabling HARQ-ACK feedback is optional.

# Priority and multiplexing rules

NR multicast can be applied to diverse use cases, including V2X, public safety, living video, etc. These use cases have different requirements about latency and reliability as shown in Table 1.

Table 1. Requirements for different MBS use cases

| MBS use cases | Latency | Reliability |
| --- | --- | --- |
| V2X | 5-100ms | 90% to 99.9999% |
| Living Video | 150ms | 99.9% |
| IOT Software update | Latency Tolerant | Higher reliability is beneficial |
| Industry applications | 0.5ms | 99.9999% |

For example, use cases with same requirement for reliability have different requirement for latency, such as V2X and industry applications and vice versa. To meet these latency and reliability requirements, it is not enough to introduce only HARQ feedback when MBS feedback conflicts with other feedbacks or uplink transmissions. It is necessary to introduce some prioritizations and multiplexing rules among different MBS priorities and among MBS and unicast priorities.

Intra-UE prioritization/multiplexing rules and the related priority indication and configuration schemes have been already introduced in NR Rel-16 URLLC, and multiplexing rules with different priorities will be discussed in NR Rel-17 URLLC.

Therefore, priority indication and/or configuration schemes should be introduced for NR MBS. Moreover, the prioritization and multiplexing rules defined in URLLC can be the starting point for NR MBS.

Proposal 6: Priority indication and/or configuration schemes should be introduced for NR MBS. Moreover, the prioritization and multiplexing rules defined in URLLC can be the starting point for NR MBS.

# PDSCH repetition and CSI feedback

In NR unicast, PDSCH repetition mechanism configured semi-statically or indicated dynamically by corresponding DCI is supported to improve the reliability. The number of repetitions that can be configurable seems sufficient for MBS, and it is up to gNB implementation to adjust the number of repetition if needed without the need of further standardization work.

In NR unicast, CSI report used for link adaption with flexible configuration of CSI measurement and CSI reporting does improve the reliability and spectrum efficiency. For NR MBS, the framework of CSI measurement and reporting can be reused for MBS, and it’s not necessary to identify whether the CSI report is used for unicast transmission or MBS. Some modifications e.g. how to do link adaption according to CSI reports from multiple UEs, can be up to gNB implementation without any further standardization work.

Proposal 7: The PDSCH repetition mechanism and CSI feedback mechanism used in NR unicast can be applied to NR MBS without any enhancements.

# Conclusions

In this contribution, the following proposals are made:

Observation 1: The ACK/NACK feedback mode for NR MBS can reuse the procedure and configuration of HARQ feedback for NR unicast, but may lead to large feedback overhead.

Observation 2: NACK only feedback mode can save feedback overhead by letting multiple UEs share a common NACK resource, but may be not able to reuse unicast HARQ framework directly.

Proposal 1: ACK/NACK feedback mode should be adopted to NR MBS as baseline.

Proposal 2: NACK only feedback mode could apply to use cases for a large number of UEs and even IDLE/ INACTIVE states.

Proposal 3: Disabling and enabling HARQ feedback should be introduced to NR MBS which can help with a good trade-off between HARQ feedback overhead and system reliability for NR MBS.

Proposal 4: Disabling and enabling HARQ feedback is indicated by DCI.

Proposal 5: The function of enabling/disabling HARQ-ACK feedback is optional.

Proposal 6: Priority indication and/or configuration schemes should be introduced for NR MBS. Moreover, the prioritization and multiplexing rules defined in URLLC can be the starting point for NR MBS.

Proposal 7: The PDSCH repetition mechanism and CSI feedback mechanism used in NR unicast can be applied to NR MBS without any enhancements.

# References

RP-201038, “WID revision: NR Multicast and Broadcast Services”, Huawei, HiSilicon, E-meeting, June 29 - July 3, 2020.

RAN1 Chairman’s Notes, RAN1#102-e, e-Meeting, 17th August – 28th August, 2020.

RP-201310, “Revised WID Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR”, Nokia, Nokia Shanghai Bell, E-meeting, June 29 – July 3, 2020.

R1-2007562, “Resource configuration and group scheduling for RRC_CONNECTED UEs”, Huawei, HiSilicon, E-meeting, October 26-November 13, 2020.