3GPP TSG RAN WG1 Meeting #103-e	R1-2007594

E-meeting, October 26th – November 13th, 2020

Agenda Item:	8.10.1

Source:	Huawei, HiSilicon

Title:	Resource multiplexing between backhaul and access for IAB duplexing enhancements

Document for:	Discussion and Decision

# Introduction

In RAN1#102-e [1], the following agreements were achieved:

Agreement Reuse by IAB-MT of existing Inter-frequency DC is considered as a starting point to support concurrent BH links to two parents. - FFS: Reuse of multi-TRP transmission resource allocation features (if intra-freq DC scenario is supported for IAB) - FFS: Additional specification effort to support IAB Agreement At least existing Rel-16 bands supporting IAB can be considered when evaluating the feasibility/impact of supporting different multiplexing cases. Agreement The Rel-16 semi-static and dynamic resource allocation mechanisms are the starting point for supporting Rel-17 multiplexing cases. - FFS: Applicability for different IAB-DU resource types - FFS: Cell-specific/semi-static signals and channels at the IAB-DU and/or IAB-MT Agreement - Based on the WID, the following multiplexing cases are in scope for potential support in Rel-17: - Multiplexing Case A: Simultaneous MT-Tx/DU-Tx - Multiplexing Case B: Simultaneous MT-Rx/DU-Rx - Multiplexing Case C: Simultaneous MT-Rx/DU-Tx - Multiplexing Case D: Simultaneous MT-Tx/DU-Rx - Further study for for Case A and Case B at least the following scenarios: - Single or multi-panel IAB nodes operating in unpaired spectrum (FR1 and FR2 bands) - Further study for Case C and Case D at least for the following scenarios: - Multi-panel IAB nodes operating in unpaired spectrum (FR1 and FR2 bands) - FFS: Required level of specification impact to support the different cases. Any additional specification support in Rel-17 should be conditioned on feasibility from an interference and reliability perspective on a per-link and network basis

This paper focuses on the enhancements to resource multiplexing between child and parent links of an IAB node. Specifically, the potential specification impact to support different multiplexing cases are discussed.

# Discussion

## Unrestricted and restricted capability of simultaneous operation

In RAN1#102e, the following issues were recommended for further discussion.

For companies to further consider: Whether the following characteristics of the IAB node implementation will impact the operation of different resource multiplexing cases, including resource partitioning (i.e. identify whether there is a need for potential specification impact/enhancements compared to Rel-16 if the characteristic is or is not supported by an IAB node): - Baseband (mis)timing alignment between IAB-MT and IAB-DU - Separate vs. shared antenna panels/RF front-end for IAB-MT/IAB-DU - Separate vs. shared baseband for IAB-MT/IAB-DU - Transmitter/receiver implementation - Self-interference cancellation - Power control mechanisms

It is clear that all of the above characteristics will impact the operation of resource multiplexing. However, depending on the potential standardization impact, they can be divided into two groups:

- Implementation related IAB node characteristics which do not require specification changes

- Capability related IAB node characteristics which would require specification changes

First, we discuss the characteristics related to specific implementations of an IAB node:

Separate vs. shared antenna panels/RF front-end for IAB-MT/IAB-DU

As one possible implementation, an IAB node can have separate antenna panels with opposite orientations or geographically separated. In this case, the interference between MT and DU can be quite small and the IAB node may be able to support resource multiplexing between the parent and child links without any restriction. However, if the spatial isolation between the MT and DU is not sufficient, the IAB node may still need some timing alignment or power control mechanisms to suppress the interference level between the MT and DU so that resource multiplexing can be supported.

Separate vs. shared baseband for IAB-MT/IAB-DU

There are two different baseband architectures for IAB node implementation: separate vs. shared baseband. An IAB node with shared baseband for MT and DU is more beneficial for the operation of resource multiplexing. With shared baseband, an IAB node can perform joint precoding and reception, which can potentially enhance the performance of simultaneous operation. However, separate baseband typically have lower cost and implementation complexity, and thus may also be adopted. In any case, there is no need to specify the implementation choices for the support of resource multiplexing.

Transmitter/receiver implementation

Transmitter/receiver implementation refers to the transceiver baseband or RF implementations that are adopted by an IAB node. Similar to the previous issue, the algorithms adopted at the transceiver can be left to implementations, e.g., some implementations may be able to support multiplexing Case B with larger power gap due to the adoption of more advanced receivers.

Self-interference cancellation

For full duplex, IAB node may suffer from high self-interference if the MT and DU are co-located. In this case, self-interference cancellation is required. But if the MT and DU are deployed in different locations, self-interference cancellation may not be necessary. If an IAB node reports full duplex, the parent node does not need to be aware of if the capability is achieved by self-interference cancellation or geographic isolation.

In summary, IAB nodes can have various architectures and implementation choices, and their capabilities to support the resource multiplexing can also be different. However, a specific implementation does not always require the definition of a certain IAB node capability. For an IAB node, the multiplexing capability for a given {MT CC, DU cell}-pair has been defined in Rel-16. The architecture and/or implementation related characteristics of an IAB node can be readily covered by the existing signalling framework. On the other hand, even if the donor node or parent node knows the implementation choice of IAB node, it may not be able to figure out whether resource multiplexing is supported or not.

Observation 1: There is no need to define implementation related IAB node characteristics in specification since there is no direct relation between implementation and the resource multiplexing capability.

In the following, we discuss some capability related IAB node characteristics which require specification changes to support resource multiplexing. First of all, it should be noted that the Rel-16 IAB node capability with respect to resource multiplexing are not sufficient since there is no any restrictions for the operation of resource multiplexing which can only be met under certain conditions.

Therefore, we have the following observation:

Observation 2: There are two types of capabilities with respect to simultaneous operations:

- Unrestricted capabilities of simultaneous operations: IAB node can support simultaneous operations without restriction

- Restricted capabilities of simultaneous operations: IAB node can support simultaneous operations only if some conditions are met, e.g. timing alignment between IAB-MT and IAB-DU is adopted or a specific power control scheme is applied

When an IAB node only has restricted capabilities of simultaneous operation, the conditions for the support of the simultaneous operations should be known among adjacent nodes.

Baseband timing (mis)alignment between IAB-MT and IAB-DU

To support multiplexing Case A, an IAB node may need transmission timing alignment for joint processing. In this case, the parent node should schedule the IAB-MT using Case 6 timing; therefore, the parent node needs to be informed that if Case 6 timing is required or not by the IAB node. To implement multiplexing Case B, reception timing alignment at IAB node can be beneficial for interference rejection. In this case, the IAB-DU should schedule the child IAB-MT or UE with Case 7 timing given Case 7 timing supported by the child IAB-MT or UE.

Power control mechanisms

To implement a specific multiplexing case, IAB-MT or IAB-DU may have requirements on power control mechanisms. Similar to the case of timing alignment, if the requirement is for IAB-MT, the parent node should be informed in order to set proper power control parameters since the UL Tx power of IAB-MT is controlled by the parent node, but if the requirement is for IAB-DU, it can be agnostic to the parent node.  Of course, the enhanced power control scheme needs to be supported by the child node.

Therefore, we have the following observation and proposal:

Observation 3: To support a specific resource multiplexing case, the requirements for the transmission/reception of IAB-DU can be met by IAB node itself while the requirements for the transmission/reception of IAB-MT should be coordinated between parent node and IAB node.

Proposal 1: To facilitate simultaneous operations, new IAB node capabilities to indicate the support of restricted simultaneous operations should be introduced

- The conditions required in the restricted simultaneous operation include at least the requirement for the transmission/reception of IAB-MT.

## Resource types for simultaneous operation

In this section, we discuss which of the IAB-DU resource types can support simultaneous operation.

In Rel-16, an IAB-DU can use its soft resources only if

-	the IAB-MT does not transmit or receive in the symbol, or

-	the IAB-MT would transmit or receive in the symbol, and the transmission or reception in the symbol is not changed due to a use of the symbol by the IAB-DU, or

-	the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft symbol as available

It can be concluded from the second bullet that simultaneous operation are already allowed in soft resources of IAB-DU based on the current specification. To be more specific, when the parent node does not indicate a soft resource as available, the IAB-DU can use the resource if the transmission or reception of IAB-MT is not impacted by the operation of IAB-DU.

Obviously, simultaneous operation cannot be implemented in not-available resources of IAB-DU according to the definition; specifically, there is simply no transmission or reception in IAB-DU.

Observation 4: Based on the Rel-16 framework, simultaneous operation cannot be implemented in not-available resources of IAB-DU, but can be implemented at least in soft resource of IAB-DU.

Therefore, the remaining question is that whether the simultaneous operation can be implemented in hard resource of IAB-DU. According to the definition, the IAB-DU can always transmit or receive signals in hard resources; therefore, if the parent node would schedule the IAB-MT in the corresponding resource, the transmission or reception should not impact the operation of IAB-DU. This does not seem to require any change to the definition of the hard resources.

Proposal 2: Simultaneous operations can be implemented in hard resources of IAB-DU only if the transmission or reception of IAB-MT does not impact the operation of IAB-DU.

## Scenarios for simultaneous operation

In RAN1#102-e, the following issues for the scenarios of simultaneous operation were left for further study.

For companies to further consider: Different resource partitioning scenarios for access and backhaul links, including their respective implication on interference, for different resource multiplexing cases. Examples include: - Whether a given case is only applicable for certain resource types: e.g. DL only, UL only, DL + UL - Whether a given case is only applicable for backhaul links or both access and backhaul links - Note: This should have no impact on legacy UE behavior

The scenarios for multiplexing Cases C and D are similar to TDM operation since the parent link and child link have same link directions; therefore, we mainly discuss multiplexing Cases A and B in this section. In general, there are two typical scenarios for multiplexing Case A and B.

- Scenario 1: IAB nodes implement multiplexing Cases A and B at donor cell UL occasion, which is shown in Figure 1. (a).

- Scenario 2: IAB nodes implement multiplexing Cases A and B at donor cell DL occasion, which is shown in Figure 1. (b).

In scenario 1, the IAB node at 1st hop implements simultaneous transmission and the IAB node at 2nd hop implement simultaneous reception. In scenario 2, the IAB node at 1st hop implements simultaneous reception and the IAB node at 2nd hop implement simultaneous transmission. In Figure 1, we assume that simultaneous operations are applicable for both access links and backhaul links.

- Multiplexing Cases A and B at donor cell UL occasion

- Multiplexing Cases A and B at donor cell DL occasion

Figure 1: Two typical scenarios for multiplexing cases

As discussed in [2], when an IAB node implements multiplexing Case A and B between access link and backhaul link, and the access link and donor cell are in opposite link directions, there may be severe cross link interferences between UEs. Overall, the interference situation is same to the case when the slot formats are dynamically changed in different neighboring cells. Although interference measurement between UEs was studied and standardized in Rel-16, this may not be a typical deployment in particular for macro cells due to the considerations of performance and complexity. Therefore, we have the following proposal.

Proposal 3: For multiplexing Case A and Case B, the following scenarios should be prioritized in Rel-17:

- Multiplexing between parent backhaul link and child backhaul

- Multiplexing between backhaul link and access link, and the direction of access link is same with the direction of donor cell

In other words, when the directions of access link and donor cell are opposite, the multiplexing between backhaul link and access link should be deprioritized. It should be noted, the de-prioritization means RAN1 should not spend much time to study this case, but the IAB node would still be able to implement the multiplexing by using existing CLI measurement and management mechanisms. When this multiplexing case is precluded, scenarios 1 and 2 are illustrated by Figure 2.

- Multiplexing Cases A and B at donor cell UL occasion

- Multiplexing Cases A and B at donor cell DL occasion

Figure 2: Two typical scenarios for multiplexing cases with priority

In scenario 1, MT of IAB node#1 is performing traditional downlink reception, and DU of IAB node#2 is performing traditional downlink transmission. Meanwhile, the DU of IAB node#1 is receiving the uplink from MT of IAB node#2. For UEs, the uplink transmission leads to similar downlink inter-cell interference.

In scenario 2, MT of IAB node#1 is performing traditional uplink transmission, and DU of IAB node#2 is performing traditional uplink reception. Meanwhile, the MT of IAB node#2 is receiving the downlink from DU of IAB node#2. For other network nodes, the downlink transmission leads to similar uplink inter-cell interference.

In conclusion, both scenarios 1 and 2 have smaller impact on the legacy UEs and network, and thus can be supported. Therefore, we have the following proposal:

Proposal 4: Both DL and UL can be used to implement different resource multiplexing cases.

## TDD configurations enhancement for IAB-MT and IAB-DU

A prerequisite to enable simultaneous operation of IAB-node’s child and parent links is that the resource configurations of MT and DU should be in line with the TX/RX combinations on the resources where simultaneous operations occurs

- In case of full duplexing, i.e. simultaneous operation w.r.t. MT-TX/DU-RX and MT-RX/DU-TX, the IAB-MT and IAB-DU should have same link directions.

- In case of SDM, i.e. simultaneous operation w.r.t. MT-TX/DU-TX and MT-RX/DU-RX, the IAB-MT and IAB-DU should have opposite link directions.

It was agreed that the Rel-16 resource allocation schemes will be reused as a baseline to enable the simultaneous operation. In the following, some potential enhancements are discussed.

### TDD configuration for MT

In Rel-16, the slot formats with the sequence order UL-Flexible-DL for IAB node MT were introduced, and the following agreement on the dedicated RRC signalling (referred as TDD-UL-DL-ConfigDedicated-IAB-MT) was achieved in RAN1#99:

Agreements: If the IAB-MT is additionally provided TDD-UL-DL-ConfigDedicated-IAB-MT, the parameter TDD-UL-DL-ConfigDedicated-IAB-MT overrides all symbols (with a limitation that effectively only flexible symbols can be overwritten in Rel-16) per slot over the number of slots as provided by TDD-UL-DL-ConfigurationCommon.

The legacy dedicated TDD configuration can only override the flexible symbols per slot over the number of slots as provided by tdd-UL-DL-ConfigurationCommon. According to the agreement, an enhanced overriding rule for IAB-MT is introduced, i.e., tdd-UL-DL-ConfigDedicated-IAB-MT can override all symbols per slot over the number of slots as provided by tdd-UL-DL-ConfigurationCommon.

However, Rel-16 IAB focuses on TDM operation, and therefore the enhanced overriding rule was not captured in specification, i.e., only flexible symbols can be overridden by tdd-UL-DL-ConfigDedicated-IAB-MT in Rel-16. To increase the opportunity of simultaneous operation, the limitation of the overriding rule in Rel-16 should be removed in Rel-17.

Proposal 5: If an IAB-MT is additionally provided TDD-UL-DL-ConfigDedicated-IAB-MT, the parameter tdd-UL-DL-ConfigurationDedicated-IAB-MT overrides all symbols per slot over the number of slots as provided by tdd-UL-DL-ConfigurationCommon.

Figure 3. tdd-UL-DL-ConfigurationDedicated-IAB-MT without Rel-16 limitation

The UE behaviors related to tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are captured in TS 38.213. In Rel-16, when tdd-UL-DL-ConfigDedicated-IAB-MT is provided, IAB-MT follows the UE behaviors by replacing tdd-UL-DL-ConfigurationDedicated with tdd-UL-DL-ConfigDedicated-IAB-MT. However, if tdd-UL-DL-ConfigurationDedicated-IAB-MT can override all symbols in Rel-17, the behavior of IAB-MT with tdd-UL-DL-ConfigurationDedicated-IAB-MT should be revisited.

For example, the following UE behaviors about TDD configurations and cell-specific signals/channels are specified in TS 38.213:

For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot that are indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, for reception of SS/PBCH blocks, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot. The UE does not expect the set of symbols of the slot to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, when provided to the UE. For a set of symbols of a slot corresponding to a valid PRACH occasion and Ngap symbols before the valid PRACH occasion, as described in Sublcause 8.1, the UE does not receive PDCCH, PDSCH, or CSI-RS in the slot if a reception would overlap with any symbol from the set of symbols. The UE does not expect the set of symbols of the slot to be indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated. For a set of symbols of a slot indicated to a UE by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, the UE does not expect the set of symbols to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated.

It can be observed from the above that the TDD configurations (tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated) should be compatible with the cell-specific channels and signals.

However, retaining the above constraint for tdd-UL-DL-ConfigurationDedicated-IAB-MT can dramatically reduce the opportunities for simultaneous operations. Specifically, most of the downlink slots indicated by tdd-UL-DL-ConfigurationCommon contain SS/PBCH blocks in typical TDD UL/DL configurations. This is due to the fact that the slot configuration period is usually smaller than the period of SS/PBCH blocks, which means that these slots cannot be indicated as uplink by tdd-UL-DL-ConfigurationDedicated-IAB-MT, i.e. SDM Rx/Tx cannot be implemented in these slots. As an example in Figure 2, assuming the TDD configuration periodicity is 2.5 ms, it can be observed that all the first 4 slots within one TDD configuration period cannot be indicated as uplink according to current specification since the SS/PBCH blocks may be transmitted every 20 ms.

Figure 4. tdd-UL-DL-ConfigurationDedicated-IAB-MT colliding with SS/PBCH blocks

To solve this problem, the specification should allow the collision between cell-specific signals/channels and tdd-UL-DL-ConfigurationDedicated-IAB-MT. In the slots with the collision, the IAB node should ignore tdd-UL-DL-ConfigurationDedicated-IAB-MT and it should be able to transmit/receive the cell-specific signals/channels. And in the TDD configuration periods or slots without collision, the IAB node can perform simultaneous operation according to the TDD configuration. As is shown in Figure 4, the tdd-UL-DL-ConfigurationDedicated-IAB-MT is ignored in the first TDD period because of the collision, but it can be valid in other TDD periods, which increase the resources for simultaneous operation significantly.

In conclusion, we have the following proposal:

Proposal 6: To increase the resources for simultaneous operation, the specification should allow the collision between tdd-UL-DL-ConfigurationDedicated-IAB-MT and cell-specific signals/channels. In the slots with the collision, the IAB node should ignore tdd-UL-DL-ConfigurationDedicated-IAB-MT. The list of cell-specific signals/channels includes:

- SS/PBCH block

- CORESET for Type0-PDCCH CSS set

- PRACH

### TDD configuration for DU

In Rel-16, slot formats with the sequence order UL-Flexible-DL are also supported by IAB node DU and donor DU. For a specific slot, if an UL-Flexible-DL slot format is provided to DU, the DU can only use the configuration to schedule child IAB-MT because the legacy UE cannot support the slot format as shown in Figure 5. However, the slot cannot be occupied by child IAB-MT in every TDD configuration period, i.e., in some TDD configuration periods, DU may still need to schedule UE in the slot. Therefore, a legacy DL-Flexible-UL configuration should also be provided to the DU for the slot.

Figure 5: DU TDD configuration with UL-Flexible-DL slots

Therefore, the IAB node DU or donor DU should have two separate DU resource configurations for the different purposes. Specifically, a legacy DU resource configuration, which is compatible with Rel-15 slot formats of DL-flexible-UL order, can be used for both the access UEs and child node MTs. Besides, an additional DU resource configuration with the new slot formats with UL-flexible-DL order can only be used for child node MTs. When the DU communicates with the access UEs, it should use the basic DU resource configuration. When the DU communicates with the child node MTs, it can use both of the basic DU resource configuration and the additional DU resource configuration, as illustrated in Figure 6.

Figure 6: Example of separate DU TDD-UL-DL configurations

Therefore, we have the following proposal

Proposal 7: Donor CU can provide two sets of resource configurations to each DU cell: a basic resource configuration for access UEs and one additional resource configuration for child IAB node MTs, and the DU cell should maintain two sets of DU resource configurations simultaneously.

# Conclusions

Based on the discussion, we have following observations and proposals:

Observation 1: There is no need to define implementation related IAB node characteristics in specification since there is no direct relation between implementation and the resource multiplexing capability.

Observation 2: There are two types of capabilities with respect to simultaneous operations:

- Unrestricted capabilities of simultaneous operations: IAB node can support simultaneous operations without restriction

- Restricted capabilities of simultaneous operations: IAB node can support simultaneous operations only if some conditions are met, e.g. timing alignment between IAB-MT and IAB-DU is adopted or a specific power control scheme is applied

Observation 3: To support a specific resource multiplexing case, the requirements for the transmission/reception of IAB-DU can be met by IAB node itself while the requirements for the transmission/reception of IAB-MT should be coordinated between parent node and IAB node.

Observation 4: Based on the Rel-16 framework, simultaneous operation cannot be implemented in not-available resources of IAB-DU, but can be implemented at least in soft resource of IAB-DU.

Proposal 1: To facilitate simultaneous operations, new IAB node capabilities to indicate the support of restricted simultaneous operations should be introduced

- The conditions required in the restricted simultaneous operation include at least the requirement for the transmission/reception of IAB-MT.

Proposal 2: Simultaneous operations can be implemented in hard resources of IAB-DU only if the transmission or reception of IAB-MT does not impact the operation of IAB-DU.

Proposal 3: For multiplexing Case A and Case B, the following scenarios should be prioritized in Rel-17:

- Multiplexing between parent backhaul link and child backhaul

- Multiplexing between backhaul link and access link, and the direction of access link is same with the direction of donor cell

Proposal 4: Both DL and UL can be used to implement different resource multiplexing cases.

Proposal 5: If an IAB-MT is additionally provided TDD-UL-DL-ConfigDedicated-IAB-MT, the parameter tdd-UL-DL-ConfigurationDedicated-IAB-MT overrides all symbols per slot over the number of slots as provided by tdd-UL-DL-ConfigurationCommon.

Proposal 6: To increase the resources for simultaneous operation, the specification should allow the collision between tdd-UL-DL-ConfigurationDedicated-IAB-MT and cell-specific signals/channels. In the slots with the collision, the IAB node should ignore tdd-UL-DL-ConfigurationDedicated-IAB-MT. The list of cell-specific signals/channels includes:

- SS/PBCH block

- CORESET for Type0-PDCCH CSS set

- PRACH

Proposal 7: Donor CU can provide two sets of resource configurations to each DU cell: a basic resource configuration for access UEs and one additional resource configuration for child IAB node MTs, and the DU cell should maintain two sets of DU resource configurations simultaneously.

# References

RAN1 Chairman note 102-e.

R1-2005261, Enhancements for simultaneous operation of MT and DU, Huawei, HiSilicon, Aug. 2020.