3GPP TSG RAN WG1 Meeting #103-e 	R1-2007612

E-meeting, October 26 – November 13, 2020

Agenda Item:	7.2.4

Source:	Huawei, HiSilicon

Title:	Remaining details of sidelink resource allocation mode 2

Document for:	Discussion and Decision

- Introduction

In this paper, we discuss the remaining details of sidelink resource allocation for mode 2, including the determination of time resource and frequency resource, the details in the sensing and resource (re-)selection procedure.

- Mode 2 resource allocation

  - Backward indication

In RAN1#100bis-e, the following proposals for backward indication are achieved in FL summary R1-2003038:

Proposal 1a (1bit)

- When periodic reservations are enabled in a resource pool, and when a (pre-)configuration indicates that “resource index” signaling is enabled in a resource pool, a separate field of 1 bit in the first stage SCI indicates a “resource index” for the purpose of backward indication

  - If resource index = 0, then

    - For Nmax = 2 or 3: t0’ = 0, t1’ = t1

    - For Nmax = 3: t2’ = t2

  - If resource index = 1, then

    - For Nmax = 2 or 3: t0’ = -t1 , t1’ = 0

    - For Nmax = 3: t2’ = t2 - t1

  - where t0, t1, t2 are logical slot offsets with respect to the slot where SCI 0-1 received, obtained from “Time resource assignment”, and t0’, t1’, t2’ are actual logical slot offsets with respect to the slot where SCI 0-1 received

Proposal 1b (full)

- When periodic reservations are enabled in a resource pool, and when a (pre-)configuration indicates that “resource index” signaling is enabled in a resource pool, a separate field of ceil(log2(Nmax)) bit in the first stage SCI indicates a “resource index” for the purpose of backward indication

  - If resource index = 0, then

    - For Nmax = 2 or 3: t0’ = 0, t1’ = t1

    - For Nmax = 3: t2’ = t2

  - If resource index = 1, then

    - For Nmax = 2 or 3: t0’ = -t1 , t1’ = 0

    - For Nmax = 3: t2’ = t2 - t1

  - If resource index = 2, then

    - t0’ = -t2, t1’ = t1 - t2, t2’ = 0

  - where t0, t1, t2 are logical slot offsets with respect to the slot where SCI 0-1 received, obtained from “Time resource assignment”, and t0’, t1’, t2’ are actual logical slot offsets with respect to the slot where SCI 0-1 received

In LTE-V2X, section 14.1.1.4.C of TS 36.213 specifies the set of subframes and resource blocks for PSSCH transmission which are determined by the "Frequency resource location of the initial transmission and retransmission" field, "Retransmission index" field and "Time gap between initial transmission and retransmission" field in the associated SCI format 1. To avoid missing reservation due to undetected SCI, backward indication is supported in LTE-V2X.

Although the maximum number of (re)-transmissions for one TB in NR-V2X can be up to 32, the backward indication is also needed when a group of consecutive SCIs are missed due to the fluctuation of the channel condition. As shown in Figure 1, assume only forward indication is supported, and a group of consecutive SCIs which are sent on resources R6, R7 and R8 are missed, then the re-transmission on R8 cannot be detected by other sensing UE. Thus, the backward indication is necessary even though more (re)-transmissions for one TB are supported in NR-V2X.

Figure 1: E.g., assume only forward indication is supported, and a group of consecutive SCIs (R6/R7/R8) are missed

Observation 1: The backward indication is needed when a group of consecutive SCIs are missed due to the fluctuation of the channel condition.

In LTE-V2X, since a given TB can be transmitted only once or twice, so it is enough to use only 1 bit for such forward/backward indication. However, for NR-V2X, a SCI can indicate up to 3 resources for a given TB when Nmax=3. So to enable full flexibility for backward indication, a field of ceil(log2(Nmax)) bit should be used. Thus, Proposal 1b in the FL summary R1-2002038 of RAN1#100bis-e is preferred.

Proposal 1: Support full backward indication, i.e., support Proposal 1b (full) in the FL summary R1-2003038.

A proposed text for this correction is provided as follows for Section 8.1.5 of TS 38.214.

-------------------------- Start of Text Proposal for TS 38.214 -------------------------

<Unchanged parts omitted>

8.1.5    UE procedure for determining slots and resource blocks for PSSCH transmission associated with an SCI format 0-1

<Unchanged parts omitted>

If sl-MaxNumPerReserve is 2 then

FRIV=nsubCH,1start+i=1LsubCH-1N subchannel SL+1-i

If sl-MaxNumPerReserve is 3 then

FRIV=nsubCH,1start+nsubCH,2start⋅N subchannel SL+1-LsubCH+i=1LsubCH-1N subchannel SL+1-i2

where

-	nsubCH,1start denotes the starting sub-channel index for the second resource

-	nsubCH,2start denotes the starting sub-channel index for the third resource

-	N subchannel SL is the number of sub-channels in a resource pool provided according to the higher layer parameter numSubchannel

For the SCI format 1-A transmitted on the PSCCH resource m  (described in Subclause 16.4 TS 38.213) in slot tnSL, the set of slots and sub-channels for the corresponding PSSCH are determined as follows:

- if  "Time resource assignment" field in  [8, TS 38.212] is zero,

  - the time and frequency resources for the corresponding PSSCH is given by

    - sub-channel(s) m, m+1,…, m+LsubCH-1, in slot  tnSL

- elseif  "Time resource assignment" field in  [8, TS 38.212] is smaller than 32,

  - if "Retransmission index" in the SCI format 1-A is zero,

    - the time and frequency resources for the corresponding PSSCH is given by

        - sub-channel(s)  m, m+1,…, m+LsubCH-1, in slot tnSL, and

        - sub-channel(s) nsubCHstart1, nsubCHstart1+1,…, nsubCHstart1+LsubCH -1 , in slot tn+T1SL

  - else (i.e. that  "Retransmission index" in the SCI format 1-A is one),

    - the time and frequency resources for the corresponding PSSCH is given by

        - sub-channel(s) nsubCHstart1, nsubCHstart1+1,…, nsubCHstart1+LsubCH -1 , in slot tn- T1SL , and

        - sub-channel(s)  m, m+1,…, m+LsubCH-1, in slot tnSL

- else

  - if "Retransmission index" in the SCI format 1-A is zero,

    - the time and frequency resources for the corresponding PSSCH is given by

        - sub-channel(s)  m, m+1,…, m+LsubCH-1, in slot tnSL, and

        - sub-channel(s) nsubCHstart1, nsubCHstart1+1,…, nsubCHstart1+LsubCH -1 , in slot tn+ T1SL, and

        - sub-channel(s) nsubCHstart2,nsubCHstart2+1,…, nsubCHstart2+LsubCH -1 , in slot  tn+  T2SL

  - elseif "Retransmission index" in the SCI format 1-A is one,

    - the time and frequency resources for the corresponding PSSCH is given by

        - sub-channel(s) nsubCHstart1, nsubCHstart1+1,…, nsubCHstart1+LsubCH -1 , in slot tn- T1SL, and

        - sub-channel(s)  m, m+1,…, m+LsubCH-1, in slot tnSL, and

        - sub-channel(s) nsubCHstart2, nsubCHstart2+1,…, nsubCHstart2+LsubCH -1 , in slot  tn+ T2-T1SL

  - else (i.e. that "Retransmission index" in the SCI format 1-A is two),

    - the time and frequency resources for the corresponding PSSCH is given by

        - sub-channel(s) nsubCHstart1, nsubCHstart1+1,…, nsubCHstart1+LsubCH -1 , in slot  tn- T2SL, and

        - sub-channel(s) nsubCHstart2, nsubCHstart2+1,…, nsubCHstart2+LsubCH -1 , in slot  tn- T1SL, and

sub-channel(s) m, m+1,…, m+LsubCH-1, in slot tnSL

<Unchanged parts omitted>

------------------------------------End of Text Proposal -------------------------------

  - Re-evaluation and pre-emption

During the resource exclusion procedure in step 1, whether a candidate resource Rx,y needs to be excluded depends on the SCI detected in the sensing window. According to the current definition of sensing window in TS 38.214, the UE shall monitor slots which belong to a sidelink resource pool within the sensing window [n –T0,n–Tproc,0) when resource (re)-selection is triggered in slot n. Moreover, besides the re-evaluation or pre-emption performed at the moment ‘m-T3’, it has been agreed that the re-evaluation or pre-emption check before the moment ‘m-T3’ or after ‘m-T3’ are up to UE implementation. When re-evaluation or pre-emption are performed in more than one slot and based on the current definition of sensing window, it is possible that some SCIs which will cause collision are included in previous sensing window but not included in the final sensing window, or equivalently some candidate resource which has already been excluded from the previous resource selection window would not be excluded from the final resource selection window.

As shown in Figure 2, assume the re-evaluation or pre-emption check are performed twice in slot n1  and n2 , separately. For the first resource exclusion triggered in slot n1, assume one SCI on resource S2 within the first sensing window [n1 –T0,n1–Tproc,0) indicates the resource reservation on resource R2, then the reserved resource R2 can be excluded from the candidate resource set SA. Similarly, for the second resource exclusion triggered in slot n2, the resource R3 can be excluded by the SCI received on resource S1. However, if the second resource exclusion only considers the resource reservation sensed from the second sensing window [n2 –T0,n2–Tproc,0), the SCI on resource S2 will not be included in the second sensing window, resulting in the resource R2 which has been previously excluded would be considered as the identified candidate resource again.

To avoid such problem, the sensing results derived from multiple sensing window should be considered jointly, i.e., a candidate resource will be excluded from the identified candidate resource set SA reported to higher layer if, in any one or more resource selection window associated with the re-evaluation or pre-emption check for a pre-selected or reserved resource, it is not regarded as an identified candidate resource.

Moreover, since the SCI with reservation has already been decoded in the previous sensing window, the extra UE complexity for the joint window exclusion is minimal.

Figure 2: the identified candidate resource of re-evaluation

Observation 2: It is possible that the resource which has already been excluded in the previous resource selection window would be considered as the identified candidate resource again when the sensing window is changed.

Proposal 2: A candidate resource will be excluded from the identified candidate resource set SA reported to higher layer if, in any one or more resource selection window associated with the re-evaluation or pre-emption check for a pre-selected or reserved resource, it is not regarded as an identified candidate resource.

A proposed text for this correction is provided as follows for Section 8.1.4 of TS 38.214.

--------------------------- Start of Text Proposal for TS 38.214 ------------------------

<Unchanged parts omitted>

8.1.4   UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged parts omitted>

The following steps are used:

<Unchanged parts omitted>

7)	If the number of candidate single-slot resources remaining in the set SA is smaller than X⋅Mtotal, then  Th(pi,pj) is increased by 3 dB for each priority value Th(pi,pj) and the procedure continues with step 4.

One candidate resource will be excluded from the identified candidate resource set  SA reported to higher layer if, in any one or more resource selection window associated with the re-evaluation or pre-emption check for a pre-selected or reserved resource, it is not regarded as an identified candidate resource.

The UE shall report set SA to higher layers.

<Unchanged parts omitted>

------------------------------------End of Text Proposal -------------------------------

In RAN1#102-e, the following proposal for re-evaluation are discussed [6]:

Proposal 1-1

- If periodic reservation is in use by a UE selecting resources, the UE performs re-evaluation procedure only for resource(s) in the first period after the initial resource re-selection trigger or for resources in non-initial resource after the re-selection triggered by pre-emption

  - Note, this is intended to be captured in MAC specification

  - Note, the initial resource re-selection trigger refers to the initial (re-)selection (i.e. not including the resource re-selection triggered by re-evaluation and pre-emption)

In RAN1#98bis, the re-evaluation is defined as the resource exclusion and (re-)selection procedure before transmission of SCI with reservation. As shown in Figure 3, assume one set of periodic resources R1#1, R1#2, R1#3…. are selected in slot n, and the SCI with “Resource reservation period” field set to P is to be transmitted on resource R1#1 in slot k. Before slot k, it would be inaccurate to predict future resource occupation in slots k+P, k+2*P, …, since the channel condition or collision situation may largely change between slot k and k+P, k+2*P, …. Hence, we propose that before slot k, the UE can perform re-evaluation for resource in occasion k, but cannot perform re-evaluation for resources in occasions k+P, k+2*P,…, k+Cresel*P.

Proposal 3: When SCI in slot ‘k’ signals resource reservation with “Resource reservation period” set to P, re-evaluation for resource(s) in logical slots of a resource pool corresponding to occasions k+P, k+2*P,…, k+Cresel*P is not triggered before the slot ‘k’.

Figure 3: Illustration of re-revaluation for periodic reservation

  - Resource exclusion

2.3.1 Non-monitored slots

During the resource determination procedure for mode 2 in TS 38.214, the set  SA for all the candidate single-slot resources is defined in step 4) in section 8.1.4. Then the sensing UE excludes the resources from  SA in step 5) and step 6). In step 5), the slots on which the UE has not monitored are considered. Since the sensing results are absent for these non-monitored slots, all periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList for all the sub-channels are assumed. In this case, one candidate single-slot resource Rx,y which overlaps with any the reservation by step 5) will be excluded. While in step 6), the monitored slots on which the UE received SCI are considered, and the RSRP measurements are performed on these slots. One candidate single-slot resource Rx,y which overlaps with the reserved resources by the received  SCI will be excluded if the RSRP measurements result is higher than the associated threshold. After performing step 5) and step 6), the UE will check if the number of the remaining candidate single-slot resources in SA is smaller than pre-configured ratio X⋅Mtotal , where the Mtotal denotes the total number of candidate single-slot resources in SA. If the ratio condition cannot be met, then the RSRP threshold is increased by 3 dB and the procedure continues with step 4).

It can be observed from the resource exclusion procedure that the RSRP measurement is only associated with step 6). If the number of the remaining candidate single-slot resources in SA is smaller than X⋅Mtotal, the increased 3 dB on RSRP threshold only impacts the exclusion results in step 6), and some excluded resources can be recovered by the increased RSRP threshold. However, the resource exclusion by step 5) is not related with the RSRP threshold, so the excluded resources by step 5) cannot be recovered. Therefore, when the number of the remaining resources in SA  after performing step 5) is already smaller than X⋅Mtotal, the operation of increasing RSRP threshold cannot help to ensure the final resources set satisfies the requirement.

Observation 3: When the number of the remaining resources in SA after performing step 5) is already smaller than X⋅Mtotal, the operation of increasing RSRP threshold cannot help to ensure the final resources set satisfies the requirement since step 5) is not related with the RSRP threshold.

To guarantee the remaining number of candidate single-slot resources in SA is not smaller than X⋅Mtotal, it is proposed to add one additional condition after performing step 5), i.e., the UE need to determine the condition that whether the number of the excluded resources in step 5) is larger than (1-X )⋅Mtotal. If the above condition is met, then randomly select some excluded resources in step 5) and add them back in the set  SA  until the number of the candidate single-slot resources remaining in the set  SA is not smaller than  X⋅Mtotal.

Proposal 4: If number of the excluded resources in step 5) is larger than (1-X)⋅Mtotal , then randomly select some excluded resources in step 5) and add them back in the set SA until the number of the candidate single-slot resources remaining in the set  SA is not smaller than  X⋅Mtotal.

A proposed text for this correction is provided as follows for Section 8.1.4 of TS 38.214.

--------------------------- Start of Text Proposal for TS 38.214 ------------------------

<Unchanged parts omitted>

8.1.4    UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

In resource allocation mode 2, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission. To trigger this procedure, in slot n, the higher layer provides the following parameters for this PSSCH/PSCCH transmission:

<Unchanged parts omitted>

The following steps are used:

<Unchanged parts omitted>

4)	The set SA is initialized to the set of all the candidate single-slot resources.

5-1)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

-	the UE has not monitored slot tmSL in Step 2.

-	for any periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList-r16 and a hypothetical SCI format 1-A received in slot tmSL with "Resource reservation period" field set to that periodicity value and indicating all subchannels of the resource pool in this slot, condition c in step 6 would be met.

5-2)	If number of the excluded resources by step 5-1) is larger than (1- X)⋅ Mtotal , randomly select some excluded resources in step 5-1) and add them back in the set SA until the number of the candidate single-slot resources remaining in the set SA is not smaller than  X⋅Mtotal

<Unchanged parts omitted>

------------------------------------End of Text Proposal -------------------------------

2.3.2 Resource exclusion for different TBs

In RAN1#102-e meeting, the resource exclusion rule that a UE cannot select more than one resource in the same slot for the same TB has been clarified by the following conclusion:

Conclusion

- RAN1 understands that the agreement in RAN1#100-bis-e / 101-e cited below already prevents selection of the resources in the same slot and no further clarification is necessary

Agreements:

- In Step 2, a UE shall select resources so that HARQ retransmission resources can be reserved by a prior SCI, except that

  - In case no resource can be found for reservation (e.g., based on the identified candidate set after Step 1) for a retransmission of a TB, the re-transmission can be transmitted on a resource that is not reserved

  - After the resource selection is performed, HARQ retransmission on a resource not reserved by a prior SCI is allowed due to transmission dropping caused by prioritization, pre-emption and congestion control

However, the resource exclusion for different TBs is still unclear. Two options are discussed in Proposal 2-4 in R1-2007220.

Proposal 2-4

- Option 1:

  - When resource (re-)selection for a given TB is performed, the slots containing resources selected for other TBs are excluded from the candidates before RSRP-based exclusion procedure

- Option 2:

  - When resource (re-)selection for a given TB is performed, it is up to RAN2 and MAC specification how to ensure that resources for different TBs are selected in different slots.

The difference between the above two options lies on the timing of excluding the resources for different TBs. In Option 1, the slots containing resources selected for other TBs are excluded before the RSRP-based exclusion, i.e., step 6) in resource determination procedure in PHY layer. With this option, the resource collision due to other TBs can be avoided and the number of remaining candidate resource reported to higher layer can be guaranteed.

While for Option 2, the resource exclusion for different TBs is performed after the remaining candidate resource set SA is reported to MAC layer, and the transmission resource for current TB is selected after excluding the resource for the other TBs. If the UE has selected multiple resources for the other TBs, then it is highly possible that there are not enough candidate resources left for current TB. Moreover, NR-V2X introduced more restrictions during resource selection, such as HARQ RTT minimum gap, chain reservation. So if there are not enough candidate resources, then MAC can only select very limited resources, and cannot satisfy the demanding QoS requirement.

According to the resource exclusion procedure for mode 2 in TS 38.214, one candidate single-slot resource Rx,y would be excluded from the set of all the candidate single-slot resources due to the non-monitored slots and the slots on which UE received an SCI from the other UE. For the slots on which UE has not monitored, i.e., UE transmitted on these slots. The hypothetical reservation considers any periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList. In this case, both the reservation from the sensing UE itself and the reservation from the other transmitting UE are included. Hence, only the resources which have been selected and periodically reserved for different TBs can be excluded by step 5). The resources which have been pre-selected but not reserved and the resources which have not been periodically reserved are not excluded by the current resource determination procedure.

For example, as shown in Figure 4, assume the resource determination is triggered in slot n, after the resource exclusion for the selection window [n+T1,n+T2], UE reports a set of the remaining candidate single-slot resources SA  to higher layer, and further assume the higher layer randomly selects resources R1, R2, R3, R4, R5 and R6. The retransmission resources R2 and R3 are reserved by the transmission on resource R1, and the resources R4, R5 and R6 have been pre-selected but not reserved. Assume a new MAC PDU arrives at the higher layer in slot n+T and triggers another resource exclusion procedure in PHY layer. Since the slots for the resources R2, R3, R4, R5 and R6 are later than slot  n+T,  based on the current exclusion procedure defined in TS 38.214, those resources will not be excluded in the resource selection procedure triggered at slot n+T, and probably they would also be selected as candidate resources in SA and be reported to higher layer. And since MAC layer selects resources randomly from SA, it’s possible that those resources are also selected for MAC PDU arriving at slot n+T, thus causing collision between different TBs.

Figure 4: Illustration of resource exclusion for different TBs

Therefore, to avoid the resource collision caused by the resources which have been pre-selected but not reserved and the resources which have not been periodically reserved, one more condition should be considered for resource exclusion.

Proposal 5: The resources which are pre-selected but not reserved, and the resources which have not been periodically reserved, are considered in the resource exclusion procedure.

A proposed text for this correction is provided as follows for Section 8.1.4 of TS 38.214.

--------------------------- Start of Text Proposal for TS 38.214 ------------------------

<Unchanged parts omitted>

8.1.4    UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

In resource allocation mode 2, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission. To trigger this procedure, in slot n, the higher layer provides the following parameters for this PSSCH/PSCCH transmission:

<Unchanged parts omitted>

-	if the higher layer requests the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission as part of re-evaluation or pre-emption procedure, the higher layer provides a set of resources (r0,r1,r2,…) which may be subject to re-evaluation and a set of resources (r0',r1',r2',…) which may be subject to pre-emption.

-	it is up to UE implementation to determine the subset of resources as requested by higher layers before or after the slot ri'' - T3, where ri'' is the slot with the smallest slot index among (r0,r1,r2,…) and (r0',r1',r2',…) , and T3 is equal to Tproc,1SL, where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP.

- the higher layer provides a set of resources (r0,r1,r2,…) which were selected before n for PSSCH/PSCCH transmission.

The following steps are used:

<Unchanged parts omitted>

4)	The set SA is initialized to the set of all the candidate single-slot resources.

5-1)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

-	the UE has not monitored slot tmSL in Step 2.

-	for any periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList-r16 and a hypothetical SCI format 1-A received in slot tmSL with "Resource reservation period" field set to that periodicity value and indicating all subchannels of the resource pool in this slot, condition c in step 6 would be met.

5-2)	For remaining resources from the set (r0,r1,r2,…) which have not been excluded by step 5-1), the UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

a) Remaining resource from the set (r0,r1,r2,…)  overlaps with Rx,y+j×Prsvp_TX' for j=0, 1, …, Cresel-1

<Unchanged parts omitted>

------------------------------------End of Text Proposal -------------------------------

2.3.3 Periodically reserved resources during pre-emption check

According to the agreements in RAN1#100b-e, the procedure of check whether a reserved resource is pre-empted follows the same steps as the regular resource determination in section 8.1.4 of 38.214.

Agreements:

- The procedure to check whether a reserved resource to be signaled in slot ‘m’ should be re-selected due to pre-emption:

  - A regular Step 1 (as in 8.1.4 in 38.214) of the resource (re-)selection procedure is performed

  - If the reserved resource is still in the identified candidate resource set after the Step 1 execution, then Step 2 for reselection of the reserved resource(s) is not triggered

  - If the reserved resource is NOT in the identified candidate resource set after the Step 1 execution

    - If the resource is excluded by comparison with the RSRP measurement for an SCI associated with a priority which can trigger pre-emption, then Step 2 for reselection of the reserved resource(s) is triggered

    - If the resource is excluded by comparison with the RSRP measurement for an SCI associated with a priority which cannot trigger pre-emption, then Step 2 for reselection of the reserved resource(s) is not triggered

According to the step 5) of resource exclusion for mode 2 in TS 38.214, for the slots on which UE has not monitored, a hypothetical reservation considers any periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList for all the sub-channels. If the resources provided by higher layer for pre-emption check have already been periodically reserved, i.e., the pervious transmission is performed on the non-monitored slots, the resources for pre-emption check will be excluded directly in step 5).

For example, as shown in Figure 5, assume one transmission is performed on resource R1 in slot k, and the transmission on resource R1 reserves the next periodic resource R2 in slot m. When the pre-emption check for resource R2 is triggered in slot m-T3, assume the sensing window [n-T0,n-Tproc,0] includes the slot k, then the slot k will be regarded as the non-monitored slots, the resource R2 will be excluded by the current exclusion rule in step 5).

Observation 4: If the resources provided by higher layer for pre-emption check have already been periodically reserved, these resources will be excluded directly in step 5).

Therefore, to prevent the resources for pre-emption check are excluded directly by current step 5), the period associated with these resources is excluded in step 5).

Proposal 6: If resources subject to pre-emption check have already been periodically reserved, the period associated with these resources is excluded in step 5) during pre-emption check.

Figure 5: Illustration of periodical reservation for pre-emption check

A proposed text for this correction is provided as follows for Section 8.1.4 of TS 38.214.

--------------------------- Start of Text Proposal for TS 38.214 ------------------------

<Unchanged parts omitted>

8.1.4    UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

In resource allocation mode 2, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission. To trigger this procedure, in slot n, the higher layer provides the following parameters for this PSSCH/PSCCH transmission:

<Unchanged parts omitted>

-	if the higher layer requests the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission as part of re-evaluation or pre-emption procedure, the higher layer provides a set of resources (r0,r1,r2,…) which may be subject to re-evaluation and a set of resources (r0',r1',r2',…) which may be subject to pre-emption.

-	it is up to UE implementation to determine the subset of resources as requested by higher layers before or after the slot ri'' - T3, where ri'' is the slot with the smallest slot index among (r0,r1,r2,…) and (r0',r1',r2',…) , and T3 is equal to Tproc,1SL, where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP.

The following steps are used:

<Unchanged parts omitted>

4)	The set SA is initialized to the set of all the candidate single-slot resources.

5)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

-	the UE has not monitored slot tmSL in Step 2.

-	for any periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList-r16 and a hypothetical SCI format 1-A received in slot tmSL with "Resource reservation period" field set to that periodicity value and indicating all subchannels of the resource pool in this slot, condition c in step 6 would be met.

Note: If the resource subject to pre-emption check has already been periodically reserved, the period associated with this resource is excluded from step 5) during pre-emption check

<Unchanged parts omitted>

------------------------------------End of Text Proposal -------------------------------

2.3.4 Exclude resources based on PSSCH decoding results

In TS 38.321, the early termination of re-transmission is supported if the ACK is received before the last transmission. Therefore, the Tx UE will release the reserved retransmission resource.

5.22.1.1 SL Grant reception and SCI transmission 1>if a selected sidelink grant is available for retransmission(s) of a MAC PDU which has been positively acknowledged as specified in clause 5.22.1.3.3: 2> clear the PSCCH duration(s) and PSSCH duration(s) corresponding to retransmission(s) of the MAC PDU from the selected sidelink grant.

For the HARQ feedback based retransmission in unicast, to increase the resource utilization, the reserved resources on which the sensing UE expects to receive PSCCH transmission should not be excluded if the decoding results for the previous transmission is successful. For example, as shown in Figure 6, assume the resource exclusion is triggered in slot n, an SCI received in slot tmSL indicates the resource R1, and the fields of “Frequency resource assignment” and “Time resource assignment” reserves the retransmission resources R2 and R3 within the selection window  [n+T1,n+T2] . Assume the field of “Destination ID” indicates the receiver UE is the sensing UE itself and the field of “Cast type indicator” indicates unicast transmission. If the sensing UE decoded the PSSCH/PSCCH transmission successfully in slot tmSL, it means the resources R2 and R3 will not be used by the Tx UE, so the resources R2 and R3 should not be excluded even if the conditions of RSRP measurement and resources overlapping in TS 38.214 are met.

Proposal 7: A sensing UE is allowed to exclude the resources based on its PSSCH decoding results.

Figure 6: Illustration of resource exclusion for the expected reception

A proposed text for this correction is provided as follows for Section 8.1.4 of TS 38.214.

--------------------------- Start of Text Proposal for TS 38.214 ------------------------

<Unchanged parts omitted>

8.1.4    UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

In resource allocation mode 2, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission. To trigger this procedure, in slot n, the higher layer provides the following parameters for this PSSCH/PSCCH transmission:

<Unchanged parts omitted>

The following steps are used:

<Unchanged parts omitted>

6)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions unless the receiver is the UE selecting resources, and the field “Cast type indicator” indicates  unicast, and the PSSCH/PSCCH transmission in slot tmSL is successfully received”:

a)	the UE receives an SCI format 1-A in slot tmSL, and "Resource reservation period" field, if present, and "Priority" field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause 16.4 in [6, TS 38.213];

b)	the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than  ThprioRX;

c)	the SCI format received in slot tmSLor the same SCI format which, if and only if the "Resource reservation period" field is present in the received SCI format 1-A,  is assumed to be received in slot(s) tm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=TscalPrsvp_RX  if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX', where tn'SL = n if slot n belongs to the set t0SL,t1SL,...,tTmaxSL, otherwise slot tn'SL is the first slot after slot n belonging to the set t0SL,t1SL,...,tTmaxSL; otherwise Q=1. Tscal is set to selection window size T2 converted to units of ms.

7)	If the number of candidate single-slot resources remaining in the set SA is smaller than X⋅Mtotal, then Th(pi,pj) is increased by 3 dB for each priority value Th(pi,pj) and the procedure continues with step 4.

The UE shall report set SA to higher layers.

<Unchanged parts omitted>

------------------------------------End of Text Proposal -------------------------------

  - Clarification on UE procedure for determining the number of logical slots for a reservation period

In subclause 8.1.7 of TS 38.214, a procedure to convert the reservation period from millisecond to the logical slot is specified. The number of slots, which can be used for SL transmission within 20ms is involved for the conversion. Technically, the UE needs to determine which slots can be used for SL transmission based on clause 8. In addition to the tdd-UL-DL configuration, the SL slots exclude the S-SSB slots, reserved slots and are further decided based on the bitmap indication, but these are not mentioned in subclause 8.1.7, resulting in the SL slots determination for reservation period conversion incomplete. To make the specification clearer, the following TP is proposed.

Proposal 8: The following text proposal is adopted to clarify that the number of SL slots within 20ms used for reservation period conversion is determined according to clause 8 of TS 38.214.

--------------------------- Start of Text Proposal for TS 38.214 ------------------------

<Unchanged parts omitted>

8.1.7	UE procedure for determining the number of logical slots for a reservation period

A given resource reservation period Prsvp in milliseconds is converted to a period Prsvp' in logical slots as:

Prsvp'=N20 ms×Prsvp

where N is the number of slots that can be used for SL transmission within 20 ms according to clause 8.

<Unchanged parts omitted>

------------------------------------End of Text Proposal -------------------------------

- Conclusions

In this contribution, we discussed the resource allocation for UE autonomous transmission for NR V2X sidelink transmission. We have the following observations and proposals:

Observation 1: The backward indication is needed when a group of consecutive SCIs are missed due to the fluctuation of the channel condition.

Observation 2: It is possible that the resource which has already been excluded in the previous resource selection window would be considered as the identified candidate resource again when the sensing window is changed.

Observation 3: When the number of the remaining resources in SA after performing step 5) is already smaller than X⋅Mtotal, the operation of increasing RSRP threshold cannot help to ensure the final resources set satisfies the requirement since step 5) is not related with the RSRP threshold.

Observation 4: If the resources provided by higher layer for pre-emption check have already been periodically reserved, these resources will be excluded directly in step 5).

Proposal 1: Support full backward indication, i.e., support Proposal 1b (full) in the FL summary R1-2003038.

Proposal 2: A candidate resource will be excluded from the identified candidate resource set SA reported to higher layer if, in any one or more resource selection window associated with the re-evaluation or pre-emption check for a pre-selected or reserved resource, it is not regarded as an identified candidate resource.

Proposal 3: When SCI in slot ‘k’ signals resource reservation with “Resource reservation period” set to P, re-evaluation for resource(s) in logical slots of a resource pool corresponding to occasions k+P, k+2*P,…, k+Cresel*P is not triggered before the slot ‘k’.

Proposal 4: If number of the excluded resources in step 5) is larger than (1-X)⋅Mtotal , then randomly select some excluded resources in step 5) and add them back in the set SA until the number of the candidate single-slot resources remaining in the set  SA is not smaller than  X⋅Mtotal.

Proposal 5: The resources which are pre-selected but not reserved, and the resources which have not been periodically reserved, are considered in the resource exclusion procedure.

Proposal 6: If resources subject to pre-emption check have already been periodically reserved, the period associated with these resources is excluded in step 5) during pre-emption check.

Proposal 7: A sensing UE is allowed to exclude the resources based on its PSSCH decoding results.

Proposal 8: The following text proposal is adopted to clarify that the number of SL slots within 20ms used for reservation period conversion is determined according to clause 8 of TS 38.214.

References

- Chairman’s Note 3GPP TSG RAN WG1 #100, Mar.2020

- 3GPP TS 38.211

- 3GPP TS 38.212

- 3GPP TS 38.213

- 3GPP TS 38.214

- R1-2007219, “Outcome and TPs from email discussion [102-e-NR-5G_V2X_NRSL-Mode-2-01]”, Moderator (Intel Corporation), RAN1#102-e, August 17th – 28th, 2020