3GPP TSG RAN WG1 Meeting #103-e	R1-2007639

Electronic meeting, October 26th – November 13th, 2020

Agenda item:	8.12.3

Source:	Chengdu TD Tech, TD Tech

Title:	Basic functions for MBS for RRC_IDLE/RRC_INACTIVE UEs

Document for:	Discussion and Decision

- Introduction

The work item “NR Multicast and Broadcast Services” [1] was agreed in RAN#88-e. This work item focuses on providing both the NR MBS solution for RRC_CONNECTED UEs and the NR MBS solution for RRC_IDLE/RRC_INACTIVE UEs. In the contribution, the basic functions for MBS for RRC_ILE/RRC_INACTIVE UEs are discussed and the related proposals are made. For simplicity, II-UE is used to denote an RRC_IDLE/RRC_INACTIVE UE. In detail, the following content is discussed in the contribution.

- BWP configuration for MBS for II-UEs

- Logical channels for PTM bearer

- Group scheduling for PTM bearer

- BWP configuration for MBS for II-UEs

In order to provide the MBS to II-UEs with no influence on the actions by II-UEs on the initial DL BWP, the frequency range [F1, F2] for MBS for II-UEs with the initial DL BWP included can be found based on the bandwidth requirement on the MBS for II-UEs. See Figure 1 for the frequency range [F1, F2] for MBS for II-UEs and the frequency range [f1, f2] for the initial DL BWP.

In order to lower the power consumption of II-UE, the frequency range [F1, F2] can be divided into several BWPs where each generated BWP contains the initial DL BWP. In Figure 1, the frequency range [F1, F2] is divided into five BWPs (BWP1~BWP5). For simplicity, use II-UE BWP to indicate each generated BWP.

Configure at least one CORESET and Search Space ( SS ) group for each II-UE BWP where one CORESET and SS group indicates one CORESET and one SS on the indicated CORESET.

The different II-UE may have the different RF channel bandwidths. If the bandwidth of an II-UE BWP is beyond the RF channel bandwidth of an II-UE, the UE can’t receive the MBS on the II-UE BWP. In order for all II-UEs to receive the MBS on the frequency range [F1, F2], both (F2-f1) and (f2-F1) should be no more than the RF channel bandwidth of the II-UE with the minimum RF channel bandwidth. Such condition limits the bandwidth of the frequency range [F1, F2]. If the frequency range [F1, F2] satisfying such condition can’t provide enough bandwidth for the MBS for II-UE, at least one secondary II-UE BWP needs to be configured to provide the radio resource for the MBS for II-UE.

Each secondary II-UE BWP is configured outside of the frequency range [F1, F2]. Each secondary II-UE BWP acts as the initial DL BWP does, which means each secondary II-UE BWP provides the same information to an II-UE as the initial DL BWP does. An II-UE camps on the secondary II-UE BWP just as it camps on the initial DL BWP. With the secondary II-UE BWP, the bandwidth requirement for the MBS for II-UEs can be met with the extra radio resource consumption for the system information and paging information on the secondary II-UE BWP. In order to provide the scheduling information, at least one CORESET and SS group needs to be configured for each secondary II-UE BWP, where one CORESET and SS groups indicate one CORESET and one SS on the indicated CORESET.

Based on the above discussion, the following proposals are made.

Proposal 1: The frequency range [F1, F2] with the initial DL BWP contained can be set to provide the MBS for II-UEs. The frequency range [F1, F2] can be divided into several II-UE BWPs to lower the power consumption of II-UEs with each II-UE BWP containing the initial DL BWP.

Proposal 2: One or several secondary II-UE BWPs can be configured outside the frequency range [F1, F2] to meet the bandwidth requirement for MBS for II-UE. Each secondary II-UE BWP can not only provide the MBS for II-UEs, but also acts as the initial DL BWP does.

Proposal 3: At least one CORESET and SS group needs to be configured for each II-UE BWP or each secondary II-UE BWP.

- Logical channels for PTM bearer

The PTM bearer is used to carry the MBS to II-UEs. The logical channel for the PTM bearer shall be a common logical channel. SC-MTCH in the LTE SC-PTM is reused for the PTM bearer of an MBS for II-UEs. If the PTM bearer has K RBs, K SC-MTCHs can be configured with one-to-one mapping between K RBs and K SC-MTCHs.

In order for II-UEs to receive the PTM bearer configuration information of the PTM bearer of an MBS for II-UEs, SC-MCCH in LTE SC-PTM is reused.

Based on the above discussion, the following proposals are made.

Proposal 4: SC-MTCH is used to carry each RB of the PTM bearer of an MBS for II-UEs

Proposal 5: SC-MCCH is used to carry the PTM bearer configuration information for the PTM bearer of an MBS for II-UEs.

- Group scheduling for PTM bearer

The RBs of the PTM bearer can be classified into P-RBs and NP-RBs. Each P-RB has the feature of transmitting periodically with the same fixed data amount during each period. Except the P-RBs, each of the other RBs is NP-RB.

For the PTM bearer of an MBS for II-UEs, the same processing for the PTM bearer of an MBS for RRC_CONNECTED UEs can be reused. In detail, the following proposals are made.

Proposal 6: G-RNTI and SPS G-RNTI are configured for an MBS.

Proposal 7: For a P-RB of the PTM bearer, the following items need to be supported.

- Configure the SPS PDSCH resource to transmit the data of a P-RB of the PTM bearer periodically

- PDCCH with CRC scrambled with SPS G-RNTI is used to activate/de-activate the SPS PDSCH resource.

- SPS G-RNTI is used in the bit scrambling of the PDSCH carrying a P-RB of the PTM bearer.

Proposal 8: For all the NP-RBs of the PTM bearer, the following items need to be supported.

- The dynamic scheduling is used to transmit the data of all the NP-RBs of the PTM bearer with the SC-MTCHs of all the NP-RBs multiplexed onto one DL-SCH.

- Each time the PTM bearer is scheduled, the PDCCH and PDSCH resource is allocated. The PDCCH with CRC scrambled with G-RNTI is transmitted with the allocated PDCCH resource. The PDSCH with G-RNTI used in the bit scrambling is transmitted with the allocated PDSCH resource

- Each time the PTM bearer is scheduled, one TB is generated and repeatedly transmitted N1 times in each beam coverage area where at least one RRC_CONNECTED UE is located.

Proposal 9: The beams for transmitting the SS/PBCH BLOCK are used to transmit the PTM bearer.

For SC-MCCH, reuse the processing of SC-MCCH in LTE SC-PTM. In detail, the following proposals are made.

Proposal 10: A new SIB is used to carry the configuration information of SC-MCCH. The same transmission manner with the medication period and repetition period as in LTE SC-PTM is reused.

Proposal 11: PDCCH with CRC scrambled by SC-RNTI I is used to carry the dynamic scheduling information of SC-MCCH.

Proposal 12: The SC-MCCH notification is used to indicate a new MBS session in the NR cell. The SC-MCCH notification is sent by PDCCH with CRC scrambled by SC-N-RNTI.

- Conclusion

Based on the discussion in the above sections, the following proposals for the NR MBS solution for RRC_CONNECTED UEs are made and expected to be discussed and agreed.

Proposal 1: The frequency range [F1, F2] with the initial DL BWP contained can be set to provide the MBS for II-UEs. The frequency range [F1, F2] can be divided into several II-UE BWPs to lower the power consumption of II-UEs with each II-UE BWP containing the initial DL BWP.

Proposal 2: One or several secondary II-UE BWPs can be configured outside the frequency range [F1, F2] to meet the bandwidth requirement for MBS for II-UE. Each secondary II-UE BWP can not only provide the MBS for II-UEs, but also acts as the initial DL BWP does.

Proposal 3: At least one CORESET and SS group needs to be configured for each II-UE BWP or each secondary II-UE BWP.

Proposal 4: SC-MTCH is used to carry each RB of the PTM bearer of an MBS for II-UEs

Proposal 5: SC-MCCH is used to carry the PTM bearer configuration information for the PTM bearer of an MBS for II-UEs.

Proposal 6: G-RNTI and SPS G-RNTI are configured for an MBS.

Proposal 7: For a P-RB of the PTM bearer, the following items need to be supported.

- Configure the SPS PDSCH resource to transmit the data of a P-RB of the PTM bearer periodically

- PDCCH with CRC scrambled with SPS G-RNTI is used to activate/de-activate the SPS PDSCH resource.

- SPS G-RNTI is used in the bit scrambling of the PDSCH carrying a P-RB of the PTM bearer.

Proposal 8: For all the NP-RBs of the PTM bearer, the following items need to be supported.

- The dynamic scheduling is used to transmit the data of all the NP-RBs of the PTM bearer with the SC-MTCHs of all the NP-RBs multiplexed onto one DL-SCH.

- Each time the PTM bearer is scheduled, the PDCCH and PDSCH resource is allocated. The PDCCH with CRC scrambled with G-RNTI is transmitted with the allocated PDCCH resource. The PDSCH with G-RNTI used in the bit scrambling is transmitted with the allocated PDSCH resource

- Each time the PTM bearer is scheduled, one TB is generated and repeatedly transmitted N1 times in each beam coverage area where at least one RRC_CONNECTED UE is located.

Proposal 9: The beams for transmitting the SS/PBCH BLOCK are used to transmit the PTM bearer.

Proposal 10: A new SIB is used to carry the configuration information of SC-MCCH. The same transmission manner with the medication period and repetition period as in LTE SC-PTM is reused.

Proposal 11: PDCCH with CRC scrambled by SC-RNTI I is used to carry the dynamic scheduling information of SC-MCCH.

Proposal 12: The SC-MCCH notification is used to indicate a new MBS session in the NR cell. The SC-MCCH notification is sent by PDCCH with CRC scrambled by SC-N-RNTI.

Reference

- RP-201038, Revised Work Item on NR Multicast and Broadcast Services, RAN Meeting #88-e.