3GPP TSG RAN WG1 #103-e	                                              R1-2007699

e-Meeting, October 26th – November 13th, 2020

Source:	vivo

Title:	Performance evaluation of XR traffic

Agenda Item:	8.14.2

Document for:	Discussion and Decision

- Introduction

In RAN#86, the new study item on XR evaluation for NR was approved with the following objectives [1],

The following applications are to be considered as starting points for this study:

- VR1: “Viewport dependent streaming”

- VR2: “Split Rendering: Viewport rendering with Time Warp in device”

- AR1: “XR Distributed Computing”

- AR2: “XR Conversational”

- CG: Cloud Gaming

Note: Use cases in quotes are from TR26.928.

The following traffic parameters for the different applications are to be considered as starting point for the study:

Traffic characteristics:

- UL and DL File Size distribution (e.g., Pareto with given parameters)

- UL and DL File arrival time distribution (e.g., Periodic every 1/60 seconds)

Traffic requirements:

- Round-trip-time or UL and DL one-way Packet delay budget (PDB)

- UL and DL Packet error rate (PER)

The objective of this study item are as follows:

- Confirm XR and Cloud Gaming applications of interest

- Identify the traffic model for each application of interest taking outcome of SA WG4 work as input, including considering different upper layer assumptions, e.g. rendering latency, codec compression capability etc.

- Identify evaluation methodology to assess XR and CG performance along with identification of KPIs of interest for relevant deployment scenarios

- Once traffic model and evaluation methodologies are agreed, carry out performance evaluations towards characterization of identified KPIs

Note 1: eURLLC SI/WI work relevant to XR should be taken into consideration.

Note 2: Traffic model for the performance evaluation shall be based on the standardization in SA WG4

In our companion contribution [2], we provide our views on XR applications, traffic models and evaluation methodologies. According to our proposed traffic models and evaluation methodologies, this contribution presents our initial performance evaluation results on XR.

- Performance evaluation results

In this section, the system performance of capacity, power consumption and coverage are evaluated, considering both Indoor Hotspot and Dense Urban scenarios. The overall evaluation assumptions for XR are listed in Appendix A.

  - Capacity

The initial simulation results of capacity for XR and Cloud Gaming are described for downlink and uplink separately as follows.

    - DL

The following different cases are corresponding to different DL traffic models, and the detailed parameters are shown in Table 1.  Case 1 and Case 2 can be regarded as 2K by 2K per eye, while Case 3 can be regarded as 4K by 4K per eye. For Case 1, packets of two eyes are generated and transmitted at the same time. For Case 2 and Case 3, packets of two eyes are generated and transmitted alternately. Case 4 corresponds to the traffic model of Cloud Gaming, in which the same frame is rendered for both eyes. The key metric of DL capacity evaluation is the percentage of UEs with ≥99% packets successfully delivered within PDB.

Table 1. Different traffic models for XR and Cloud Gaming

| Cases | Use case | Packet Mean | Packet STD | Packet Max | Packet Min | Packet Period | PDB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | VR/AR | 104000Bytes | 13000Bytes | 162500Bytes | 67Bytes | 16.67ms | 10ms |
| Case 2 |  | 52000Bytes | 6500Bytes | 81250Bytes | 67Bytes | 8.33ms | 10ms |
| Case 3 |  | 104000Bytes | 13000Bytes | 162500Bytes | 67Bytes | 8.33ms | 10ms |
| Case 4 | CG | 52000Bytes | 6500Bytes | 81250Bytes | 67Bytes | 16.67ms | 15ms |

- VR/AR

For VR/AR cases, the system performance for FR1 and FR2 are evaluated separately.

- FR1

  - Indoor Hotspot, 4GHz, DDDSUDDSUU

| - Capacity | - RU |
| --- | --- |

Figure 1. FR1 VR/AR capacity simulation results in Indoor Hotspot scenario

  - Dense Urban, 4GHz, DDDSUDDSUU

| - Capacity | - RU |
| --- | --- |

Figure 2. FR1 VR/AR capacity simulation results in Dense Urban scenario

Based on the system-level simulation results above, it can be observed that the percentage of UEs with ≥99% packets successfully delivered within PDB is decreased with the increasing of the number of UEs per cell, due to resource utilization increasing rapidly.

Compared to Case 1, the capacity performance for Case 2 is better. This is because the PDB for two-eye frame transmission in Case 1 is bounded to 10 ms, while the PDB of each frame transmission in Case 2 is independent, implying that the total PDB for two-eye frame transmission in Case 2 is longer. As for Case 3, the percentage of UEs with ≥99% packets successfully delivered within PDB is lower than that in Case 1 and Case 2, resulting from the smaller packet period compared to Case 1, and the data-rate in Case 3 is about twice than Case 1. In Case2 and Case 3, the same packet arrival period is applied, but in Case 3 bigger packets are assumed, which means a higher resolution and more data will be transmitted.

Observation 1: For VR/AR traffic in both Indoor Hotspot and Dense Urban scenarios in FR1, the capacity performance is degraded with the increasing of the number of UEs per cell.

- FR2

  - Indoor Hotspot, 28GHz, DDDSU

| - Capacity | - RU |
| --- | --- |

Figure 3. FR2 VR/AR capacity simulation results in Indoor Hotspot scenario

  - Dense Urban, 28GHz, DDDSU

| - Capacity | - RU |
| --- | --- |

Figure 4. FR2 VR/AR capacity simulation results in Dense Urban scenario

As shown in Figure 3 and Figure 4, with less than or equal to 30 UEs per cell, the percentage of UEs with ≥99% packets successfully delivered within PDB for Case 1 and Case 2 are above 98%, and the resource utilization is only about 60% with 30 users per cell. For Case 3, at least 15 users per cell can be supported in FR2, where there is more than 90% UEs with ≥99% packets delivered within PDB both in Indoor Hotspot and Dense Urban scenario. Compared to FR1, it is obvious that FR2 can accommodate more users due to wider bandwidth and more transmission opportunities.

Observation 2: For VR/AR traffic in both Indoor Hotspot and Dense Urban scenarios in FR2, at least 30, 30 and 15 users per cell can be supported for Case1, Case2 and Case3 respectively, with more than 90% UEs with ≥99% packets delivered within PDB.

- Cloud Gaming

For Cloud Gaming, Case 4 in Table 1 is evaluated, and corresponding simulation results for FR1 are depicted as follows.

  - Indoor Hotspot, 4GHz, DDDSUDDSUU

| - Capacity | - RU |
| --- | --- |

Figure 5. Cloud Gaming capacity simulation results in Indoor Hotspot scenario

  - Dense Urban, 4GHz, DDDSUDDSUU

| - Capacity | - RU |
| --- | --- |

Figure 6. Cloud Gaming capacity simulation results in Dense Urban scenario

It can be observed that the percentage of UEs with ≥99% packets successfully delivered within PDB could reach nearly 100% for Cloud Gaming, because of smaller packet sizes, larger packet arrival period and looser PDB requirements. As shown in the above figure, the resource utilization is significantly lower for Cloud Gaming.

Observation 3: For Cloud Gaming traffic in both Indoor Hotspot and Dense Urban scenarios in FR1, at least 15 users per cell can be supported with the percentage of users with ≥99% packets successfully delivered within PDB larger than 90%.

    - UL

In FR1 uplink simulation, interaction/pose information delivering and scene updating are both evaluated. The traffic models for interaction and pose information delivering are shown in Table 2. For the case of interaction and pose information delivering, the key metric of UL capacity evaluation is the 95th percentile user interaction delay. For the case of scene updating, DL capacity evaluation methodology can be reused.

Table 2. UL traffic models for interaction and pose information

| Pose cases | Packet Size | Packet Period | PDB |
| --- | --- | --- | --- |
| Case 1 | Fixed, 100Bytes | 1ms | 10ms |
| Case 2 | Fixed, 100Bytes | 2ms | 10ms |

- Interaction/pose information delivering

- FR1

  - 4GHz, DDDSUDDSUU

| - Indoor Hotspot scenario | - Dense Urban scenario |
| --- | --- |

Figure 7. UL capacity simulation results in FR1 for interaction/pose information

As shown in Figure 7, it can be observed that the 95th percentile user interaction delay for Case 2 is nearly same for different numbers of UEs per cell. For Case 1, the 95th percentile user interaction delay is slightly higher than that for Case 2 because of the higher traffic load for Case 1. Moreover, the 95th percentile user interaction delay is far below the uplink PDB requirement.

Observation 4: For uplink traffic of interaction/pose information delivering in FR1, when the number of UEs per cell is no more than 16, 95th percentile user interaction delay is far below 10ms.

- FR2

  - Indoor Hotspot, 28GHz, DDDSU

| - Indoor Hotspot scenario | - Dense Urban scenario |
| --- | --- |

Figure 8. UL capacity simulation results in FR2 for interaction/pose information

As shown in Figure 8, it can be observed that the 95th percentile user interaction delay increases with the increasing of the number of UEs per cell in both Indoor Hotspot and Dense Urban scenarios. Compared to FR1, it is obvious that FR2 has poor performance since beam-based single-user scheduling is assumed.

Observation 5: For uplink traffic of interaction/pose information delivering in FR2, the 95th percentile user interaction delay increases significantly with the increasing of the number of UEs per cell.

- Scene updating

For the case of scene updating, evaluation is performed only in Indoor Hotspot scenario.

- FR1

  - Indoor Hotspot, 4GHz, DDDSUDDSUU

| - Capacity | - RU |
| --- | --- |

Figure 9. UL capacity simulation results in FR1 for scene information

It can be observed that the percentage of UEs with ≥99% packets successfully delivered within PDB decreases rapidly as the increase of the number of UEs per cell in both Indoor Hotspot and Dense Urban scenarios. Especially, when up to 10 UEs per cell are assumed, the percentage of UEs with ≥99% packets successfully delivered within PDB is close to 0%. The resources are nearly exhausted when 7 users per cell are assumed for Case 1 and Case 2, and 4 users for Case 3.

- FR2

  - Indoor Hotspot, 28GHz, DDDSU

| - Capacity | - RU |
| --- | --- |

Figure 10. UL capacity simulation results in FR2 for scene information

As shown in Figure 10, on one hand, similar to DL capacity evaluation results, it is obvious that FR2 can accommodate more users due to wider bandwidth and more transmission opportunities compared to FR1. On the other hand, UL evaluation results for both FR1 and FR2 are significantly worse than DL evaluation results, due to less uplink resources.

Observation 6: For uplink traffic of scene updating, the uplink evaluation results for both FR1 and FR2 are significantly worse than the downlink evaluation results.

  - Power consumption

In the following power consumption evaluation, the retransmission timers such as drx-RetransmissionTimer and drx-HARQ-RTT-Timer are not modelled, and only a single user per cell is assumed in Dense Urban scenario. In addition, the DL traffic model for Case 1 is adopted. Table 3 provides two sets of DRX configurations, both of which can satisfy the DL capacity requirement, that is, the percentage of UEs with ≥99% packets successfully delivered within PDB is not less than 90%.

Table 3. DRX configurations

| DRX parameters | DRX cycle (ms) | drx-onDurationTimer (ms) | drx-InactivityTimer(ms) |
| --- | --- | --- | --- |
| DRX configuration 1 | 8 | 4 | 1 |
| DRX configuration 2 | 10 | 5 | 1 |

Figure 11. The ratio of different power state time

Figure 12. Power consumption results for different DRX configurations

Figure 13. Percentage of UEs satisfying requirements in different DRX configurations

From Figure 11, it can be observed that the PDCCH-only proportion can be reduced by half which is changed into micro sleep with DRX configurations, compared to that without DRX. However, due to the short traffic arrival time interval, there is no time for deep sleep, light sleep and related state transition. From Figure 12, it is obvious that the total power consumption can be reduced by 21.8% by using DRX configurations. Figure 13 compares DL capacity performance metrics for different cases further. Compared to the case without DRX, the UE capacity will suffer a little bit decrease due to packet scheduling latency caused by DRX configuration. The smaller the DRX cycle, the lower latency and smaller impact on UE capacity. Capacity evaluation with DRX configuration for more than 1 user per cell could be evaluated further if required.

Observation 7: Up to 21.8% power saving gain can be achieved by appropriate DRX configuration, at the same time DL capacity performance is guaranteed, compared to the case without DRX.

Observation 8: Due to the short time interval of traffic arrival, there is no time for deep sleep and light sleep with DRX configuration.

Observation 9: Enabling DRX has a slight impact on the percentage of UEs satisfying requirements for DL.

  - Coverage

In our companion contribution, we propose to use max isotropic loss (MIL) as the XR coverage evaluation metric. The MIL can be deduced based on link budget template. Evaluation assumptions for capacity evaluation can be reused, and the evaluation results of MIL for different scenarios are shown below.

    - DL

- FR1

Following carrier frequencies and frame structures are considered in FR1, and only NLOS outdoor-to-indoor is considered for Dense Urban scenario. In XR evaluation, the target data rate of PDSCH is 50Mbps or 100Mbps, so massive data need to be transmitted in limited resources, which results in decreasing of transmission performance. The evaluation results of Dense Urban scenario are shown in Figure 14. It can be observed that PDSCH is the bottleneck channel among all DL channels, based on MIL.

  - 4GHz, DDDSUDDSUU

Figure 14. DL MIL in Dense Urban scenario for 4GHz, DDDSUDDSUU

Observation 10: In Dense Urban scenario in FR1, PDSCH is the bottleneck channel among downlink channels.

- FR2

Following carrier frequency and frame structures are considered in FR2, and only NLOS indoor-to-indoor is considered for Indoor Hotspot scenario. And for Dense Urban scenario, only NLOS outdoor-to-indoor is evaluated. The evaluation results for Indoor Hotspot and Dense Urban scenarios in FR2 are shown in Figure 15. It can be observed that PDSCH is the bottleneck channel among all DL channels due to the requirement of high data rate.

  - 28GHz, DDDSU

| - Indoor Hotspot scenario | - Dense Urban scenario |
| --- | --- |

Figure 15. DL MIL for 28GHz, DDDSU

Observation 11: In Indoor Hotspot and Dense Urban scenarios in FR2, PDSCH is the bottleneck channel among downlink channels.

    - UL

- FR1

For XR UL coverage evaluation, data rate requirements 0.4Mbps, 2Mbps, 50Mbps, 100Mbps are evaluated, which correspond to two cases for interaction/pose information delivering and two cases for scene updating respectively. The evaluation results of FR1 Dense Urban scenario are shown in Figure 16, where it can be observed that the bottleneck channel among uplink channels is PUSCH, especially the PUSCH with high data-rate requirement.

  - 4GHz, DDDSUDDSUU

Figure 16. UL MIL in Dense Urban scenario for 4GHz, DDDSUDDSUU

Observation 12: For Dense Urban scenario in FR1, PUSCH is the bottleneck channel among uplink channels .

- FR2

For FR2, similar conclusions can be observed in both Indoor Hotspot and Dense Urban scenarios, compared to FR1. PUSCH is the bottleneck channel among all uplink channels.

  - 28GHz, DDDSU

| - Indoor Hotspot scenario | - Dense Urban scenario |
| --- | --- |

Figure 17. UL MIL for 28GHz, DDDSU

Observation 13: For Indoor Hotspot scenario and Dense Urban scenario in FR2, PUSCH is the bottleneck channel among all uplink channels .

- Conclusion

In this contribution, we provide our initial simulation results on XR capacity, power consumption and coverage with the following observations and proposals:

Observation 1: For VR/AR traffic in both Indoor Hotspot and Dense Urban scenarios in FR1, the capacity performance is degraded with the increasing of the number of UEs per cell.

Observation 2: For VR/AR traffic in both Indoor Hotspot and Dense Urban scenarios in FR2, at least 30, 30 and 15 users per cell can be supported for Case1, Case2 and Case3 respectively, with more than 90% UEs with ≥99% packets delivered within PDB.

Observation 3: For Cloud Gaming traffic in both Indoor Hotspot and Dense Urban scenarios in FR1, at least 15 users per cell can be supported with the percentage of users with ≥99% packets successfully delivered within PDB larger than 90%.

Observation 4: For uplink traffic of interaction/pose information delivering in FR1, when the number of UEs per cell is no more than 16, 95th percentile user interaction delay is far below 10ms.

Observation 5: For uplink traffic of interaction/pose information delivering in FR2, the 95th percentile user interaction delay increases significantly with the increasing of the number of UEs per cell.

Observation 6: For uplink traffic of scene updating, the uplink evaluation results for both FR1 and FR2 are significantly worse than the downlink evaluation results.

Observation 7: Up to 21.8% power saving gain can be achieved by appropriate DRX configuration, at the same time DL capacity performance is guaranteed, compared to the case without DRX.

Observation 8: Due to the short time interval of traffic arrival, there is no time for deep sleep and light sleep with DRX configuration.

Observation 9: Enabling DRX has a slight impact on the percentage of UEs satisfying requirements for DL.

Observation 10: In Dense Urban scenario in FR1, PDSCH is the bottleneck channel among downlink channels.

Observation 11: In Indoor Hotspot and Dense Urban scenarios in FR2, PDSCH is the bottleneck channel among downlink channels.

Observation 12: For Dense Urban scenario in FR1, PUSCH is the bottleneck channel among uplink channels .

Observation 13: For Indoor Hotspot scenario and Dense Urban scenario in FR2, PUSCH is the bottleneck channel among all uplink channels .

# References

- RP-193241, 3GPP TSG RAN Meeting #86, Sitges, Spain, December 9-12, 2019

- R1-2007698, 3GPP TSG RAN WG1 #103-e, e-Meeting, October 26th – November 13th, 2020

# Appendix A – Simulation assumptions

Table I. Simulation assumption for FR1 DL

| Parameter | value |  |
| --- | --- | --- |
| Scenarios | Scenario-1: Indoor Hotspot 12 nodes in 50 m x 120 m | Scenario-2: Dense Urban hexagonal layout with 7, 3 Sectors |
| Carrier frequency | 4GHz |  |
| Bandwidth | 100 MHz, 1.72% Guard Band |  |
| Subcarrier spacing | 30 KHz |  |
| Frame structure | DDDSUDDSUU (S: 10D:2G:2U) |  |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 32T: (4,4,2,1,1;4,4) (dH,dV) = (0.5, 0.5)λ | For 64T: (12,8,2,1,1;4,8) (dH, dV)=(0.5, 0.8)λ |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | For 4R: (1,2,2,1,1;1,2) (dH, dV)=(0.5, N/A)λ |  |
| BS antenna pattern | Ceiling-mount pattern, 5 dBi | 3-TRxP pattern, 8 dBi |
| UE antenna pattern | Omnidirectional, 0 dBi |  |
| BS Power | 24 dBm | 53 dBm |
| ISD | 20 m | 200 m |
| BS height | 3 m | 25 m |
| UE height | 1.5 m | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) |
| Noise Figure | BS:5 dB, UE:9 dB |  |
| Scheduler | MU-MIMO Proportional Fair |  |
| Max MCS | 256QAM |  |
| Device deployment | 100% indoor | 80% outdoor, 20% indoor |
| Down-tilt | 90 degrees | 105 degrees |
| UE speed | 3 km/h |  |

Table II. Simulation assumption for FR1 UL

| Parameter | value |  |
| --- | --- | --- |
| Scenarios | Scenario-1: Indoor Hotspot 12 nodes in 50 m x 120 m | Scenario-2: Dense Urban hexagonal layout with 7, 3 Sectors |
| Carrier frequency | 4GHz |  |
| Bandwidth | 20 MHz, 8.2% Guard Band for interaction/pose information delivering 100 MHz, 1.72% Guard Band for scene updating |  |
| Subcarrier spacing | 30 KHz |  |
| Frame structure | DDDSUDDSUU (S: 10D:2G:2U) |  |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 32R: (4,4,2,1,1;4,4) (dH,dV) = (0.5, 0.5)λ | For 64R: (12,8,2,1,1;4,8) (dH, dV)=(0.5, 0.8)λ |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | For 4T: (1,2,2,1,1;1,2) (dH, dV)=( 0.5, N/A)λ |  |
| BS antenna pattern | Ceiling-mount pattern, 5 dBi | 3-TRxP pattern, 8 dBi |
| UE antenna pattern | Omnidirectional, 0 dBi |  |
| UE max Power | 23 dBm |  |
| Power control | P0 = -80 dBm, alpha = 0.8 |  |
| Noise Figure | BS:5 dB, UE:9 dB |  |
| Scheduler | MU-MIMO Proportional Fair |  |
| Max MCS | 256QAM |  |
| Device deployment | 100% indoor | 80% outdoor, 20% indoor |
| Down-tilt | 90 degrees | 105 degrees |
| UE speed | 3 km/h |  |

Table III. Simulation assumption for FR2 DL

| Parameter | value |  |
| --- | --- | --- |
| Scenarios | Scenario-1: Indoor Hotspot 12 nodes in 50 m x 120 m | Scenario-2: Dense Urban hexagonal layout with 7, 3 Sectors |
| Carrier frequency | 28 GHz |  |
| Bandwidth | 400 MHz, 4.96% Guard Band |  |
| Subcarrier spacing | 120 KHz |  |
| Frame structure | DDDSU (S: 10D:2G:2U) |  |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 8T: (8,8,2,1,1;2,2) (dH, dV)=(0.5, 0.5)λ | For 8T: (4,8,2,2,2;1,1) (dH, dV)=(0.5, 0.5)λ (dg,H,dg,V) = (4.0, 2.0) λ |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | For 8R: (2,4,2,1,2;1,2) (dH,dV) = (0.5, 0.5)λ (dg,V,dg,H) = (0, 0)λ |  |
| BS antenna pattern | Ceiling-mount pattern, 5 dBi | 3-TRxP pattern, 8dBi |
| UE antenna pattern | Directional antenna panel, 5 dBi |  |
| BS Power | 23dBm, EIRP should not exceed 58 dBm | 40dBm, EIRP should not exceed 73 dBm |
| Noise Figure | BS:7 dB, UE:13 dB |  |
| Scheduler | SU-MIMO Proportional Fair |  |
| Max MCS | 256QAM |  |
| Device deployment | 100% indoor | 100% outdoor |
| UE speed | 3 km/h |  |

Table Ⅳ. Simulation assumption for FR2 UL

| Parameter | value |  |
| --- | --- | --- |
| Scenarios | Scenario-1: Indoor Hotspot 12 nodes in 50 m x 120 m | Scenario-2: Dense Urban hexagonal layout with 7, 3 Sectors |
| Carrier frequency | 28 GHz |  |
| Bandwidth | 100 MHz, 4.96% Guard Band for pose/control information uploading 400MHz, 4.96% Guard Band for scene updating |  |
| Subcarrier spacing | 120 KHz |  |
| Frame structure | DDDSU (S: 10D:2G:2U) |  |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 8R: (8,8,2,1,1;2,2) (dH, dV)=(0.5, 0.5)λ | For 8R: (4,8,2,2,2;1,1) (dH, dV)=(0.5, 0.5)λ (dg,H,dg,V) = (4.0, 2.0) λ |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | For 8T: (2,4,2,1,2;1,2) (dH,dV) = (0.5, 0.5)λ (dg,V,dg,H) = (0, 0)λ |  |
| BS antenna pattern | Ceiling-mount pattern, 5 dBi | 3-TRxP pattern, 8dBi |
| UE antenna pattern | Directional antenna panel, 5 dBi |  |
| UE max Power | 23dBm, EIRP should not exceed 43 dBm |  |
| Power control | P0 = -60, alpha = 0.6 | P0 = -74, alpha = 0.6 |
| Noise Figure | BS:7 dB, UE:13 dB |  |
| Scheduler | SU-MIMO Proportional Fair |  |
| Max MCS | 256QAM |  |
| Device deployment | 100% indoor | 100% outdoor |
| UE speed | 3 km/h |  |