3GPP TSG RAN WG1 #103-e                                                                                   R1-2007741

e-Meeting, October 26th – November 13th, 2020

Source:	ZTE

Title:                 Discussion on baseline coverage performance for FR1

Agenda item:	8.8.1.1

Document for: Discussion and Decision

Introduction

According to the revised SID on NR coverage enhancements [1], one objective is to identify the baseline performance for specific scenarios for FR1, where the scenarios include urban scenario and rural scenario. In the RAN1#101-e and RAN1#102-e meeting, the evaluation methodology, LLS simulation assumptions, link budget template were discussed and the output can be found in [2][3][4][5].

In this contribution, we further discuss the remaining issues on the simulation assumptions/link budget template, and provide the baseline coverage performance for DL/UL channels in different scenarios in FR1.

Discussion on LLS simulation assumptions

- DMRS configuration for PUCCH

On PUCCH simulation assumption, there is still one FFS on number of DMRS symbols for PUCCH Format 3 to be resolved. As the simulation results shown in [6], we find the performance of PUCCH format 3 with additional DMRS (i.e. 4 DMRS symbols for 14-OS PUCCH) has better performance for the concerned scenarios. This also aligns with what has been agreed for FR2. Thus, we have the following proposal.

Proposal 1: Support additional DMRS (i.e. 4 DMRS symbols for 14-OS PUCCH) for link level simulation for PUCCH Format 3 for FR1.

- PUCCH carrying HARQ-ACK for Msg4

- In Section 4 of this contribution, we find PUCCH carrying HARQ-ACK for Msg4 could potentially have coverage issues. However, it’s unfortunate that RAN1 missed to discuss the evaluation on this channel and its corresponding simulation assumptions. Compare to normal PUCCH evaluation, the main difference of PUCCH carrying HARQ-ACK for Msg4 is the HARQ-ACK payload is one bit and no PUCCH repetition is supported. As a result, we have the following proposal.

- Proposal 2: For link level simulation of PUCCH format 1 with 1-bit HARQ-ACK for Msg4 in FR1, reuse the simulation assumptions of normal PUCCH with assuming no PUCCH repetition.

Discussion on parameters for link budget template

- Transmitter/Receiver array gain

In RAN1#102-e, the following agreements are reached for antenna gain. Two correction factors 1 and 2 are introduced for antenna gain component 2 and antenna gain component 3/4 respectively. In the following, we provide our views on determination of antenna gain.

Agreements: Further clarify the agreement on antenna gain and antenna gain components including antenna gain correction factors as follows: - For both TDL option 1 (table A below) and TDL option 2 & CDL (table B below) - The gain of antenna gain component 1 is included in LLS results - The gain of antenna gain component 2 is included in link budget template - The gain is expressed by 10 * log 10( N/k ) - 1 - For TDL option 2 & CDL, the gain is 0 dB - The gain of antenna gain component 3 is included in link budget template - The gain of antenna gain component 4 is included in link budget template - The gain of antenna gain components 3 and 4 is expressed by Antenna Element Gain + 10 * log 10( M/N ) -2 - For Tx, One row is used represent the gain of antenna gain component 3 + 4, i.e. row No. (4) - For Rx, One row is used represent the gain of antenna gain component 3 + 4, i.e. row No. (11) - Note: more appropriate name or explanation will be added to row No.(4) and (11). Details can be discussed when the link budget template is updated. Table A. antenna gain components for TDL option 1

Take rural 700MHz as an example,  there are two 2 RF chains with totally 16 elements at gNB side. In such case, Δ1 is zero and Δ2 can be determined by UE’s angular location in reference to the bore sight of the selected beam via SLS. In Figure 1, the CDF curve for analog beamforming gain of all UEs in the layout is provided. In the simulation, 8 analog beams per sector are formulated for channels in connected mode while 4 SSB beams for channels in idle mode.

- Figure 1 CDF curve for analog beamforming gain (antenna gain component 3) of all UEs for 700MHz rural scenario —— DL/UL channels in connected mode (left figure),  DL/UL channels in idle mode (right figure)

As can be observed in Figure 1, if the 95th percentile beamforming gain is chosen, the correction Δ2 is 3.86 dB (9.03 dB - 5.17 dB) for DL/UL channels in RRC-connected mode, and Δ2=7.55 dB (9.03 dB - 1.48 dB) for DL/UL channels in RRC-idle mode. Basically, the beam forming gain of channels in RRC-idle mode is about 4dB worse due to less number of analog beams used.

Observation 1: For rural scenario at 700MHz, Δ2 is 3.86 dB for DL/UL channels in RRC-connected mode, and Δ2 is 7.55 dB for DL/UL channels in RRC-idle mode.

Take 4GHz urban as another example, antenna configuration of (4 RF chains, 64 TxRU, 192 elements) can be assumed at gNB side. In such case, antenna component 1 representing the baseband digital beamforming can be dynamically controlled by gNB. For DL/UL channels in RRC-connected mode, Δ1 could be assumed as zero assuming the bore sight of the digital beam can be directed to the UE without taking the channel estimation error into account. For Δ2, it can be determined by UE’s angular location in reference to the bore sight of the selected beam in SLS (assuming 3 analog beams to cover one sector).

For DL channels in idle mode, all the elements across multiple TxRU per RF chain could be used to form the 8 SSB beams to cover one sector, i.e., using all 48 elements per RF chain. In such case, the antenna gain component 1 and component 2 could be determined jointly. For UL channels in idle mode, considering there are actually 64 TxRUs for receiving the UL transmissions, gNB could enjoy the full diversity and accumulated reception among all TxRUs. In this sense, there is expected no much difference on the antenna gain between UL channels in connected mode and UL channels in idle mode in such scenario.

- Figure 2 CDF curve for beamforming gain of all UEs for 4GHz urban scenario —— DL channels in connected mode or UL channels (left figure),  DL channels in idle mode (right figure)

As can be observed in Figure 2, if the 95th percentile beamforming gain is chosen, the correction Δ2 is 2.95 dB (4.77 dB -1.82 dB) for DL channels in RRC-connected mode and all UL channels, and Δ1+Δ2 = 13.38 dB (16.81 dB - 3.43 dB) for DL channels in RRC-idle mode.

Observation 2: For urban scenario at 4GHz, Δ2 is 2.95 dB for DL channels in RRC-connected mode and all UL channels, and Δ1+Δ2 is 13.38 dB for DL channels in RRC-idle mode.

Similarly, the correction factors could be obtained for other scenarios. In Table 1 below, the correction factors are summarized.

Table 1 Antenna gain correction factors for FR1

| Scenario | Ideal beamforming gain per TxRU (dB) | Antenna gain correction factors (dB) |
| --- | --- | --- |
| DL/UL channels in RRC connect mode or UL channels in RRC idle mode in 2.6/4GHz urban scenario with 4 Rx | 4.77 | Δ2 = 2.95 |
| DL channels in RRC idle mode in 2.6/4GHz urban scenario with 4Rx | 16.81 | Δ1+Δ2 = 13.38 |
| DL/UL channels in RRC connect mode in 2.6/4GHz rural scenario with 4Rx | 0 | Δ2 = 3 |
| DL channels in RRC idle mode or UL channels in RRC idle mode in 2.6/4GHz rural scenario with 4Rx | 12.04 | Δ1+Δ2 = 7.16 |
| DL/UL channels in RRC connect mode in 2GHz rural scenario with 2Rx | 12.04 | Δ2 = 3.98 |
| DL/UL channels in RRC idle mode in 2GHz rural scenario with 2Rx | 12.04 | Δ2 = 9.79 |
| DL/UL channels in RRC connect mode in 700MHz rural scenario with 2Rx | 9.03 | Δ2 =3.86 |
| DL/UL channels in RRC idle mode in 700MHz rural scenario with 2Rx | 9.03 | Δ2 = 7.55 |

- Proposal 3: Consider the antenna gain correction factors in Table 1 for link budget calculation in FR1.

- Penetration margin

In ITU self-evaluation, the penetration margin for O2I is based on the model in Table 7.4.3-3 of TR 38.901. However, this model is only for backwards compatibility purpose. More accurate model in Table 7.4.3-1 and Table 7.4.3-2 of TR 38.901, which is frequency and penetration material dependent, should be used. For urban scenario, 50% low-loss and 50% high-loss models can be considered. Only the low-loss model is applicable to rural scenario. In Table 2, the penetration margins for different O2I cases are given.

Table 2. Penetration margins for different O2I cases

| Fc(Hz) | 4G (Urban) | 2.6G (Urban) | 4G (Rural) | 2.6G (Rural) | 2G (Rural) | 700MHz(Rural) |
| --- | --- | --- | --- | --- | --- | --- |
| ITU Template | 26.25 | - | - | - | - | 12.5 |
| TR 38.901 | 26.68 | 24.56 | 15.38 | 14.76 | 14.33 | 12.74 |

For O2O case, the typical penetration margin is 9 dB according to TR 38.901, which aligns with the assumptions in ITU self-evaluation.

Proposal 4: Consider the penetration margins for different O2I cases in Table 2 for link budget calculation in FR1.

Evaluation results

In this section, the link budget based baseline performance for different channels in different scenarios are provided. Also, we give some SLS based results, namely the 5th percentile SNR and the gap with the required SNR, for some of the scenarios.

- Urban at 4GHz

In Table 3, the evaluation results for urban 4GHz assuming PSD of 33 dBm/MHz are provided. In the evaluation, the following assumptions are used:

- Correction factors for antenna array gain: Using values in Table 1 above.

- Interference density:  Reusing IMT-2020 values.

- Penetration margins: Using values in Table 2 above for 4G urban case.

- Shadow fading margin:  Reusing IMT-2020 values.

- For other MPL specific values:  Reusing IMT-2020 values.

Table 3 Evaluation results for urban 4GHz with PSD of 33 dBm/MHz

| Evaluation methodology | Item | PUSCH-eMBB | PUSCH -VoIP | MSG3 | PRACH B4 | MSG4 ACK | PUCCH-2bits | PUCCH-11bits | PUCCH -22bits | PDCCH-unicast | PDCCH-broadcast | PBCH | PDSCH-eMBB | MSG4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Required SNR | -7.07 | -7.66 | -7.33 | -13.1 | -11.6 | -10.3 | -8.57 | -6.05 | -11.4 | -11.4 | -12.7 | -10.95 | -12.61 |
| LLS based methodology | MCL | 134.31 | 143.15 | 145.83 | 140.74 | 150.04 | 148.74 | 147.01 | 132.45 | 159.43 | 149.00 | 150.30 | 159.48 | 150.21 |
|  | MIL | 140.13 | 148.97 | 151.65 | 146.56 | 155.86 | 154.56 | 152.83 | 150.31 | 165.25 | 154.82 | 156.12 | 165.30 | 156.03 |
|  | MPL | 108.97 | 117.81 | 120.49 | 112.32 | 121.62 | 120.32 | 118.59 | 116.07 | 131.01 | 120.58 | 121.88 | 134.14 | 124.87 |
|  | - Target MPL with ISD=500m | 121.75 | 121.75 | 121.75 | 121.75 | 121.75 | 121.75 | 121.75 | 121.75 | 121.75 | 121.75 | 121.75 | 121.75 | 121.75 |
| SLS based methodology | 5th percentile SNR via SLS | -21.72 | -11.52 | -9.43 | -18.30 | - | -14.56 | -12.96 | -10.69 | -3.63 | -3.37 | -3.37 | -3.63 | -3.37 |
|  | Performance gap via SLS | 14.65 | 3.86 | 2.1 | 5.2 | - | 4.26 | 4.29 | 4.64 | NAN | NAN | NAN | NAN | NAN |

For LLS based methodology, if the target is defined by MPL with ISD = 500m, the dark yellow highlighted channels including all UL channels and PDCCH broadcast channels need coverage enhancement. For service dependent targets, the target could reach to MCL=159.04 dB [7]. In such case, only PDCCH unicast channl and PDSCH carrying eMBB do not need further enhancement. However, if a PSD of 24 dBm/MHz is assumed, more channels are required to be enhanced.

For SLS based methodology, basically we find all UL channels need further enhancement.

Observation 3: In urban scenario at 4GHz,

  - for LLS based methodology, all UL channels and PDCCH broadcast channels need coverage enhancement if the target is defined by MPL with ISD = 500m.

  - for SLS based methodology, all UL channels need coverage enhancement.

- Urban at 2.6GHz

In Table 4, the evaluation results for urban 2.6 GHz assuming PSD of 33 dBm/MHz are provided. In the evaluation, the following assumptions are used:

- Correction factors for antenna array gain: Using values in Table 1 above.

- Interference density:  Reusing IMT-2020 values.

- Penetration margins: Using values in Table 2 above for 2.6 GHz urban case.

- Shadow fading margin:  Using the modified values suggested in [4].

- For other MPL specific values:  Reusing IMT-2020 values.

Table 4 Evaluation results for urban 2.6GHz

| Evaluation methodology | Item | PUSCH-eMBB | PUSCH -VoIP | MSG3 | PRACH B4 | MSG4 ACK | PUCCH-2bits | PUCCH-11bits | PUCCH -22bits | PDCCH-unicast | PDCCH-broadcast | PBCH | PDSCH-eMBB | MSG4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Required SNR | -4.85 | -6.98 | -7.06 | -13.07 | -11.76 | -10.34 | -8.6 | -6.08 | -11.63 | -11.63 | -12.98 | -9.63 | -12.74 |
| LLS based methodology | MCL | 132.09 | 142.47 | 145.56 | 140.71 | 150.20 | 148.78 | 147.04 | 144.52 | 159.66 | 149.23 | 150.58 | 158.16 | 150.34 |
|  | MIL | 137.91 | 148.29 | 151.38 | 146.53 | 156.02 | 154.60 | 152.86 | 150.34 | 165.48 | 155.05 | 156.40 | 163.98 | 156.16 |
|  | MPL | 108.87 | 119.25 | 122.34 | 114.41 | 123.90 | 122.48 | 120.74 | 118.22 | 133.36 | 122.93 | 124.28 | 134.94 | 127.12 |
|  | - Target MPL with ISD=500m | 118.01 | 118.01 | 118.01 | 118.01 | 118.01 | 118.01 | 118.01 | 118.01 | 118.01 | 118.01 | 118.01 | 118.01 | 118.01 |

For LLS based methodology, if the target is defined by MPL with ISD = 500 m, only PUSCH carrying eMBB service and RRACH need coverage enhancement. For service dependent targets, the target could reach to MCL=159.04 dB. In such case, only PDCCH unicast channel does not need further enhancement.

Observation 4: For LLS based methodology in urban scenario at 2.6GHz, PUSCH carrying eMBB service and RRACH need coverage enhancement if the target is defined by MPL with ISD = 500m.

- Rural at 4GHz & O2I

In Table 5, the evaluation results for rural 4GHz & O2I assuming PSD of 33 dBm/MHz are provided. In the evaluation, the following assumptions are used:

- Correction factors for antenna array gain: Using values in Table 1 above.

- Interference density:  Reusing IMT-2020 values.

- Penetration margins: Using values in Table 2 above for 4G rural case.

- Shadow fading margin:  Reusing IMT-2020 values.

- For other MPL specific values:  Reusing IMT-2020 values.

Table 5 Evaluation results for rural 4GHz & O2I

| Evaluation methodology | Item | PUSCH-eMBB | MSG3 | PRACH B4 | MSG4 ACK | PUCCH-2bits | PUCCH-11bits | PUCCH -22bits | PDCCH-unicast | PDCCH-broadcast | PBCH | PDSCH-eMBB | MSG4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Required SNR | -5.23 | -7.33 | -13.1 | -11.6 | -10.3 | -8.57 | -6.05 | -11.44 | -11.44 | -12.68 | -12.5 | -12.45 |
| LLS based methodology | MCL | 141.22 | 145.83 | 140.74 | 150.04 | 148.74 | 147.01 | 144.49 | 162.48 | 155.31 | 156.55 | 164.04 | 156.32 |
|  | MIL | 142.22 | 146.83 | 141.74 | 155.86 | 149.74 | 148.01 | 145.49 | 163.48 | 156.31 | 157.55 | 165.04 | 157.32 |
|  | MPL | 121.71 | 126.32 | 117.91 | 127.21 | 125.91 | 124.18 | 121.66 | 139.65 | 132.48 | 133.72 | 144.53 | 136.81 |
|  | - Target MPL with ISD=1732m | 131.57 | 131.57 | 131.57 | 131.57 | 131.57 | 131.57 | 131.57 | 131.57 | 131.57 | 131.57 | 131.57 | 131.57 |
| SLS based methodology | 5th percentile SNR via SLS | -11.12 | -10.41 | -17.24 | - | -14.49 | -12.88 | -10.61 | -3.62 | 0.05 | 0.05 | -3.62 | 0.05 |
|  | Performance gap via SLS | 5.89 | 3.08 | 4.14 | - | 4.19 | 4.31 | 4.56 | NAN | NAN | NAN | NAN | NAN |

For LLS based methodology, if the target is defined by MPL with ISD = 1732m, all UL channels need coverage enhancement. For service dependent targets, the target could reach to MCL=159.04 dB. In such case, only PDCCH unicast channel and PDSCH carrying eMBB do not need further enhancement. However, if a PSD of 24 dBm/MHz is assumed, all most all channels are required to be enhanced.

For SLS based methodology, basically we find all UL channels need further enhancement.

Observation 5: In rural O2I scenario at 4GHz,

  - for LLS based methodology, all UL channels need coverage enhancement if the target is defined by MPL with ISD = 1732m.

  - for SLS based methodology, all UL channels need coverage enhancement.

- Rural at 4GHz & O2O

In Table 6, the evaluation results for rural 4GHz & O2O assuming PSD of 33 dBm/MHz are provided. In the evaluation, the following assumptions are used:

- Correction factors for antenna array gain: Using values in Table 1 above.

- Interference density:  Reusing IMT-2020 values.

- Penetration margins: Using 9dB for O2O.

- Shadow fading margin:  Reusing IMT-2020 values.

- For other MPL specific values:  Reusing IMT-2020 values.

Table 6 Evaluation results for rural 4GHz & O2O

| Evaluation methodology | Item | PUSCH-eMBB | MSG3 | PRACH B4 | MSG4 ACK | PUCCH-2bits | PUCCH-11bits | PUCCH -22bits | PDCCH-unicast | PDCCH-broadcast | PBCH | PDSCH-eMBB | MSG4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Required SNR | -5.4 | -8.01 | -13.25 | -11.65 | -10.3 | -8.6 | -6.1 | -11.49 | -11.49 | -14.94 | -8.59 | -8.46 |
| LLS based methodology | MCL | 141.39 | 146.51 | 140.89 | 150.09 | 148.74 | 147.04 | 144.54 | 162.53 | 155.36 | 158.81 | 160.13 | 152.33 |
|  | MIL | 142.39 | 147.51 | 141.89 | 151.09 | 149.74 | 148.04 | 145.54 | 163.53 | 156.36 | 159.81 | 161.13 | 153.33 |
|  | MPL | 126.78 | 131.90 | 122.44 | 131.64 | 130.29 | 128.59 | 126.09 | 144.08 | 136.91 | 140.36 | 145.52 | 137.72 |
|  | - Target MPL with ISD=1732m | 131.57 | 131.57 | 131.57 | 131.57 | 131.57 | 131.57 | 131.57 | 131.57 | 131.57 | 131.57 | 131.57 | 131.57 |

For LLS based methodology, if the target is defined by MPL with ISD = 1732m, PUSCH carrying eMBB service, RRACH, PUCCH need coverage enhancement. For service dependent targets, the target could reach to MCL=159.04 dB. In such case, only PDCCH unicast channel does not need further enhancement.

Observation 6: For LLS based methodology in rural O2O scenario at 4GHz, PUSCH carrying eMBB service, RRACH, PUCCH need coverage enhancement if the target is defined by MPL with ISD = 1732m.

- Rural at 2.6GHz & O2I

In Table 7, the evaluation results for rural 2.6GHz & O2I assuming PSD of 33 dBm/MHz are provided. In the evaluation, the following assumptions are used:

- Correction factors for antenna array gain: Using values in Table 1 above.

- Interference density:  Reusing IMT-2020 values.

- Penetration margins: Using values in Table 2 above for 2.6GHz rural case.

- Shadow fading margin:  Using the modified values suggested in [4].

- For other MPL specific values:  Reusing IMT-2020 values.

Table 7 Evaluation results for rural 2.6GHz & O2I

| Evaluation methodology | Item | PUSCH-eMBB | MSG3 | PRACH B4 | MSG4 ACK | PUCCH-2bits | PUCCH-11bits | PUCCH -22bits | PDCCH-unicast | PDCCH-broadcast | PBCH | PDSCH-eMBB | MSG4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Required SNR | -4.2 | -7.06 | -13.07 | -11.76 | -10.2 | -8.62 | -6.14 | -11.64 | -11.64 | -12.99 | -12.84 | -12.81 |
| LLS based methodology | MCL | 140.19 | 145.56 | 140.71 | 150.20 | 148.64 | 147.06 | 144.58 | 162.68 | 155.51 | 156.86 | 164.38 | 156.68 |
|  | MIL | 141.19 | 146.56 | 141.71 | 151.20 | 149.64 | 148.06 | 145.58 | 163.68 | 156.51 | 157.86 | 165.38 | 157.68 |
|  | MPL | 121.3 | 126.67 | 118.50 | 127.99 | 126.57 | 124.83 | 122.31 | 140.47 | 133.30 | 134.65 | 145.49 | 137.79 |
|  | - Target MPL with ISD=1732m | 127.83 | 127.83 | 127.83 | 127.83 | 127.83 | 127.83 | 127.83 | 127.83 | 127.83 | 127.83 | 127.83 | 127.83 |
| SLS based methodology | 5th percentile SNR via SLS | -9.31 | -10.41 | -17.18 | - | -14.50 | -12.90 | -10.64 | -2.56 | 1.43 | 1.43 | -2.56 | 1.43 |
|  | Performance gap via SLS | 5.11 | 3.35 | 4.11 | - | 4.3 | 4.28 | 4.5 | NAN | NAN | NAN | NAN | NAN |

For LLS based methodology, if the target is defined by MPL with ISD = 1732m, all UL channels except for PUCCH for HARQ-ACK of Msg 4 need coverage enhancement. For service dependent targets, the target could reach to MCL=159.04 dB. In such case, only PDCCH unicast channl and PDSCH carrying eMBB do not need further enhancement.

For SLS based methodology, basically we find all UL channels need further enhancement (except for Msg4 HARQ-ACK which lacks of SLS results for now).

Observation 7: In rural O2I scenario at 2.6GHz,

  - for both LLS based and SLS based methodology, all UL channels except for PUCCH for Msg4 HARQ-ACK need coverage enhancement if the target is defined by MPL with ISD = 1732m.

- Rural at 2.6GHz & O2O

In Table 8, the evaluation results for rural 2.6GHz & O2O assuming PSD of 33 dBm/MHz are provided. In the evaluation, the following assumptions are used:

- Correction factors for antenna array gain: Using values in Table 1 above.

- Interference density:  Reusing IMT-2020 values.

- Penetration margins: Using 9dB for O2O.

- Shadow fading margin:  Using the modified values suggested in [4].

- For other MPL specific values:  Reusing IMT-2020 values.

Table 8 Evaluation results for rural 2.6GHz & O2O

| Evaluation methodology | Item | PUSCH-eMBB | MSG3 | PRACH B4 | MSG4 ACK | PUCCH-2bits | PUCCH-11bits | PUCCH -22bits | PDCCH-unicast | PDCCH-broadcast | PBCH | PDSCH-eMBB | MSG4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Required SNR | -4.63 | -7.68 | -13.26 | -11.67 | -10.3 | -8.6 | -6.08 | -11.61 | -11.61 | -15.95 | -8.63 | -8.49 |
| LLS based methodology | MCL | 140.62 | 146.18 | 140.90 | 150.11 | 148.74 | 147.04 | 144.52 | 162.65 | 155.48 | 159.82 | 160.17 | 152.36 |
|  | MIL | 141.62 | 147.18 | 141.90 | 151.11 | 149.74 | 148.04 | 145.52 | 163.65 | 156.48 | 160.82 | 161.17 | 153.36 |
|  | MPL | 126.01 | 131.57 | 122.45 | 131.66 | 130.29 | 128.59 | 126.07 | 144.20 | 137.03 | 141.37 | 145.56 | 137.75 |
|  | - Target MPL with ISD=1732m | 127.83 | 127.83 | 127.83 | 127.83 | 127.83 | 127.83 | 127.83 | 127.83 | 127.83 | 127.83 | 127.83 | 127.83 |

For LLS based methodology, if the target is defined by MPL with ISD = 1732m, PUSCH carrying eMBB service, RRACH, PUCCH carrying 11 or 22 bits need coverage enhancement. For service dependent targets, the target could reach to MCL=159.04 dB. In such case, broadcast PDCCH and Msg4 also need further enhancement.

Observation 8: For LLS based methodology in rural O2O scenario at 2.6GHz, PUSCH carrying eMBB service, RRACH, PUCCH  carrying 11 or 22 bits need coverage enhancement if the target is defined by MPL with ISD = 1732m.

- Rural at 2GHz & O2I

In Table 9, the evaluation results for rural 2GHz & O2I assuming PSD of 36 dBm/MHz are provided. In the evaluation, the following assumptions are used:

- Correction factors for antenna array gain: Using values in Table 1 above.

- Interference density:  Reusing IMT-2020 values.

- Penetration margins: Using values in Table 2 above for 2GHz rural case.

- Shadow fading margin:  Using the modified values suggested in [4].

- For other MPL specific values:  Reusing IMT-2020 values.

Table 9 Evaluation results for rural 2GHz & O2I

| Evaluation methodology | Item | PUSCH-eMBB | PUSCH -VoIP | PUSCH-SIP | MSG3 | PRACH B4 | MSG4 ACK | PUCCH-2bits | PUCCH-11bits | PUCCH -22bits | PDCCH-unicast | PDCCH-broadcast | PBCH | PDSCH-eMBB | MSG4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Required SNR | -3.11 | -7.68 | -4.4 | -3.04 | -11.52 | -7.53 | -6.1 | -4.1 | -1.27 | -8.43 | -8.43 | -7.47 | -8.66 | -8.56 |
| LLS based methodology | MCL | 130.07 | 134.14 | 130.86 | 132.51 | 130.13 | 136.94 | 135.51 | 133.51 | 130.68 | 147.42 | 147.42 | 146.46 | 148.15 | 147.55 |
|  | MIL | 142.13 | 146.20 | 142.92 | 138.76 | 136.38 | 143.19 | 147.57 | 145.57 | 142.74 | 159.48 | 153.67 | 152.71 | 160.21 | 153.80 |
|  | MPL | 122.67 | 126.74 | 123.46 | 119.30 | 113.60 | 120.41 | 124.79 | 122.79 | 119.96 | 136.70 | 130.89 | 129.93 | 140.75 | 134.34 |
|  | - Target MPL with ISD=1732m | 125.55 | 125.55 | 125.55 | 125.55 | 125.55 | 125.55 | 125.55 | 125.55 | 125.55 | 125.55 | 125.55 | 125.55 | 125.55 | 125.55 |

For LLS based methodology, if the target is defined by MPL with ISD = 1732m, all UL channels except for PUSCH carrying VoIP need coverage enhancement. For service dependent targets, the target could reach to MCL=159.04 dB. In such case, all channels need further enhancement.

Observation 9: For LLS based methodology in rural O2O scenario at 2.6GHz, all UL channels except for PUSCH carrying VoIP need coverage enhancement if the target is defined by MPL with ISD = 1732m.

- Rural at 2GHz & O2O

In Table 10, the evaluation results for rural 2GHz & O2O assuming PSD of 36 dBm/MHz are provided. In the evaluation, the following assumptions are used:

- Correction factors for antenna array gain: Using values in Table 1 above.

- Interference density:  Reusing IMT-2020 values.

- Penetration margins: Using 9dB for O2O.

- Shadow fading margin:  Using the modified values suggested in [4].

- For other MPL specific values:  Reusing IMT-2020 values.

Table 10 Evaluation results for rural 2GHz & O2O

| Evaluation methodology | Item | PUSCH-eMBB | PUSCH -VoIP | PUSCH-SIP | MSG3 | PRACH B4 | MSG4 ACK | PUCCH-2bits | PUCCH-11bits | PUCCH -22bits | PDCCH-unicast | PDCCH-broadcast | PBCH | PDSCH-eMBB | MSG4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Required SNR | -3.46 | -10.79 | -6.74 | -3.16 | -10.84 | -7.52 | -6.1 | -4.07 | -1.58 | -8.41 | -8.41 | -10.04 | -5.48 | -5.35 |
| LLS based methodology | MCL | 130.42 | 137.25 | 133.20 | 132.63 | 129.45 | 136.93 | 135.51 | 133.48 | 130.99 | 147.40 | 147.40 | 149.03 | 144.97 | 144.34 |
|  | MIL | 142.48 | 149.31 | 145.26 | 138.88 | 135.70 | 143.18 | 147.57 | 145.54 | 143.05 | 159.46 | 153.65 | 155.28 | 157.03 | 150.59 |
|  | MPL | 126.87 | 133.70 | 129.65 | 123.27 | 116.25 | 123.73 | 128.12 | 126.09 | 123.60 | 140.01 | 134.20 | 135.83 | 141.42 | 134.98 |
|  | - Target MPL with ISD=1732m | 125.55 | 125.55 | 125.55 | 125.55 | 125.55 | 125.55 | 125.55 | 125.55 | 125.55 | 125.55 | 125.55 | 125.55 | 125.55 | 125.55 |

For LLS based methodology, if the target is defined by MPL with ISD = 1732m, Msg3, PRACH, PUCCH carrying Msg4 HARQ-ACK and PUCCH carrying more than 22 bits need coverage enhancement. For service dependent targets, the target could reach to MCL=159.04 dB. In such case, all channels need further enhancement.

Observation 10: For LLS based methodology in rural O2I scenario at 2GHz, Msg3, PRACH, PUCCH carrying Msg4 HARQ-ACK and PUCCH carrying more than 22 bits need coverage enhancement if the target is defined by MPL with ISD = 1732m.

- Rural at 700MHz & O2O

In Table 11, the evaluation results for rural 700MHz & O2O assuming PSD of 36 dBm/MHz are provided. In the evaluation, the following assumptions are used:

- Correction factors for antenna array gain: Using values in Table 1 above.

- Interference density:  Reusing IMT-2020 values.

- Penetration margins: Using 9dB for O2O.

- Shadow fading margin:  Reusing IMT-2020 values.

- For other MPL specific values:  Reusing IMT-2020 values.

Table 11 Evaluation results for rural 700MHz & O2O

| Evaluation methodology | Item | PUSCH-eMBB | PUSCH -VoIP | PUSCH-SIP | MSG3 | PRACH B4 | MSG4 ACK | PUCCH-2bits | PUCCH-11bits | PUCCH -22bits | PDCCH-unicast | PDCCH-broadcast | PBCH | PDSCH-eMBB | MSG4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Required SNR | -3.26 | -11.68 | -7.81 | -3.47 | -10.93 | -7.49 | -6.02 | -3.97 | -1.26 | -7.43 | -7.43 | -9.41 | -5.87 | -5.75 |
| LLS based methodology | MCL | 130.22 | 138.14 | 134.27 | 132.94 | 129.54 | 136.90 | 135.43 | 133.38 | 130.67 | 146.42 | 146.42 | 148.40 | 145.36 | 144.74 |
|  | MIL | 139.39 | 147.31 | 146.33 | 138.42 | 135.02 | 142.38 | 144.60 | 142.55 | 139.84 | 155.59 | 151.90 | 153.88 | 154.53 | 150.22 |
|  | MPL | 123.78 | 131.70 | 130.72 | 122.81 | 115.57 | 122.93 | 125.15 | 123.10 | 120.39 | 136.14 | 132.45 | 134.43 | 138.92 | 134.61 |
|  | - Target MPL with ISD=1732m | 116.44 | 116.44 | 116.44 | 116.44 | 116.44 | 116.44 | 116.44 | 116.44 | 116.44 | 116.44 | 116.44 | 116.44 | 116.44 | 116.44 |
| SLS based methodology | 5th percentile SNR via SLS | 0.12 | -3.74 | - | -0.24 | -4.61 | - | -1.56 | -0.42 | 0.03 | 3.45 | -0.09 | -0.09 | 3.45 | -0.09 |
|  | Performance gap via SLS | NAN | NAN | - | NAN | NAN | - | NAN | NAN | NAN | NAN | NAN | NAN | NAN | NAN |

For LLS based methodology, if the target is defined by MPL with ISD = 1732m, only PRACH need coverage enhancement. For SLS based methodology, we find all channels can reach the target and no enhancement is needed.

Observation 11: In rural O2O scenario at 700MHz,

  - for LLS based methodology, only PRACH needs coverage enhancement if the target is defined by MPL with ISD = 1732m.

  - for SLS based methodology, no channels need coverage enhancement.

- Rural at 700MHz & O2O with long distance

In Table 12, the evaluation results for rural 700MHz & O2O with long distance assuming PSD of 36 dBm/MHz are provided. In the evaluation, the following assumptions are used:

- Correction factors for antenna array gain: Using values in Table 1 above.

- Interference density:  Reusing IMT-2020 values.

- Penetration margins: Using 9dB for O2O.

- Shadow fading margin:  Reusing IMT-2020 values.

- For other MPL specific values:  Reusing IMT-2020 values.

Table 12 Evaluation results for rural 700MHz & O2O with long distance

| Evaluation methodology | Item | PUSCH-eMBB | PUSCH -VoIP | MSG3 | PRACH B4 | MSG4 ACK | PUCCH-2bits | PUCCH-11bits | PUCCH -22bits | PDCCH-unicast | PDCCH-broadcast | PBCH | PDSCH-eMBB | MSG4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Required SNR | -6.18 | -12.05 | -6.4 | -13.7 | -10.65 | -8.45 | -7.37 | -4.93 | -7.68 | -7.68 | -11.6 | -5.85 | -5.76 |
| LLS based methodology | MCL | 133.14 | 138.51 | 135.87 | 132.31 | 140.06 | 137.86 | 136.78 | 134.34 | 146.67 | 146.67 | 150.59 | 145.34 | 144.75 |
|  | MIL | 142.31 | 147.68 | 141.35 | 137.79 | 149.23 | 147.03 | 145.95 | 143.51 | 155.84 | 152.15 | 156.07 | 154.51 | 150.23 |
|  | MPL | 128.52 | 133.89 | 127.56 | 122.79 | 134.23 | 132.03 | 130.95 | 128.51 | 140.84 | 137.15 | 141.07 | 140.72 | 136.44 |
|  | - Target MPL with ISD=12000 | 127.00 | 127.00 | 127.00 | 127.00 | 127.00 | 127.00 | 127.00 | 127.00 | 127.00 | 127.00 | 127.00 | 127.00 | 127.00 |
| SLS based methodology | 5th percentile SNR via SLS | -1.3 | -0.35 | -3.12 | -8.79 | - | -2.9 | -2.29 | -1.26 | 1.74 | -1 | -1 | 1.74 | -1 |
|  | Performance gap via SLS | NAN | NAN | NAN | NAN | - | NAN | NAN | NAN | NAN | NAN | NAN | NAN | NAN |

For LLS based methodology, if the target is defined by MPL with ISD = 12km, only PRACH need coverage enhancement. For SLS based methodology, we find all channels can reach the target and no enhancement is needed.

Observation 12: In rural O2O long distance scenario at 700MHz,

  - for LLS based methodology, only PRACH needs coverage enhancement if the target is defined by MPL with ISD = 12km.

  - for SLS based methodology, no channels need coverage enhancement.

- Summary of evaluation results

Based on above analysis on evaluation results for different scenarios, we can find all UL channels including PUSCH, PUCCH, PRACH, Msg3, PUCCH carrying Msg HARQ-ACK need further coverage enhancement for most of the scenarios at 2GHz/2.6GHz/4GHz based on relative target MPL. Broadcast PDCCH also has coverage problem in rural or urban scenario at 4GHz in case of 24 dBm/MHz DL PSD.

As a result, we have the following proposal.

Proposal 5: Study enhancement techniques for UL channels including PUSCH, PUCCH, PRACH, Msg3, PUCCH carrying Msg4 HARQ-ACK and broadcast PDCCH in NR coverage enhancement SI.

Conclusion

According to the analysis given above, we have the following observations and proposals:

Observation 1: For rural scenario at 700MHz, Δ2 is 3.86 dB for DL/UL channels in RRC-connected mode, and Δ2 is 7.55 dB for DL/UL channels in RRC-idle mode.

Observation 2: For urban scenario at 4GHz, Δ2 is 2.95 dB for DL channels in RRC-connected mode and all UL channels, and Δ1+Δ2 is 13.38 dB for DL channels in RRC-idle mode.

Observation 3: In urban scenario at 4GHz,

  - for LLS based methodology, all UL channels and PDCCH broadcast channels need coverage enhancement if the target is defined by MPL with ISD = 500m.

  - for SLS based methodology, all UL channels need coverage enhancement.

Observation 4: For LLS based methodology in urban scenario at 2.6GHz, PUSCH carrying eMBB service and RRACH need coverage enhancement if the target is defined by MPL with ISD = 500m.

Observation 5: In rural O2I scenario at 4GHz,

  - for LLS based methodology, all UL channels need coverage enhancement if the target is defined by MPL with ISD = 1732m.

  - for SLS based methodology, all UL channels need coverage enhancement.

Observation 6: For LLS based methodology in rural O2O scenario at 4GHz, PUSCH carrying eMBB service, RRACH, PUCCH need coverage enhancement if the target is defined by MPL with ISD = 1732m.

Observation 7: In rural O2I scenario at 2.6GHz,

  - for both LLS based and SLS based methodology, all UL channels except for PUCCH for Msg4 HARQ-ACK need coverage enhancement if the target is defined by MPL with ISD = 1732m.

Observation 8: For LLS based methodology in rural O2O scenario at 2.6GHz, PUSCH carrying eMBB service, RRACH, PUCCH  carrying 11 or 22 bits need coverage enhancement if the target is defined by MPL with ISD = 1732m.

Observation 9: For LLS based methodology in rural O2O scenario at 2.6GHz, all UL channels except for PUSCH carrying VoIP need coverage enhancement if the target is defined by MPL with ISD = 1732m.

Observation 10: For LLS based methodology in rural O2I scenario at 2GHz, Msg3, PRACH, PUCCH carrying Msg4 HARQ-ACK and PUCCH carrying more than 22 bits need coverage enhancement if the target is defined by MPL with ISD = 1732m.

Observation 11: In rural O2O scenario at 700MHz,

  - for LLS based methodology, only PRACH needs coverage enhancement if the target is defined by MPL with ISD = 1732m.

  - for SLS based methodology, no channels need coverage enhancement.

Observation 12: In rural O2O long distance scenario at 700MHz,

  - for LLS based methodology, only PRACH needs coverage enhancement if the target is defined by MPL with ISD = 12km.

  - for SLS based methodology, no channels need coverage enhancement.

Proposal 1: Support additional DMRS (i.e. 4 DMRS symbols for 14-OS PUCCH) for link level simulation for PUCCH Format 3 for FR1.

- Proposal 2: For link level simulation of PUCCH format 1 with 1-bit HARQ-ACK for Msg4 in FR1, reuse the simulation assumptions of normal PUCCH with assuming no PUCCH repetition.

- Proposal 3: Consider the antenna gain correction factors in Table 1 for link budget calculation in FR1.

Proposal 4: Consider the penetration margins for different O2I cases in Table 2 for link budget calculation in FR1.

Proposal 5: Study enhancement techniques for UL channels including PUSCH, PUCCH, PRACH, Msg3, PUCCH carrying Msg4 HARQ-ACK and broadcast PDCCH in NR coverage enhancement SI.

Reference

- 3GPP RAN#88-e, RP-200861, “Revised SID on NR coverage enhancement”, China Telecom.

- 3GPP, RAN1#101-e, Chairman’s notes.

- 3GPP, RAN1#102-e, Chairman’s notes.

- 3GPP, RAN1#102-e, R1-2007479, [102-e-Post-NR-CovEnh-01] Summary on email discussion/approval of remaining simulation assumptions, Moderator (SoftBank, Nokia).

- 3GPP, RAN1#102-e, R1-2007480, [102-e-Post-NR-CovEnh-02] Summary on email discussion/approval of link budget template, Moderator (SoftBank, Nokia).

- 3GPP, RAN1#102-e, R1-2005425, Discussion on baseline coverage performance for FR1, ZTE.

- 3GPP, RAN1#103-e, R1-2007746, Discussion on target performance for NR coverage enhancements, ZTE.

- Appendix-A LLS assumptions

- A.1 LLS assumptions and results for PUSCH

Table A.1-1 Simulation assumption for baseline performance of PUSCH for FR1

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenario | Urban , O2I | Rural, O2O or , O2I | Rural with long distance, O2O |
| Duplexing scheme and frequency | 2.6GHz/4GHz TDD | 2.6GHz/4GHz TDD 700MHz /2GHz FDD | 700MHz FDD |
| Frame structure for TDD | DDDSUDDSUU (S: 10D:2G:2U) for 4GHz DDDDDDDSUU (S: 6D:4G:4U) for 2.6GHz |  |  |
| Transmission bit rate for data channel (bit/s) | 1 Mbps for eMBB 12.2 kbps for VoIP | 100 kbps for eMBB 12.2 kbps for VoIP | 100 kbps for eMBB 12.2 kbps for VoIP |
| Subcarrier spacing | 30kHz | 30kHz for TDD and 15kHz for FDD | 15kHz |
| Channel model for link-level simulation | TDL-C | TDL-C | TDL-D |
| Pathloss model (select from LoS or NLoS) | NLos | NLos | Los |
| Delay Spread | 300ns | 300ns | 30ns |
| UE velocity | 3 km/h | 120 km/h for O2O 3 km/h for O2I | 120 km/h |
| Number of RF chains for BS | 4 | 4 for 2.6GHz/4GHz and 2 for 700MHz /2GHz | 2 |
| Number of UE antennas | 2 | 1 | 1 |
| Number of RF chains for UE | 2 with PMI adaption | 1 | 1 |
| Number of occupied RBs | 30 | 4 | 4 |
| MCS | MCS4 for 2.6GHz MCS3 for 4GHz | MCS0 for 700MHz MCS0 for 2GHz MCS3 for 2.6GHz MCS2 for 4GHz | MCS0 |
| TBS for VoIP and SIP invite messsage | 320bits for VoIP 560bits for SIP invite message |  |  |
| DMRS overhead | - For Urban scenario : Type I, one DMRS symbol, no multiplexing with data. - For other case: Type I, 2 DMRS symbol (one front- loaded and one additional), no multiplexing data |  |  |
| Frequency hopping | - For Rural and Rural with long distance scenario : w/ frequency hopping - For Urban scenario : w/o frequency hopping |  |  |
| Channel estimation | Practical |  |  |
| Receiver type | MMSE |  |  |
| BLER Target | 10% iBLER for eMBB 2%rBLER for VoIP |  |  |
| Waveform | DFT-s-OFDM |  |  |
| Number of repetitions and re-transmissions | 3 repetitions and maximum 4 (re)-transmissions for VoIP for TDD@4GHz 2 repetitions and maximum 4 (re)-transmissions for VoIP for TDD@2.6GHz 8 repetitions and maximum 4 (re)-transmissions for VoIP for FDD 8 repetitions and maximum 1 (re)-transmissions for SIP invite message |  |  |

- A.2 LLS assumptions and results for MSG3

Table A.2-1 Simulation assumption for baseline performance of Msg3 for FR1

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenario | Urban , O2I | Rural, O2O or , O2I | Rural with long distance, O2O |
| Duplexing scheme and frequency | 2.6GHz/4GHz TDD | 2.6GHz/4GHz TDD 700MHz /2GHz FDD | 700MHz FDD |
| Frame structure for TDD | DDDSUDDSUU (S: 10D:2G:2U) for 4GHz DDDDDDDSUU (S: 6D:4G:4U) for 2.6GHz |  |  |
| TB size | 56bits |  |  |
| Subcarrier spacing | 30kHz | 15kHz | 15kHz |
| Channel model for link-level simulation | TDL-C | TDL-C | TDL-D |
| Pathloss model (select from LoS or NLoS) | NLos | NLos | Los |
| Delay Spread | 300ns | 300ns | 30ns |
| UE velocity | 3 km/h | 120 km/h for O2O 3 km/h for O2I | 120 km/h |
| Number of RF chains for BS | 4 | 4 for 2.6GHz/4GHz and 2 for 700MHz /2GHz | 2 |
| Number of UE antennas | 2 | 1 | 1 |
| Number of RF chains for UE | 1 | 1 | 1 |
| Number of occupied RBs | 2 | 2 | 2 |
| DMRS overhead | Type I, 3 DMRS symbol (one front- loaded and one additional), no multiplexing data |  |  |
| Frequency hopping | w/o frequency hopping |  |  |
| Channel estimation | Practical |  |  |
| Receiver type | MMSE |  |  |
| BLER Target | 10% iBLER |  |  |
| Waveform | DFT-s-OFDM |  |  |

- A.3 LLS assumptions and results for PUCCH

Table A.3-1 Simulation assumption for baseline performance of PUCCH for FR1

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenario | Urban | Rural | Rural with long distance |
| Duplexing scheme and frequency | 2.6GHz/4GHz TDD | 2.6G/4GHz TDD 700MHz/2GHz FDD | 700MHz FDD |
| Subcarrier spacing | 30kHz | 30kHz for TDD and 15kHz for FDD | 15kHz |
| Channel model for link-level simulation | TDL-C | TDL-C | TDL-D |
| Pathloss model (select from LoS or NLoS) | NLos | NLos | Los |
| Delay Spread | 300ns | 300ns | 30ns |
| UE velocity | 3 km/h | 120 km/h for O2O 3 km/h for O2I | 120 km/h |
| Number of RF chains for UE | 1 |  |  |
| Number of RF chains for BS | 4 for 2.6GHz/4GHz and 2 for 700MHz /2GHz |  |  |
| Format type/UCI payload | Format 1: 2bits or 1-bit for HARQ-ACK for Msg4 Format 3: 11bits, 22bits. |  |  |
| Number of occupied RBs | 1 PRB |  |  |
| PUCCH duration | 14OS |  |  |
| DMRS overhead | Additional DMRS for PUCCH format 3 |  |  |
| Channel estimation | Practical |  |  |
| Receiver type | MMSE |  |  |
| BLER Target | DTX to ACK probability: 1% ; NACK to ACK probability: 0.1%; ACK missed detection probability: 1% CSI block error probability: 1% |  |  |

- A.4 LLS assumptions and results for PDCCH

Table A.4-1 Simulation assumption for baseline performance of PDCCH for FR1

| Parameter | Value |  |  |
| --- | --- | --- | --- |
| Scenario | Urban | Rural | Rural with long distance |
| Duplexing scheme and frequency | 2.6GHz/4GHz TDD | 2.6G/4GHz TDD 700MHz/2GHz FDD | 700MHz FDD |
| Subcarrier spacing | 30kHz | 30kHz for TDD and 15kHz for FDD | 15kHz |
| Channel model for link-level simulation | TDL-C | TDL-C | TDL-D |
| Pathloss model (select from LoS or NLoS) | NLos | NLos | Los |
| Delay Spread | 300ns | 300ns | 30ns |
| UE velocity | 3 km/h | 120 km/h for O2O 3 km/h for O2I | 120 km/h |
| Number of RF chains for UE | 4 for 2.6GHz/4GHz and 2 for 700MHz /2GHz |  |  |
| Number of RF chains for BS | 4 for 2.6GHz/4GHz and 2 for 700MHz /2GHz |  |  |
| Number of symbols for CORESET | 2 |  |  |
| Transmission type | Interleaved(R=3 for 3OS,others,R=2) |  |  |
| DCI payload (excluding 24bits CRC) | 40 bits |  |  |
| Number of occupied RBs | 48 |  |  |
| REG bundling size | 6 |  |  |
| Modulation | QPSK |  |  |
| Channel coding | Polar code (DCI) |  |  |
| Transmission scheme | 1-port precoder cycling |  |  |
| Channel estimation | Practical |  |  |
| Receiver type | MMSE |  |  |
| BLER target | 1% |  |  |

- A.5 LLS assumptions and results for PBCH

Table A.5-1 Simulation assumption for baseline performance of PBCH for FR1

| Parameter | Value |  |  |
| --- | --- | --- | --- |
| Scenario | Urban | Rural | Rural with long distance |
| Duplexing scheme and frequency | 2.6GHz/4GHz TDD | 2.6G/4GHz TDD 700MHz/2GHz FDD | 700MHz FDD |
| Subcarrier spacing | 30kHz | 30kHz for TDD and 15kHz for FDD | 15kHz |
| Channel model for link-level simulation | TDL-C | TDL-C | TDL-D |
| Pathloss model (select from LoS or NLoS) | NLos | NLos | Los |
| Delay Spread | 300ns | 300ns | 30ns |
| UE velocity | 3 km/h | 120 km/h for O2O 3 km/h for O2I | 120 km/h |
| PBCH payload (excluding 24bits CRC) | 32 bits |  |  |
| Number of symbols for CORESET | 2 |  |  |
| Modulation | QPSK |  |  |
| Channel coding | Polar code (DCI) |  |  |
| Transmission scheme | 1-port precoder cycling |  |  |
| Channel model | TDL-C (delay spread: 300ns) |  |  |
| Channel estimation | Practical |  |  |
| Receiver type | MMSE |  |  |
| BLER target | 1% |  |  |
| Number of RF chains for UE | 4 for 2.6GHz/4GHz and 2 for 700MHz /2GHz |  |  |
| Number of RF chains for BS | 4 for 2.6GHz/4GHz and 2 for 700MHz /2GHz |  |  |
| Number of interfering TRPs | 0 TRP |  |  |
| Combined number | 4 SSBs |  |  |
| Frequency Offset | Initial acquisition TRP: uniform distribution +/- 0.05 ppm UE: uniform distribution +/- 5, 10, 20 ppm (each company to choose one) Non-initial acquisition TRP: uniform distribution +/- 0.05 ppm UE: uniform distribution +/- 0.1 ppm |  |  |

- A.6 LLS assumptions and results for PRACH

Table A.6-1 Simulation assumption for baseline performance of PRACH for FR1

| Parameter | Value |  |  |
| --- | --- | --- | --- |
| Scenario | Urban | Rural | Rural with long distance |
| Duplexing scheme and frequency | 2.6GHz/4GHz TDD | 2.6G/4GHz TDD 700MHz/2GHz FDD | 700MHz FDD |
| Subcarrier spacing | 30kHz | 30kHz for TDD and 15kHz for FDD | 15kHz |
| Channel model for link-level simulation | TDL-C | TDL-C | TDL-D |
| Pathloss model (select from LoS or NLoS) | NLos | NLos | Los |
| Delay Spread | 300ns | 300ns | 30ns |
| UE velocity | 3 km/h | 120 km/h for O2O 3 km/h for O2I | 120 km/h |
| Number of occupied RBs | NR PRACH preamble format 0 : 6 RBs NR PRACH preamble format 4B : 12 RBs |  |  |
| Number of RF chains for UE | 1 |  |  |
| Number of RF chains for BS | 4 for 2.6GHz/4GHz and 2 for 700MHz /2GHz |  |  |
| Miss-detection target | 1% |  |  |
| Preamble | NR PRACH preamble format B4 with Ncs = 13, Logical sequence index = 1, v=0 for O2O and O2I |  |  |

- A.7 LLS assumptions and results for PDSCH/Msg 4

Table A.7-1 Simulation assumption for baseline performance of PDSCH for FR1

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenario | Urban | Rural | Rural with long distance |
| Duplexing scheme and frequency | 2.6GHz/4GHz TDD | 2.6G/4GHz TDD 700MHz/2GHz FDD | 700MHz FDD |
| Frame structure for TDD | DDDSUDDSUU (S: 10D:2G:2U) |  |  |
| Transmission bit rate for data channel (bit/s) | 10 Mbps | 1 Mbps | 1 Mbps |
| Subcarrier spacing | 30kHz | 30kHz for TDD and 15kHz for FDD | 15kHz |
| Channel model for link-level simulation | TDL-C | TDL-C | TDL-D |
| Pathloss model (select from LoS or NLoS) | NLos | NLos | Los |
| Delay Spread | 300ns | 300ns | 30ns |
| UE velocity | 3 km/h | 120 km/h for O2O 3 km/h for O2I | 120 km/h |
| Number of RF chains for UE | 4 for 2.6GHz/4GHz and 2 for 700MHz /2GHz |  |  |
| Number of RF chains for BS | 4 for 2.6GHz/4GHz and 2 for 700MHz /2GHz |  |  |
| Payload for MSG4 | 1000 bits |  |  |
| DMRS configuration | 3 DMRS symbols for O2O case 2 DMRS symbols for O2I case |  |  |
| Number of occupied RBs/MCS | 246 RBs with MCS#0 for PDSCH in urban scenario |  |  |
| PMI adaption | ideal |  |  |
| Channel estimation | Practical |  |  |
| Receiver type | 2D-MMSE |  |  |
| BLER Target | 10% |  |  |
| Duration | 12 OS |  |  |

- Appendix-B SLS assumptions

The simulation assumptions for SLS can reuse that of defined for LLS as above. In the following, SLS specific simulation assumptions are provided.

Table B-1 Simulation assumptions for system-level simulation for FR1

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenario | Urban , O2I | Rural, O2O or , O2I | Rural with long distance, O2O |
| Layout | Single layer - Macro layer: Hex. Grid |  |  |
| Channel model | UMa in TR 38.901 | RMa in TR 38.901 |  |
| Number of BS antennas & antenna configuration | 192 for 2.6G and 4G: (M,N,P,Mg,Ng;Mp,Np) = (12,8,2,1,1;1,1) | 64 for 2.6G and 4G: (M,N,P,Mg,Ng;Mp,Np) = (8,4,2,1,1;1,1); 32 for 2G: (M,N,P,Mg,Ng;Mp,Np) = (8,2,2,1,1;1,1); 16 for 700M: (M,N,P,Mg,Ng;Mp,Np) = (4,2,2,1,1;1,1); | 16 for 700M: (M,N,P,Mg,Ng;Mp,Np) = (4,2,2,1,1;1,1) |
| Min distance of UE2gNB | 35m |  |  |