3GPP TSG RAN WG1 Meeting #103-e                                       R1-2007754

e-Meeting, October 26th – November 13th, 2020

Source:         ZTE

Title:            Evaluation of achievable accuracy and latency

Agenda item:    8.5.2

Document for:	Discussion and Decision

Introduction

In 3GPP TSG RAN Meeting #86, a new study item was agreed [1]. The study item is intended to firstly study achievable performance based on Rel-16 positioning methods for different scenarios, especially for accuracy and latency. After that, we can find the performance gap to fulfill Rel-17 positioning requirements. Therefore, some evaluation assumptions have been approved in last two meetings [2][3].

In this contribution, we provide some evaluation results of positioning accuracy and latency according to agreed evaluation assumptions.

Evaluation results of positioning accuracy

In this section, we provide some simulation results on InF-SH and InF-DH scenarios. All detailed configurations and algorithms are listed in appendix titled Table 13 and Table 14. In addition, Table 15 in the appendix reveals some controlled variables of all simulation cases.

  - Horizontal positioning accuracy based on Rel-16 DL-TDOA method

Firstly, the horizontal positioning errors of InF-SH scenario for FR1 and FR2 at some specific percentiles are shown in Table 1 and Table 2 respectively, and the corresponding CDF curves are attached in appendix in Figure 1-1 and Figure 1-2.

Table 1 CDF of horizontal positioning accuracy for InF-SH, FR1 scenario

| Simulation cases | UE horizontal drop procedure | gNB Tx calibration error | Horizontal positioning accuracy (m) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 67% | 80% | 90% |
| Case 1 | Uniformly distributed | 0 ns | 0.282 | 0.370 | 0.467 | 0.603 |
| Case 2 | Inside convex hull | 0 ns | 0.264 | 0.350 | 0.469 | 0.568 |
| Case 3 | Inside convex hull | 0.5 ns | 0.301 | 0.411 | 0.549 | 0.704 |
| Case 4 | Inside convex hull | 1 ns | 0.475 | 0.616 | 0.767 | 0.943 |
| Case 5 | Inside convex hull | 2 ns | 0.841 | 1.042 | 1.239 | 1.479 |

Table 2 CDF of horizontal positioning accuracy for InF-SH, FR2 scenario

| Simulation cases | UE horizontal drop procedure | gNB Tx calibration error | Horizontal positioning accuracy (m) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 67% | 80% | 90% |
| Case 6 | Uniformly distributed | 0 ns | 0.044 | 0.059 | 0.072 | 0.092 |
| Case 7 | Inside convex hull | 0 ns | 0.041 | 0.055 | 0.070 | 0.090 |
| Case 8 | Inside convex hull | 0.5 ns | 0.171 | 0.223 | 0.257 | 0.300 |
| Case 9 | Inside convex hull | 1 ns | 0.419 | 0.480 | 0.545 | 0.615 |
| Case 10 | Inside convex hull | 2 ns | 0.833 | 0.971 | 1.086 | 1.224 |

Secondly, the horizontal positioning errors of InF-DH scenario for FR1 and FR2 at some specific percentiles are shown in Table 3 and Table 4 respectively, and the corresponding CDF curves are attached in appendix in Figure 1-3 and Figure 1-4.

Table 3 CDF of horizontal positioning accuracy for InF-DH, FR1 scenario

| Simulation cases | UE horizontal drop procedure | gNB Tx calibration error | Horizontal positioning accuracy (m) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 67% | 80% | 90% |
| Case 11 | Uniformly distributed | 0 ns | 0.552 | 5.708 | 10.120 | 12.433 |
| Case 12 | Inside convex hull | 0 ns | 0.414 | 1.060 | 7.337 | 12.345 |
| Case13 | Inside convex hull | 0.5 ns | 0.885 | 1.719 | 7.544 | 12.386 |
| Case 14 | Inside convex hull | 1 ns | 1.242 | 3.560 | 8.249 | 12.368 |
| Case 15 | Inside convex hull | 2 ns | 1.632 | 4.127 | 7.701 | 12.458 |

Table 4 CDF of horizontal positioning accuracy for InF-DH, FR2 scenario

| Simulation cases | UE horizontal drop procedure | gNB Tx calibration error | Horizontal positioning accuracy (m) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 67% | 80% | 90% |
| Case 16 | Uniformly distributed | 0 ns | 0.076 | 0.150 | 10.255 | 14.759 |
| Case 17 | Inside convex hull | 0 ns | 0.062 | 0.097 | 0.174 | 12.174 |
| Case 18 | Inside convex hull | 0.5 ns | 0.323 | 0.458 | 0.665 | 10.815 |
| Case 19 | Inside convex hull | 1 ns | 0.542 | 0.738 | 1.045 | 12.285 |
| Case 20 | Inside convex hull | 2 ns | 0.958 | 1.220 | 11.075 | 14.845 |

Observation 1: For horizontal positioning accuracy of InF-SH scenario based on Rel-16 DL-TDOA method,

- The positioning accuracy of UE inside convex hull is slightly better than the case where all UEs are uniformly distributed over the factory, because the high quality links for positioning are always enough under high LOS probability scenario.

- When there is no network synchronization error and gNB Tx calibration error, the horizontal positioning accuracy of 90% UEs inside convex hull are less than 0.568 m in FR1, while the value is 0.090 m in FR2.

- The sub-meter level horizontal positioning accuracy at the percentile of 90% UEs will be fulfilled, if gNB Tx calibration error is larger than 1 ns in FR1 and 2 ns in FR2.

Observation 2: For horizontal positioning accuracy of InF-DH scenario based on Rel-16 DL-TDOA method,

- The positioning accuracy of UE inside convex hull is much better than the case where all UEs are uniformly distributed over the factory, because the high quality links for positioning may not be enough under low LOS probability scenario.

- When there is no network synchronization error and gNB Tx calibration error, sub-meter level horizontal positioning accuracy will be fulfilled for UEs inside convex hull at the percentile of 67% in FR1 and 80% in FR2.

- gNB Tx calibration error significantly degrades the positioning performance, e.g. 1ns in FR1 and 2 ns in FR2.

  - Vertical positioning accuracy based on Rel-16 DL-TDOA method

Thirdly, the vertical positioning errors at some specific percentiles are shown in Table 5, and the corresponding CDF curves are attached in appendix in Figure 2-1 and Figure 2-2. It needs to be clarified that estimated vertical location of all UEs for InF-SH and InF-DH are restricted to 0.5-2 m and 0.5-3 m respectively.

Table 5 CDF of vertical positioning accuracy

| Simulation cases | FR1/FR2 | scenario | Vertical positioning accuracy (m) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 67% | 80% | 90% |
| Case 21 | FR1 | InF-SH | 0.411 | 0.623 | 0.815 | 0.979 |
| Case 22 | FR2 | InF-SH | 0.185 | 0.293 | 0.362 | 0.459 |
| Case 23 | FR1 | InF-DH | 0.522 | 0.814 | 1.062 | 1.419 |
| Case 24 | FR2 | InF-DH | 0.222 | 0.414 | 0.705 | 1.271 |

Observation 3: For vertical positioning accuracy based on Rel-16 DL-TDOA method, all cases of InF-SH scenario can meet the loose vertical accuracy requirement (i.e. 1 m for 90% of UEs), but in the InF-DH scenario, only 67% UEs in FR1 and 80% UEs in FR2 can meet sub-meter level requirement.

  - Rel-17 enhancements with LOS identification

Lastly, some Rel-17 enhancements with LOS identification have been investigated, horizontal positioning errors of InF-DH scenario for FR1 and FR2 at some specific percentiles are shown in Table 6 and Table 7 respectively, and the corresponding CDF curves are attached in appendix in Figure 3-1 and Figure 3-2.

Table 6 CDF of horizontal positioning accuracy for InF-DH, FR1 scenario (Rel-17 enhancements)

| Simulation cases | Whether Rel-17 enhancement is implemented | Horizontal positioning accuracy (m) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50% | 67% | 80% | 90% |
| Case 12 | NO | 0.414 | 1.060 | 7.337 | 12.345 |
| Case 25 | Ideal LOS identification | 0.261 | 0.371 | 0.493 | 0.648 |
| Case 26 | Rician K-factor as assistance information | 0.294 | 0.4207 | 0.786 | 7.205 |
| Case 27 | Coherence bandwidth as assistance information | 0.300 | 0.4232 | 0.659 | 2.003 |

Table 7 CDF of horizontal positioning accuracy for InF-DH, FR2 scenario (Rel-17 enhancements)

| Simulation cases | Whether Rel-17 enhancement is implemented | Horizontal positioning accuracy (m) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50% | 67% | 80% | 90% |
| Case 17 | NO | 0.062 | 0.097 | 0.174 | 12.174 |
| Case 28 | Ideal LOS identification | 0.047 | 0.062 | 0.080 | 0.107 |
| Case 29 | Rician K-factor as assistance information | 0.053 | 0.074 | 0.101 | 9.248 |
| Case 30 | Coherence bandwidth as assistance information | 0.051 | 0.069 | 0.091 | 0.228 |

Observation 4: For InF-DH scenario with enhancements on LOS identification,

- When positioning is done without assistance information, the positioning performance degrades rapidly for UEs connected with small LOS communication links.

- If ideal classification of LOS and NLOS links is assumed, all cases meet sub-meter level positioning accuracy requirement.

- By utilizing Rician K-factor and coherence bandwidth as assistance information, positioning accuracy is significantly improved compared to without such assistance information, and sub-meter level positioning accuracy requirement can be fulfilled at the percentile of 90% UEs in FR2.

- Evaluation results of positioning latency

There are some agreements in RAN1#102e meeting [3] regarding physical layer latency for positioning, where defines start and end times to derive physical layer latency, and detailed information for latency analysis. Meanwhile, RAN1 sent an LS in RAN1#102-e meeting to ask the input from RAN2 on latency components of NR/NG-RAN/5GC higher layer positioning protocols. Therefore, RAN1 will only focus on physical layer latency. In this section, we provide some exemplary components and corresponding values of physical layer latency based on Rel-16 positioning procedures for DL-TDOA and DL-ECID. All the evaluation results are under the following assumptions,

- UE is already in RRC_CONNECTED state, and some LPP messages such as capability data are not considered in physical layer latency.

- The latency due to any additional delays (e.g. RACH procedures, RRC reconfiguration, handover, etc.) are ignored.

  - UE-assisted positioning based on DL-TDOA method

Table 8 physical layer latency analysis for UE-assisted positioning based on DL-TDOA method

| Source [UE]/Destination [NW] Positioning technique [DL-TDOA],type [DL], mode [UE-A] Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| Start trigger (step 1) | N/A | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message |
| UE decodes and applies the Location Request (step 2) | 10 ms | RRC Processing time at the UE are captured in 38.331 Section 12, where defines the following RRC procedure delay, - reception of the network -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> network response message |
| Measurement gap request (step 3) | Corresponding values can be found in right column. | UL user plane latency defined in 37.910 clause 5.7.1.2.2 is applicable to measurement gap request, where the latency includes - UE processing delay - UL frame alignment (transmission alignment) - TTI for UL data packet transmission - BS processing delay The grant free based UL transmission is assumed in this contribution, some typical cases can be found in 37.910 clause 5.7.1.2.2. We choose following cases as examples, UL user plane latency for NR FDD with grant free transmission: UL user plane latency (Grant free)(ms) – NR FDD; UE capability 2. SCS. 30 KHz; 60 KHz. Resource mapping Type B; M=7 (7OS non-slot); p=0; 0.43; 0.30. UL user plane latency for NR TDD with grant free transmission: UL user plane latency (Grant free,DDDXU, with ‘X’=’U’ for UL traffic)(ms) – NR TDD; UE capability 2. SCS. 30 KHz; 60 KHz. Resource mapping Type B; M=7 (7OS non-slot); p=0; 1.09; 0.64. |
| Serving gNB decodes and interprets the measurement gap request (step 4) | 3 ms | Regarding the RRC processing time at the gNB, 37.910 clause 5.7.1.1.1 defines processing delay in gNB for L2 and RRC. |
| Measurement gap configuration(step 5) | Corresponding values can be found in right column. | DL user plane latency defined in 37.910 clause 5.7.1.2.1 is applicable to measurement gap configuration, where the latency includes - BS processing delay - DL Frame alignment (transmission alignment) - TTI for DL data packet transmission - UE processing delay The latency may be different according to various configurations,some typical cases can be found in in 37.910 clause 5.7.1.2.2. We choose following cases as examples, DL user plane latency for NR FDD: DL user plane latency (ms) – NR FDD; UE capability 2. SCS. 30 KHz; 60 KHz. Resource mapping Type B; M=4 (7OS non-slot); p=0; 0.37; 0.27. DL user plane latency for NR TDD: DL user plane latency (ms – NR TDD (DDDXU,’X’=’D’ for DL traffic); UE capability 2. SCS. 30 KHz; 60 KHz. Resource mapping Type B; M=4 (7OS non-slot); p=0; 0.45; 0.32. |
| UE interprets and applies the measurement gap configuration(step 6) | 10 ms | RRC Processing time at the UE are captured in 38.331 Section 12, where defines the following RRC procedure delay, reception of the network -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> network response message |
| UE positioning measurement(step 7) | 82 ms ( FR1) or 644 ms ( FR2) | Measurement period requirements defined in 38.133 clause 9.9.2.5 should be satisfied, more detailed configurations can refer to Table 9. |
| UE positioning measurement transmission(step 8) | Corresponding values can be found in right column. | The same latency is assumed as “Measurement gap request”. |
| End trigger (step 9) | N/A | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values (1) | 106.23 ms | FDD and 30KHz SCS |
| Total values (2) | 667.87 ms | FDD and 60KHz SCS |
| Total values (3) | 107.63 ms | TDD and 30KHz SCS |
| Total values (4) | 668.60 ms | TDD and 60KHz SCS |

Table 9 Measurement period requirements for DL-TDOA method

| Parameters | FR1 | FR2 | Additional Explanations |
| --- | --- | --- | --- |
| MGRP ( Measurement Gap Repetition Period ) | 20 ms | 20 ms |  |
| MGL ( Measurement Gap Length ) | 4 ms | 4 ms | PRS is configured in a compact way, we assume the MGL can cover all PRS occasions from different TRPs. |
| TPRS ( DL PRS periodicity ) | 4 ms | 4 ms | All PRS resource sets have the same periodicity. |
| (N,T) ( Duration of DL PRS symbols N in units of ms a UE can process every T ms ) | (4, 20) ms | (4, 20) ms | The DL PRS buffering time fully aligns the MGL. |
| L ( Number of positioning frequency layer ) | 1 | 1 |  |
| Number of TRP | 4 | 4 |  |
| SCS ( Sub-carrier Spacing ) | 30 KHz | 60 KHz | MGL will span 8 slots for FR1, while the value is 16 slots for FR2. |
| LPRS ( the duration of DL PRS symbols within any a window ) | 2 ms | 4 ms | Type II DL PRS buffering capability is assumed. |
| ( carrier-specific scaling factor ) | 1 | 1 |  |
| ( UE Rx beam sweeping factor ) | 1 | 8 |  |
| ( Number of PRS RSTD samples ) | 4 | 4 |  |
| Total measurement period | 82 ms | 644 ms |  |
| Other assumptions | - The number of PRS resources in a slot as configured by the assistance data is smaller the UE capability of max number of DL PRS resources that UE can process in a slot. - UE is not required to measure PRS resources from multiple TRPs on the same OFDM symbol in FR2. |  |  |

  - UE-based positioning based on DL-TDOA method

Regarding the UE-based positioning, the physical layer latency varies when different start and end times are assumed. The components that are different from UE-assisted positioning are listed in Table 10.

Table 10 physical layer latency for UE-based positioning based on DL-TDOA method

| Start trigger | End trigger | Components that are different from UE-assisted positioning in Table 8. | Total values |
| --- | --- | --- | --- |
| Transmission of the PDSCH from the gNB carrying the LPP Request Location Information | Successful decoding of the PUSCH at gNB carrying the LPP Provide Location Information message | - Update the start and end triggers accordingly. - All other steps are the same as UE-assisted positioning in Table 1. | Total values (1); 106.23 ms. Total values (2); 667.87 ms. Total values (3); 107.63 ms. Total values (4); 668.60 ms. |
| Transmission of the PDSCH from the gNB carrying the LPP Request Location Information | Calculation of Location Estimate at the UE | - Update the start and end triggers accordingly. - Step 8 should be removed since UE is the location consumer, which is replaced by “ UE location calculation” ( assume the latency is [1 slot ]) | Total values (1); 106.30 ms. Total values (2); 667.82 ms. Total values (3); 107.08 ms. Total values (4); 668.51 ms. |
| Transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data | Successful decoding of the PUSCH at gNB carrying the LPP Provide Location Information message | - Update the start and end triggers accordingly. - Step 2 is replaced by “UE decodes and applies the LPP message containing the assistance data”. - Another step for “ UE location calculation” ( assume the latency is [1 slot ] ) is added between step 7 and step 8. | Total values (1); 106.23 ms. Total values (2); 667.87 ms. Total values (3); 107.63 ms. Total values (4); 668.60 ms. |
| Transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data | Calculation of Location Estimate at the UE | - Update the start and end triggers accordingly. - Step 2 is replaced by “UE decodes and applies the LPP message containing the assistance data”. - Step 8 should be removed since UE is the location consumer, which is replaced by “ UE location calculation” ( assume the latency is [1 slot ] ) | Total values (1); 106.30 ms. Total values (2); 667.82 ms. Total values (3); 107.08 ms. Total values (4); 668.51 ms. |

  - UE-assisted positioning based on DL-ECID method

Table 11 physical layer latency for UE-based positioning based on DL-ECID method

| Source [UE]/Destination [NW] Positioning technique [DL-ECID],type [DL], mode [UE-A], Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| Start trigger | N/A | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message |
| UE interprets and applies the measurement configuration | 10 ms | RRC Processing time at the UE are captured in 38.331 Section 12, where defines the following RRC procedure delay, reception of the network -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> network response message |
| UE ECID measurement time | Refer to Table 5. | Measurement period requirements defined in 38.133 section 9.2 and section 9.3 should be satisfied, more detailed configurations can refer to Table 11. Depends on when the UE ECID measurement is required, the average time for UE ECID measurement approximates to a half of RRM measurement period. Otherwise if RRM is available(i.e. buffered) at UE side, UE can forward the ECID measurement directly without consuming additional time. |
| UE positioning measurement transmission | Corresponding values can be found in right column. | UL user plane latency defined in 37.910 clause 5.7.1.2.2 is applicable to measurement gap request, where the latency includes - UE processing delay - UL frame alignment (transmission alignment) - TTI for UL data packet transmission - BS processing delay The grant free based UL transmission is assumed in this contribution, some typical cases can be found in 37.910 clause 5.7.1.2.2. We choose following cases as examples, UL user plane latency for NR FDD with grant free transmission: UL user plane latency (Grant free)(ms) – NR FDD; UE capability 2. SCS. 30 KHz; 60 KHz. Resource mapping Type B; M=7 (7OS non-slot); p=0; 0.43; 0.30. UL user plane latency for NR TDD with grant free transmission: UL user plane latency (Grant free,DDDXU, with ‘X’=’U’ for UL traffic)(ms) – NR TDD; UE capability 2. SCS. 30 KHz; 60 KHz. Resource mapping Type B; M=7 (7OS non-slot); p=0; 1.09; 0.64. |
| End trigger | N/A | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | Corresponding values can be found in right column. | FDD and 30KHz SCS: RRM measurement is available; RRM measurement is not available. without associated SSB index; 10.43 ms; 410.43 ms. with associated SSB index; 10.43 ms; 470.43 ms. FDD and 60KHz SCS: RRM measurement is available; RRM measurement is not available. without associated SSB index; 10.30 ms; 510.43 ms. with associated SSB index; 10.30 ms; 570.43 ms. TDD and 30KHz SCS: RRM measurement is available; RRM measurement is not available. without associated SSB index; 11.09 ms; 411.09 ms. with associated SSB index; 11.09 ms; 471.09 ms. TDD and 60KHz SCS: RRM measurement is available; RRM measurement is not available. without associated SSB index; 10.64 ms; 510.64 ms. with associated SSB index; 10.64 ms; 570.64 ms. |

Table 12 RRM measurement period requirements for inter-frequency/intra-frequency with measurement gaps

| Parameters | FR1 | FR2 |
| --- | --- | --- |
| Without the associated SSB index | 600ms+200ms=800ms | 600ms+400ms=1000ms |
| With the associated SSB index | 600ms+120ms+200ms=920ms | 600ms+120ms+400ms=1120ms |
| Note: - No DRX is configured. - CSSFintra=1; CSSFinter=1. - Only minimum time periods for TPSS/SSS_sync_intra, TSSB_time_index_intra and T SSB_measurement_period_intra are assumed. - Only minimum time periods for TPSS/SSS_sync_inter, TSSB_time_index_inter and T SSB_measurement_period_inter are assumed. |  |  |

Observation 5: The dominant contributors of physical layer latency for DL-TDOA method are UE positioning measurement and MG request procedures.

Observation 6: TDD or FDD configuration is not the dominant contributor on physical layer latency.

Observation 7: UE requires additional time for beam sweeping (or beam alignment) in FR2, which leads to much higher physical layer latency over FR1.

Observation 8: Based on Rel-16 positioning procedures, DL-TDOA method is hard to meet stringent physical layer latency requirements in Rel-17.

Observation 9: DL-ECID method consumes small physical layer latency if RRM measurement is available at UE side.

- 4 Conclusions

In this contribution, we provide evaluation results of NR positioning accuracy and latency. Based on the evaluation, we have the following observations,

Observation 1: For horizontal positioning accuracy of InF-SH scenario based on Rel-16 DL-TDOA method,

- The positioning accuracy of UE inside convex hull is slightly better than the case where all UEs are uniformly distributed over the factory, because the high quality links for positioning are always enough under high LOS probability scenario.

- When there is no network synchronization error and gNB Tx calibration error, the horizontal positioning accuracy of 90% UEs inside convex hull are less than 0.568 m in FR1, while the value is 0.090 m in FR2.

- The sub-meter level horizontal positioning accuracy at the percentile of 90% UEs will be fulfilled, if gNB Tx calibration error is larger than 1 ns in FR1 and 2 ns in FR2.

Observation 2: For horizontal positioning accuracy of InF-DH scenario based on Rel-16 DL-TDOA method,

- The positioning accuracy of UE inside convex hull is much better than the case where all UEs are uniformly distributed over the factory, because the high quality links for positioning may not be enough under low LOS probability scenario.

- When there is no network synchronization error and gNB Tx calibration error, sub-meter level horizontal positioning accuracy will be fulfilled for UEs inside convex hull at the percentile of 67% in FR1 and 80% in FR2.

- gNB Tx calibration error significantly degrades the positioning performance, e.g. 1ns in FR1 and 2 ns in FR2.

Observation 3: For vertical positioning accuracy based on Rel-16 DL-TDOA method, all cases of InF-SH scenario can meet the loose vertical accuracy requirement (i.e. 1 m for 90% of UEs), but in the InF-DH scenario, only 67% UEs in FR1 and 80% UEs in FR2 can meet sub-meter level requirement.

Observation 4: For InF-DH scenario with enhancements on LOS identification,

- When positioning is done without assistance information, the positioning performance degrades rapidly for UEs connected with small LOS communication links.

- If ideal classification of LOS and NLOS links is assumed, all cases meet sub-meter level positioning accuracy requirement.

- By utilizing Rician K-factor and coherence bandwidth as assistance information, positioning accuracy is largely improved compared to without assistance information and sub-meter level positioning accuracy requirement can be fulfilled at the percentile of 90% UEs in FR2.

Observation 5: The dominant contributors of physical layer latency for DL-TDOA method are UE positioning measurement and MG request procedures.

Observation 6: TDD or FDD configuration is not the dominant contributor on physical layer latency.

Observation 7: UE requires additional time for beam sweeping (or beam alignment) in FR2, which leads to much higher physical layer latency over FR1.

Observation 8: Based on Rel-16 positioning procedures, DL-TDOA method is hard to meet stringent physical layer latency requirements in Rel-17.

Observation 9: DL-ECID method consumes small physical layer latency if RRM measurement is available at UE side.

- 5 References

- RP-193237, New SID on NR Positioning Enhancements, Qualcomm Incorporated

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #101-e, May 25th – June 5th, 2020

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #102-e, August 17th – 28th, 2020

- Study on self evaluation towards IMT-2020 submission (Release 16), 3GPP TR 37.910

- Radio Resource Control (RRC) protocol specification (Release 16), 3GPP TS 38.331

- Requirements for support of radio resource management (Release 16), 3GPP TS 38.133

- Appendix

## Table 13 Common scenario parameters

| Parameter | FR1 | FR2 |
| --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |
| Carrier frequency | 3.5 GHz | 28 GHz |
| Subcarrier spacing | 30 KHz | 120 KHz |
| Reference Signal Transmission Bandwidth | 100 MHz | 400 MHz |
| gNB noise Figure, dB | 5dB | 7dB |
| UE noise Figure, dB | 9dB | 13dB |
| UE max. TX power, dBm | 23dBm | 23dBm EIRP should not exceed 43 dBm. |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL-PRS-CombSizeN-r16 = 6 DL-PRS-ReOffset-r16 = {0,3,1,4,2,5} DL-PRS-NumSymbols-r16 = 6 |  |
| Number of sites | 18 |  |
| UE number per site | 20 |  |
| number of occasions used per positioning estimate | 4 |  |
| Power-boosting level | 7.8dB |  |
| Uplink power control (applied/not applied) | Not applied |  |
| interference modelling (ideal muting, or other) | Ideal muting |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | MUSIC algorithm |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA, Guass-Newton algorithm |  |
| Network synchronization assumptions | Without sync error |  |
| gNB Tx calibration Error | 0ns, 0.5ns, 1ns, 2ns |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | DFT codebook |  |
| Additional notes, if any | The absolute time of arrival is applied according to TR 38.901 |  |

## Table 14 Parameters common to InF scenario(s)

|  |  | FR1 Specific Values |  | FR2 Specific Values |
| --- | --- | --- | --- | --- |
| Channel model |  | InF-SH, InF-DH |  | InF-SH, InF-DH |
| Layout | Hall size | InF-SH: (baseline) 300x150 m InF-DH: (baseline) 120x60 m |  |  |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |  |  |
|  | Room height | 10m |  |  |
| Total gNB TX power, dBm |  | 24dBm | 24dBm EIRP should not exceed 58 dBm |  |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |  |
| gNB antenna radiation pattern |  | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |  |
| Peneteration loss |  | 0dB |  |  |
| Number of floors |  | 1 |  |  |
| UE horizontal drop procedure |  | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be at least the convex hull of the horizontal BS deployment. It can also be the whole hall area if the CDF values for positioning accuracy is obtained from whole hall area. |  |  |
| UE antenna height |  | Horizontal evaluation: 1.5m Vertical evaluation: uniformly distributed within [0.5, X2] m, where X2 = 2 m for InF-SH and X2=hc for InF-DH |  |  |
| UE mobility |  | 3km/h |  |  |
| Min gNB-UE distance (2D), m |  | 0m |  |  |
| gNB antenna height |  | Horizontal evaluation: 8 m Vertical evaluation: {4, 8} m |  |  |
| Clutter parameters: {density , height ,size } |  | Low clutter density: {20%, 2m, 10m} High clutter density: {40%, 2m, 2m} {40%, 3m, 5m} |  |  |
| Note 1: According to Table A.2.1-7 in 3GPP TR 38.802 |  |  |  |  |

## Table 15 All simulation cases for positioning accuracy evaluation

| Simulation cases | Scenario | FR1/FR2 | Whether Rel-17 enhancement is implemented | UE horizontal drop procedure | gNB Tx calibration error | Clutter parameters |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 | InF-SH | FR1 | NO | Uniformly distributed | 0ns | {20%, 2m, 10m} |
| Case 2 | InF-SH | FR1 | NO | Inside convex hull | 0ns | {20%, 2m, 10m} |
| Case 3 | InF-SH | FR1 | NO | Inside convex hull | 0.5ns | {20%, 2m, 10m} |
| Case 4 | InF-SH | FR1 | NO | Inside convex hull | 1ns | {20%, 2m, 10m} |
| Case 5 | InF-SH | FR1 | NO | Inside convex hull | 2ns | {20%, 2m, 10m} |
| Case 6 | InF-SH | FR2 | NO | Uniformly distributed | 0ns | {20%, 2m, 10m} |
| Case 7 | InF-SH | FR2 | NO | Inside convex hull | 0ns | {20%, 2m, 10m} |
| Case 8 | InF-SH | FR2 | NO | Inside convex hull | 0.5ns | {20%, 2m, 10m} |
| Case 9 | InF-SH | FR2 | NO | Inside convex hull | 1ns | {20%, 2m, 10m} |
| Case 10 | InF-SH | FR2 | NO | Inside convex hull | 2ns | {20%, 2m, 10m} |
| Case 11 | InF-DH | FR1 | NO | Uniformly distributed | 0ns | {40%, 2m, 2m} |
| Case 12 | InF-DH | FR1 | NO | Inside convex hull | 0ns | {40%, 2m, 2m} |
| Case 13 | InF-DH | FR1 | NO | Inside convex hull | 0.5ns | {40%, 2m, 2m} |
| Case 14 | InF-DH | FR1 | NO | Inside convex hull | 1ns | {40%, 2m, 2m} |
| Case 15 | InF-DH | FR1 | NO | Inside convex hull | 2ns | {40%, 2m, 2m} |
| Case 16 | InF-DH | FR2 | NO | Uniformly distributed | 0ns | {40%, 2m, 2m} |
| Case 17 | InF-DH | FR2 | NO | Inside convex hull | 0ns | {40%, 2m, 2m} |
| Case 18 | InF-DH | FR2 | NO | Inside convex hull | 0.5ns | {40%, 2m, 2m} |
| Case 19 | InF-DH | FR2 | NO | Inside convex hull | 1ns | {40%, 2m, 2m} |
| Case 20 | InF-DH | FR2 | NO | Inside convex hull | 2ns | {40%, 2m, 2m} |
| Case 21 | InF-SH | FR1 | NO | Inside convex hull | 0ns | {20%, 2m, 10m} |
| Case 22 | InF-SH | FR2 | NO | Inside convex hull | 0ns | {20%, 2m, 10m} |
| Case 23 | InF-DH | FR1 | NO | Inside convex hull | 0ns | {40%, 3m,5m} |
| Case 24 | InF-DH | FR2 | NO | Inside convex hull | 0ns | {40%, 3m, 5m} |
| Case 25 | InF-DH | FR1 | Ideal LOS identification | Inside convex hull | 0ns | {40%, 2m, 2m} |
| Case 26 | InF-DH | FR1 | Rician K-factor | Inside convex hull | 0ns | {40%, 2m, 2m} |
| Case 27 | InF-DH | FR1 | Coherence bandwidth | Inside convex hull | 0ns | {40%, 2m, 2m} |
| Case 28 | InF-DH | FR2 | Ideal LOS identification | Inside convex hull | 0ns | {40%, 2m, 2m} |
| Case 29 | InF-DH | FR2 | Rician K-factor | Inside convex hull | 0ns | {40%, 2m, 2m} |
| Case 30 | InF-DH | FR2 | Coherence bandwidth | Inside convex hull | 0ns | {40%, 2m, 2m} |

## Figure 1 Horizontal positioning accuracy based on Rel-16 DL-TDOA method

Figure 1-1 InF-SH, FR1                        Figure 1-2 InF-SH, FR2

Figure 1-3 InF-DH, FR1                        Figure 1-4 InF-DH, FR2

## Figure 2 Vertical positioning accuracy based on Rel-16 DL-TDOA method

Figure 2-1 InF-SH                               Figure 2-2 InF-DH

## Figure 3 Rel-17 enhancements with LOS identification

Figure 3-1 InF-DH, FR1                             Figure 3-2 InF-DH, FR2