3GPP TSG-RAN WG1 #103-e	R1- 2007804

# e-Meeting, October 26th – November 13th, 2020‎

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | - | rev | - | Current version: | 15.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on HARQ-ACK generation for DL transmission with single TB when multi-TB is configured |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In current specification for Type-2 HARQ-ACK codebook, for a serving cell configured with maximum two transport blocks, the definition for HARQ-ACK feedback generation for a SPS PDSCH release is missing, which may lead to misunderstanding between UE and gNB for the HARQ-ACK feedback. When harq-ACK-SpatialBundlingPUCCH is not provided, UE may map the HARQ-ACK for the SPS PDSCH release to either position of the two TBs and UE may generate either ACK or NACK for the non-scheduled TB. When harq-ACK-SpatialBundlingPUCCH is provided, a wrong bundled feedback may be generated by assuming NACK for the non-scheduled TB. In current specification for Type-1 HARQ-ACK codebook, for a serving cell configured with maximum two transport blocks, the HARQ-ACK feedback generation for a PDSCH reception with single TB is only defined for the case of DCI format 1_1 scheduling. There is no definition for the case of PDSCH scheduling with DCI format 1_0 and for SPS PDSCH release, which may lead to misunderstanding between UE and gNB for the HARQ-ACK feedback smilar as Type-2 HARQ-ACK codebook when harq-ACK-SpatialBundlingPUCCH is not provided. Since there is description in the pseudo-code as “if the UE receives one transport block, the UE assumes ACK for the second transport block”, there is no problem when harq-ACK-SpatialBundlingPUCCH is provided. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the HARQ-ACK assumption of the second TB for PDSCH scheduled by DCI format 0_0 for Type-1 HARQ-ACK codebook and for SPS PDSCH release for Type-1 and Type-2 HARQ-ACK codebooks when multi-TB is configured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear UE behavior for HARQ-ACK feedback bit generation for a DL transmission with a single TB when multi-TB is configured for a serving cell. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.2.1, 9.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR has isolated impact on HARQ-ACK feedback bit generation for a DL transmission with a single TB when multi-TB is configured for a serving cell. The corrections in this CR are common understandings which may have no impact on both UE and gNB implementation. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

For a serving cell , an active DL BWP, and an active UL BWP, as described in Clause 12, the UE determines a set of  occasions for candidate PDSCH receptions for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot . If serving cell  is deactivated, the UE uses as the active DL BWP for determining the set of  occasions for candidate PDSCH receptions a DL BWP provided by firstActiveDownlinkBWP-Id. The determination is based:

a)	on a set of slot timing values  associated with the active UL BWP

a)	If the UE is configured to monitor PDCCH for DCI format 1_0 and is not configured to monitor PDCCH for DCI format 1_1 on serving cell ,  is provided by the slot timing values {1, 2, 3, 4, 5, 6, 7, 8} for DCI format 1_0

b)	If the UE is configured to monitor PDCCH for DCI format 1_1 for serving cell ,  is provided by dl-DataToUL-ACK for DCI format 1_1

b)	on a set of row indexes  of a table that is provided either by a first set of row indexes of a table that is provided by pdsch-TimeDomainAllocationList in pdsch-ConfigCommon or by Default PDSCH time domain resource allocation A [6, TS 38.214], or by the union of the first set of row indexes and a second set of row indexes, if provided by pdsch-TimeDomainAllocationList in pdsch-Config, associated with the active DL BWP and defining respective sets of slot offsets , start and length indicators SLIV, and PDSCH mapping types for PDSCH reception as described in [6, TS 38.214]

c)	on the ratio  between the downlink SCS configuration  and the uplink SCS configuration  provided by subcarrierSpacing in BWP-Downlink and BWP-Uplink for the active DL BWP and the active UL BWP, respectively

d)	if provided, on tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated as described in Clause 11.1.

For the set of slot timing values , the UE determines a set of  occasions for candidate PDSCH receptions or SPS PDSCH releases according to the following pseudo-code. A location in the Type-1 HARQ-ACK codebook for HARQ-ACK information corresponding to a SPS PDSCH release is same as for a corresponding SPS PDSCH reception.

Set  - index of occasion for candidate PDSCH reception or SPS PDSCH release

Set

Set

Set  to the cardinality of set

Set k =0 – index of slot timing values , in descending order of the slot timing values, in set  for serving cell

while

if

Set  – index of a DL slot within an UL slot

while

Set  to the set of rows

Set  to the cardinality of

Set  – index of row in set

if slot  starts at a same time as or after a slot for an active DL BWP change on serving cell  or an active UL BWP change on the PCell and slot  is before the slot for the active DL BWP change on serving cell  or the active UL BWP change on the PCell

nD=nD+1;

else

while

if the UE is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot  to slot , at least one symbol of the PDSCH time resource derived by row  is configured as UL where  is the k-th slot timing value in set ,

;

else

;

end if

end while

if the UE does not indicate a capability to receive more than one unicast PDSCH per slot and ,

;

;

The UE does not expect to receive SPS PDSCH release and unicast PDSCH in a same slot;

else

Set  to the cardinality of

Set  to the smallest last OFDM symbol index, as determined by the SLIV, among all rows of

while

Set

while

if  for start OFDM symbol index  for row

; - index of occasion for candidate PDSCH reception or SPS PDSCH release associated with row

;

;

else

;

end if

end while

;

Set  to the smallest last OFDM symbol index among all rows of ;

end while

end if

nD=nD+1;

end if

end while

end if

;

end while

If the UE indicates a capability to receive more than one PDSCH per slot, for occasions of candidate PDSCH receptions corresponding to rows of associated with a same value of , where , the UE does not expect to receive more than one PDSCH in a same DL slot.

If a UE receives a SPS PDSCH, or a SPS PDSCH release, or a PDSCH that is scheduled by a DCI format 1_0 and if

-	the UE is configured with one serving cell, and

-	, and

-	PDSCH-CodeBlockGroupTransmission is provided to the UE

the UE generates HARQ-ACK information only for the transport block in the PDSCH or only for the SPS PDSCH release.

If a UE receives a SPS PDSCH, or a SPS PDSCH release, or a PDSCH that is scheduled by a DCI format 1_0 and if

-	the UE is configured with more than one serving cells, or

-	, and

-	PDSCH-CodeBlockGroupTransmission is provided to the UE

the UE repeats  times the HARQ-ACK information for the transport block in the PDSCH or for the SPS PDSCH release.

A UE does not expect to detect a DCI format switching a DL BWP within  symbols prior to a first symbol of a PUCCH transmission where the UE multiplexes HARQ-ACK information, where  is defined in Clause 9.2.3.

If a UE is provided dl-DataToUL-ACK, the UE does not expect to be indicated by DCI format 1_0 a slot timing value for transmission of HARQ-ACK information that does not belong to the intersection of the set of slot timing values {1, 2, 3, 4, 5, 6, 7, 8} and the set of slot timing values provided by dl-DataToUL-ACK for the active DL BWP of a corresponding serving cell.

If  maxNrofCodeWordsScheduledByDCI indicates reception of two transport blocks, when the UE receives a PDSCH with one transport block or a SPS PDSCH release, the HARQ-ACK information is associated with the first transport block and the UE generates a NACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is not provided and generates HARQ-ACK information with value of ACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is provided.

A UE determines  HARQ-ACK information bits, for a total number of  HARQ-ACK information bits, of a HARQ-ACK codebook for transmission in a PUCCH according to the following pseudo-code. In the following pseudo-code, if the UE does not receive a transport block or a CBG, due to the UE not detecting a corresponding DCI format 1_0 or DCI format 1_1, the UE generates a NACK value for the transport block or the CBG. The cardinality of the set  defines a total number  of occasions for PDSCH reception or SPS PDSCH release for serving cell  corresponding to the HARQ-ACK information bits.

Set  – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell

Set - HARQ-ACK information bit index

Set  to the number of serving cells configured by higher layers for the UE

while

Set  – index of occasion for candidate PDSCH reception or SPS PDSCH release

while

if harq-ACK-SpatialBundlingPUCCH is not provided, PDSCH-CodeBlockGroupTransmission is not provided, and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell ,

= HARQ-ACK information bit corresponding to a first transport block of this cell;

;

= HARQ-ACK information bit corresponding to a second transport block of this cell;

;

elseif harq-ACK-SpatialBundlingPUCCH is provided, and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell ,

= binary AND operation of the HARQ-ACK information bits corresponding to first and second transport blocks of this cell - if the UE receives one transport block, the UE assumes ACK for the second transport block;

;

elseif PDSCH-CodeBlockGroupTransmission is provided, and  CBGs are indicated by maxCodeBlockGroupsPerTransportBlock for serving cell ,

Set - CBG index

while

= HARQ-ACK information bit corresponding to CBG  of the first transport block;

if the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell

= HARQ-ACK information bit corresponding to CBG  of the second transport block;

end if

;

end while

, where  is the value of maxNrofCodeWordsScheduledByDCI for the active DL BWP of serving cell ;

else

= HARQ-ACK information bit of serving cell ;

;

end if

;

end while

;

end while

If , the UE determines a number of HARQ-ACK information bits  for obtaining a transmission power for a PUCCH, as described in Clause 7.2.1, as  where

-	 is the number of transport blocks the UE receives in PDSCH reception occasion  for serving cell  if harq-ACK-SpatialBundlingPUCCH and PDSCH-CodeBlockGroupTransmission are not provided, or the number of transport blocks the UE receives in PDSCH reception occasion  for serving cell  if PDSCH-CodeBlockGroupTransmission is provided and the PDSCH reception is scheduled by a DCI format 1_0, or the number of PDSCH receptions if harq-ACK-SpatialBundlingPUCCH is provided or SPS PDSCH release in PDSCH reception occasion  for serving cell  and the UE reports corresponding HARQ-ACK information in the PUCCH.

-	 is the number of CBGs the UE receives in a PDSCH reception occasion  for serving cell  if PDSCH-CodeBlockGroupTransmission is provided and the PDSCH reception is scheduled by a DCI format 1_1 and the UE reports corresponding HARQ-ACK information in the PUCCH.

<unchanged part is omitted>

9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

A UE determines monitoring occasions for PDCCH with DCI format 1_0 or DCI format 1_1 for scheduling PDSCH receptions or SPS PDSCH release on an active DL BWP of a serving cell , as described in Clause 10.1, and for which the UE transmits HARQ-ACK information in a same PUCCH in slot  based on

-	PDSCH-to-HARQ_feedback timing indicator field values for PUCCH transmission with HARQ-ACK information in slot  in response to PDSCH receptions or SPS PDSCH release

-	slot offsets  [6, TS 38.214] provided by time domain resource assignment field in DCI format 1_0 or DCI format 1_1 for scheduling PDSCH receptions or SPS PDSCH release and by pdsch-AggregationFactor, when provided.

The set of PDCCH monitoring occasions for DCI format 1_0 or DCI format 1_1 for scheduling PDSCH receptions or SPS PDSCH release is defined as the union of PDCCH monitoring occasions across active DL BWPs of configured serving cells, ordered in ascending order of start time of the search space set associated with a PDCCH monitoring occasion. The cardinality of the set of PDCCH monitoring occasions defines a total number  of PDCCH monitoring occasions.

A value of the counter downlink assignment indicator (DAI) field in DCI format 1_0 or DCI format 1_1 denotes the accumulative number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s) or SPS PDSCH release associated with DCI format 1_0 or DCI format 1_1 is present, up to the current serving cell and current PDCCH monitoring occasion, first in ascending order of serving cell index and then in ascending order of PDCCH monitoring occasion index , where .

The value of the total DAI, when present [5, TS 38.212], in DCI format 1_1 denotes the total number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s) or SPS PDSCH release associated with DCI format 1_0 or DCI format 1_1 is present, up to the current PDCCH monitoring occasion  and is updated from PDCCH monitoring occasion to PDCCH monitoring occasion.

Denote by  the value of the counter DAI in DCI format 1_0 or DCI format 1_1 for scheduling on serving cell  in PDCCH monitoring occasion  according to Table 9.1.3-1. Denote by  the value of the total DAI in DCI format 1_1 in PDCCH monitoring occasion  according to Table 9.1.3-1. The UE assumes a same value of total DAI in all DCI formats 1_1 in PDCCH monitoring occasion .

If the UE transmits HARQ-ACK information in a PUCCH in slot  and for any PUCCH format, the UE determines the , for a total number of  HARQ-ACK information bits, according to the following pseudo-code:

Set  – PDCCH with DCI format 1_0 or DCI format 1_1 monitoring occasion index: lower index corresponds to earlier PDCCH with DCI format 1_0 or DCI format 1_1 monitoring occasion

Set

Set

Set

Set

Set  to the number of serving cells configured by higher layers for the UE

Set  to the number of PDCCH monitoring occasion(s)

while

Set  – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell

while

if PDCCH monitoring occasion  is before an active DL BWP change on serving cell  or an active UL BWP change on the PCell and an active DL BWP change is not triggered by a DCI format 1_1 in PDCCH monitoring occasion

;

else

if there is a PDSCH on serving cell  associated with PDCCH in PDCCH monitoring occasion , or there is a PDCCH indicating SPS PDSCH release on serving cell

if

end if

if

else

end if

if harq-ACK-SpatialBundlingPUCCH is not provided and  is a monitoring occasion for PDCCH with DCI format 1_0 or DCI format 1_1 and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of at least one serving cell,

= HARQ-ACK information bit corresponding to the first transport block of this cell

= HARQ-ACK information bit corresponding to the second transport block of this cell

elseif harq-ACK-SpatialBundlingPUCCH is provided to the UE and  is a monitoring occasion for PDCCH with DCI format 1_1 and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks in at least one configured DL BWP of a serving cell,

= binary AND operation of the HARQ-ACK information bits corresponding to the first and second transport blocks of this cell

else

= HARQ-ACK information bit of this cell

end if

end if

end if

end while

end while

if

end if

if harq-ACK-SpatialBundlingPUCCH is not provided to the UE and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of a serving cell,

else

end if

for any

Set

while

if SPS PDSCH reception is activated for a UE and the UE is configured to receive SPS PDSCH in a slot  for serving cell , where  is the PDSCH-to-HARQ-feedback timing value for SPS PDSCH on serving cell

= HARQ-ACK information bit associated with the SPS PDSCH reception

end if

;

end while

For a PDCCH monitoring occasion with DCI format 1_0 or DCI format 1_1 in the active DL BWP of a serving cell, when a UE receives a PDSCH with one transport block or a SPS PDSCH release, and the value of maxNrofCodeWordsScheduledByDCI is 2, the HARQ-ACK information is associated with the first transport block and the UE generates a NACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is not provided and generates HARQ-ACK information with value of ACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is provided.

If a UE is not provided PDSCH-CodeBlockGroupTransmission for each of the  serving cells, or for PDSCH receptions scheduled by DCI format 1_0, or for SPS PDSCH reception, or for SPS PDSCH release, and if , the UE determines a number of HARQ-ACK information bits  for obtaining a transmission power for a PUCCH, as described in Clause 7.2.1, as

where

-	if ,  is the value of the counter DAI in the last DCI format 1_0 or DCI format 1_1 scheduling PDSCH reception or indicating SPS PDSCH release for any serving cell  that the UE detects within the  PDCCH monitoring occasions.

-	if

-	if the UE does not detect any DCI format 1_1 in a last PDCCH monitoring occasion within the  PDCCH monitoring occasions where the UE detects at least one DCI format scheduling PDSCH reception or indicating SPS PDSCH release for any serving cell ,  is the value of the counter DAI in a last DCI format 1_0 the UE detects in the last PDCCH monitoring occasion

-	if the UE detects at least one DCI format 1_1 in a last PDCCH monitoring occasion within the  PDCCH monitoring occasions where the UE detects at least one DCI format scheduling PDSCH reception or indicating SPS PDSCH release for any serving cell ,  is the value of the total DAI in the at least one DCI format 1_1

-	 if the UE does not detect any DCI format 1_0 or DCI format 1_1 scheduling PDSCH reception or indicating SPS PDSCH release for any serving cell  in any of the  PDCCH monitoring occasions.

-	 is the total number of DCI format 1_0 and DCI format 1_1 scheduling PDSCH receptions or indicating SPS PDSCH release that the UE detects within the  PDCCH monitoring occasions for serving cell .  if the UE does not detect any DCI format 1_0 or DCI format 1_1 scheduling PDSCH reception or indicating SPS PDSCH release for serving cell  in any of the  PDCCH monitoring occasions.

-	 if the value of maxNrofCodeWordsScheduledByDCI is 2 for any serving cell  and harq-ACK-SpatialBundlingPUCCH is not provided; otherwise, .

-	 is the number of transport blocks the UE receives in a PDSCH scheduled by DCI format 1_0 or DCI format 1_1 that the UE detects in PDCCH monitoring occasion  for serving cell  if harq-ACK-SpatialBundlingPUCCH is not provided, or the number of PDSCH scheduled by DCI format 1_0 and DCI format 1_1 that the UE detects in PDCCH monitoring occasion  for serving cell  if harq-ACK-SpatialBundlingPUCCH is provided, or the number of DCI format 1_0 that the UE detects and indicate SPS PDSCH release in PDCCH monitoring occasion  for serving cell .

-	 is the number of SPS PDSCH receptions by the UE on serving cell  for which the UE transmits corresponding HARQ-ACK information in the same PUCCH as for HARQ-ACK information corresponding to PDSCH receptions within the  PDCCH monitoring occasions.

If a UE

-	is provided PDSCH-CodeBlockGroupTransmission for  serving cells; and

-	is not provided PDSCH-CodeBlockGroupTransmission, for  serving cells where

the UE determines the  according to the previous pseudo-code with the following modifications

-	 is used for the determination of a first HARQ-ACK sub-codebook for SPS PDSCH release, SPS PDSCH reception, and for TB-based PDSCH receptions scheduled by DCI formats 1_0 on the  serving cells and by DCI formats 1_0 and DCI formats 1_1 on the  serving cells

-	 is replaced by  for the determination of a second HARQ-ACK sub-codebook corresponding to the  serving cells for CBG-based PDSCH receptions scheduled by DCI format 1_1, and

-	Instead of generating one HARQ-ACK information bit per transport block for a serving cell from the  serving cells, the UE generates  HARQ-ACK information bits, where  is the maximum value of  across all  serving cells and  is the value of maxNrofCodeWordsScheduledByDCI for serving cell . If for a serving cell  it is , the UE generates NACK for the last  HARQ-ACK information bits for serving cell

-	The pseudo-code operation when harq-ACK-SpatialBundlingPUCCH is provided is not applicable

-	The counter DAI value and the total DAI value apply separately for each HARQ-ACK sub-codebook

-	The UE generates the HARQ-ACK codebook by appending the second HARQ-ACK sub-codebook to the first HARQ-ACK sub-codebook

If , the UE also determines  for obtaining a PUCCH transmission power, as described in Clause 7.2.1, with

where

-	if ,  is the value of the counter DAI in the last DCI format 1_1 scheduling CBG-based PDSCH reception for any serving cell  that the UE detects within the  PDCCH monitoring occasions

-	if ,  is the value of the total DAI in the last DCI format 1_1 scheduling CBG-based PDSCH reception for any serving cell  that the UE detects within the  PDCCH monitoring occasions

-	 if the UE does not detect any DCI format 1_1 scheduling CBG-based PDSCH reception for any serving cell  in any of the  PDCCH monitoring occasions

-	 is the total number of DCI format 1_1 scheduling CBG-based PDSCH receptions that the UE detects within the  PDCCH monitoring occasions for serving cell .  if the UE does not detect any DCI format 1_1 scheduling CBG-based PDSCH reception for serving cell  in any of the  PDCCH monitoring occasions

-	 is the number of CBGs the UE receives in a PDSCH scheduled by DCI format 1_1 that the UE detects in PDCCH monitoring occasion  for serving cell  and the UE reports corresponding HARQ-ACK information in the PUCCH

Table 9.1.3-1: Value of counter DAI in DCI format 1_0 and of counter DAI or total DAI DCI format 1_1

| DAI MSB, LSB | or | Number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH transmission(s) associated with PDCCH or PDCCH indicating SPS PDSCH release is present, denoted as and |
| --- | --- | --- |
| 0,0 | 1 |  |
| 0,1 | 2 |  |
| 1,0 | 3 |  |
| 1,1 | 4 |  |