3GPP TSG RAN WG1 #103-e		                                                      R1-2007816

e-Meeting, October 26th – November 13th, 2020

Source:	CATT

Title:	Remaining issues on intra-UE prioritization

Agenda Item:	7.2.5

Document for:	Discussion and Decision

Introduction

At the RAN1#102 e-meeting, several issues on intra-UE multiplexing/prioritization were discussed and multiple agreements were reached. In this contribution, we discuss the remaining issues for intra-UE prioritization.

Discussion

Timeline requirement for cancellation

When a high-priority UL transmission overlaps with a low-priority UL transmission in a slot, cancellation timeline was defined in specification according to agreements approved in RAN1#101 meeting as following:

If a UE is scheduled by a DCI format in a first PDCCH reception to transmit a first PUCCH or a first PUSCH of larger priority index that overlaps with a second PUCCH or a second PUSCH transmission of smaller priority index that, if any, is scheduled by a DCI format in a second PDCCH - Tproc,2 is based on a value of μ corresponding to the smallest SCS configuration of the first PDCCH, the second PDCCHs, the first PUCCH or the first PUSCH, and the second PUCCHs or the second PUSCHs - if the overlapping group includes the first PUCCH - if processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell where the UE receives the first PDCCH and for all serving cells where the UE receives the PDSCHs corresponding to the second PUCCHs, and if processingType2Enabled of PUSCH-ServingCellConfig is set to enable for the serving cells with the second PUSCHs, N2 is 5 for μ=0, 5.5 for μ=1 and 11 for μ=2 - else, N2 is 10 for μ=0, 12 for μ=1, 23 for μ=2, and 36 for μ=3; - if the overlapping group includes the first PUSCH - if processingType2Enabled of PUSCH-ServingCellConfig is set to enable for the serving cells with the first PUSCH and the second PUSCHs and if processingType2Enabled of PDSCH-ServingCellConfig is set to enable for all serving cells where the UE receives the PDSCHs corresponding to the second PUCCHs, N2 is 5 for μ=0, 5.5 for μ=1 and 11 for μ=2 - else, N2 is 10 for μ=0, 12 for μ=1, 23 for μ=2, and 36 for μ=3;

For determination the value of  μ for Tproc,2, it can be seen that the SCS configuration of the PDCCH providing the DCI for the low/high priority channel, and the SCS configuration of the low/high priority channels are considered. However, in case the low/high priority channel is PUCCH, the SCS configuration of PDSCH corresponding to the low/high priority channels is not considered. In cancellation timeline, the processing time of all channels corresponding to the high and low priority channels should be considered, since the processing time of high priority DCI is related with all channels corresponding to high priority channel. For PUCCH transmission, the SCS configuration of corresponding PDCCH and PDSCH should be considered in processing time since different SCS may be used for PDCCH and PDSCH for cross-carrier scheduling, then it is proposed that the SCS configuration of the PDSCH is considered in cancellation time.

Proposal 1: The SCS configuration of the PDSCH corresponding to the overlapping PUCCH should be considered in cancellation time.

A text proposal is provided below for cancellation timeline in section 9 of 38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

If a UE is scheduled by a DCI format in a first PDCCH reception to transmit a first PUCCH or a first PUSCH of larger priority index that overlaps with at least a second PUCCH or a second PUSCH transmission of smaller priority index that, if any, is scheduled by a DCI format in a second PDCCH

-	Tproc,2 is based on a value of μ corresponding to the smallest SCS configuration of the first PDCCH, the second PDCCHs, the PDSCHs corresponding to the first PUCCH, the PDSCHs corresponding to the second PUCCHs, the first PUCCH or the first PUSCH, and the second PUCCHs or the second PUSCHs

-	if the overlapping group includes the first PUCCH

-	if processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell where the UE receives the first PDCCH and for all serving cells where the UE receives the PDSCHs corresponding to the second PUCCHs, and if processingType2Enabled of PUSCH-ServingCellConfig is set to enable for the serving cells with the second PUSCHs, N2 is 5 for μ=0, 5.5 for μ=1 and 11 for μ=2

-	else, N2 is 10 for μ=0, 12 for μ=1, 23 for μ=2, and 36 for μ=3;

-	if the overlapping group includes the first PUSCH

-	if processingType2Enabled of PUSCH-ServingCellConfig is set to enable for the serving cells with the first PUSCH and the second PUSCHs and if processingType2Enabled of PDSCH-ServingCellConfig is set to enable for all serving cells where the UE receives the PDSCHs corresponding to the second PUCCHs, N2 is 5 for μ=0, 5.5 for μ=1 and 11 for μ=2

-	else, N2 is 10 for μ=0, 12 for μ=1, 23 for μ=2, and 36 for μ=3;

----------------------------------------------------- End of text proposal ------------------------------------------------------

Furthermore, for determining the value of N2 for Tproc,2, it is not clear whether PDSCH processing capability of a serving cell with a PDSCH transmission is the same as the PUSCH processing capability of a serving cell with a PUCCH transmission which carries HARQ-ACK for the PDSCH transmission.

For the sake of simple and clear description, the following expressions are used:

Cap PDCCH: PDSCH processing capability of a serving cell with a PDCCH transmission

Cap PDSCH: PDSCH processing capability of a serving cell with a PDSCH transmission

Cap PUCCH: PUSCH processing capability of a serving cell with a PUCCH transmission

Cap PUSCH: PUSCH processing capability of a serving cell with a PUSCH transmission

There are two understandings on the relationship between Cap PDSCH and Cap PUCCH which carries HARQ-ACK for the PDSCH transmission as below:

- Understanding 1: Cap PDSCH should be the same as Cap PUCCH

- Understanding 2: Cap PDSCH can be different from Cap PUCCH

Based on understanding 1, there is a redundant condition when determining the value of N2 for Tproc,2. For example, if the overlapping group consists of a high priority PUCCH carrying HARQ-ACK, then

- Cap PUCCH for first PUCCH is the same as Cap PUCCH for second PUCCHs since they are configured on the same serving cell.

- Cap PDSCH corresponding to the second PUCCHs is the same as Cap PUCCH corresponding to the second PUCCHs based on understanding 1, where the second PUCCHs and the PDSCHs corresponding to the second PUCCHs belongs to the same cell group or PUCCH group;

- Since Cap PDSCH is the same as Cap PDCCH corresponding to the PDSCH considering that cross-carrier scheduling is not supported for processing capability 2, and Cap PDSCH is the same as Cap PUCCH based on understanding 1, then Cap PDCCH is the same as Cap PUCCH;

It can be seen that Cap PDCCH for the first PDCCH is the same as Cap PDSCH for second PUCCHs. Hence, the following text proposal corresponding to understanding 1 should be considered for 38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

If a UE is scheduled by a DCI format in a first PDCCH reception to transmit a first PUCCH or a first PUSCH of larger priority index that overlaps with a second PUCCH or a second PUSCH transmission of smaller priority index that, if any, is scheduled by a DCI format in a second PDCCH

-	Tproc,2 is based on a value of μ corresponding to the smallest SCS configuration of the first PDCCH, the second PDCCHs, the first PUCCH or the first PUSCH, and the second PUCCHs or the second PUSCHs

-	if the overlapping group includes the first PUCCH

-	if processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell where the UE receives the first PDCCH, and if processingType2Enabled of PUSCH-ServingCellConfig is set to enable for the serving cells with the second PUSCHs, N2 is 5 for μ=0, 5.5 for μ=1 and 11 for μ=2

-	else, N2 is 10 for μ=0, 12 for μ=1, 23 for μ=2, and 36 for μ=3;

-	if the overlapping group includes the first PUSCH

-	if processingType2Enabled of PUSCH-ServingCellConfig is set to enable for the serving cells with the first PUSCH and the second PUSCHs and if processingType2Enabled of PDSCH-ServingCellConfig is set to enable for all serving cells where the UE receives the PDSCHs corresponding to the second PUCCHs, N2 is 5 for μ=0, 5.5 for μ=1 and 11 for μ=2

-	else, N2 is 10 for μ=0, 12 for μ=1, 23 for μ=2, and 36 for μ=3;

----------------------------------------------------- End of text proposal ------------------------------------------------------

Based on understanding 2, some conditions are missing when determining the value of N2 for Tproc,2.

If the overlapping group includes the first PUCCH, PUSCH processing capability for the serving cell with the first PUCCH and second PUCCHs are not considered. For example, if processing capability 2 for PUSCH is not enabled for the serving cell with the first PUCCH or the second PUCCHs, then the smaller value of N2 should not be used in cancellation timeline. But without considering the processing capability 2 for PUSCH in the serving cell with the first PUCCH or the second PUCCHs in cancellation timeline, the smaller value of N2 may be determined, in which case UE may not have sufficient cancellation time. Considering that the processing capability 2 for PUSCH is the same between the first PUCCH and the second PUCCHs, the serving cell of the first PUCCH can be added in the condition to resolve the issue.

In addition, if the overlapping group includes the first PUSCH, PUSCH processing capability for the serving cell with the second PUCCHs is not considered. Similar as above, without considering the processing time capability 2 for PUSCH in the serving cell with the second PUCCHs in cancellation timeline, the smaller value of N2 may be determined, in which case UE may not have sufficient cancellation time.

Hence, the following text proposal corresponding to understanding 2 should be considered in 38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

If a UE is scheduled by a DCI format in a first PDCCH reception to transmit a first PUCCH or a first PUSCH of larger priority index that overlaps with a second PUCCH or a second PUSCH transmission of smaller priority index that, if any, is scheduled by a DCI format in a second PDCCH

-	Tproc,2 is based on a value of μ corresponding to the smallest SCS configuration of the first PDCCH, the second PDCCHs, the first PUCCH or the first PUSCH, and the second PUCCHs or the second PUSCHs

-	if the overlapping group includes the first PUCCH

-	if processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell where the UE receives the first PDCCH and for all serving cells where the UE receives the PDSCHs corresponding to the second PUCCHs, and if processingType2Enabled of PUSCH-ServingCellConfig is set to enable for the serving cells with the second PUSCHs and for the serving cell with the first PUCCH, N2 is 5 for μ=0, 5.5 for μ=1 and 11 for μ=2

-	else, N2 is 10 for μ=0, 12 for μ=1, 23 for μ=2, and 36 for μ=3;

-	if the overlapping group includes the first PUSCH

-	if processingType2Enabled of PUSCH-ServingCellConfig is set to enable for the serving cells with the first PUSCH and the second PUSCHs and for the serving cell with the second PUCCHs and if processingType2Enabled of PDSCH-ServingCellConfig is set to enable for all serving cells where the UE receives the PDSCHs corresponding to the second PUCCHs, N2 is 5 for μ=0, 5.5 for μ=1 and 11 for μ=2

-	else, N2 is 10 for μ=0, 12 for μ=1, 23 for μ=2, and 36 for μ=3;

----------------------------------------------------- End of text proposal ------------------------------------------------------

Proposal 2: It should be clarified which is the correct understanding below and the corresponding text proposal should be adopted:

- Understanding 1: PDSCH processing capability of a serving cell with a PDSCH transmission should be the same as the PUSCH processing capability of a serving cell with a PUCCH transmission corresponding to the PDSCH transmission

- Understanding 2: PDSCH processing capability of a serving cell with a PDSCH transmission can be different with the PUSCH processing capability of a PUCCH transmission corresponding to the PDSCH transmission

Processing order of UL cancellation by SFI/DG and UL multiplexing

For intra-UE prioritization/multiplexing, the following was agreed in last meeting.

Agreement Confirm the following working assumption and remove the brackets as follows: - A UE behavior of handling intra-UE prioritization/multiplexing for overlapping UL transmissions on semi-static flexible symbols is not affected by UL cancellation due to dynamic SFI or [DL grant] Note: The UE performs prioritization/multiplexing first and once done applies dynamic SFI

However, when a UE is configured by higher layers with parameter SlotFormatIndicator and the UE does not detect a DCI format 2_0 providing a slot format for the slot, the agreement is not reflected in the specification. Hence, a text proposal is provided below for intra-UE prioritization/multiplexing in section 11.1.1 of 38.213.

Proposal 3: Adopt the following TP for section 11.1.1 of TS38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

11.1.1      UE procedure for determining slot format

*** Unchanged text is omitted ***

For a set of symbols of a slot that are indicated as flexible by tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated are not provided to the UE, and if the UE does not detect a DCI format 2_0 providing a slot format for the slot

-    the UE receives PDSCH or CSI-RS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format

-    the UE transmits PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format, a RAR UL grant, fallbackRAR UL grant, or successRAR

-    the UE receives PDCCH as described in Clause 10.1

-    if the UE is configured by higher layers to receive PDSCH or CSI-RS in the set of symbols of the slot, the UE does not receive the PDSCH or the CSI-RS in the set of symbols of the slot

-    if the UE is configured by higher layers to receive DL PRS in the set of symbols of the slot, the UE receives the DL PRS

-    if the UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in the set of symbols of the slot and the UE is not provided EnableConfiguredUL-r16, the UE

-     does not transmit the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], as determined in Clauses 9 and 9.2.5 or in Clause 6.1 of [6. TS 38.214], or the PRACH in the slot and does not transmit the SRS in symbols from the set of symbols in the slot, if any, starting from a symbol that is after PUSCH preparation time  for the corresponding PUSCH timing capability [6, TS 38.214] assuming  after a last symbol of a CORESET where the UE is configured to monitor PDCCH for DCI format 2_0 and  corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format 2_0 and the SCS configuration of the SRS, PUCCH, PUSCH or r, where r corresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise r=0

-     does not expect to cancel the transmission of the SRS, or the PUCCH, or the PUSCH, or the PRACH in symbols from the set of symbols in the slot, if any, starting before a symbol that is after the PUSCH preparation time  for the corresponding PUSCH timing capability [6, TS 38.214] assuming  after a last symbol of a CORESET where the UE is configured to monitor PDCCH for DCI format 2_0 and  corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format 2_0 and the SCS configuration of the SRS, PUCCH, PUSCH or r, where r corresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise r=0

-    if the UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in the set of symbols of the slot and the UE is provided EnableConfiguredUL-r16, the UE can transmit the SRS, or PUCCH, or PUSCH, or PRACH, respectively.

----------------------------------------------------- End of text proposal ------------------------------------------------------

Conclusion

In this contribution, we analyse the remaining issues for intra-UE prioritization of URLLC and give the following proposals:

Proposal 1: The SCS configuration of the PDSCH corresponding to the overlapping PUCCH should be considered in cancellation time.

Proposal 2: It should be clarified which is the correct understanding below and the corresponding text proposal should be adopted:

- Understanding 1: PDSCH processing capability of a serving cell with a PDSCH transmission should be the same as the PUSCH processing capability of a serving cell with a PUCCH transmission corresponding to the PDSCH transmission

- Understanding 2: PDSCH processing capability of a serving cell with a PDSCH transmission can be different with the PUSCH processing capability of a PUCCH transmission corresponding to the PDSCH transmission

Proposal 3: Adopt the following TP for section 11.1.1 of TS38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

11.1.1      UE procedure for determining slot format

*** Unchanged text is omitted ***

For a set of symbols of a slot that are indicated as flexible by tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated are not provided to the UE, and if the UE does not detect a DCI format 2_0 providing a slot format for the slot

-    the UE receives PDSCH or CSI-RS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format

-    the UE transmits PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format, a RAR UL grant, fallbackRAR UL grant, or successRAR

-    the UE receives PDCCH as described in Clause 10.1

-    if the UE is configured by higher layers to receive PDSCH or CSI-RS in the set of symbols of the slot, the UE does not receive the PDSCH or the CSI-RS in the set of symbols of the slot

-    if the UE is configured by higher layers to receive DL PRS in the set of symbols of the slot, the UE receives the DL PRS

-    if the UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in the set of symbols of the slot and the UE is not provided EnableConfiguredUL-r16, the UE

-     does not transmit the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], as determined in Clauses 9 and 9.2.5 or in Clause 6.1 of [6. TS 38.214], or the PRACH in the slot and does not transmit the SRS in symbols from the set of symbols in the slot, if any, starting from a symbol that is after PUSCH preparation time  for the corresponding PUSCH timing capability [6, TS 38.214] assuming  after a last symbol of a CORESET where the UE is configured to monitor PDCCH for DCI format 2_0 and  corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format 2_0 and the SCS configuration of the SRS, PUCCH, PUSCH or r, where r corresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise r=0

-     does not expect to cancel the transmission of the SRS, or the PUCCH, or the PUSCH, or the PRACH in symbols from the set of symbols in the slot, if any, starting before a symbol that is after the PUSCH preparation time  for the corresponding PUSCH timing capability [6, TS 38.214] assuming  after a last symbol of a CORESET where the UE is configured to monitor PDCCH for DCI format 2_0 and  corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format 2_0 and the SCS configuration of the SRS, PUCCH, PUSCH or r, where r corresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise r=0

-    if the UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in the set of symbols of the slot and the UE is provided EnableConfiguredUL-r16, the UE can transmit the SRS, or PUCCH, or PUSCH, or PRACH, respectively.

----------------------------------------------------- End of text proposal ------------------------------------------------------